Confidential▼

# PON OLT Equipment MIB Specifications

Project Name: CIANET Contract

Unit: ZTE do Brasil

Date: 18/02/2015

#### Totally 158 Pages (Cover Included)

Confidential▼

Version No.	Prepared by

/

Revised by

Revision History

Revision Date	Reason for Revision

Main Contents Revised (Main Points)

Zhu Min	2012-11-23

Zhu Min	2013-8-16	Consummate	relevant contents

Confidential▼

## Overview

This document is applicable to the PON OLT products, such as C200, C220, C300 and C320.

Most information of MIB objects shall be got from the corresponding MIB, and the MIB file is the final and the most accurate source of information of all the MIB objects. This document is only to list key information and the belonging MIB file.

If there are no special specifications, the corresponding MIB has no change among versions, and is suitable for all the OLT versions released.

Confidential▼

## Definition of Composite Index

In the OLT equipment, to get index of some MIB tables, you need to combine some fields into 32 bytes, such index is called as composite index in this document.

Corresponding to it, the index directly using numbers like 1, 2, 3 is called as regular index in this document.

The composite index used in the OLT equipment management can be divided into two categories: one is platform composite index, and the other is PON equipment composite index. General functions of different kinds of equipment usually use composite index, while the management of the PON equipment uses PON composite index.

  - Platform Composite Index

The platform index of MIB tables usually consists of a MIB scalar. When accessing an MIB table, you need to select corresponding method to constitute the composite index according to the type of the interface.

The platform composite index is a 32-bit integer without a symbol, and the composite index—s high 4 bits are used to indicate the type of the index:

    - Slot Conversion of Platform Composite Index

Slot fields of the platform composite index needs to fill in a logic slot value, and the logic slot value is got via the conversion of physical slot value. The conversion modes of different OLT equipment are different, for details, refer to relevant section.

Slot COjVerSiOj Of C220 Shelf

if (slot <= 6)

{

composite index slot = slot - 1;

Confidential▼

}

else if ((slot >= 9) && (slot <= 14))

{

}

else

{

}

composite index slot = slot - 3;

composite index slot = 0;

Slot COjVerSiOj Of C200 Shelf

if (slot <= 3)

{

composite index slot = slot;

}

else if (slot == 6)

{

composite index slot = 5;

}

else if ((slot >= 4) && (slot <= 5))

{

if (The main control card is inserted in the slot)

{

}

else

{

}

}

composite index slot = 0;

composite index slot = 4;

Slot COjVerSiOj Of C300 Shelf

if ((slot >= 2) && (slot <= 9))

{

composite index slot = slot - 2;

}

else if ((slot >= 12) && (slot <= 22))

{

composite index slot = slot - 4;

}

Confidential▼

else

{

}

composite index slot = 0;

Slot COjVerSiOj Of C320 Shelf

if((0 < vSlot) && (ucSlotnum >= vSlot))

{

*vpCardId = vSlot - 1;

}

    - Type 1 Index

Type 1 index indicates port or OLT. From high bit to low bit:

bit31 to bit28 indicates Type, which is 1,

bit27 to bit24 indicates Shelf No., which is 0,

bit23 to bit16 indicates Slot No., to fill in Slot No. after conversion, refer to 1.2 bit15to bit8 indicates Port No. or OLT No., be equal to Port No. or (OLT No. ’ 1) bit7 to bit0 fill in 0

|	4	|	4	|	8	|	8	|	8	|

|Type: 1 |	Shelf	|	Cardid	|	Port	(Olt)	|	Vc/SrvPortId	|

For example: gpon-olt_1/2/3—s index is 0x10000200.

    - Type 3 Index

Type 3 index indicates ONU. The PON cards having 8 ports or less than 8 ports use this index type.

From high bit to low bit:

bit31 to bit28 indicates Type, which is 3, bit27 to bit24 indicates Shelf No., which is 0,

bit23 to bit19 indicates Slot No., to fill in Slot No. after conversion, refer to 1.2

bit18 to bit16 indicates OLT NO., be equal to (OLT NO. ’ 1) bit15 to bit8 indicates ONU NO., be equal to (ONU NO. ’ 1)

Confidential▼

bit7 to bit0 fill in 0

|	4	|	4	|	8	|	8	|	8	|

|Type: 3 |	Shelf	| Cardid(5b)Olt( 3b) |	Onu	|	|

For example: gpon-onu_1/2/3:2—s index is 0x30020100.

    - Type 4 Index

Type 4 index indicates ONU virtual link. The PON cards having 8 ports or less than 8 ports use this index type.

From high bit to low bit:

bit31 to bit28 indicates Type, which is 3, bit27 to bit24 indicates Shelf No., which is 0,

bit23 to bit19 indicates Slot No., to fill in Slot No. after conversion, refer to1.2

bit18 to bit16 indicates OLT NO., be equal to (OLT NO. ’ 1) bit15 to bit8 indicates ONU NO., be equal to (ONU NO. ’ 1)

bit7 to bit0 indicates ONU virtual link, be equal to 0 for EPON equipment

|	4	|	4	|	8	|	8	|	8	|

|Type: 4 |	Shelf	| Cardid(5b)Olt(3b) |	Onu	| Vport/SrvPortId	|

Of them, Vport and SrvPortId are differentiated via the highest bit of the belonging field. If it is 1, it indicates srvport.

For	example:	gpon-onu_1/2/3:2	VPORT1—s	index	is	0x40020101,                                                    gpon-onu_1/2/3:2 service port1—s index is 0x40020181.

    - Type 9 Index

Type 9 index indicates 16-port PON card—s ONU. The PON cards having 8 ports or less than 8 ports use Type 3 index.

From high bit to low bit:

bit31 to bit28 indicates Type, which is 9, bit27 to bit24 indicates Shelf No., which is 0,

bit23 to bit19 indicates Slot No., to fill in Slot No. after conversion, refer to1.2

bit18 to bit16 indicates OLT NO., be equal to (OLT NO. ’ 1) bit15 to bit8 indicates ONU NO., be equal to (ONU NO. ’ 1)

Confidential▼

bit7 to bit0 fill in 0

|	4	|	3	|	9	|	8	|	8	|

|Type: 9 |	Shelf(3b)	| Cardid(5b)Olt(4b) |	Onu	|	|

    - Type 10 Index

Type 10 index indicates 16-port PON card—s ONU virtual link. The PON cards having 8 ports or less than 8 ports use Type 4 index.

From high bit to low bit:

bit31 to bit28 indicates Type, which is 10, bit27 to bit24 indicates Shelf No., which is 0,

bit23 to bit19 indicates Slot No., to fill in Slot No. after conversion, refer to1.2 bit18 to bit16 indicates OLT NO., be equal to (OLT NO. ’ 1)

bit15 to bit8 indicates ONU NO., be equal to (ONU NO. ’ 1)

bit7 to bit0 indicates ONU virtual link, be equal to 0 for EPON equipment

|	4	|	3	|	9	|	8	|	8	|

|Type: 10 |	Shelf(3b)	| Cardid(5b)Olt( 4b) |	Onu	| Vport/SrvPort Id	|

  - PON Composite Index

The index of PON MIB tables usually consists of one or multiple MIB scalars. When accessing an MIB table, if the index consists of one MIB scalar, you need to select corresponding method to constitute the composite index according to the type of the interface; if the index consists of multiple MIB scalars, only the first scalar constituting the index needs you to select corresponding method to constitute the composite index according to the type of the interface, while other scalars are all regular indices, which use numbers like 1, 2, 3, etc.

PON composite index is a 32-bit integer without symbols, and the composite index—s high 4 bits are used to indicate the type of the index.

The slot No. in the PON composite index adopts physical slot No., with no need to be converted.

GPON PON port-level objects are all Type 1composite index, ONU -level objects are (PON port Type 1composite index + ONU No.).

EPON PON port-level objects are all Type 1composite index, ONU -level objects are Type 3composite index.

Confidential▼

    - Type 1 Composite Index

Type 1indicates indicates port or OLT. From high bit to low bit:

bit31 to bit28 indicates Type, which is 1, bit27 to bit24 indicates Shelf No., which is 0,

bit23 to bit16 indicates Slot No., fill in Slot No.

bit15 to bit8 indicates Port No. or OLT No., fill in Port No. or OLT No. bit7 to bit0 fill in 0

|	4	|	4	|	8	|	8	|	8	|

| Type: 1 | Shelf | Slot	| Port(Olt) |	|

For example: gpon-olt_1/2/3's index is 0x10020300.

    - Type 3 Composite Index

Type 3 indicates ONU. From high bit to low bit:

bit31 to bit28 indicates Type, which is 3, bit27 to bit24 indicates Shelf No., which is 0,

bit23 to bit19 indicates Slot No., fill in Slot No. bit18 to bit16 indicates OLT NO., fill in OLT NO. bit15 to bit8 indicates ONU NO., fill in ONU NO. bit7 to bit0 fill in 0

Note: For C300 and C320 NEs, bit18 to bit16 indicates OLT NO., fill in (OLT NO. ‐ 1) while for other NEs, fill in OLT NO.

|	4	|	4	|	8	|	8	| 8 |

| Type: 3 | Shelf |Card(5bit)Olt(3bit)| Onu ID |	|

For example: gpon-onu_1/2/3:1's index is 0x30120100.

    - Type 6 Composite Index

Type 6 indicates slot. From high bit to low bit:

bit31 to bit28 indicates Type, which is 6, bit27 to bit24 indicates Shelf No., which is 0,

bit23 to bit16 indicates Slot No., fill in Slot No.

Confidential▼

bit15 to bit8 fill in 0 bit7 to bit0 fill in 0

|	4	|	4	|	8	|	8	|	8	|

| Type: 6 | Shelf |	slot |	|	|

For example: The index of the GPON card in Slot 2 is 0x60020000.

2.2.4 Type 7 Composite Index

Type 7 indicates profile.

From high bit to low bit:

bit31 to bit28 indicates Type, which is 7, bit27 to bit24 indicates Shelf No., which is 0, bit23 to bit16 fill in 0

bit15 to bit0 indicates profile No., fill in profile No.

|

4

|

4	|	8 |	8	|

| Type: 7 | Shelf |	0 |	profile No.

8	|

|

For example: Profile index is 0x70000001.

2.2.5 Type 9 Composite Index

Type 9 indicates ONU (The composite index is specially used for the ONUs of the PON cards

having more than 8 ports, while the ONUs of the PON cards having 8 ports or less than 8 ports use Type 3).

From high bit to low bit:

bit31 to bit28 indicates Type, which is 9, bit27 to bit25 indicates Shelf No., which is 0,

bit24 to bit20 indicates Slot No., fill in Slot No. bit19 to bit16 indicates OLT NO., fill in OLT NO.

bit15 to bit8 indicates ONU NO., fill in ONU NO.

bit7 to bit0 fill in 0

Note: For C300 and C320 NEs, bit19 to bit16 indicates OLT NO., fill in (OLT NO. ‐ 1) while for other NEs, fill in OLT NO.

|	4	|	4	|	8	|	8	| 8 |

| Type: 3 | Shelf (3bit)|Slot(5bit)Olt(4bit)| Onu ID |	|

Confidential▼

## System Control

  - NE Information

Basic NE information includes ObjectID, description information, system operation time, system location and so on.

Index Specification

Global variable, index.0

sysContact

sysName sysLocation

.1.3.6.1.2.1.1.4

.1.3.6.1.2.1.1.5

.1.3.6.1.2.1.1.6

OID Specification

| MIB Variable | OID | Specification |
| --- | --- | --- |
| sysDescr | .1.3.6.1.2.1.1.1 | Read Only |
| sysObjectID | .1.3.6.1.2.1.1.2 | Read Only |
| sysUpTime | .1.3.6.1.2.1.1.3 | Read Only |

  - Shelf Information

Get actual shelf type of the NE and configured shelf type, and set the configured shelf type of the NE

Index Specification

The index is zxAnRackNo (Rack No.) and zxAnShelfNo (Shelf No.), and Rack No. and Shelf No. are numbered from 0.

OID Specification

| MIB Variable | OID | Specification |
| --- | --- | --- |
| zxAnShelfActType | .1.3.6.1.4.1.3902.1015.2.1.1.2.1.3 | Read Only |
| zxAnShelfCfgType | .1.3.6.1.4.1.3902.1015.2.1.1.2.1.4 |  |
| zxAnShelfRowStatus | .1.3.6.1.4.1.3902.1015.2.1.1.2.1.15 |  |

- Definitions	of	actual	shelf	type	zxAnShelfActType	and	configured	shelf	type zxAnShelfCfgType

Confidential▼

0x00010101		C220 equipment Type-A shelf 0x00010102		C220 equipment Type-B shelf 0x00010103	C200 equipment Type-A shelf 0x00010104	C100 equipment Type-A shelf

  - Card Information

Get the configured card list on the NE

Index Specification

The index is zxAnRackNo (Rack No.), zxAnShelfNo (Shelf No.) and zxAnSlotNo (Slot No.).

Rack No. and Shelf No. are numbered from 0, while Slot No. is numbered from 1.

OID Specification

| MIB Variable zxAnCardCfgMainType | OID .1.3.6.1.4.1.3902.1015.2.1.1.3.1.2 | Specification Configured card type |
| --- | --- | --- |
| zxAnCardActMainType | .1.3.6.1.4.1.3902.1015.2.1.1.3.1.3 | Actual card type (Read |
| zxAnCardActType | .1.3.6.1.4.1.3902.1015.2.1.1.3.1.4 | Only) Card name (Read Only) |
| zxAnCardOperStatus | .1.3.6.1.4.1.3902.1015.2.1.1.3.1.5 | Card status (Read |
|  |  | Only) |
| zxAnCardCpuLoad | .1.3.6.1.4.1.3902.1015.2.1.1.3.1.9 | CPU load (Read Only) |
| zxAnCardMemUsage | .1.3.6.1.4.1.3902.1015.2.1.1.3.1. 1 | Memory usage rate (%) |
|  |  | (Read Only) |
| zxAnCardStandbyStatus | .1.3.6.1.4.1.3902.1015.2.1.1.3.1.13 | Card active / standby |
|  |  | status (Read Only) |
| zxAnCardRowStatus | .1.3.6.1.4.1.3902.1015.2.1.1.3.1.30 |  |

- Definitions of zxAnCardCfgMainType and zxAnCardActMainType

0x00010108	EC4GH

0x00010104	EC4GM

0x00010102	EC4G

0x00011002	EIG

0x00011001	EIGM

0x00012405	EIGMP

0x00011003	EIGMF

0x00011004	EIT1F

0x00011005	EIT2F

0x00012301	EPEB

0x00012302	EPFC

0x00012303	EPFB

Confidential▼

0x00010101	GCSA

0x00010103	GCSC

0x00010105	GCSD

0x00010107	GCSE

0x00011101	CE1B

0x00011102	CE1BB

0x00011103	CL1A

0x00012402	GPTAB

0x00012403	GPTCB

0x00012404	GPFAB

0x00012401	GPTBB

0x00080101	MS8E

0x00080102	MS8EP

0x00081001	EI8E

0x00081002	EI8EP

0x00081101	ETC8B

0x00082301	EPUA

0x00082401	GPUA

0x00082601	V08B

0x00082602	V16B

- Card status 0 On-line

- Off-line

- Fail to provide services 3 hw on-line

- Off-line

- Being configured

- Configuration failed

- The configured card type isn't matched with actual type 8 De-activate

- Fault

- Failure

- Card active / standby status 1 Active status

2 Standby status

15 Unknown status

  - Version Information

To view the information of the version currently running of the NE (active version, boot

Confidential▼

version and firmware version)

Index Specification

The index is zxAnRackNo (Rack No.), zxAnShelfNo (Shelf No.) and zxAnSlotNo (Slot No.). Rack No. and Shelf No. are numbered from 0, while Slot No. is numbered from 1.

OID Specification

| MIB Variable | OID | Specification |
| --- | --- | --- |
| zxAnCardHardVersion | .1.3.6.1.4.1.3902.1015.2.1.2.2.1.1 | Card hardware version |
| zxAnVersionFileType | .1.3.6.1.4.1.3902.1015.2.1.2.2.1.3 | Active version type |
| zxAnVersionTag | .1.3.6.1.4.1.3902.1015.2.1.2.2.1.4 | Active version No. |
| .zxAnVersionFileLen | .1.3.6.1.4.1.3902.1015.2.1.2.2.1.5 | Length of the active |
|  |  | version file |
| zxAnVersionBuildTime | .1.3.6.1.4.1.3902.1015.2.1.2.2.1.6 | Active version time |
| zxAnBootromFileType | .1.3.6.1.4.1.3902.1015.2.1.2.2.1.8 | Boot version type |
| zxAnBootromTag | .1.3.6.1.4.1.3902.1015.2.1.2.2.1.9 | Boot version No. |
| zxAnBootromFileLen | .1.3.6.1.4.1.3902.1015.2.1.2.2.1.10 | Length of the boot |
| zxAnBootromBuildTime | .1.3.6.1.4.1.3902.1015.2.1.2.2.1. 1 | version file Boot version time |
| zxAnFirmware1FileType | .1.3.6.1.4.1.3902.1015.2.1.2.2.1.13 | Firmware type |
| zxAnFirmware1Tag | .1.3.6.1.4.1.3902.1015.2.1.2.2.1.14 | Firmware No. |
| zxAnFirmware1FileLen | .1.3.6.1.4.1.3902.1015.2.1.2.2.1.15 | Length of the firmware |
|  |  | file |
| zxAnFirmware1BuildTime | .1.3.6.1.4.1.3902.1015.2.1.2.2.1.16 | Firmware time |

  - Fan Information

Get the information of the fan operation.

Index Specification

The index is zxAnEnvFanIndex (fan unit id), which starts from 1. The maximum number is the number of actual fan units. If lower layers fail to read the number of fans correctly, there are 3 19-inch shelves and 4 21-inch shelves.

OID Specification

MIB Variable

zxAnEnvFanActualSpeedLevel

OID

.1.3.6.1.4.1.3902.1015.2.1.3.10.10.10.1.3

Specification

Actual

speed

level  of  the  fan

unit

.zxAnEnvFanOperStatus	.1.3.6.1.4.1.3902.1015.2.1.3.10.10.10.1.5	Operation  status

|  |  | of the fan unit |
| --- | --- | --- |
| zxAnEnvFanOnlineStatus | .1.3.6.1.4.1.3902.1015.2.1.3.10.10.10.1.6 | Online status of the fan unit |
| zxAnEnvFanActualSpeed | .1.3.6.1.4.1.3902.1015.2.1.3.10.10.10.1.7 | Actual speed |

Confidential▼

(RPM) of the fan

unit

zxAnEnvFanActualSpeedLevel:

{ lowSpeed ( 1 ) , standardSpeed ( 2 ) , highSpeed ( 3 ) , superSpeed ( 4 ) , other ( 10 ) }

zxAnEnvFanOperStatus:

{ up ( 1 ) , down ( 2 ) , unknown ( 3 ) }

zxAnEnvFanOnlineStatus:

{ online ( 1 ) , offline ( 2 ) , unknown ( 3 ) }

  - Address Information

| {"zxAnL3IfIpAddressTable" | , |  |  | "1.3.6.1.4.1.3902.1015.4.1.3"} , |
| --- | --- | --- | --- | --- |
| {"zxAnL3IfIpAddressEntry" | , |  |  | "1.3.6.1.4.1.3902.1015.4.1.3.1"} , |
| {"zxAnL3IfIp" |  |  |  | "1.3.6.1.4.1.3902.1015.4.1.3.1.1"} , |
| {"zxAnL3IfMask" |  |  | , | "1.3.6.1.4.1.3902.1015.4.1.3.1.2"} , |
| {"zxAnL3IfIpCatagory" | , |  |  | "1.3.6.1.4.1.3902.1015.4.1.3.1.3"} , |
| {"zxAnL3IfIpRowStatus" | , |  |  | "1.3.6.1.4.1.3902.1015.4.1.3.1.4"} , |

,

Confidential▼

## Platform

  - VLAN

Relevant MIB file is: ZTE-AN-VLAN-MIB.mib

    - Create VLAN Interface

| {"zxAnL3IfTable" | , "1.3.6.1.4.1.3902.1015.4.1.1"} , |
| --- | --- |
| {"zxAnL3IfEntry" | , "1.3.6.1.4.1.3902.1015.4.1.1.1"} , |
| {"zxAnL3IfIndex" | , "1.3.6.1.4.1.3902.1015.4.1.1.1.1"} , |
| {"zxAnL3IfName" | , "1.3.6.1.4.1.3902.1015.4.1.1.1.2"} , |
| {"zxAnIfReferIndex" | , "1.3.6.1.4.1.3902.1015.4.1.1.1.3"} , |
| {"zxAnL3IfArpProxyEnable" | , "1.3.6.1.4.1.3902.1015.4.1.1.1.4"} , |
| {"zxAnL3IfRowStatus" | , "1.3.6.1.4.1.3902.1015.4.1.1.1.5"} , |

Name	 Oid	Range	Description zzxAnL3IfIndex	.1.3.6.1.4.1.3902.1015.4.1.1.1.1

zxAnL3IfName	.1.3.6.1.4.1.3902.1015.4.1.1.1.2	STRING ( SIZE ( 1 ..

32 ) )

zxAnIfReferIndex	.1.3.6.1.4.1.3902.1015.4.1.1.1.3

able

zxAnL3IfArpProxyEn

.1.3.6.1.4.1.3902.1015.4.1.1.1.4

zxAnL3IfRowStatus	.1.3.6.1.4.1.3902.1015.4.1.1.1.5	rowStatus

    - Configure IP Address of VLAN Interface

{"zxAnL3IfIp"	, "1.3.6.1.4.1.3902.1015.4.1.3.1.1"} ,

{"zxAnL3IfMask"	,	"1.3.6.1.4.1.3902.1015.4.1.3.1.2"} ,

{"zxAnL3IfIpCatagory"	, "1.3.6.1.4.1.3902.1015.4.1.3.1.3"} ,

{"zxAnL3IfIpRowStatus"	, "1.3.6.1.4.1.3902.1015.4.1.3.1.4"} ,

Name	Oid	Range	Description

Confidential▼

| zzxAnL3IfIndex | .1.3.6.1.4.1.3902.1015.4.1.1.1.1 |  |
| --- | --- | --- |
| zxAnL3IfIp | .1.3.6.1.4.1.3902.1015.4.1.3.1.1 |  |
| zxAnL3IfMask | .1.3.6.1.4.1.3902.1015.4.1.3.1.2 |  |
| zxAnL3IfIpCatagory | .1.3.6.1.4.1.3902.1015.4.1.3.1.3 |  |
| zxAnL3IfIpRowStatus | .1.3.6.1.4.1.3902.1015.4.1.3.1.4 | rowStatus |

    - Configure Switchport

{"zxAnVlanPortConfVlanCmdTable" ,	"1.3.6.1.4.1.3902.1015.20.4"} ,

{"zxAnVlanPortConfVlanCmdEntry" ,	"1.3.6.1.4.1.3902.1015.20.4.1"} ,

{"zxAnVlanPortConfVlanCmd"	,	"1.3.6.1.4.1.3902.1015.20.4.1.1"} ,

{"zxAnVlanPortConfVlanId"	,	"1.3.6.1.4.1.3902.1015.20.4.1.2"} ,

    - Configure Serviceport

| {"zxAnServicePort" | , |  | "1.3.6.1.4.1.3902.1015.8"} , |
| --- | --- | --- | --- |
| {"zxAnServicePortObjects" |  | , | "1.3.6.1.4.1.3902.1015.8.1"} , |
| {"zxAnServicePortConfTable" |  | , | "1.3.6.1.4.1.3902.1015.8.1.1"} , |
| {"zxAnServicePortConfEntry" |  | , | "1.3.6.1.4.1.3902.1015.8.1.1.1"} , |
| {"zxAnBridgePortIndex" |  | , | "1.3.6.1.4.1.3902.1015.8.1.1.1.1"} , |
| {"zxAnUserOutSVid" |  |  | , "1.3.6.1.4.1.3902.1015.8.1.1.1.10"} , |
| {"zxAnServicePortRowStatus" {"zxAnUserTlsVlan" |  | , | "1.3.6.1.4.1.3902.1015.8.1.1.1.100"} , , "1.3.6.1.4.1.3902.1015.8.1.1.1.11"} , |
| {"zxAnUserSVidCos" |  |  | , "1.3.6.1.4.1.3902.1015.8.1.1.1.12"} , |
| {"zxAnVlanTransMode" |  |  | , "1.3.6.1.4.1.3902.1015.8.1.1.1.13"} , |
| {"zxAnUserInStartVid" |  | , | "1.3.6.1.4.1.3902.1015.8.1.1.1.14"} , |

{"zxAnUserInEndVid"	,	"1.3.6.1.4.1.3902.1015.8.1.1.1.15"} ,

{"zxAnUserCVidCos"	,	"1.3.6.1.4.1.3902.1015.8.1.1.1.16"} ,

{"zxAnUserInSVid"	,	"1.3.6.1.4.1.3902.1015.8.1.1.1.17"} ,

{"zxAnSrvPortVlanXconnectEnable",	"1.3.6.1.4.1.3902.1015.8.1.1.1.18"} ,

{"zxAnServicePortUserDscp"	,	"1.3.6.1.4.1.3902.1015.8.1.1.1.19"} ,

{"zxAnServicePortID"	,	"1.3.6.1.4.1.3902.1015.8.1.1.1.2"} ,

{"zxAnServicePortIngressTrafficPrf",	"1.3.6.1.4.1.3902.1015.8.1.1.1.20"} ,

{"zxAnServicePortEgressTrafficPrf",	"1.3.6.1.4.1.3902.1015.8.1.1.1.21"} ,

{"zxAnServicePortAdminStatus",  "1.3.6.1.4.1.3902.1015.8.1.1.1.22"} ,

Confidential▼

{"zxAnServicePortQueueId",	"1.3.6.1.4.1.3902.1015.8.1.1.1.23"} ,

{"zxAnServicePortDesc"	,	"1.3.6.1.4.1.3902.1015.8.1.1.1.3"} ,

{"zxAnServicePortServiceMode"	,	"1.3.6.1.4.1.3902.1015.8.1.1.1.4"} ,

{"zxAnUserInVid"	,	"1.3.6.1.4.1.3902.1015.8.1.1.1.5"} ,

{"zxAnUserInPriority"	,	"1.3.6.1.4.1.3902.1015.8.1.1.1.6"} ,

{"zxAnUserEthType"	,	"1.3.6.1.4.1.3902.1015.8.1.1.1.7"} ,

{"zxAnUserEthFilter"	,	"1.3.6.1.4.1.3902.1015.8.1.1.1.8"} ,

{"zxAnUserOutCVid"	,	"1.3.6.1.4.1.3902.1015.8.1.1.1.9"} ,

{"zxAnUserSceneTable"	,	"1.3.6.1.4.1.3902.1015.8.1.2"} ,

{"zxAnUserSceneEntry"	,	"1.3.6.1.4.1.3902.1015.8.1.2.1"} ,

{"zxAnCpeIndex"	,	"1.3.6.1.4.1.3902.1015.8.1.2.1.1"} ,

{"zxAnSceneMode"	,	"1.3.6.1.4.1.3902.1015.8.1.2.1.2"} ,

{"zxAnServicePortGlobalObjects" ,	"1.3.6.1.4.1.3902.1015.8.1.50"} ,

{"zxAnServicePortCompatible"	,	"1.3.6.1.4.1.3902.1015.8.1.50.1"} ,

  - Multicast

Please refer to ZTE-AN-IGMP-MIB.mib.

    - Global Management

MIB Variable/ OID	Type	Description

zxAnIgmpAdminStatus 1.3.6.1.4.1.3902.1015.31.1.1.1.1

zxAnIgmpAging 1.3.6.1.4.1.3902.1015.31.1.1.1.2

zxAnIgmpWorkingMode 1.3.6.1.4.1.3902.1015.31.1.1.1.3

INTEGER	The administrative status of global

igmp service. Default: enable

Integer32	[30..3600] (Def: 300)

The timeout period in seconds for

aging out Multicast Groups dynamically learned with IGMP.

Note that aging operates on a per interface per VLAN per multicast group basis. The value is 0 means that no igmp aging.

INTEGER	[proxy(1), routing(2), snooping(3)]

(Def: 1)

IGMP working mode, igmp entity can work as a igmp proxy, igmp routing or igmp snooping.

zxAnIgmpMcastInAllVlan 1.3.6.1.4.1.3902.1015.31.1.1.1.4

INTEGER	[enable(1), disable(2) ] (Def: 1)

enable: multicast forward data in all

Confidential▼

zxAnIgmpDefaultMvlan 1.3.6.1.4.1.3902.1015.31.1.1.1.5

vlans.

disable: multicast play role in their vlans only

Integer32	[0..4095](Def:1)

Default multicast vlan ID.

It is used to auto config (add or delete) multicast group to this default Mvlan. That is if this variable is set then all the operation (add or delete) about a group will not need to operate mvlan manually again to add or delete group to (from) a mvlan.

The agent should add or delete a group to (from) the default mvlan automatically.

0 means no default multicast

zxAnIgmpAutoConfigGrpToDefaultM vlan

1.3.6.1.4.1.3902.1015.31.1.1.1.6

zxAnIgmpFastQureyBasedIpPool 1.3.6.1.4.1.3902.1015.31.1.1.1.7

zxAnIgmpHandleGeneralLeaveMsg 1.3.6.1.4.1.3902.1015.31.1.1.1.8

zxAnIgmpUserSideRoutingIp 1.3.6.1.4.1.3902.1015.31.1.1.1.9

zxAnIgmpV1AdminStatus 1.3.6.1.4.1.3902.1015.31.1.1.1.10

vlan.

INTEGER	[enable(1), disable(2) ] (Def: 1)

Enable/disable auto add or delete grp	to	default	mvlan.	If	it	is enable(1),zxAnIgmpDefaultMvlan must be set a non zero value at first.

INTEGER	[enable(1), disable(2) ] (Def: 2)

enable(1): enable fast query function based on user's IP log pool. This function helps to speed up the query in case of multi users in same port.

disable(2): disable fast uery function.

INTEGER	[enable(1),disable(2)]  (Def: 2)

enable(1): enable handling general leave msg.

disable(2): disable handling general leave msg.

IpAddress	The routing ip for user side when

the device is looked as a igmp router.

INTEGER	[accept(1), ignore(2), drop(3)]

(Def: 1)

The administrative status of IGMP

v1 packet. zxAnIgmpV2AdminStatus	INTEGER	[accept(1), ignore(2), drop(3)]

Confidential▼

1.3.6.1.4.1.3902.1015.31.1.1.1.11	(Def: 1)

The administrative status of IGMP v2 packet.

zxAnIgmpV3AdminStatus 1.3.6.1.4.1.3902.1015.31.1.1.1.12

zxAnIgmpBandwidthCtrl 1.3.6.1.4.1.3902.1015.31.1.1.1.13

zxAnIgmpRobustnessVariable 1.3.6.1.4.1.3902.1015.31.1.1.1.14

zxAnIgmpQueryInterval 1.3.6.1.4.1.3902.1015.31.1.1.1.15

zxAnIgmpQueryMaxResponseTime 1.3.6.1.4.1.3902.1015.31.1.1.1.16

zxAnIgmpLastMembQueryInterval 1.3.6.1.4.1.3902.1015.31.1.1.1.17

zxAnIgmpLastMembQueryCount

1.3.6.1.4.1.3902.1015.31.1.1.1.18

zxAnIgmpV1QuerierTimeout 1.3.6.1.4.1.3902.1015.31.1.1.1.19

zxAnIgmpUnsolicitedReportInterval1. 3.6.1.4.1.3902.1015.31.1.1.1.20

rfc2662:adslAturConfMinSnrMgn 1.3.6.1.2.1.10.94.1.1.14.1.20

INTEGER	[accept(1), ignore(2), drop(3)]

(Def: 1)

The administrative status of IGMP v3 packet.

INTEGER	[enable(1), disable(2) ] (Def: 2)

Adminstrative status of bandwidth traffic control for multicast service.

Integer32	[2-5] (Def:2)

IGMP proxy robustness variable.

Integer32	[60-300](Def:125)

IGMP proxy general query interval.

Integer32	[10-250] (Def:100),

IGMP proxy query max response time.

Integer32	[1-255] (Def:10),

IGMP proxy last member query interval.

Integer32	[2-5] (Def:2)

IGMP proxy last member query count.

Integer32	[60-3600] (Def:400)

IGMP	proxy	version	1	router present timeout.

Integer32	[1-310](Def:310)

IGMP	proxy	unsolicited	report interval.

INTEGER	[1-60](Def:10)

IGMP	proxy	unsolicited	report interval.

    - Multicast VLAN Management

Object name

zxAnIgmpVlanId 1.3.6.1.4.1.3902.1015.31.1.1.22.1.1

Type

Integer32

zxAnMvlanAdminStatus

INTEGER

Description

[1-4094]

Multicast vlan ID

[enable(1), disable(2),drop(3) ] (Def: 1)

1.3.6.1.4.1.3902.1015.31.1.1.22.1.2

The administrative status of IGMP

packet on this MVLAN.

Confidential▼

zxAnMvlanWorkMode 1.3.6.1.4.1.3902.1015.31.1.1.22.1.3

zxAnMvlanNetworkSideHostIp 1.3.6.1.4.1.3902.1015.31.1.1.22.1.4

zxAnMvlanGroupFilterEnable

INTEGER

IpAddress

[igmpv1(1), igmpv2(2), igmpv3(3)] (Def: 2)

The	IGMP	woke	mode	of	this MVLAN.

The host ip of this MVLAN when working on proxy mode. (Def: 0)

1.3.6.1.4.1.3902.1015.31.1.1.22.1.5	TruthValue

zxAnMvlanMaxGroups

Integer32

1.3.6.1.4.1.3902.1015.31.1.1.22.1.6

zxAnMvlanCurrActGroups

Max groups in mvlan,the value lies on	the	device's	ability. Dsl8000 is 255, dsl9000 is 512,9800

is 1024. (Def:512)

1.3.6.1.4.1.3902.1015.31.1.1.22.1.7

zxAnMvlanPriority

Gauge32	Current active groups in mvlan

[0-7] (Def: 0)

1.3.6.1.4.1.3902.1015.31.1.1.22.1.8

zxAnMvlanHostVersion 1.3.6.1.4.1.3902.1015.31.1.1.22.1.9

zxAnMvlanRowStatus

Integer32

INTEGER

IGMP protocol packet's priority in

proxy/router mode.

[auto(1),	igmpv2(2),	igmpv3(3)]

(Def: 2)

Igmp  version  of	host  on  proxy

mode.

1.3.6.1.4.1.3902.1015.31.1.1.21.1.20

RowStatus	RowStatus

    - Multicast Group Management

Object name

type

zxAnIgmpSrcIp

description

Multicast	source	IpAddress	in

IpAddress

1.3.6.1.4.1.3902.1015.31.1.1.24.1.1

IGMPv3.If the device can't support

IGMPv3, this object is 0.0.0.0

zxAnIgmpGrpIp

1.3.6.1.4.1.3902.1015.31.1.1.24.1.2

IpAddress

Multicast group IpAddress.

zxAnIgmpGroupBandwidthCost

1.3.6.1.4.1.3902.1015.31.1.1.24.1.3

Estimated  Bandwidth  cost  by  this

Integer32	group,it	is	used	as	a	traffic

and bandwidth control parameter.

zxAnIgmpGroupPrejoinEnable

1.3.6.1.4.1.3902.1015.31.1.1.24.1.4

zxAnIgmpGroupMaxHosts 1.3.6.1.4.1.3902.1015.31.1.1.24.1.5

zxAnIgmpGroupActHosts

TruthValue

(Def: false)

Integer32

Max User in this Multicast Group.

Integer32	Current User in this Multicast Group.

1.3.6.1.4.1.3902.1015.31.1.1.24.1.6

zxAnIgmpGroupRowStatus

RowStatus

RowStatus of this row.

Confidential▼

1.3.6.1.4.1.3902.1015.31.1.1.24.1.15

    - Multicast Vort Management

Add and delete multicast source and receiving ports via the table.

Object name	Type	Description

The command to operate param list of zxAnMvlanGroupPortListTable.

Each command is related to each meaming	of

zxAnMvlanGroupStaticPortList. Command	operation

zxAnIgmpParamListCmd 1.3.6.1.4.1.3902.1015.31.1.1.26.1.1

INTEGER

addSrcPortToMvlan(1)

Bind source port with mvlan

delSrcPortFromMvlan(2) unbind source Port with mvlan

addRecPortToMvlan(3)

bind recievingPort with mvlan

delRecPortFromMvlan(4) unbind recievingPort with mvlan addStaticGroup(5)	Bind port with Static Multicast

delStaticGroup(6)	unbind port with Static Multicast

The value of object ID in param list,eg.

zxAnIgmpParamObject 1.3.6.1.4.1.3902.1015.31.1.1.26.1.2

IDENTIFIER

addSrcPortToMvlan(1) zxAnIgmpParamObject is 'mvid.ifid'

delSrcPortFromMvlan(2) zxAnIgmpParamObject is 'mvid.ifid'

addRecPortToMvlan(3) zxAnIgmpParamObject is 'mvid.ifid'

delRecPortFromMvlan(4) zxAnIgmpParamObject is 'mvid.ifid'

addStaticGroup(5) zxAnIgmpParamObject	is 'mvid.srcip.gpip.ifid'

delStaticGroup(6) zxAnIgmpParamObject	is 'mvid.srcip.gpip.ifid'

Confidential▼

    - Receiving Port Property Management

Object name	Type	Description

The logical port No,ie. layer 2 bridge

zxAnIgmpIfIndex

1.3.6.1.4.1.3902.1015.31.1.1.21.1.1

zxAnIgmpPortAdminStatus 1.3.6.1.4.1.3902.1015.31.1.1.21.1.2

zxAnIgmpPortProtoVersion 1.3.6.1.4.1.3902.1015.31.1.1.21.1.3

zxAnIgmpPortFastLeaveEnable 1.3.6.1.4.1.3902.1015.31.1.1.21.1.4

zxAnIgmpPortUsage 1.3.6.1.4.1.3902.1015.31.1.1.21.1.5

zxAnIgmpDataPort 1.3.6.1.4.1.3902.1015.31.1.1.21.1.6

zxAnIgmpPortProxyIp 1.3.6.1.4.1.3902.1015.31.1.1.21.1.7

zxAnIgmpPortPacketLimit 1.3.6.1.4.1.3902.1015.31.1.1.21.1.8

zxAnIgmpPortMaxBandwidth 1.3.6.1.4.1.3902.1015.31.1.1.21.1.9

ZxAnIfindex

INTEGER

INTEGER

TruthValue

INTEGER

ZxAnIfindex

IpAddress

Integer32

Integer32

port. (ie: DSLPVC, PON_VPORT, VDSL)

[enable(1), disable(2),drop(3) ] (Def: 1)

The administrative status of port's igmp service.

[igmpv1(1), igmpv2(2), igmpv3(3)] (Def: 2)

The version of IGMP protocol which is running on this port.

The status of fast-leave property, true:enable fast leave;

false: disable fast leave.(Def: true) [all(1), igmp(2), data(3)] (Def:1)

The usage of the igmp port,igmp signalling only,data only or both signalling and data.

If	the	logical		port		indicated		by zxAnIgmpIfIndex is used as igmp signaling				 transporting,					 ie. zxAnIgmpPortUsage									is data(2),zxAnIgmpDataPort must be configured to indicate which port will recieve		the	igmp	data.	If		the zxAnIgmpPortUsage			is		All(1), zxAnIgmpDataPort is same as the zxAnIgmpIfIndex. zxAnIgmpPortProxyIp is the ip of igmp				proxy						when zxAnIgmpWorkingMode							 is igmp_proxy(1). (Def: 0)

The maxium IGMP packet num can be processed in 1 second on this port, exceeding packets will be discarded. The maximum permitted traffic in this logical port. (Def: 2048)

zxAnIgmpPortMaxConcurrGroups

1.3.6.1.4.1.3902.1015.31.1.1.21.1.10

The max multicast groups supported

Integer32

by this port concurrently. (Def: 1)

zxAnIgmpPortCurrActGroups	Gauge32	The active multicast groups on this

Confidential▼

1.3.6.1.4.1.3902.1015.31.1.1.21.1.11	port currentlly.

zxAnIgmpPortQueryInterval 1.3.6.1.4.1.3902.1015.31.1.1.21.1.12

zxAnIgmpPortLastMembQueryIntvl 1.3.6.1.4.1.3902.1015.31.1.1.21.1.13

zxAnIgmpPortQueryMaxResponseTime 1.3.6.1.4.1.3902.1015.31.1.1.21.1.14

zxAnIgmpPortEtherPriority 1.3.6.1.4.1.3902.1015.31.1.1.21.1.15

Integer32	Def: 125

Integer32	[0-255] (Def: 10)

Integer32	[0-255] (Def: 100)

Integer32	[0-7]

Ether	priority	assigned	to	the upstream IGMP packet on the port.

    - Query MVLAN Line Card Port List

Object name

zxAnMvlanPortListShelf 1.3.6.1.4.1.3902.1015.31.1.1.23.1.1

zxAnMvlanPortListCard 1.3.6.1.4.1.3902.1015.31.1.1.23.1.2

zxAnMvlanPortSrcPortList 1.3.6.1.4.1.3902.1015.31.1.1.23.1.3

zxAnMvlanPortRecvPortList

1.3.6.1.4.1.3902.1015.31.1.1.23.1.4

Type

Integer32

Integer32

Description

The shelf that contains the logical port list.

The card that contains the logical port

list.

ZxAnPortList

Source port in mvlan

ZxAnPortList

Receiving port in mvlan

    - Query MVLAN Dynamic and Static Interfaces

Object name	Type	Description zxAnIgmpGroupStaticPortList

1.3.6.1.4.1.3902.1015.31.1.1.25.1.1

zxAnIgmpGroupDynamicPortList 1.3.6.1.4.1.3902.1015.31.1.1.25.1.2

ZxAnPortList	static port in a mvlan group

dynamic port in a mvlan group

ZxAnPortList

  - Port Location

    - DHCP82 Global Switch

DHCP82 global packet extracting switch: Disable / enable of DHCP82 globally. Disable-the drive doesn—t extract DHCP82 packet; Enable-extract packet.

Index Specification:

Confidential▼

OID Specification:

MIB Variable	OID	Specification

zxAnPortLocatingDhcp82E nable zxAnPortLocatingDhcp82E nable_enable

1.3.6.1.4.1.3902.1015.32.21.1	INTEGER	{	enable

( 1 ) , disable ( 2 ) } Default value: 2.

    - DHCP82 Port Switch

DHCP82 Port Switch: DHCP82 disable / enable at the port interface level. Port disable-process the port locating information in DHCP82 message; Port enable-do not process the port locating information in DHCP82 message.

Index Specification:

Platform composite index Type 4

OID Specification:

MIB Variable	OID	Specification

zxAnPortLocatingDhcpInde x zxAnPortLocatingPortDhcp 82Enable

1.3.6.1.4.1.3902.1015.32.21.20.1.1	platform  composite	index

Type 4 1.3.6.1.4.1.3902.1015.32.21.20.1.2	INTEGER	{	enable

( 1 ) , disable ( 2 ) }.

Default value: 2.

    - PPPOE Port Switch

(config-if)#pppoe-plus enable vport 2

.1.3.6.1.4.1.3902.1015.32.40.10.1.2

    - Port Trust Status

Port Trust Status: port-level trust status. True-trust the port locating information in the uplink packet; False-not trust

Index Specification:

Platform composite index Type 4

OID Specification:

| MIB Variable | OID | Specification |
| --- | --- | --- |
| zxAnPortLocatingDhcpIndex | 1.3.6.1.4.1.3902.1015.32.21.20.1.1 | Platform composite index Type 4 |
| zxAnPortLocatingPortTrust | 1.3.6.1.4.1.3902.1015.32.21.20.1.4 | INTEGER { true ( 1 ) , false ( 2 ) }. Default value: 2. |

Confidential▼

    - Port Trust Measure

Port-level trust measure: when the trust status is true, you can set trust measure as keep or replace; when the trust status is false, you can set trust measure as discard or add. Of them, keep-keep the port locating information in the uplink packet, replace-replace, discard-discard the uplink packet, add-add one port locating information again.

Index Specification:

Platform composite index Type 4

replace ( 2 ) , discard ( 3 ) ,

add ( 4 ) }. Default value: 4.

OID Specification:

| MIB Variable | OID | Specification |
| --- | --- | --- |
| zxAnPortLocatingDhcpIndex | 1.3.6.1.4.1.3902.1015.32.21.20.1.1 | Platform composite index Type 4 |
| zxAnPortLocatingPortPolicy | 1.3.6.1.4.1.3902.1015.32.21.20.1.5 | INTEGER { keep ( 1 ) , |

    - Port Locating Format

Port Locating Format: configure the format of the port locating information.

Index Specification:

Platform composite index Type 4

OID Specification:

MIB Variable	OID	Specification

zxAnPortLocatingIndex	1.3.6.1.4.1.3902.1015.32.20.1.1	Platform	composite	index

Type 4

zxAnPortLocatingIfaceForm at

1.3.6.1.4.1.3902.1015.32.20.1.2	china_tel,	dsl_forum,

china_net,	ti,

flexible_syntax, ft, gt, vf.

    - Port Remote ID Switch

RID Switch: RID disable-do not insert RID in the message; enable-insert RID in the message.

Index Specification:

Platform composite index Type 4

MIB Variable

zxAnPortLocatingIndex

OID

1.3.6.1.4.1.3902.1015.32.20.1.1

Specification

Platform

Type 4

INTEGER

composite

index

zxAnPortLocatingIfaceRem

oteIdEnable

1.3.6.1.4.1.3902.1015.32.20.1.3

{ enable ( 1 ) ,

disable ( 2 ) }

OID Specification:

Confidential▼

    - Port Remote ID Content

Port RID Content: Configure specific content of RID, 1-64 characters.

Index Specification:

Platform composite index Type 4

MIB Variable

zxAnPortLocatingIndex

OID

1.3.6.1.4.1.3902.1015.32.20.1.1

Specification

Platform

Type 4

composite

index

zxAnPortLocatingIfaceRem

oteId

1.3.6.1.4.1.3902.1015.32.20.1.4

DisplayString ( SIZE ( 0 ..

64

) )

OID Specification:

  - DHCP SNOOPING

ZTE-AN-DHCP-SNOOPING-MIB

Confidential▼

## PON Port for Public Use

  - IP Address Pool

{"zxAnXponMgmtIpAddress"	,	"1.3.6.1.4.1.3902.1015.1010.5.9.1.1"} ,

{"zxAnXponMgmtIpMask"	,	"1.3.6.1.4.1.3902.1015.1010.5.9.1.2"} ,

{"zxAnXponMgmtPriority"	,	"1.3.6.1.4.1.3902.1015.1010.5.9.1.3"} ,

{"zxAnXponMgmtCVlan"	,	"1.3.6.1.4.1.3902.1015.1010.5.9.1.4"} ,

{"zxAnXponMgmtSVlan"	,	"1.3.6.1.4.1.3902.1015.1010.5.9.1.5"} ,

{"zxAnXponMgmtNetworkIp"	,	"1.3.6.1.4.1.3902.1015.1010.5.9.1.6"} ,

{"zxAnXponMgmtNetworkMask"	,	"1.3.6.1.4.1.3902.1015.1010.5.9.1.7"} ,

{"zxAnXponMgmtNetworkGateway"	,	"1.3.6.1.4.1.3902.1015.1010.5.9.1.8"} ,

{"zxAnXponMgmtIpEnableStatus"	,	"1.3.6.1.4.1.3902.1015.1010.5.9.1.9"} ,

{"zxAnXponMgmtIpAssignationMode",	"1.3.6.1.4.1.3902.1015.1010.5.9.1.10"} ,

Confidential▼

# EPON OLT

  - ONU Management

The table is used to create and delete the ONU under a certain OLT interface, to configure the authentication mode, authentication information, etc., and to perform basic management.

OID Specification

onuAdminObjectTable—s OID is .1.3.6.1.4.1.3902.1015.1010.1.7.4.

Index Specification

{ ifIndex }, PON composite index Type 3 or 9.

MIB Specification

For detailed description, refer to ZXEPON-SERVICE-PRIVATE-MIB.mib.

Of them:

onuAuthLoid is LOID information, you can modify the LOID via setting this MIB object. onuLatelyPassDot1xAuthTime is the final authorized pass time, and it can be used as ONU

uptime.

onuAdminObjectTable OBJECT-TYPE

SYNTAX	SEQUENCE OF OnuAdminObjectEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

""

::= { privateObjects 4 }

onuAdminObjectEntry OBJECT-TYPE

SYNTAX	OnuAdminObjectEntry

MAX-ACCESS		not-accessible STATUS	current DESCRIPTION

""

INDEX	{ ifIndex }

::= { onuAdminObjectTable 1}

OnuAdminObjectEntry ::= SEQUENCE {

onuDescript	DisplayString,

onuSplitterSn	INTEGER,

onuOpticalLineSn	INTEGER,

Confidential▼

onuUserInfo	DisplayString,

onuType	DisplayString,

onuAdminState	INTEGER,

onuAuthMACAddress	MacAddress,

onuRegisterMACAddress	MacAddress, onuAuthMACSn		OCTET STRING,

onuRegisterSn	OCTET STRING,

onuCurrentRegState	INTEGER,

onuRegisterTime	DisplayString,

onuCurrAdminAuthState	INTEGER,

onuLatelyPassAdminAuthTime	DisplayString,

onuCurrDot1xAuthState	INTEGER,

onuLatelyPassDot1xAuthTime	DisplayString,

onuMgmtOnlineStatus	INTEGER,

onuActiveStatus	INTEGER,

onuMgmtEntryStatus	RowStatus, onuMgmtIpCfgMode		INTEGER,

onuAuthLoid	DisplayString,

onuAuthPassword	DisplayString,

onuRegisterLoid	DisplayString,

onuRegisterPassword	DisplayString

}

  - Unauthenticated ONU

EPON uncfg onu [ZXEPON-SERVICE-PRIVATE-MIB.mib] 6.2	Unauthenticated ONU is the whole table

.iso.org.dod.internet.private.enterprises.zte.zxAn.zxAnPonMib.zxAnEponMib.privateObjects.onuUnPa ssedAdminAuthInfoTable

.1.3.6.1.4.1.3902.1015.1010.1.7.14.1

  - FEC Configuration

zxGponFECMode @ zxGponservice.mib

.1.3.6.1.4.1.3902.1012.3.11.3.1.1

Confidential▼

# EPON ONU

  - ONU Remote Operation

OID Specification

zxAnEponOnuActionTable—s OID is .1.3.6.1.4.1.3902.1015.1010.1.1.2.1.

Index Specification

{ zxAnEponOnuIfIndex}, is PON composite index Type 3 or 9.

MIB Specification

For detailed description, refer to ZXANEPON-ONUMGMT-MIB.mi2.

At present, there is only one zxAnEponOnuAction object, and its role is to reset the ONU.

zxAnEponOnuActionTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnEponOnuActionEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"Update ONU parameters during run-time."

::= { zxAnEponOnuExtendedActionMgmt 1 }

zxAnEponOnuActionEntry OBJECT-TYPE SYNTAX	ZxAnEponOnuActionEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION " "

INDEX	{ zxAnEponOnuIfIndex}

::= { zxAnEponOnuActionTable 1 }

ZxAnEponOnuActionEntry ::= SEQUENCE {

zxAnEponOnuAction	INTEGER

}

Confidential▼

  - LAN port

    - Port Status

Index Specification

Type 3—s PON composite index and port No.

OID Specification

| MIB Variable | OID | Specification |
| --- | --- | --- |
| zxAnEponOnuPhyAdminState | .1.3.6.1.4.1.3902.1015.1010.1.1.3.1.1.1 | Management |
| zxAnEponOnuEthPortLinkState | .1.3.6.1.4.1.3902.1015.1010.1.1.1.5.1.2 | status Online status |

- zxAnEponOnuPhyAdminState definition

      - disable

      - enable

- zxAnEponOnuEthPortLinkState definition

- down

- up

    - Auto-Negotiation Status

Index Specification

Type 3—s PON composite index and port No.

OID Specification

| MIB Variable | OID | Specification |
| --- | --- | --- |
| zxAnEponOnuAutoNegAdminState | .1.3.6.1.4.1.3902.1015.1010.1.1.3.2.1.1 | Enable auto-negotiation |
| zxAnEponOnuAutoNegCapability | .1.3.6.1.4.1.3902.1015.1010.1.1.3.2.1.2 | Auto-negotiation capacity |
| zxAnEponOnuAutoNegCapAdvertised | .1.3.6.1.4.1.3902.1015.1010.1.1.3.2.1.3 | Auto-negotiation |
|  |  | advertisement capacity |

Confidential▼

- zxAnEponOnuAutoNegAdminState definition

- disable

- enable

- Meaning that each BIT of zxAnEponOnuAutoNegCapability indicates

zx-GLOBAL ( 0 ) ,

zx-OTHER ( 1 ) ,

zx-UNKNOWN ( 2 ) , zx-10BASE-T ( 3 ) ,

zx-10BASE-TFD ( 4 ) , zx-100BASE-T4 ( 5 ) , zx-100BASE-TX ( 6 ) ,

zx-100BASE-TXFD ( 7 ) , zx-FDX-PAUSE ( 8 ) ,

zx-FDX-APAUSE ( 9 ) , zx-FDX-SPAUSE ( 10 ) , zx-FDX-BPAUSE ( 11 ) , zx-100BASE-T2 ( 12 ) ,

zx-100BASE-T2FD ( 13 ) , zx-1000BASE-X ( 14 ) ,

zx-1000BASE-XFD ( 15 ) , zx-1000BASE-T ( 16 ) ,

zx-1000BASE-TFD ( 17 ) , zx-REM-FAULT1 ( 18 ) , zx-REM-FAULT2 ( 19 ) ,

zx-ISO-ETHERNET ( 20 ) }

- Meaning that each BIT of zxAnEponOnuAutoNegCapAdvertised indicates

zx-GLOBAL ( 0 ) ,

zx-OTHER ( 1 ) ,

zx-UNKNOWN ( 2 ) , zx-10BASE-T ( 3 ) ,

zx-10BASE-TFD ( 4 ) , zx-100BASE-T4 ( 5 ) , zx-100BASE-TX ( 6 ) ,

zx-100BASE-TXFD ( 7 ) , zx-FDX-PAUSE ( 8 ) ,

zx-FDX-APAUSE ( 9 ) ,

zx-FDX-SPAUSE ( 10 ) , zx-FDX-BPAUSE ( 11 ) ,

Confidential▼

zx-100BASE-T2 ( 12 ) ,

zx-100BASE-T2FD ( 13 ) , zx-1000BASE-X ( 14 ) ,

zx-1000BASE-XFD ( 15 ) , zx-1000BASE-T ( 16 ) ,

zx-1000BASE-TFD ( 17 ) , zx-REM-FAULT1 ( 18 ) , zx-REM-FAULT2 ( 19 ) ,

zx-ISO-ETHERNET ( 20 ) }

    - Port VLAN Mode

Index Specification

Type 3's PON composite index and port No.

OID Specification

MIB Variable

zxAnEponOnuVlanMode

OID

.1.3.6.1.4.1.3902.1015.1010.1.1.1.10.1.1.1

Specification

- zxAnEponOnuVlanMode definition

- Transparent transmission mode

- Tag mode

- Translation mode

- Trunk mode

- Hybrid mode

    - Tag VLAN

Index Specification

Type 3's PON composite index and port No.

OID Specification

| MIB Variable | OID | Specification |
| --- | --- | --- |
| zxAnEponOnuVlanTagVid | .1.3.6.1.4.1.3902.1015.1010.1.1.1.10.2.1.1 | VID |
| zxAnEponOnuVlanTagTpid | .1.3.6.1.4.1.3902.1015.1010.1.1.1.10.2.1.2 | TPID |
| zxAnEponOnuVlanTagCfi | .1.3.6.1.4.1.3902.1015.1010.1.1.1.10.2.1.3 | CFI |
| zxAnEponOnuVlanPriority | .1.3.6.1.4.1.3902.1015.1010.1.1.1.10.2.1.4 | Priority |

Confidential▼

    - Translation VLAN

Translation VLAN mode's port can configure multiple translation VLAN entries.

Index Specification

Type 3's PON composite index, port NO. and SN (start from 1).

OID Specification

| MIB Variable | OID | Specification |
| --- | --- | --- |
| zxAnEponOnuVlanTransOriginalTag | .1.3.6.1.4.1.3902.1015.1010.1.1.1.10.3.1.2 | Old VLAN |
| zxAnEponOnuVlanTransNewTag 7.2.6 Trunk VLAN | .1.3.6.1.4.1.3902.1015.1010.1.1.1.10.3.1.3 | New VLAN |

Index Specification

Type 3's PON composite index and port No.

OID Specification

MIB Variable		OID	Specification zxAnEponOnuVlanTrunkModeVlan	.1.3.6.1.4.1.3902.1015.1010.1.1.1.10.4.1.1	VLAN list

    - Hybrid VLAN

Index Specification

Type 3's PON composite index and port No.

OID Specification

| MIB Variable | OID | Specification |
| --- | --- | --- |
| zxAnEponOnuVlanTrunkModeVlan | .1.3.6.1.4.1.3902.1015.1010.1.1.1.10.4.1.1 | VLAN list |
| zxAnEponOnuVlanTagVid | .1.3.6.1.4.1.3902.1015.1010.1.1.1.10.2.1.1 | VID |

    - Port Receiving and Sending Byte Numbers

Both sending byte number and receiving byte number are 32-bit integer without a symbol, and are accumulative amount. The port's receiving rate and sending rate can be got through calculating receiving byte number in a period of time ó time.

Confidential▼

Index Specification

Type 3's PON composite index and port No.

| OID Specification |  |  |
| --- | --- | --- |
| MIB Variable | OID | Specification |
| parameter1 | .1.3.6.1.4.1.3902.1015.1010.1.1.7.1.1.2 | Receiving byte number |
| parameter6 | .1.3.6.1.4.1.3902.1015.1010.1.1.7.1.1.7 | Sending byte number |

    - Port Traffic Control

Ethernet port traffic control

Index Specification

Type 3's PON composite index and port No.

ream

Stream

OID Specification

| MIB Variable | OID | Specification |
| --- | --- | --- |
| zxAnEponOnuPortBackPressure | .1.3.6.1.4.1.3902.1015.1010.1.1.1.6.1.1 | Traffic control |
| zxAnEponOnuPortPolicing | .1.3.6.1.4.1.3902.1015.1010.1.1.1.7.1.1 | Uplink traffic policing |
| zxAnEponOnuPortPolicingCir | .1.3.6.1.4.1.3902.1015.1010.1.1.1.7.1.2 | Uplink CIR |
| zxAnEponOnuPortPolicingBucketDepth | .1.3.6.1.4.1.3902.1015.1010.1.1.1.7.1.3 | Uplink CBS |
| zxAnEponOnuPortPolicingExtraBurstSize | .1.3.6.1.4.1.3902.1015.1010.1.1.1.7.1.4 | Uplink EBS |
| zxAnEponOnuPortPolicingDownStream | .1.3.6.1.4.1.3902.1015.1010.1.1.1.7.1.5 | Downlink traffic |
|  |  | policing |
| zxAnEponOnuPortPolicingCirDownStream | .1.3.6.1.4.1.3902.1015.1010.1.1.1.7.1.6 | Downlink CIR |
| zxAnEponOnuPortPolicingBucketDepthDownSt | .1.3.6.1.4.1.3902.1015.1010.1.1.1.7.1.7 | Downlink CBS |
| zxAnEponOnuPortPolicingExtraBurstSizeDown | .1.3.6.1.4.1.3902.1015.1010.1.1.1.7.1.8 | Downlink EBS |

- zxAnEponOnuPortBackPressure definition

      - deactive

      - active

- zxAnEponOnuPortPolicing definition

- deactive

- active

- zxAnEponOnuPortPolicingDownStream definition

Confidential▼

- deactive

- active

  - Multicast VLAN

multicast vlan port eth_0/2 add vlanlist 40

Name:	zxAnEponOnuMulticastVlanTable

Type:	OBJECT-TYPE

OID:	1.3.6.1.4.1.3902.1015.1010.1.1.1.12

Full path:

iso(1).org(3).dod(6).internet(1).private(4).enterprises(1).zte(3902).zxAn(1015).z xAnPonMib(1010).zxAnEponMib(1).zxAnEponOnuRemoteMgmt(1).zxAnEponOnuExtendedAttrMg mt(1).zxAnEponOnuMulticastVlanTable(12)

Module:	ZXANEPON-ONUMGMT-MIB

Status:	current

Max access:	not-accessible

Sequences:	1: zxAnEponOnuMulticastVlanAction - INTEGER(2 - integer (32 bit)) 2: zxAnEponOnuMulticastVlanList - OCTET STRING(4 - octets)

  - VOIP

  - ONU E1 port

    - Port Management Status

Index Specification

Type 3's PON composite index and port No.

OID Specification

MIB Variable

zxAnEponOnuE1PortEnable

OID

.1.3.6.1.4.1.3902.1015.1010.1.1.1.9.1.1

Specification

- zxAnEponOnuE1PortEnable definition

- disable

- enable

Confidential▼

  - POTS Port Management

    - Port Management Status

Index Specification

Type 3's PON composite index and port No.

OID Specification

MIB Variable

zxAnEponOnuVoipPortEnable

OID

.1.3.6.1.4.1.3902.1015.1010.1.1.1.8.1.1

Specification

- zxAnEponOnuVoipPortEnable definition

- disable

- enable

  - ONU Fiber Length

View the fiber length between the OLT and ONU

Index Specification

{ ifIndex }, Type 3 or 9 PON composite index.

OID Specification

For details, refer to ZXEPON-SERVICE-MIB.mib	dot3MpcpStatTable, the table's OID is .1.3.6.1.4.1.3902.1015.1010.1.2.1.

MIB Variable

dot3MpcpRoundTripTime

OID

.1.3.6.1.4.1.3902.1015.1010.1.2.1.1.10

Specification

- dot3MpcpRoundTripTime

dot3MpcpRoundTripTimeindicates RoundTrip time between the OLT and ONU Fiber length = dot3MpcpRoundTripTim * 1.635 / 1000

  - ONU Version and Model

ONU model, software version, hardware version, etc.

Index Specification

Confidential▼

Type 3's PON composite index

OID Specification

| MIB Variable | OID | Specification |
| --- | --- | --- |
| zxAnEponOnuModel | .1.3.6.1.4.1.3902.1015.1010.1.1.1.1.1.3 | ONU model |
| zxAnEponOnuSoftwareVersion | .1.3.6.1.4.1.3902.1015.1010.1.1.1.1.1.6 | Software version |
| zxAnEponOnuHardwareVersion | .1.3.6.1.4.1.3902.1015.1010.1.1.1.1.1.5 | Hardware version |

Confidential▼

# GPON OLT

  - ONU Management

Refer to zxGponOntDevMgmtTable defined in zxGponService.mib. It is used to create and delete the ONU under a certain OLT interface, to configure the authentication mode, authentication information, etc., and to perform basic management.

Of them, zxGponOntRegId corresponds to PW, LOID authentication information, which can be modified via modifying the field.

Name	Oid	Range	Description

zxGponOntDevMgmtTypeNa me zxGponOntDevMgmtProvisio

nSn zxGponOntDevMgmtEntrySta

tus

.1.3.6.1.4.1.3902.10

12.3.28.1.1.1

.1.3.6.1.4.1.3902.10

12.3.28.1.1.5

.1.3.6.1.4.1.3902.10

12.3.28.1.1.9

STRING	(	SIZE	(	0	.. 64	) )

( SIZE ( 8	) )

SN configured by Olt for the purpose of register

regModeSn(1), regModePw(2),

regModeSnPlusPw(3),

zxGponOntRegMode

.1.3.6.1.4.1.3902.10

12.3.28.1.1.12

regModeRegisterId(4),

regModeRegisterIdPlus8021x( 5),

regModeRegisterIdPlusMutual (6),

regModeTefPw(7), regModeSnPlusTefPw(8), regModeLoid(9), regModeLoidPlusPw(10)

This object indicates the onu registration mode,'regModePwAutoBindSn' mode	s	is	an	extension	of 'regModePw'. 'regModePwAutoBindSn'

mode indicates serial number will	binds	password automatically.

Confidential▼

  - ONU Status

Name: zxGponOntPhaseState

OID: .1.3.6.1.4.1.3902.1012.3.28.2.1.4

Description:

INTEGER	{ logging ( 0 ) , los ( 1 ) , syncMib ( 2 ) , working ( 3 ) , dyinggasp ( 4 ) , authFailed ( 5 ) , offline ( 6 ) }

  - T-CONT Profile

Refer to zxGponBandwidthProfileTable defined in zxGponService.mib.

Name	Oid	Range	Description

.1.3.6.1.4.1.3902.1012

zxGponBWProfileIndex

zxGponBWProfileName zxGponBWProfileFixed

zxGponBWProfileAssur

ed

zxGponBWProfileMaxi

.3.26.1.1.1

.1.3.6.1.4.1.3902.1012

.3.26.1.1.2

.1.3.6.1.4.1.3902.1012

.3.26.1.1.3

.1.3.6.1.4.1.3902.1012

.3.26.1.1.4

.1.3.6.1.4.1.3902.1012

(  SIZE  (  0  ..

64	) )

mum

zxGponBWProfileType

zxGponBWProfileEntry Status

.3.26.1.1.5

|  | { type1 | ( 1 ) , |
| --- | --- | --- |
| .1.3.6.1.4.1.3902.1012 | type2 ( 2 ) | , type3 |
| .3.26.1.1.6 | ( 3 ) , type4 | ( 4 ) , |
|  | type5 ( 5 ) } |  |

.1.3.6.1.4.1.3902.1012

.3.26.1.1.8

  - Gemport Traffic Profile

Refer to zxGponTrafficProfileTable defined in zxGponService.mib.

Name	Oid	Range	Description

ndex

zxGponTrafficProfileI

zxGponTrafficProfile

.1.3.6.1.4.1.3902.1012.3.26

.2.1.1

.1.3.6.1.4.1.3902.1012.3.26

( SIZE ( 0 ..

Name

.2.1.2

64 ) )

zxGponTrafficProfileS	.1.3.6.1.4.1.3902.1012.3.26

Confidential▼

ir

zxGponTrafficProfileP

ir

zxGponTrafficProfile EntryStatus

.2.1.3

.1.3.6.1.4.1.3902.1012.3.26

.2.1.4

.1.3.6.1.4.1.3902.1012.3.26

.2.1.6

  - T-CONT

Refer to zxOnuTrafficMgmtUnitTable defined in zxGponService.mib.

Name	Oid	Range	Description

An	arbitrary(unique)	integer

zxOnuTrafficMgmtUnitTcont

Idx

zxOnuTrafficMgmtUnitTcont

UpBWIdxPtr zxOnuTrafficMgmtUnitEntry Status

.1.3.6.1.4.1.3902.1012.3.30

.1.1.1

.1.3.6.1.4.1.3902.1012.3.30

.1.1.3

.1.3.6.1.4.1.3902.1012.3.30

.1.1.8

for identifying an entry

in the table, from 1 to N.

This attribute is the index of template used by T-CONT BW configuration.

This entry can be create modified and delete.

  - GEM-Port

Refer to zxGponGemPortTable defined in zxGponService.mib.

Name	Oid	Range	Description

An	arbitrary(unique)	integer

zxGponGemPortIdx

zxGponGemPortName

zxGponGemPortType

zxGponGemPortTcontIdx

zxGponGemPortEntryStatus

.1.3.6.1.4.1.3902.1012.3.30

.2.1.1

.1.3.6.1.4.1.3902.1012.3.30

.2.1.2

.1.3.6.1.4.1.3902.1012.3.30

.2.1.3

.1.3.6.1.4.1.3902.1012.3.30

.2.1.4

.1.3.6.1.4.1.3902.1012.3.30

.2.1.10

( SIZE ( 0 .. 64	) )

{ unicast ( 1 ) , multicast ( 2 ) }

for identifying an entry

in the table. It is the virtual portid index

This attribute specifies the name of the GEM port.

This attribute specifies the type of the GEM port. Only 'unicast' is supported at present.

It is equal to the 'zxOnuTrafficMgmtUnitTcontI dx'	of

'zxOnuTrafficMgmtUnitTable'. This entry can be create modified and delete.

Confidential▼

  - Gemport Traffic Limit

Refer to zxGponGemPortLimitRateTable defined in zxGponService.mib.

Name	Oid	Range	Description

The	GEM	PORT

zxGponGemPortLimitUpTrafficPtr

zxGponGemPortLimitDownTrafficPtr

.1.3.6.1.4.1.3902.1012.

3.30.3.1.1

.1.3.6.1.4.1.3902.1012.

3.30.3.1.2

upstream	traffic descriptor profile.

The	GEM	PORT

downstream	traffic descriptor profile.

  - VLAN (Service Port)

Refer to zxAnServicePortConfTable defined in ZTE-AN-SERVICEPORT-MIB.mib.

Name	Oid	Range	Description 1.3.6.1.4.1.3902.1015.8.

zxAnServicePortServiceMode

zxAnUserInVid zxAnUserOutCVid zxAnUserOutSVid zxAnServicePortRowStatus

1.1.1.4

1.3.6.1.4.1.3902.1015.8.

1.1.1.5

1.3.6.1.4.1.3902.1015.8.

1.1.1.9

1.3.6.1.4.1.3902.1015.8.

1.1.1.10

1.3.6.1.4.1.3902.1015.8.

1.1.1.100

1-15

0-4094

0-4094

0-4094

  - Un-configured ONU

Please refer to zxGponUnCfgSnOntInfoTable defined in zxGponService.mib.

Name	 Oid	Range	Description zxGponUnCfgSnIdx	.1.3.6.1.4.1.3902.1012.3.13.3.1.1

zxGponUnCfgSnOntSN	.1.3.6.1.4.1.3902.1012.3.13.3.1.2

(	SIZE

( 8	) )

zxGponUnCfgSnOntRID	.1.3.6.1.4.1.3902.1012.3.13.3.1.3	STRING	The register ID of the

Confidential▼

(	SIZE	un-configured ONT. ( 4		) )

This table is read only, and is used to get uncfg ONT SN information.

  - Traffic-profile

Please refer to zxAnQos3TrafficConfigTable defined in ZTE-AN-QOS3-MIB.mib

| Name | Oid | Range | Description |
| --- | --- | --- | --- |
| zxAnQos3TrafficConfigTable | .1.3.6.1.4.1.3902.1015.21.4.6.2 |  |  |
| zxAnQos3TrafficConfigEntry | .1.3.6.1.4.1.3902.1015.21.4.6.2.1 |  |  |
| zxAnQos3TrafficPrf | .1.3.6.1.4.1.3902.1015.21.4.6.2.1.2 |  |  |
| zxAnQos3TrafficDirection | .1.3.6.1.4.1.3902.1015.21.4.6.2.1.1 |  |  |
| zxAnQos3TrafficConfigRowStatus | .1.3.6.1.4.1.3902.1015.21.4.6.2.1.20 |  |  |

The relationship between profile and ONU interface

| Name | Oid | Range | Description |
| --- | --- | --- | --- |
| zxAnQos3Rack | .1.3.6.1.4.1.3902.1015.21.4.3.1.1.1 |  |  |
| zxAnQos3Shelf | .1.3.6.1.4.1.3902.1015.21.4.3.1.1.2 |  |  |
| zxAnQos3Slot | .1.3.6.1.4.1.3902.1015.21.4.3.1.1.3 |  |  |
| zxAnQos3Port | .1.3.6.1.4.1.3902.1015.21.4.6.2.1.4 |  |  |
| zxAnQos3Onu | .1.3.6.1.4.1.3902.1015.21.4.6.2.1.5 |  |  |
| zxAnQos3VCircuitType | .1.3.6.1.4.1.3902.1015.21.4.3.1.1.6 |  |  |
| zxAnQos3LogicalId | .1.3.6.1.4.1.3902.1015.21.4.3.1.1.7 |  |  |

Confidential▼

# GPON ONU

  - Service

Via	creating	the	mapping	from	serviceConfigure	user	flow	to	gemport,	refer	to zxGponServiceTable defined in zxGponOntMgmt.mib.

| Name | Oid | Range | Description |
| --- | --- | --- | --- |
| zxGponServiceIndex | .1.3.6.1.4.1.3902.1012.3.50.11.1.1.1 |  |  |
| zxGponServiceName | .1.3.6.1.4.1.3902.1012.3.50.21.1.1.2 | STRING ( SIZE ( 1 .. 32 ) ) |  |

zxGponServiceType	.1.3.6.1.4.1.3902.1012.3.50.21.1.1.2

zxGponServiceGemPo

.1.3.6.1.4.1.3902.1012.3.50.21.1.1.4

rt

zxGponServiceMapTy

internet(1),iptv(2),voip(3),inter net-iptv(4),hybrid(5)

pe zxGponServiceMapVl

an

zxGponServiceEntryS tatus

.1.3.6.1.4.1.3902.1012.3.50.21.1.1.5	cos(1), vlan (2),cos-vlan (3)

.1.3.6.1.4.1.3902.1012.3.50.21.1.1.7	STRING(SIZE(24))

.1.3.6.1.4.1.3902.1012.3.50.11.1.1.1	2 bytes

This entry can be create modified and delete.

  - LAN Port

    - Port Management

Refer to zxGponPptpEthUNITable defined in zxGponOntMgmt.mib.

Name

zxGponPptpEthUNIAd minState

Oid

1.3.6.1.4.1.3902.1012.3.50.14.

1.1.5

Range

Description

    - Traffic Management

Confidential▼

  - LAN Port VLAN

Please refer to following tables defined in zxGponOntMgmt.mib. Add and delete ONU UNI port—s VLAN via configuring following tables.

Configuring ONU user port—s vlan filter usually concerns two MIB tables, for the meaning of each MIB variable, refer to definitions in the MIB file:

{"zxGponVlanPortTable"	,	"1.3.6.1.4.1.3902.1012.3.50.15.100.1"} ,

{"zxGponVlanPortEntry"	,	"1.3.6.1.4.1.3902.1012.3.50.15.100.1.1"} ,

{"zxGponVlanPortType"	,	"1.3.6.1.4.1.3902.1012.3.50.15.100.1.1.1"} ,

{"zxGponVlanPortIndex"	,	"1.3.6.1.4.1.3902.1012.3.50.15.100.1.1.2"} ,

{"zxGponVlanPortMode"	,	"1.3.6.1.4.1.3902.1012.3.50.15.100.1.1.3"} ,

{"zxGponVlanPortPvid"	,	"1.3.6.1.4.1.3902.1012.3.50.15.100.1.1.4"} ,

{"zxGponVlanPortConfVlanCmdTable",	"1.3.6.1.4.1.3902.1012.3.50.15.100.2"} ,

{"zxGponVlanPortConfVlanCmdEntry",	"1.3.6.1.4.1.3902.1012.3.50.15.100.2.1"} ,

{"zxGponVlanPortConfVlanCmd"	,	"1.3.6.1.4.1.3902.1012.3.50.15.100.2.1.1"} ,

{"zxGponVlanPortConfVlanId"	,	"1.3.6.1.4.1.3902.1012.3.50.15.100.2.1.2"} ,

In the following, take the configuration of gpon-onu_1/3/1:1 eth_0/1 to specify:

- Transparent transmission mode:

ZXAN(gpon-onu-mng)#show onu running config gpon-onu_1/3/1:1 pon-onu-mng gpon-onu_1/3/1:1

vlan port eth_0/1 mode transparent

／

The corresponding MIB configuration is:

snmpset -v2c -c public ipaddr .1.3.6.1.4.1.3902.1012.3.50.15.100.1.1.3 i 4

- Trunk mode:

ZXAN(gpon-onu-mng)#show onu running config gpon-onu_1/3/1:1 pon-onu-mng gpon-onu_1/3/1:1

vlan port eth_0/1 mode trunk vlan port eth_0/1 vlan 100

／

The corresponding MIB operation:

snmpset -v2c -c public ipaddr .1.3.6.1.4.1.3902.1012.3.50.15.100.1.1.3 i 2

Confidential▼

snmpset -v2c -c public ipaddr .1.3.6.1.4.1.3902.1012.3.50.15.100.2.1.1.268632320.2.1.1 i 1 .1.3.6.1.4.1.3902.1012.3.50.15.100.2.1.2.268632320.2.1.1 i 100

Other vlan processing modes are similar. Try to configure according to MIB definition by yourselves.

    - ZxGponVlanPortTable

Name	Oid	Range	Description ethuni(1),

gemport-unicast(2),

zxGponVlanPortType

zxGponVlanPortIndex

zxGponVlanPortMode

.1.3.6.1.4.1.3902.1012.3.50

.15.100.1.1.1

.1.3.6.1.4.1.3902.1012.3.50

.15.100.1.1.2

.1.3.6.1.4.1.3902.1012.3.50

.15.100.1.1.3

.1.3.6.1.4.1.3902.1012.3.50

gemport-multicast(3), wifiuni(4),

veip(5), wan(6)

4 bytes

iaccess(1),trunk(2), hybrid(3),transparent( 4), na(5)

This attribute indicates the type of port

This attribute indicates the port index.

For eth UNI: The third byte specifies the slot Id associated with this UNI; the forth byte specifies the port Id associated with this UNI.

For flow: The forth byte specifies the flow Id.(4 bytes)

For IPHost: The third and forth bytes provides a unique number for each instance of this managed entity.

the access mode of the port

zxGponVlanPortPvid

zxGponVlanPortDefa ultPriority

.15.100.1.1.4

.1.3.6.1.4.1.3902.1012.3.50

.15.100.1.1.5

1..4094	the pvid of the port

0..7	the default priority of the port

    - ZxGponVlanPortConfVlanCmdTable

Name

zxGponVlanPortConf VlanCmd

Oid

.1.3.6.1.4.1.3902.1012.3.50

.15.100.2.1.1

Range

addTaggedVlan(1),

Description

The command to operate Vlan list of

Confidential▼

zxGponVlanPortConf VlanId

.1.3.6.1.4.1.3902.1012.3.50

.15.100.2.1.2

delTaggedVlan(2),	zxGponVlanPortConfTable.Each

command	is	related	to	each meaming					of

zxGponVlanPortConfVlanObjNa me.

The value of object name in Vlan list,eg.pkgName,chanName.Differ ent  command  has  different

operated Vlan list,so this object

1..4094

has different meanings.please see

the	DESCRIPTION	of zxGponVlanPortConfVlanListCm d.

Confidential▼

    - ZxGponVlanTranslateTable

| Name zxGponVlanTranslate | Oid .1.3.6.1.4.1.3902.1012.3.50 |
| --- | --- |
| VlanId | .15.100.3.1.1 |
| zxGponVlanTranslate | .1.3.6.1.4.1.3902.1012.3.50 |
| Priority | .15.100.3.1.2 |
| zxGponVlanTranslate | .1.3.6.1.4.1.3902.1012.3.50 |
| CVlanId | .15.100.3.1.3 |
| zxGponVlanTranslate | .1.3.6.1.4.1.3902.1012.3.50 |
| CVlanPriority | .15.100.3.1.4 |
| zxGponVlanTranslate | .1.3.6.1.4.1.3902.1012.3.50 |
| SVlanId | .15.100.3.1.5 |
| zxGponVlanTranslate | .1.3.6.1.4.1.3902.1012.3.50 |
| SVlanPriority | .15.100.3.1.6 |
| zxGponVlanTranslate | .1.3.6.1.4.1.3902.1012.3.50 |
| RowStatus | .15.100.3.1.7 |

Range

Description

1 .. 4094

VlanId for Vlan translate.

Priority	for	Vlan	translate,the

0..8	scope of priority is from 0 to 8

and '8' indicates no configuration.

Inner VLAN for Vlan translate,'0'

0..4094

indicates no translation.

Inner  VLAN  priority  for  Vlan

0..8

translate,'8'

translation.

indicates

no

0..4094

0..8

Outer VLAN for Vlan translate,'0'

indicates no SVLAN.

Outer VLAN priority for Vlan translate,'8' indicates copy priority from inner VLAN priority.

This object indicates the status of

this entry

Multicast VLAN

Please refer to zxGponMCastVlanTable defined in zxGponOntMgmt.mib.

Name	Oid	Range	Description

.1.3.6.1.4.1.3902.1012.3.50.

zxGponMCastVlanId

zxGponMCastVlanEn tryStatus

15.24.1.1

.1.3.6.1.4.1.3902.1012.3.50.

15.24.1.2

  - VOIP

Please refer to zxGponOntMgmt.mib.

Confidential▼

    - IP Host

Please refer to zxGponIPHostConfigDataTable defined in zxGponOntMgmt.mib.

Name	Oid	Range	Description

This attribute provides a unique number for each instance  of  this  managed

zxGponIPHostConfigDataId

zxGponIPHostConfigDataIP Address

zxGponIPHostConfigDataIP Mask

.1.3.6.1.4.1.3902.1012.3.50

.11.1.1.1

.1.3.6.1.4.1.3902.1012.3.50

.16.1.1.5

.1.3.6.1.4.1.3902.1012.3.50

.16.1.1.6

(1~n) read-only

IpAddress 4 bytes

IpAddress 4 bytes

entity.

The ONT creates as many instances as there are independent IP stacks on the ONT.

Address used for all IP services hosted by the ONT, and has a default value of 0x0 [not set].

If this value is set, it will override any values returned in DHCP.

Subnet mask for the IP services hosted by the ONT, and has a default value of 0x0 [not set].

If this value is set, it will override any values returned in DHCP.

zxGponIPHostConfigDataGa teway

.1.3.6.1.4.1.3902.1012.3.50

.16.1.1.7

IpAddress 4 bytes

Default Gateway Address used for all IP services hosted by

the ONT, and has a default value of 0x0 [not set].

If this value is set, it will override any values returned in DHCP.

Confidential▼

    - VoIP Protocol Configuration

Name	Oid	Range	Description 0x00 = None

zxGponVoIPConfigDa taSignalProtocolUsed

.1.3.6.1.4.1.3902.1012.3.

50.17.2.1.2

0x01 = SIP

0x02 = H.248

0x03 = MGCP

0xFF  =  Selected  by Non-OMCI Management Interface

This attribute identifies the type of VoIP signaling protocol used for ONT.

Only one type of protocol is allowed

    - SIP Agent

Name	Oid	Range	Description

This	attribute	provides	a

zxGponSIPAgentConfigData Id

zxGponSIPAgentConfigData ProxyServer

zxGponSIPAgentConfigData PrimaryDNS

zxGponSIPAgentConfigData UDPTCPPort

zxGponSIPAgentConfigData HostId

.1.3.6.1.4.1.3902.1012.3.50

.17.3.1.1

.1.3.6.1.4.1.3902.1012.3.50

.17.3.1.2

.1.3.6.1.4.1.3902.1012.3.50

.17.3.1.4

.1.3.6.1.4.1.3902.1012.3.50

.17.3.1.6

.1.3.6.1.4.1.3902.1012.3.50

.17.3.1.7

(1~n) (2 bytes)

STRING(SIZE(0..12 8))

4 bytes

2 bytes

2 bytes

unique number for each instance of this managed entity.

This attribute provides a pointer to a LargeString ME that contains the name (IP address or URI) of the SIP Proxy Server for SIP signaling messages.

This attribute defines the Primary SIP DNS IP Address. If this value is zero, the Primary SIP DNS should not be used. Default value of 0.

This attribute indicates the UDP/TCP port num.

This attribute associates the SIP Agent with the TCP/UDP service to be used for communication with the SIP Server.Default value is 0xFFFF.

This attribute associates the SIP Agent with the TCP/UDP service  to  be  used  for

Confidential▼

zxGponSIPAgentConfigData EntryStatus

.1.3.6.1.4.1.3902.1012.3.50

.11.1.1.1

2 bytes

communication with the SIP Server.Default value is 0xFFFF.

This entry can be create modified and delete.

    - SIP User

Name	Oid	Range	Description

This	attribute	provides	a

zxGponUNIIndex

zxGponSIPUserDataAgentId

zxGponSIPUserDataUserPart AOR

zxGponSIPUserDataUserNa me

zxGponSIPUserDataPasswor d

.1.3.6.1.4.1.3902.1012.3.50

.11.1.1.3

.1.3.6.1.4.1.3902.1012.3.50

.17.4.1.1

.1.3.6.1.4.1.3902.1012.3.50

.17.4.1.2

.1.3.6.1.4.1.3902.1012.3.50

.17.4.1.4

.1.3.6.1.4.1.3902.1012.3.50

.17.4.1.5

(1~n) (2 bytes)

2 bytes

STRING(SIZE(0..12 8))

STRING(SIZE(0..25

))

STRING(SIZE(0..12 8))

unique number for each instance of this managed entity.

Points to the SIP Agent Config Data ME to be used for signaling.

This attribute provides a pointer to a LargeString ME that contains the user identification part of the Address of Record. This can take the form of an alphanumeric string or the directory number used to reference the user in the network. 0xFFFF indicates that no user part AOR has been defined.

This attribute defines the Customer ID used for outgoing SIP messages display attribute in ASCII string format. Default value shall be null (all zeros).

A pointer to an Authentication Security Method ME that contains a SIP user name and password used

for   authentication.

0xFFFF indicates no username/password.

Confidential▼

zxGponSIPUserDataEntrySta tus

.1.3.6.1.4.1.3902.1012.3.50

.17.4.1.17

2 bytes

This	entry	can	be	create modified and delete.

    - MGC Config

Name	Oid	Range	Description

This attribute provides a unique

zxGponMGCConfigD

ataId

zxGponMGCConfigD ataPrimaryMGC

zxGponMGCConfigD ataTCPUDPPort

zxGponMGCConfigD ataTerminationIDbase

.1.3.6.1.4.1.3902.1012.3.

50.17.7.1.1

.1.3.6.1.4.1.3902.1012.3.

50.17.7.1.2

.1.3.6.1.4.1.3902.1012.3.

50.17.7.1.12

.1.3.6.1.4.1.3902.1012.3.

50.17.7.1.19

.1.3.6.1.4.1.3902.1015.32

2 bytes

STRING(SIZE(0..128))

2 bytes

STRING (SIZE(0..25))

number for each instance of this managed entity.(1~n)

This attribute points to a network address ME that contains the name (IP address or resolved name) of the primary MGC that controls the signalling messages. The port is optional and defaults to 2944 for text message formats and 2955 for binary message formats.

This attribute specifies the port (1~65535) of TCP/UDP used for communication with the MGC.

The attribute specifies the base string for the H.248 physical termination id(s) for this ONT. This string is intended to uniquely identify an ONT. Vendor specific termination identifiers (i.e. port ids) are optionally added to this string to uniquely identify a termination on a specific ONT.

zxGponMGCConfigD ataEntryStatus

.20.1.2

.1.3.6.1.4.1.3902.1012.3.

50.17.7.1.21

STRING	Port location information

This entry can be create modified

2 bytes

and delete.

  - POTS Port

Please refer to zxGponPptpEthUNITable defined in zxGponOntMgmt.mib.

Name	Oid	Range	Description

Confidential▼

zxGponPptpPOTSUNIA dminState zxGponPptpPOTSUNIO perState

1.3.6.1.4.1.3902.1012.3.50.17.1.

1.1

1.3.6.1.4.1.3902.1012.3.50.17.1.

1.9

  - Flow Configuration

Please refer to zxGponOntMgmt.mib.

    - ZxGponFlowTable

Name	Oid	Range	Description

This	attribute indicates the index of

zxGponFlowIndex	.1.3.6.1.4.1.3902.1012.3.50.11.1.1.1	1..255

zxGponFlowEntryStatus	.1.3.6.1.4.1.3902.1012.3.50.13.1.1.3

flow in ONT, from 1 to M.(1 bytes)

This entry can be create modified and delete.

Name

Oid

Range

mode0(1),

Description

When a frame is received, the frame is processed according to the following Forward Operations. The

mode1(2),

mode2(3),

operations are invoked based on the value of VID, user priority, or the entire TCI or whether or not the

TCI field exists. This attribute indicates the received frame is treated as indicated below.(1 byte)

zxGponFlowTci

ModeForwardOp er

.1.3.6.1.4.1.390

2.1012.3.50.11.

1.1.1

mode3(4),

mode4(5),

tag-filter-action	untag-filter-action	forward-mode

'transparent'	'transparent'	'mode0'

'discard'	'transparent'	'mode1'

ZxGponFlowTciModeTable

|  | 'transparent' | 'discard' | 'mode2' |
| --- | --- | --- | --- |
| mode5(6), | 'vid-filter' | 'transparent' | 'mode3' |
|  | 'vid-filter' | 'discard' | 'mode4' |
| mode6(7), | 'prio-filter' | 'transparent' | 'mode7' |
|  | 'prio-filter' | 'discard' | 'mode8' |
| mode7(8), | 'prio-vid-filter' 'prio-vid-filter' | 'transparent' 'discard' | 'mode 1' 'mode12' |

| mode8(9), | 'vid-match' | 'transparent' | 'mode15' |
| --- | --- | --- | --- |
|  | 'vid-match' | 'discard' | 'mode16' |
| mode9(10), | 'prio-match' | 'transparent' | 'mode17' |
|  | 'prio-match' | 'discard' | 'mode18' |
| mode10(11), | 'prio-vid-match' | 'transparent' | 'mode19' |
|  | 'prio-vid-match' | 'discard' | 'mode20' |
| mode11(12), mode12(13), | 'forward' | 'discard' | 'mode21' |
| mode13(14), |  |  |  |
| mode14(15), |  |  |  |
| mode15(16), |  |  |  |
| mode16(17), |  |  |  |

Confidential▼

mode17(18),

mode18(19),

mode19(20),

mode20(21),

mode21(22)

ZxGponFlowTciTable

Name	Oid	Range	Description This	attribute specifies	the

TCI	value

zxGponFlowTciIndex	.1.3.6.1.4.1.3902.1012.3.50.13.3.1.1	2 bytes

zxGponFlowTciEntryStatus	.1.3.6.1.4.1.3902.1012.3.50.13.3.1.2

which	is

provisioned at a bridging port.

This entry can be	create modified and delete.

Confidential▼

    - GEM Port Flow Configuration

Please refer to zxGponGemFlowTable defined in zxGponOntMgmt.mib.

Name	Oid	Range	Description

zxGponGemFlowId

.1.3.6.1.4.1.3902.1012.3.50.1

3.4.1.1

1..255

This attribute indicates the flow id relate with this GEM flow.

This attribute indicates the priority bit map o this GEM flow.

This bitmap specifies the priorities used by this connection when using 802.1p mapping.Each   bit   from   bit0(LSB)   to

zxGponGemFlowPriBit

.1.3.6.1.4.1.3902.1012.3.50.1

1 byte

bit7(MSB)	indicates	whethe

Map

3.4.1.2

priority0~priority7  is	occupied.  When  the

flow	type	of	zxGponGemFlowId	is	no 'switch',zxGponGemFlowPriBitMap must be assigned non-zero; Otherwise,zxGponGemFlowPriBitMap			i optional which can be assigned as zero.

zxGponGemFlowEntry Status

.1.3.6.1.4.1.3902.1012.3.50.1

3.4.1.3

This entry can be create modified and delete.

  - Remote Operation

OID Specification

zxGponONTActionTable—s OID is .1.3.6.1.4.1.3902.1012.3.50.11.3.

Index Specification

{ zxGponOltIndex, zxGponONTIndex }, zxGponOltIndex are PON Type 1composite index, others are regular index.

MIB Specification

For detailed description, refer to zxGponOntMgmt.mib. Of them, zxGponONTReboot is to reset the ONU.

zxGponONTActionTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponONTActionEntry MAX-ACCESS		not-accessible

STATUS	current

Confidential▼

DESCRIPTION

""

::= { zxGponRmONTEquipMgmt 3 }

zxGponONTActionEntry OBJECT-TYPE SYNTAX	ZxGponONTActionEntry

MAX-ACCESS		not-accessible STATUS	current

DESCRIPTION

"It is entry in the zxGponONTActionTable." INDEX	{ zxGponOltIndex, zxGponONTIndex }

::= { zxGponONTActionTable 1 }

ZxGponONTActionEntry ::= SEQUENCE {

zxGponONTReboot	TruthValue,

zxGponONTTest	TruthValue,

zxGponONTSyncTime	TruthValue,

zxGponONTRestore	TruthValue,

zxGponONTTestResult	INTEGER,

zxGponONTRestoreFactory	TruthValue

}

  - MAC Address Table Query

MIB object name and OID:

{"zxGponBridgePortAddrListTotalNum",	"1.3.6.1.4.1.3902.1012.3.50.15.28.1.2"} ,

{"zxGponBridgePortAddrListData" ,	"1.3.6.1.4.1.3902.1012.3.50.15.28.1.4"} ,

Index Specification:

Level-5 index: INDEX	{ zxGponOltIndex, zxGponONTIndex, zxGponBridgePortType, zxGponBridgePortIndex, zxGponBridgePortAddrListIndex }

Indicate Type 1—s PON composite index respectively.ONU numbering. Bridge port type. Bridge port numbering. Last time address sequence.

Operation specification:

Only support getnext operation

MIB reference:

zxGponBridgePortAddrListTable defined in zxGponOntMgmt.mib

Confidential▼

  - WIFI Configuration

{"zxAnOnuWifiIfAdminStatus"	,	"1.3.6.1.4.1.3902.1015.1010.12.2.2.1.1"} ,

| {"zxAnOnuWifiIfRadioMeasEnable" , | "1.3.6.1.4.1.3902.1015.1010.12.2.2.1.2"} , |
| --- | --- |
| {"zxAnOnuWifiIfIsolationEnable" , | "1.3.6.1.4.1.3902.1015.1010.12.2.2.1.3"} , |
| {"zxAnOnuWifiIfWirelessWorkMode", | "1.3.6.1.4.1.3902.1015.1010.12.2.2.1.4"} , |
| {"zxAnOnuWifiIfRegulatoryDomain", | "1.3.6.1.4.1.3902.1015.1010.12.2.2.1.5"} , |
| {"zxAnOnuWifiIfArcEnable" , | "1.3.6.1.4.1.3902.1015.1010.12.2.2.1.6"} , |
| {"zxAnOnuWifiIfArcInterval" , | "1.3.6.1.4.1.3902.1015.1010.12.2.2.1.7"} , |
| {"zxAnOnuWifiIfOperationalStatus", | "1.3.6.1.4.1.3902.1015.1010.12.2.2.1.8"} , |
| {"zxAnOnuWifiIfChannelBandwidth", | "1.3.6.1.4.1.3902.1015.1010.12.2.2.1.9"} , |
| {"zxAnOnuWifiIfSgiEnable" , | "1.3.6.1.4.1.3902.1015.1010.12.2.2.1.10"} , |
| {"zxAnOnuWifiIfChannel" , | "1.3.6.1.4.1.3902.1015.1010.12.2.2.1.11"} , |
| {"zxAnOnuWifiIfTxRate" , | "1.3.6.1.4.1.3902.1015.1010.12.2.2.1.12"} , |
| {"zxAnOnuWifiIfTxPower" , | "1.3.6.1.4.1.3902.1015.1010.12.2.2.1.13"} , |
| {"zxAnOnuWifiIfQosType" , | "1.3.6.1.4.1.3902.1015.1010.12.2.2.1.14"} , |

  - VEIP Configuration

switchport-bind switch_0/1 veip 1

{"zxGponUNIMACBridgePortConfigTable",	"1.3.6.1.4.1.3902.1012.3.50.15.3"} ,

{"zxGponUNIMACBridgePortConfigEntry",	"1.3.6.1.4.1.3902.1012.3.50.15.3.1"} ,

{"zxGponUNIMACBridgePortConfigBridgeId","1.3.6.1.4.1.3902.1012.3.50.15.3.1.1"} ,

{"zxGponUNIMACBridgePortConfigBridgePort",	"1.3.6.1.4.1.3902.1012.3.50.15.3.1.2"} ,

{"zxGponUNIMACBridgePortConfigPortPrio", "1.3.6.1.4.1.3902.1012.3.50.15.3.1.3"} ,

{"zxGponUNIMACBridgePortConfigPortPathCost", "1.3.6.1.4.1.3902.1012.3.50.15.3.1.4"} ,

{"zxGponUNIMACBridgePortConfigPortSpanTreeInd", "1.3.6.1.4.1.3902.1012.3.50.15.3.1.5"} ,

{"zxGponUNIMACBridgePortConfigEncapMethod","1.3.6.1.4.1.3902.1012.3.50.15.3.1.6"}  ,

{"zxGponUNIMACBridgePortConfigLANFCSInd", "1.3.6.1.4.1.3902.1012.3.50.15.3.1.7"} ,

{"zxGponUNIMACBridgePortConfigPortMACAddr", "1.3.6.1.4.1.3902.1012.3.50.15.3.1.8"} ,

{"zxGponUNIMACBridgePortConfigEntryStatus",	"1.3.6.1.4.1.3902.1012.3.50.15.3.1.9"}  ,

{"zxGponUNIMACBridgePortConfigOutboundTDPtr", "1.3.6.1.4.1.3902.1012.3.50.15.3.1.10"} ,

{"zxGponUNIMACBridgePortConfigInboundTDPtr", "1.3.6.1.4.1.3902.1012.3.50.15.3.1.11"} ,

{"zxGponUniMacBridgePortAddrLearnDepth", "1.3.6.1.4.1.3902.1012.3.50.15.3.1.12"} ,

{"zxGponUniMacBPortOutboundTdPrf",	"1.3.6.1.4.1.3902.1012.3.50.15.3.1.13"} ,

{"zxGponUniMacBPortInboundTdPrf",	"1.3.6.1.4.1.3902.1012.3.50.15.3.1.14"} ,

Confidential▼

Level-3 index: INDEX	{ zxGponOltIndex, zxGponONTIndex, zxGponUNIIndex } Of them,	zxGponUNIIndex code is (2 << 16) | (UNI slot << 8) | UNI port

2 indicates VEIP type.

When creating VEIP bridge port, you need to specify zxGponUNIMACBridgePortConfigBridgeId.

  - UNI Port Performance Statistics

//Ethernet statistics needs to be enabled on the ONU,

you can set via operating {"zxGponOmciEthStatsEntryStatus", "1.3.6.1.4.1.3902.1012.3.50.8.1.1.17"} control performance statistics, when it is set as 4,

it indicates to start; when it is set as 6, it indicates to delete.

Level-3 index:	zxGponOltIndex, zxGponONTIndex, zxGponUNIIndex

Ethernet performance statistics Uplink:

{"zxGponOmciEthStatsUpTable"	, "1.3.6.1.4.1.3902.1012.3.50.8.16"} ,

{"zxGponOmciEthStatsUpEntry"	, "1.3.6.1.4.1.3902.1012.3.50.8.16.1"} ,

{"zxGponOmciEthStatsUpIntervalEndTime",	"1.3.6.1.4.1.3902.1012.3.50.8.16.1.1"} ,

{"zxGponOmciEthStatsUpThresDataId",	"1.3.6.1.4.1.3902.1012.3.50.8.16.1.2"} ,

{"zxGponOmciEthStatsUpDropEvents",	"1.3.6.1.4.1.3902.1012.3.50.8.16.1.3"} ,

{"zxGponOmciEthStatsUpOctets"	, "1.3.6.1.4.1.3902.1012.3.50.8.16.1.4"} ,

{"zxGponOmciEthStatsUpPkts"	, "1.3.6.1.4.1.3902.1012.3.50.8.16.1.5"} ,

| {"zxGponOmciEthStatsUpBroadcastPkts", | "1.3.6.1.4.1.3902.1012.3.50.8.16.1.6"} , |
| --- | --- |
| {"zxGponOmciEthStatsUpMulticastPkts", | "1.3.6.1.4.1.3902.1012.3.50.8.16.1.7"} , |
| {"zxGponOmciEthStatsUpCRCErrPkts", | "1.3.6.1.4.1.3902.1012.3.50.8.16.1.8"} , |
| {"zxGponOmciEthStatsUpUndersizePkts", | "1.3.6.1.4.1.3902.1012.3.50.8.16.1.9"} , |
| {"zxGponOmciEthStatsUpOversizePkts", | "1.3.6.1.4.1.3902.1012.3.50.8.16.1.10"} , |
| {"zxGponOmciEthStatsUp64OctetsPkts", | "1.3.6.1.4.1.3902.1012.3.50.8.16.1.11"} , |

{"zxGponOmciEthStatsUp65To127OctetsPkts",  "1.3.6.1.4.1.3902.1012.3.50.8.16.1.12"} ,

{"zxGponOmciEthStatsUp128To255OctetsPkts", "1.3.6.1.4.1.3902.1012.3.50.8.16.1.13"} ,

{"zxGponOmciEthStatsUp256To511OctetsPkts", "1.3.6.1.4.1.3902.1012.3.50.8.16.1.14"} ,

{"zxGponOmciEthStatsUp512To1023OctetsPkts", "1.3.6.1.4.1.3902.1012.3.50.8.16.1.15"} ,

{"zxGponOmciEthStatsUp1024To1518OctetsPkts", "1.3.6.1.4.1.3902.1012.3.50.8.16.1.16"} ,

{"zxGponOmciEthStatsUpEntryStatus",    "1.3.6.1.4.1.3902.1012.3.50.8.16.1.30"} , //Used to start the performance statistics, when it is set as 4, it indicates to start statistics; when it is set as 6, it indicates to delete statistics.

Downlink:

Confidential▼

{"zxGponOmciEthStatsDownTable"	,"1.3.6.1.4.1.3902.1012.3.50.8.17"} ,

{"zxGponOmciEthStatsDownEntry"	, "1.3.6.1.4.1.3902.1012.3.50.8.17.1"} ,

{"zxGponOmciEthStatsDownIntervalEndTime", "1.3.6.1.4.1.3902.1012.3.50.8.17.1.1"} ,

{"zxGponOmciEthStatsDownThresDataId", "1.3.6.1.4.1.3902.1012.3.50.8.17.1.2"} ,

{"zxGponOmciEthStatsDownDropEvents", "1.3.6.1.4.1.3902.1012.3.50.8.17.1.3"} ,

{"zxGponOmciEthStatsDownOctets" ,  "1.3.6.1.4.1.3902.1012.3.50.8.17.1.4"} ,

{"zxGponOmciEthStatsDownPkts"	,"1.3.6.1.4.1.3902.1012.3.50.8.17.1.5"} ,

{"zxGponOmciEthStatsDownBroadcastPkts",	"1.3.6.1.4.1.3902.1012.3.50.8.17.1.6"} ,

{"zxGponOmciEthStatsDownMulticastPkts",	"1.3.6.1.4.1.3902.1012.3.50.8.17.1.7"} ,

{"zxGponOmciEthStatsDownCRCErrPkts", "1.3.6.1.4.1.3902.1012.3.50.8.17.1.8"} ,

{"zxGponOmciEthStatsDownUndersizePkts",	"1.3.6.1.4.1.3902.1012.3.50.8.17.1.9"} ,

{"zxGponOmciEthStatsDownOversizePkts","1.3.6.1.4.1.3902.1012.3.50.8.17.1.10"} ,

{"zxGponOmciEthStatsDown64OctetsPkts","1.3.6.1.4.1.3902.1012.3.50.8.17.1.11"} ,

{"zxGponOmciEthStatsDown65To127OctetsPkts", "1.3.6.1.4.1.3902.1012.3.50.8.17.1.12"} ,

{"zxGponOmciEthStatsDown128To255OctetsPkts", "1.3.6.1.4.1.3902.1012.3.50.8.17.1.13"} ,

{"zxGponOmciEthStatsDown256To511OctetsPkts",

"1.3.6.1.4.1.3902.1012.3.50.8.17.1.14"} ,

{"zxGponOmciEthStatsDown512To1023OctetsPkts",

"1.3.6.1.4.1.3902.1012.3.50.8.17.1.15"} ,

{"zxGponOmciEthStatsDown1024To1518OctetsPkts", "1.3.6.1.4.1.3902.1012.3.50.8.17.1.16"} ,

{"zxGponOmciEthStatsDownEntryStatus",  "1.3.6.1.4.1.3902.1012.3.50.8.17.1.30"}	,

//Used to start the performance statistics, when it is set as 4, it indicates to start statistics; when it is set as 6, it indicates to delete statistics.

Confidential▼

## Configuration Process Examples

  - GPON HSI Service

    - Configuration Data

### Item	Data

T-CONT Bandwidth profile

ONU

T-CONT

GEM-Port

service

vlan port

Index 2

Type1 fixed 20M Slot 3 olt 1 onu 1 type ZTE-F622

register mode : sn CIGG00000001 Index 1

Link profile 20M Index 1

Link T-CONT 1

Index 1

Link GEM-Port 1 ServiceType:internet ServiceMapType:VLAN ServiceMapVlan:200 vlanPortMode:hybrid

untag add pvid vlanProfile0 at ethUni, then vlan 10 translate to vlan 100.

Confidential▼

    - Configuration Steps

VI

GDL Lrawser

I366

Ireote P-IAJP Vramihe

Ireote AJU

Ireote P-IAJP

Ireote CEG-Vart

Ireote serviie

Ireote vhoj part

In CLI case:

ZXAN(config-gpon)#profile tcont T1-20M type 1 fixed 20000

ZXAN(config-if)#onu 1 type ZTE-F622 sn CIGG00000001

ZXAN(config-if)#tcont 1 p T1-10M

ZXAN(config-if)#gemport 1 name Gem1 unicast tcont 1

ZXAN(gpon-onu-mng)# service vlanProfile00 type internet gemport 1 vlan 100

ZXAN(gpon-onu-mng)# vlan port eth_0/1 mode hybrid

ZXAN(gpon-onu-mng)# vlan port eth_0/1 vlan 10

ZXAN(gpon-onu-mng)# vlan port eth_0/1 translate vlan 10 svlan 100

|  |  | Confidential▼ |
| --- | --- | --- |
| STEP MIB Variable | Parameter |  |
| zxGponBWProfileIndex zxGponBWProfileName | 0x70000002 T1-20M (string) | profile name |

zxGponBWProfileFixed

20000	rate

20M

Create T-CONT Profile

zxGponBWProfileAssured	0

zxGponBWProfileMaximum	0

1	fixed

Create ONU

Create T-CONT

zxGponBWProfileType

zxGponBWProfileEntryStatus

zxGponOntDevMgmtTypeNam e

zxGponOntDevMgmtProvision Sn

zxGponOntDevMgmtEntryStatu s

zxGponOntRegMode

zxOnuTrafficMgmtUnitTcontId x zxOnuTrafficMgmtUnitTcontUp BWIdxPtr zxOnuTrafficMgmtUnitEntrySta tus

zxGponGemPortIdx

bandwidth 4

Creation

ZTE-F622 (string)	ONU type

Hex format number for ONU SN,e.g.CIGG00000001 means

'43:49:47:47:00:00:00:01'

4

creation

1	registration

mode:SN

1	T-CONT

ID

0x70000002	associated Ifid

4

creation

1	GEM-Port

ID

zxGponGemPortName	Gem1 (string)	PORT Name

1	PORT Type

Create GEM-Port

zxGponGemPortType

unicast

zxGponGemPortTcontIdx	1	associated T-CONT( ID) 4

zxGponGemPortEntryStatus

creation

zxGponServiceIndex	1

zxGponServiceName	vlanProfile00 1

Create service

zxGponServiceType

internet

zxGponServiceGemPort	gemport 1

2

zxGponServiceMapType

vlan

zxGponServiceMapVlan	100

Confidential▼

STEP

MIB Variable

Parameter

zxGponServiceEntryStatus

zxGponVlanPortType

4

creation 1

ethuni

zxGponVlanPortIndex

zxGponVlanPortMode

zxGponVlanPortPvid zxGponVlanPortConfVlanCmd

Create vlan port

zxGponVlanPortConfVlanId

zxGponVlanTranslateVlanId

zxGponVlanTranslatePriority

1

3

hybrid 10

1

addTaggedVlan 10

10

8

configuration 100

8

configuration

no

zxGponVlanTranslateSVlanId

zxGponVlanTranslateSVlanPrio rity

no

PS: The 2nd index is need for ONU/TCONT/GEMPORT creation.

The primary Index: OLT Ifindex

The 2nd Index: ONU ID (e.g. 1, 2, 3, 4, …)

  - GPON VOIP

    - SIP

#### Configuration Data

Item

Data

T-CONT Bandwidth profile

Index 2

Type1 fixed 20M

Slot 3 olt 1 onu 1

ONU

type ZTE-F622

register mode : sn CIGG00000001

Confidential▼

Item

Data

T-CONT

GEM-Port

Index 1

Link profile 20M Index 1

Link T-CONT 1

service

Index 1

Link GEM-Port 1

ServiceType:voip ServiceMapType:VLAN ServiceMapVlan:200 SignalProtocolUsed:SIP VLAN 200

ONT IP: 10.40.110.101, Mask: 255.255.255.0, Gateway:

10.40.110.254

POTS1:

AOR: 66660050

VoIP (SIP)

Username: 66660050

Password: 123 POTS2:

AOR: 66660051

Username: 66660051

Password: 123

Confidential▼

Configuration Steps

VI

GDL Lrawser

I366

Ireote P-IAJP Vramihe

Ireote AJU Ireote P-IAJP Ireote CEG-Vart Ireote Yerviie Ireote DV Vramihe Ireote ]hoj Vramihe

Iajmicure ]aDV Vrataiah Ireote ]aDV DV

Ireote YDV Vramihe

Ireote YDV Yerviie

In CLI case:

PON configuration is the same as 2.1.2.

ZXAN(gpon-onu-mng)#service voip type voip iphost 1 gemport 1 vlan 200

ZXAN(config-gpon)#onu profile ip ipProfile static gateway 10.40.110.254

ZXAN(config-gpon)# onu profile vlan vlanProfile tag-mode tag cvlan 200

ZXAN(gpon-onu-mng)#voip protocol sip

ZXAN(gpon-onu-mng)#voip-ip mode static ip-profileipProfile ip 10.40.110.101 mask 255.255.255.0 vlan-profile vlanProfile host 1

Confidential▼

ZXAN(config-gpon)#onu profile sip sipProfile proxy 10.40.123.25

ZXAN(gpon-onu-mng)#sip-service pots_0/1 profile sipProfile userid 66660050 username 66660050 password 123

ZXAN(gpon-onu-mng)#sip-service pots_0/2 profile sipProfile userid 66660051 username 66660051 password 123

### Steps	MIB Variable	Parameter

zxGponServiceIndex	1

zxGponServiceName	voip

3

Create service

zxGponServiceType

zxGponServiceIfId

voip 2

iphost

zxGponServiceGemPort	gemport 1

2

zxGponServiceMapType

vlan

zxGponServiceMapVlan	200

4

zxGponServiceEntryStatus

creation

Create IP Profile

zxGponVoipIpProfileName	ipProfile

zxGponVoipIpProfileGateway	’0a:28:6e:fe’	10.40.110.254

4

zxGponVoipIpProfileRowStatus

creation

zxGponVoipVlanProfileName	vlanProfile

1

Create Vlan

zxGponVoipVlanProfileTagMode

tag

Profile

zxGponVoipVlanProfileCVlanId	200

4

zxGponVoipVlanProfileRowStatus

creation

zxGponOnuVoipIpMode

1

static

Create VoIP IP

zxGponOnuVoipIpProfile	ipProfile

zxGponOnuVoipVlanProfile	vlanProfile

zxGponOnuVoipHostId	1

zxGponOnuVoipIpAddr	’0a:28:6e:65’	10.40.110.101

24

zxGponOnuVoipIpAddrPfxLen	255.255.255.0

Confidential▼

Steps

MIB Variable

Parameter

Configure

Protocol

VoIP

zxGponVoIPConfigDataSignalProtocolUsed

2

sip

zxGponSipProfileId

0x70000001

Create SIP

Profile

zxGponSipProfileName

zxGponSipProxyServerAddress

zxGponSipRowStatus

sipProfile

’0a:28:7b:19’

4

creation

10.40.123.25

zxGponUNIIndex

zxGponSipUserProfileId zxGponSipUserPartAor zxGponSipUsername zxGponSipUserPassword

zxGponSipUserRowStatus

1

0x70000001

66660050

66660050

123

4

creation

Create Server

SIP

zxGponUNIIndex

zxGponSipUserProfileId zxGponSipUserPartAor zxGponSipUsername zxGponSipUserPassword

zxGponSipUserRowStatus

2

0x70000001

66660051

66660051

123

4

creation

10.2.2	H.248

### Configuration Data

Item	Data

T-CONT Bandwidth profile

ONU

T-CONT

Index 2

Type1 fixed 20M Slot 3 olt 1 onu 1 type ZTE-F622

register mode : sn CIGG00000001 Index 1

Link profile 20M

GEM-Port	Index 1

Link T-CONT 1

Confidential▼

Item

Data

service

Index 1

Link GEM-Port 1

ServiceType:voip ServiceMapType:VLAN

VoIP (SIP)

ServiceMapVlan:200

Link Service 1

SignalProtocolUsed: H.248 VLAN: 200

ONT IP: 10.40.110.101, Mask: 255.255.255.0, Gateway:

10.40.110.254

Server: 10.63.198.200

user-tid: AG589, postfix-len:5, postfix-start:1

rtp-tid: RTP, postfix-len:3, postfix-start:0

Configuration Steps

VI

GDL Lrawser

I366

Ireote P-IAJP Vramihe

Ireote AJU Ireote P-IAJP Ireote CEG-Vart Ireote Yerviie Ireote DV Vramihe Ireote ]hoj Vramihe

Iajmicure ]aDV Vrataiah Ireote ]aDV DV

Ireote GCI Vramihe

Ireote GCI Yerviie

In CLI case:

PON configuration is same as 2.1.2.

Confidential▼

ZXAN(gpon-onu-mng)#service voip type voip iphost 1 gemport 1 vlan 200

ZXAN(config-gpon)#onu profile ip ipProfile static gateway 10.40.110.254

ZXAN(config-gpon)# onu profile vlan vlanProfile tag-mode tag cvlan 200

ZXAN(gpon-onu-mng)# voip protocol h248 domain domain1

ZXAN(gpon-onu-mng)#voip-ip mode static ip-profile ipProfile ip 10.40.110.101 mask 255.255.255.0 vlan-profile vlanProfile host 1

ZXAN(config-gpon)#onu profile mgc mgcProfile server1 10.63.198.200

ZXAN(config-gpon)#onu profile mgc mgcProfile user-tid prefix AG589 postfix-len 5 postfix-start 1

ZXAN(config-gpon)#onu profile mgc mgcProfile rtp-tid prefix RTP postfix-len 3 postfix-start 0

Steps

MIB Variable

Parameter

zxGponServiceIndex

zxGponServiceName

zxGponServiceType zxGponServiceIfId

Create service

zxGponServiceGemPort

zxGponServiceMapType

zxGponServiceMapVlan zxGponServiceEntryStatus

1

voip 3

voip 2

iphost

gemport 1

2

vlan 200

4

creation

zxGponVoipIpProfileName

Create IP Profile	zxGponVoipIpProfileGateway

zxGponVoipIpProfileRowStatus

ipProfile

’0a:28:6e:fe’

4

10.40.110.254

ZXAN(gpon-onu-mng)#mgc-service pots_0/1 profile mgcProfile

Confidential▼

### Steps	MIB Variable	Parameter

creation

zxGponVoipVlanProfileName	vlanProfile

1

Create Vlan

zxGponVoipVlanProfileTagMode

tag

Profile

zxGponVoipVlanProfileCVlanId	200

4

zxGponVoipVlanProfileRowStatus

creation

zxGponOnuVoipIpMode

1

static

Create VoIP IP

zxGponOnuVoipIpProfile	ipProfile

zxGponOnuVoipVlanProfile	vlanProfile

zxGponOnuVoipHostId	1

zxGponOnuVoipIpAddr	’0a:28:6e:65’	10.40.110.101

24

zxGponOnuVoipIpAddrPfxLen

255.255.255.0

Configure	VoIP Protocol

zxGponVoIPConfigDataSignalProtocolUsed

3 h248

zxGponVoIPOnuDomainName	domain1

zxGponMgcProfileId	0x70000001

zxGponMgcProfileName	mgcProfile

zxGponMgcPrimaryServer	’0a:28:7b:19’	10.40.123.25

2

zxGponMgcUserTidAssignPolicy

specified

Create Mgc Profile

zxGponMgcUserTidPrefix	AG589

zxGponMgcUserTidPostfixLen	5

zxGponMgcUserTidPostfixStartNum	1

2

zxGponMgcRtpTidAssignPolicy

specified

zxGponMgcRtpTidPrefix	RTP

zxGponMgcRtpTidPostfixLen	3

zxGponMgcRtpTidPostfixStartNum	0

4

zxGponSipRowStatus

creation

Create Mgc Server

zxGponUNIIndex	1

zxGponMgcUserProfileId	0x70000001

zxGponMgcUserRowStatus	4

Confidential▼

Steps

MIB Variable

Parameter

creation

  - EPON HIS Service

  - EPON VOIP

Confidential▼

## Optical Module and Optical Power

  - V1.2.0 PON Optical Module

OID Specification

zxAnOLTOpticalDiagTable’s OID is .1.3.6.1.4.1.3902.1015.1010.11.1.

Index Specification

{ zxAnOltIndex }, Type 1 PON composite index.

MIB Specification

For detailed description, refer to ZXAN-TRANSCEIVER-MIB.mib. Only optical modules of the PON port use this MIB.

C300V1.2.3 changes to use platform MIB, the following MIB do not support any more. Optical power unit is 0.001dBm.

zxAnOLTOpticalDiagTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnOLTOpticalDiagEntry MAX-ACCESS not-accessible

STATUS	deprecated DESCRIPTION

" "

::= { zxAnTransceiver 1 }

zxAnOLTOpticalDiagEntry OBJECT-TYPE SYNTAX	ZxAnOLTOpticalDiagEntry MAX-ACCESS		not-accessible

STATUS	deprecated DESCRIPTION

""

INDEX	{ zxAnOltIndex }

::= { zxAnOLTOpticalDiagTable 1 }

ZxAnOLTOpticalDiagEntry ::= SEQUENCE { zxAnOltIndex	Integer32, zxAnTransceiverTemp			Integer32, zxAnTransceiverVoltage		Integer32, zxAnTxBiasCurrent	Integer32, zxAnTxOpticalPower		Integer32,

zxAnRxOpticalPower		Integer32, zxAnTransceiverLinkRate	Integer32,

Confidential▼

zxAnTransceiverWaveLength		Integer32, zxAnTransceiverVendorNa me		 OCTET STRING, zxAnTransceiverVendorPn	OCTET STRING

}

  - Optical Module

All optical modules of the Ethernet port and PON port in V1.2.3 query via this MIB. CES optical modules in V1.2.5 also query via this MIB.

    - Information Query

OID Specification

zxAnOpticalModuleMonTable ’s OID is .1.3.6.1.4.1.3902.1015.3.1.13.

OLT’s transmitting optical power

zxAnOpticalPowerTxCurrValue: .1.3.6.1.4.1.3902.1015.3.1.13.1.4 The index is ifid

gpon-olt_1/3/1 corresponds to: 0x10010000

gei_1/19/1 corresponds to:0x100F0000 gei_1/19/2 corresponds to:0x100F0100

...

0Fcorresponds to logic cardid 01 corresponds to logic port No.

Index Specification

{ ifIndex }, platform Type 1 composite index.

MIB Specification

For detailed description, refer to ZTE-AN-INTERFACE-MIB.mib.

zxAnOpticalModuleMonTable	OBJECT-TYPE

SYNTAX	SEQUENCE	OF	ZxAnOpticalModuleMonEntry MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION "This table provides objects to monitor optical module in a network element. It also provides

objects for setting high and low threshold levels."

::=	{ zxAnInterfaceObjects	13 }

Confidential▼

zxAnOpticalModuleMonEntry	OBJECT-TYPE SYNTAX	ZxAnOpticalModuleMonEntry MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION "An entry in zxAnOpticalModuleMonTable." INDEX	{ ifIndex }

::=	{ zxAnOpticalModuleMonTable 1 }

ZxAnOpticalModuleMonEntry	::=	SEQUENCE { zxAnOpticalPowerRxCurrValue	Integer32,

zxAnOpticalPowerRxLowerThresh	Integer32,

zxAnOpticalPowerRxUpperThresh	Integer32,

zxAnOpticalPowerTxCurrValue	Integer32,

zxAnOpticalPowerTxLowerThresh	Integer32,

zxAnOpticalPowerTxUpperThresh	Integer32,

zxAnOpticalIfRxRate	Integer32,

zxAnOpticalIfTxRate	Integer32,

zxAnOpticalBiasCurrent	Integer32,

zxAnOpticalSupplyVoltage	Integer32,

zxAnOpticalWavelength	Integer32,

zxAnOpticalTemperature	Integer32,

zxAnOpticalVenderPn	DisplayString,

zxAnOpticalVenderName	DisplayString,

zxAnOpticalBiasCurrLowerThresh	Integer32,

zxAnOpticalBiasCurrUpperThresh	Integer32,

zxAnOpticalVoltageLowerThresh	Integer32,

zxAnOpticalVoltageUpperThresh	Integer32, zxAnOpticalTempLowerThresh		Integer32,

zxAnOpticalTempUpperThresh	Integer32,

zxAnOpticalIfRate	Integer32,

zxAnOpticalFiberType	INTEGER,

zxAnOpticalVersionLevel	DisplayString,

zxAnOpticalVendorSn				DisplayString, zxAnOpticalProductionDate		DisplayString, zxAnOpticalModuleType					DisplayString, zxAnOpticalFiberInterfaceType	DisplayString, zxAnOpticalMaterialNumber			OCTET STRING,

zxAnOpticalRegisterData	OCTET STRING

}

Confidential▼

    - Threshold Value Setting

Index Specification

Profile name, character string

zxAnOptPrfTxPwrUpperThresh

.1.3.6.1.4.1.3902.1015.3.1.23.2.1.5

lower threshold Transmitting

OID Specification

| MIB Variable | OID | Specification |
| --- | --- | --- |
| zxAnOptPrfRxPwrLowerThresh | .1.3.6.1.4.1.3902.1015.3.1.23.2.1.2 | Receiving optical power lower threshold |
| zxAnOptPrfRxPwrUpperThresh | .1.3.6.1.4.1.3902.1015.3.1.23.2.1.3 | Receiving optical power |
|  |  | upper threshold |
| zxAnOptPrfTxPwrLowerThresh | .1.3.6.1.4.1.3902.1015.3.1.23.2.1.4 | Transmitting |
|  |  | optical power |

| optical |  |  | power |
| --- | --- | --- | --- |
| upper threshold |  |  |  |
| zxAnOptPrfBiasCurrLowerThresh | .1.3.6.1.4.1.3902.1015.3.1.23.2.1.6 | Current | lower |
|  |  | threshold |  |
| zxAnOptPrfBiasCurrUpperThresh | .1.3.6.1.4.1.3902.1015.3.1.23.2.1.7 | Current | upper |
|  |  | threshold |  |
| zxAnOptPrfVoltageLowerThresh | .1.3.6.1.4.1.3902.1015.3.1.23.2.1.8 | Voltage | lower |
|  |  | threshold |  |
| zxAnOptPrfVoltageUpperThresh | .1.3.6.1.4.1.3902.1015.3.1.23.2.1.9 | Voltage | upper |
|  |  | threshold |  |
| zxAnOptPrfTempLowerThresh | .1.3.6.1.4.1.3902.1015.3.1.23.2.1.10 | Temperature lower threshold |  |
| zxAnOptPrfTempUpperThresh | .1.3.6.1.4.1.3902.1015.3.1.23.2.1.11 | Temperature |  |
|  |  | upper threshold |  |
| zxAnOptPrfOfflineTrapEnable | .1.3.6.1.4.1.3902.1015.3.1.23.2.1.12 | Online alarm |  |
|  |  | switch |  |

    - Threshold Value Loading

Index Specification

Type 1 platform composite index

OID Specification

Confidential▼

MIB Variable

zxAnOpticalIfAlmPrf

OID

.1.3.6.1.4.1.3902.1015.3.1.23.3.1.2

Specification

Profile name

  - Receiving Optical Power at OLT Side

1.3.6.1.4.1.3902.1015.1010.11.2.1.2

Level-2 index: Type 1 PON composite index and ONU ID

For example: the index that gpon-onu_1/2/2:3 corresponds to: 268567040.3 Corresponding snmp operation command should be:

snmpwalk -v2c -c public 172.17.56.250 1.3.6.1.4.1.3902.1015.1010.11.2.1.2.268567040.3

*0.002 ’ 30.

  - Optical Power at EPON ONU Side

Index Specification

Type 3 PON composite index

OID Specification

| MIB Variable | OID | Specification |
| --- | --- | --- |
| zxAnEponOnuTransTemperature | .1.3.6.1.4.1.3902.1015.1010.1.1.1.29.1.1 | Temperature |
| zxAnEponOnuSupplyVoltage | .1.3.6.1.4.1.3902.1015.1010.1.1.1.29.1.2 | Voltage |
| zxAnEponOnuTxBiasCurrent | .1.3.6.1.4.1.3902.1015.1010.1.1.1.29.1.3 | Current |
| zxAnEponOnuTxPower | .1.3.6.1.4.1.3902.1015.1010.1.1.1.29.1.4 | Transmitting |
|  |  | optical power dBm |
| zxAnEponOnuRxPower | .1.3.6.1.4.1.3902.1015.1010.1.1.1.29.1.5 | Receiving optical power dBm |

Confidential▼

  - Optical Power at GPON ONU Side

| {"zxGponRmANIMgmt" |  |  | , | "1.3.6.1.4.1.3902.1012.3.50.12"} , |
| --- | --- | --- | --- | --- |
| {"zxGponPonTable" | , |  |  | "1.3.6.1.4.1.3902.1012.3.50.12.1"} , |
| {"zxGponPonEntry" | , |  |  | "1.3.6.1.4.1.3902.1012.3.50.12.1.1"} , |
| {"zxGponPonSRInd" | , |  |  | "1.3.6.1.4.1.3902.1012.3.50.12.1.1.1"} , |
| {"zxGponPonTotalTcontNum" | , |  |  | "1.3.6.1.4.1.3902.1012.3.50.12.1.1.2"} , |
| {"zxGponPonGemBlockLen" |  | , |  | "1.3.6.1.4.1.3902.1012.3.50.12.1.1.3"} , |
| {"zxGponPonPiggybackDBAReporting", |  |  | "1.3.6.1.4.1.3902.1012.3.50.12.1.1.4"} , |  |
| {"zxGponPonWholeOnuDBAReporting", |  |  | "1.3.6.1.4.1.3902.1012.3.50.12.1.1.5"} , |  |
| {"zxGponPonSFThreshold" , |  |  | "1.3.6.1.4.1.3902.1012.3.50.12.1.1.6"} , |  |
| {"zxGponPonSDThreshold" , |  |  | "1.3.6.1.4.1.3902.1012.3.50.12.1.1.7"} , |  |
| {"zxGponPonAlarmReport" , |  |  | "1.3.6.1.4.1.3902.1012.3.50.12.1.1.8"} , |  |
| {"zxGponPonAlarmDisableInterval", |  |  | "1.3.6.1.4.1.3902.1012.3.50.12.1.1.9"} , |  |
| {"zxGponPonRxOpticalLevel" , |  |  | "1.3.6.1.4.1.3902.1012.3.50.12.1.1.10"} , |  |
| {"zxGponPonLowerRxOpticalThreshold", |  |  | "1.3.6.1.4.1.3902.1012.3.50.12.1.1.11"} , |  |
| {"zxGponPonUpperRxOpticalThreshold", |  |  | "1.3.6.1.4.1.3902.1012.3.50.12.1.1.12"} , |  |

{"zxGponPonOntResponseTime"	,	"1.3.6.1.4.1.3902.1012.3.50.12.1.1.13"} ,

{"zxGponPonTxOpticalLevel"	,	"1.3.6.1.4.1.3902.1012.3.50.12.1.1.14"} ,

{"zxGponPonLowerTxOpticalThreshold",	"1.3.6.1.4.1.3902.1012.3.50.12.1.1.15"} ,

{"zxGponPonUpperTxOpticalThreshold",	"1.3.6.1.4.1.3902.1012.3.50.12.1.1.16"} ,

{"zxGponPonPowerFeedVoltage"	,	"1.3.6.1.4.1.3902.1012.3.50.12.1.1.17"} ,

{"zxGponPonLaserBiasCurrent"	,	"1.3.6.1.4.1.3902.1012.3.50.12.1.1.18"} ,

{"zxGponPonTemperature"	,	"1.3.6.1.4.1.3902.1012.3.50.12.1.1.19"} ,

{"zxGponPonLowerVoltageThreshold",	"1.3.6.1.4.1.3902.1012.3.50.12.1.1.20"} ,

{"zxGponPonUpperVoltageThreshold",	"1.3.6.1.4.1.3902.1012.3.50.12.1.1.21"} ,

{"zxGponPonLowerCurrentThreshold",	"1.3.6.1.4.1.3902.1012.3.50.12.1.1.22"} ,

{"zxGponPonUpperCurrentThreshold",	"1.3.6.1.4.1.3902.1012.3.50.12.1.1.23"} ,

{"zxGponPonLowerTemperatureThreshold",	"1.3.6.1.4.1.3902.1012.3.50.12.1.1.24"} ,

{"zxGponPonUpperTemperatureThreshold","1.3.6.1.4.1.3902.1012.3.50.12.1.1.25"} ,

Confidential▼

## Performance Statistics

  - Ethernet Port Real-time Statistics

Both V1.2.0 and V1.2.3 support it.

    - ifTable Real-time Statistics

OID Specification

ifTable’s OID is .1.3.6.1.2.1.2.2.

Index Specification

{ ifIndex } , Type 1 platform composite index.

MIB Specification

Refer to rfc2863.mib.

ifTable OBJECT-TYPE

SYNTAX	SEQUENCE OF IfEntry

MAX-ACCESS		not-accessible STATUS	current DESCRIPTION

"A list of interface entries.	The number of entries is given by the value of ifNumber."

::= { interfaces 2 }

ifEntry OBJECT-TYPE SYNTAX	IfEntry

MAX-ACCESS		not-accessible STATUS	current DESCRIPTION

"An entry containing management information applicable to a particular interface."

INDEX	{ ifIndex }

::= { ifTable 1 }

IfEntry ::=

SEQUENCE {

ifIndex	InterfaceIndex,

ifDescr	DisplayString,

ifType	IANAifType,

Confidential▼

ifMtu	Integer32,

ifSpeed	Gauge32,

ifPhysAddress	PhysAddress,

ifAdminStatus	INTEGER,

ifOperStatus	INTEGER,

ifLastChange	TimeTicks,

ifInOctets	Counter32,

ifInUcastPkts	Counter32,

ifInNUcastPkts	Counter32,	-- deprecated

ifInDiscards	Counter32,

ifInErrors	Counter32,

ifInUnknownProtos	Counter32,

ifOutOctets	Counter32,

ifOutUcastPkts	Counter32,

ifOutNUcastPkts	Counter32,	-- deprecated

ifOutDiscards	Counter32,

ifOutErrors	Counter32,

ifOutQLen	Gauge32,	-- deprecated

ifSpecific	OBJECT IDENTIFIER -- deprecated

}

    - ifXTable Real-time Statistics

OID Specification

ifXTable’s OID is .1.3.6.1.2.1.31.1.1.

Index Specification

{ ifIndex } , Type 1 platform composite index.

MIB Specification

Refer to rfc2863.mib.

ifXTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF IfXEntry

MAX-ACCESS		not-accessible STATUS	current DESCRIPTION

"A list of interface entries.	The number of entries is

given by the value of ifNumber.	This table contains additional objects for the interface table."

::= { ifMIBObjects 1 }

Confidential▼

ifXEntry	OBJECT-TYPE SYNTAX			IfXEntry MAX-ACCESS			not-accessible STATUS		current DESCRIPTION

"An entry containing additional management information applicable to a particular interface."

AUGMENTS	{ ifEntry }

::= { ifXTable 1 }

IfXEntry ::=

SEQUENCE {

ifName	DisplayString,

ifInMulticastPkts	Counter32,

ifInBroadcastPkts	 Counter32, ifOutMulticastPkts	Counter32, ifOutBroadcastPkts		Counter32,

ifHCInOctets	Counter64,

ifHCInUcastPkts	Counter64,

ifHCInMulticastPkts	Counter64, ifHCInBroadcastPkts Counter64, ifHCOutOctets		Counter64,

ifHCOutUcastPkts			Counter64, ifHCOutMulticastPkts	Counter64, ifHCOutBroadcastPkts		Counter64, ifLinkUpDownTrapEnable					INTEGER, ifHighSpeed				Gauge32,

ifPromiscuousMode		TruthValue, ifConnectorPresent	TruthValue,

ifAlias	DisplayString, ifCounterDiscontinuityTime TimeStamp

}

    - zxAnEtherIfStatTable Real-time Statistics

ZxAnEtherIfStatEntry	::=	SEQUENCE { zxAnEtherIfStatFCSErrors	Counter32,

zxAnEtherIfStat15minFCSErrors	Counter32,

zxAnEtherIf15minFcsErrsThresh	Integer32,

zxAnEtherIfStat1dayFCSErrors	Counter32,

zxAnEtherIf1dayFcsErrsThresh	Integer32,

Confidential▼

zxAnEtherIfOutDiscardPktRatio	Integer32,

zxAnEtherIfInDiscardPktRatio	Integer32,

zxAnEtherIfInErrPktRatio	Integer32,

zxAnEtherIfOutPkts		Counter64, zxAnEtherIfInOctetsCorrect	Counter64,

zxAnEtherIfOutOctetsCorrect	Counter64,

zxAnEtherIfInOctetsError	Counter64,

zxAnEtherIfOutOctetsError	Counter64,

zxAnEthIfStat1SecFcsErrs	Counter64,

zxAnEthIfStat1SecFcsErrsThresh	Integer32,

zxAnEtherIfStatOutDiscardPkt	Counter64,

zxAnEtherIfStatInDiscardPkt	Counter64,

zxAnEtherIfStatReset	INTEGER

}

  - OLT Ethernet Real-time Statistics

Both V1.2.0 and V1.2.3 versions support it.

    - Real-time Performance

OID Specification

zxAnXponOltIfStatTable’s OID is .1.3.6.1.4.1.3902.1015.1010.5.4.

Index Specification

{ zxAnXponOltIfIndex }, Type 1 PON composite index.

MIB Specification

Refer to zxAnXpon.mib.

zxAnXponOltIfStatTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnXponOltIfStatEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"xPON interface statistics table"

::= { zxAnXpon 4 }

zxAnXponOltIfStatEntry OBJECT-TYPE

SYNTAX	ZxAnXponOltIfStatEntry MAX-ACCESS		not-accessible

STATUS	current

Confidential▼

DESCRIPTION ""

INDEX	{ zxAnXponOltIfIndex }

::= { zxAnXponOltIfStatTable 1}

ZxAnXponOltIfStatEntry ::=

SEQUENCE {

zxAnXponOltIfIndex	INTEGER,

zxAnXponOltIfRxOctes	Counter64,

zxAnXponOltIfRxPkts	Counter64,

zxAnXponOltIfRxUniPkts	Counter64,

zxAnXponOltIfRxNonUniPkts	Counter64,

zxAnXponOltIfRxMultPkts	Counter64,

zxAnXponOltIfRxBroadPkts	Counter64,

zxAnXponOltIfRxPktsDiscard	Counter64,

zxAnXponOltIfRxPktsErr	Counter64, zxAnXponOltIfRxPDR		DisplayString,

zxAnXponOltIfRxPER	DisplayString,

zxAnXponOltIfTxOctes	Counter64,

zxAnXponOltIfTxPkts	Counter64,

zxAnXponOltIfTxUniPkts	Counter64,

zxAnXponOltIfTxNonUniPkts	Counter64,

zxAnXponOltIfTxMultPkts	Counter64,

zxAnXponOltIfTxBroadPkts	Counter64,

zxAnXponOltIfTxPktsDiscard	Counter64,

zxAnXponOltIfTxPktsErr	Counter64, zxAnXponOltIfTxPDR		DisplayString,

zxAnXponOltIfTxPER			DisplayString, zxAnXponOltIfRxCRCAlignErrors		Counter64, zxAnXponOltIfRxUndersizePkts	Counter64,

zxAnXponOltIfRxOversizePkts	Counter64, zxAnXponOltIfRx64OctetPkts		Counter64,

zxAnXponOltIfTx64OctetPkts	Counter64, zxAnXponOltIfRx65To127OctetPkts Counter64, zxAnXponOltIfTx65To127OctetPkts Counter64, zxAnXponOltIfRx128To255OctetPkts Counter64, zxAnXponOltIfTx128To255OctetPkts Counter64, zxAnXponOltIfRx256To511OctetPkts Counter64, zxAnXponOltIfTx256To511OctetPkts Counter64, zxAnXponOltIfRx512To1023OctPkts Counter64, zxAnXponOltIfTx512To1023OctPkts Counter64, zxAnXponOltIfRx1024To1518OctPkts   Counter64,

zxAnXponOltIfTx1024To1518OctPkts Counter64, zxAnXponOltIfStatus	INTEGER

Confidential▼

}

  - ONU Ethernet Real-time Statistics

Both V1.2.0 and V1.2.3 versions support it.

    - Real-time Performance

OID Specification

zxAnXponOnuIfStatTable’s OID is .1.3.6.1.4.1.3902.1015.1010.5.5.

Index Specification

{ zxAnXponOnuIfIndex }, Type 3 or 9 PON composite index.

MIB Specification

Refer to zxAnXpon.mib.

zxAnXponOnuIfStatTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnXponOnuIfStatEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"xPON interface statistics table"

::= { zxAnXpon 5 }

zxAnXponOnuIfStatEntry OBJECT-TYPE

SYNTAX	ZxAnXponOnuIfStatEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

""

INDEX	{ zxAnXponOnuIfIndex }

::= { zxAnXponOnuIfStatTable 1}

ZxAnXponOnuIfStatEntry ::=

SEQUENCE {

zxAnXponOnuIfIndex	INTEGER, zxAnXponOnuIfRxOctes		Counter64,

zxAnXponOnuIfRxPkts	Counter64,

zxAnXponOnuIfRxUniPkts	Counter64,

zxAnXponOnuIfRxNonUniPkts	Counter64,

zxAnXponOnuIfRxMultPkts	Counter64,

zxAnXponOnuIfRxBroadPkts	Counter64,

Confidential▼

zxAnXponOnuIfRxPktsDiscard	Counter64,

zxAnXponOnuIfRxPktsErr	Counter64, zxAnXponOnuIfRxPDR		DisplayString,

zxAnXponOnuIfRxPER	DisplayString,

zxAnXponOnuIfTxOctes	Counter64,

zxAnXponOnuIfTxPkts	Counter64,

zxAnXponOnuIfTxUniPkts	Counter64,

zxAnXponOnuIfTxNonUniPkts	Counter64,

zxAnXponOnuIfTxMultPkts	Counter64,

zxAnXponOnuIfTxBroadPkts	Counter64,

zxAnXponOnuIfTxPktsDiscard	Counter64,

zxAnXponOnuIfTxPktsErr	Counter64, zxAnXponOnuIfTxPDR		DisplayString,

zxAnXponOnuIfTxPER		DisplayString, zxAnXponOnuIfStatus	INTEGER

}

  - GPON OLT PON-Layer Real-time Statistics

Both V1.2.0 and V1.2.3 versions support it.

    - Real-time Performance

OID Specification

.zxGponOltPmStatisRealtimeInfoTable’s OID is .1.3.6.1.4.1.3902.1012.3.12.13.

Index Specification

{ zxGponMgmtPonOltId }, Type 1 PON composite index.

MIB Specification

Refer to zxGponService.mib.

zxGponOltPmStatisRealtimeInfoTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponOltPmStatisRealtimeInfoEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"" REFERENCE

""

::= { zxGponStandardOlt 13 }

Confidential▼

zxGponOltPmStatisRealtimeInfoEntry OBJECT-TYPE SYNTAX	ZxGponOltPmStatisRealtimeInfoEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

""

INDEX	{ zxGponMgmtPonOltId }

::= { zxGponOltPmStatisRealtimeInfoTable 1 }

ZxGponOltPmStatisRealtimeInfoEntry ::= SEQUENCE {

zxGponOltPmStatisInfoCorrectNonIdleGemFramesUpstream

Unsigned32, Unsigned32,

Unsigned32,

zxGponOltPmStatisInfoCorrectIdleGemFramesUpstream

zxGponOltPmStatisInfoErroredGemFramesUpstream	Unsigned32, zxGponOltPmStatisInfoGemPayloadBytesUpstream

Unsigned32, Unsigned32,

Unsigned32,

Unsigned32, Unsigned32, Unsigned32, Unsigned32, Unsigned32,

Unsigned32,

zxGponOltPmStatisInfoCorrectEthernetFramesUpstream zxGponOltPmStatisInfoErroredEthernetFramesUpstream

zxGponOltPmStatisInfoTotalOmciFramesUpstream	Unsigned32, zxGponOltPmStatisInfoERR	Unsigned32,

zxGponOltPmStatisInfoREI	Unsigned32, zxGponOltPmStatisInfoValidEthernetPacketDownstream

zxGponOltPmStatisInfoCpuPacketDownstream		Unsigned32, zxGponOltPmStatisInfoPloamDownstream	Unsigned32, zxGponOltPmStatisInfoPloamUpstream			Unsigned32,

zxGponOltPmStatisInfoInvalidPacketUpstream	Unsigned32, zxGponOltPmFecCorrectedBytes

zxGponOltPmFecCorrectedWords zxGponOltPmFecUncorrectedWords zxGponOltPmFecTotalRxWords zxGponOltPmBipErrBits zxGponOltPmCrcErrPkts

zxGponOltPmStatisReset	INTEGER

Confidential▼

}

  - GPON ONU PON-Layer Real-time Statistics

Both V1.2.0 and V1.2.3 versions support it.

    - Real-time Performance

OID Specification

zxGponOntPmStatisRealtimeInfoTable ’s OID is .1.3.6.1.4.1.3902.1012.3.28.6.1.

Index Specification

{ zxGponMgmtPonOltId, zxGponMgmtPonOnuId }, of them. zxGponMgmtPonOltId is Type 1 PON composite index, while others are regular index.

MIB Specification

Refer to zxGponService.mib.

zxGponOntPmStatisRealtimeInfoTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponOntPmStatisRealtimeInfoEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"unused attribute" REFERENCE

"G.984.3"

::= { zxGponPrivateOnu 6 }

zxGponOntPmStatisRealtimeInfoEntry OBJECT-TYPE SYNTAX	ZxGponOntPmStatisRealtimeInfoEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

""

INDEX	{ zxGponMgmtPonOltId, zxGponMgmtPonOnuId }

::= { zxGponOntPmStatisRealtimeInfoTable 1 }

ZxGponOntPmStatisRealtimeInfoEntry ::= SEQUENCE {

zxGponOntPmStatisInfoCorrectNonIdleGemFramesUpstream Unsigned32, zxGponOntPmStatisInfoCorrectIdleGemFramesUpstream	Unsigned32, zxGponOntPmStatisInfoErroredGemFramesUpstream

Confidential▼

Unsigned32, Unsigned32,

Unsigned32, Unsigned32,

Unsigned32,

zxGponOntPmStatisInfoGemPayloadBytesUpstream

zxGponOntPmStatisInfoCorrectEthernetFramesUpstream	Unsigned32, zxGponOntPmStatisInfoErroredEthernetFramesUpstream	 Unsigned32, zxGponOntPmStatisInfoTotalOmciFramesUpstream		Unsigned32,

zxGponOntPmStatisInfoERRi zxGponOntPmStatisInfoREIi

zxGponOntPmStatisInfoUnreceivedBurstsUpstream	Unsigned32, zxGponOntPmStatisInfoBipErrorUpstream		Unsigned32,

zxGponOntPmStatisInfoCorrectedBitsUpstream	Unsigned32, zxGponOntPmStatisInfoNotCorrectedWordsUpstream		Unsigned32, zxGponOntPmStatisInfoLostBurst		 Unsigned32, zxGponOntTxCorrectNonIdleGemFrms

zxGponOntTxCorrectIdleGemFrms

Unsigned32, Unsigned32,

Unsigned32, Unsigned32, Unsigned32, Unsigned32,

Unsigned32, Unsigned32,

Unsigned32, Unsigned32,

INTEGER

}

zxGponOntTxOmciFrms zxGponOntTxPloamFrms zxGponOntRxPloamFrms zxGponOntLofiAlarms zxGponOntPmFecCorrectedBytes zxGponOntPmFecCorrectedWords zxGponOntPmFecUncorrectedWords zxGponOntPmFecTotalRxWords zxGponOntPmCrcErrPkts zxGponOntPmReset

Confidential▼

  - Ethernet Port Performance (V1.2.3)

    - Real-time Performance

OID Specification

zxAnEthIfCurrStatsPolhe’s OID  is .1.3.6.1.4.1.3902.1015.3.11.2.2.

Index Specification

{ ifIndex } , Type 1 platform composite index.

MIB Specification

Refer to ZTE-AN-INTERFACE-STATS-MIB.mib.

zxAnEthIfCurrStatsTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnEthIfCurr StatsEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"Current performance data table of the ethernet interfaces."

::= { zxAnIfPerfObjects 2 }

zxAnEthIfCurrStatsEntry	OBJECT-TYPE SYNTAX	ZxAnEthIfCurrStatsEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"Current performance data entry of the ethernet interfaces."

INDEX	{ ifIndex}

::= { zxAnEthIfCurrStatsTable 1 }

ZxAnEthIfCurrStatsEntry ::= SEQUENCE { zxAnEthIfInOctets	Counter64,

zxAnEthIfInPkts	Counter64,

zxAnEthIfInUcastPkts	Counter64,

zxAnEthIfInMcastPkts	Counter64,

zxAnEthIfInBcastPkts		Counter64, zxAnEthIfInOversizedPkts	Counter64, zxAnEthIfInUndersizedPkts	Counter64, zxAnEthIfOutOctets			Counter64,

zxAnEthIfOutPkts	Counter64,

zxAnEthIfOutUcastPkts	Counter64,

zxAnEthIfOutMcastPkts	Counter64,

Confidential▼

zxAnEthIfOutBcastPkts	Counter64,

zxAnEthIfOutPausePkts	Counter64,

zxAnEthIfOutDiscardPkts	Counter64,

zxAnEthIfOutCollisions			Counter64, zxAnEthIfSingleCollisions		Counter64, zxAnEthIfMultipleCollisions	Counter64,

zxAnEthIfFcsErrors		Counter64, zxAnEthIfAlignmentErrors	Counter64,

zxAnEthIfReset	INTEGER

}

    - 15-MiniteReal-time Performance

OID Specification

zxAnEthIfCurr15MinPerfTable ’s OID is .1.3.6.1.4.1.3902.1015.3.12.2.2.

Index Specification

{ ifIndex } , Type 1 platform composite index.

MIB Specification

Refer to ZTE-AN-INTERFACE-ETH-PERF-MIB.mib.

zxAnEthIfCurr15MinPerfTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnEthIfCurr15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 15 minutes interval performance data table of the ethernet

interfaces."

::= { zxAnEthIfPerfObjects 2 }

zxAnEthIfCurr15MinPerfEntry	OBJECT-TYPE SYNTAX	ZxAnEthIfCurr15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 15 minutes interval performance data entry of the ethernet interfaces."

INDEX	{ ifIndex}

::= { zxAnEthIfCurr15MinPerfTable 1 }

Page 90

ZxAnEthIfCurr15MinPerfEntry ::= SEQUENCE { zxAnEthIfC15MTimeElapsed	Gauge32,

Confidential▼

zxAnEthIfC15MInOctets	Counter64,

zxAnEthIfC15MInPkts	Counter64,

zxAnEthIfC15MInUcastPkts	Counter64,

zxAnEthIfC15MInMcastPkts	Counter64,

zxAnEthIfC15MInBcastPkts		Counter64, zxAnEthIfC15MInOversizedPkts	Counter64,

zxAnEthIfC15MInUndersizedPkts	Counter64, zxAnEthIfC15MOutOctets		Counter64,

zxAnEthIfC15MOutPkts	Counter64,

zxAnEthIfC15MOutUcastPkts	Counter64,

zxAnEthIfC15MOutMcastPkts	Counter64,

zxAnEthIfC15MOutBcastPkts	Counter64,

zxAnEthIfC15MOutPausePkts					Counter64, zxAnEthIfC15MOutDiscardPkts				Counter64, zxAnEthIfC15MOutCollisions			Counter64, zxAnEthIfC15MSingleCollisions		Counter64, zxAnEthIfC15MMultipleCollisions	Counter64, zxAnEthIfC15MFcsErrors				 Counter64,

zxAnEthIfC15MAlignmentErrors	Counter64, zxAnEthIfC15MReset		INTEGER

}

    - 24-Hour Real-time Performance

OID Specification

zxAnEthIfCurr1DayPerfPolhe’s OID is .1.3.6.1.4.1.3902.1015.3.12.2.3.

Index Specification

{ ifIndex } , Type 1 platform composite index.

MIB Specification

Refer to ZTE-AN-INTERFACE-ETH-PERF-MIB.mib.

zxAnEthIfCurr1DayPerfTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnEthIfCurr1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 1 day interval performance data table of the ethernet interfaces."

::= { zxAnEthIfPerfObjects 3 }

zxAnEthIfCurr1DayPerfEntry	OBJECT-TYPE SYNTAX	ZxAnEthIfCurr1DayPerfEntry

Confidential▼

MAX-ACCESS		not-accessible STATUS	current DESCRIPTION

"The 1 day interval performance data entry of the ethernet interfaces."

INDEX	{ ifIndex}

::= { zxAnEthIfCurr1DayPerfTable 1 }

ZxAnEthIfCurr1DayPerfEntry ::= SEQUENCE { zxAnEthIfC1DTimeElapsed	Gauge32,

zxAnEthIfC1DInOctets	Counter64,

zxAnEthIfC1DInPkts	Counter64,

zxAnEthIfC1DInUcastPkts	Counter64,

zxAnEthIfC1DInMcastPkts	Counter64,

zxAnEthIfC1DInBcastPkts		Counter64, zxAnEthIfC1DInOversizedPkts	Counter64, zxAnEthIfC1DInUndersizedPkts	Counter64, zxAnEthIfC1DOutOctets			Counter64,

zxAnEthIfC1DOutPkts	Counter64,

zxAnEthIfC1DOutUcastPkts	Counter64,

zxAnEthIfC1DOutMcastPkts	Counter64,

zxAnEthIfC1DOutBcastPkts	Counter64,

zxAnEthIfC1DOutPausePkts	Counter64,

zxAnEthIfC1DOutDiscardPkts	Counter64,

zxAnEthIfC1DOutCollisions		Counter64, zxAnEthIfC1DSingleCollisions	 Counter64, zxAnEthIfC1DMultipleCollisions	Counter64, zxAnEthIfC1DFcsErrors				Counter64, zxAnEthIfC1DAlignmentErrors			Counter64, zxAnEthIfC1DReset					INTEGER

}

    - 15-Minite Historical Performance

OID Specification

zxAnEthIfHis15MinPerfPolhe’s OID is .1.3.6.1.4.1.3902.1015.3.12.2.4.

Index Specification

{ ifIndex } , Type 1 platform composite index.

MIB Specification

Refer to ZTE-AN-INTERFACE-ETH-PERF-MIB.mib.

zxAnEthIfHis15MinPerfTable	OBJECT-TYPE

Confidential▼

SYNTAX	SEQUENCE OF ZxAnEthIfHis15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 15 minutes interval history performance data table of the ethernet interfaces."

::= { zxAnEthIfPerfObjects 4 }

zxAnEthIfHis15MinPerfEntry	OBJECT-TYPE SYNTAX	ZxAnEthIfHis15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 15 minutes interval performance data entry of the ethernet interfaces."

INDEX	{ ifIndex, zxAnEthIfH15MIntervalNo }

::= { zxAnEthIfHis15MinPerfTable 1 }

ZxAnEthIfHis15MinPerfEntry ::= SEQUENCE { zxAnEthIfH15MIntervalNo	Integer32,

zxAnEthIfH15MIntervalValidData	TruthValue, zxAnEthIfH15MDateTime		DateAndTime,

zxAnEthIfH15MInOctets	Counter64,

zxAnEthIfH15MInPkts	Counter64,

zxAnEthIfH15MInUcastPkts	Counter64,

zxAnEthIfH15MInMcastPkts	Counter64,

zxAnEthIfH15MInBcastPkts		Counter64, zxAnEthIfH15MInOversizedPkts	Counter64, zxAnEthIfH15MInUndersizedPkts	Counter64, zxAnEthIfH15MOutOctets			Counter64,

zxAnEthIfH15MOutPkts	Counter64,

zxAnEthIfH15MOutUcastPkts	Counter64,

zxAnEthIfH15MOutMcastPkts	Counter64,

zxAnEthIfH15MOutBcastPkts	Counter64,

zxAnEthIfH15MOutPausePkts					Counter64, zxAnEthIfH15MOutDiscardPkts				Counter64, zxAnEthIfH15MOutCollisions		Counter64, zxAnEthIfH15MSingleCollisions	 Counter64, zxAnEthIfH15MMultipleCollisions	Counter64, zxAnEthIfH15MFcsErrors				 Counter64, zxAnEthIfH15MAlignmentErrors			Counter64

}

Confidential▼

    - 24-Hour Historical Performance

OID Specification

zxAnEthIfHis1DayPerfTable’s OID is .1.3.6.1.4.1.3902.1015.3.12.2.5.

Index Specification

{ ifIndex } , Type 1 platform composite index.

MIB Specification

Refer to ZTE-AN-INTERFACE-ETH-PERF-MIB.mib.

zxAnEthIfHis1DayPerfTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnEthIfHis1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 1 day interval history performance data table of the ethernet

interfaces."

::= { zxAnEthIfPerfObjects 5 }

zxAnEthIfHis1DayPerfEntry	OBJECT-TYPE SYNTAX	ZxAnEthIfHis1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 1 day interval performance data entry of the ethernet interfaces."

INDEX	{ ifIndex, zxAnEthIfH1DIntervalNo }

::= { zxAnEthIfHis1DayPerfTable 1 }

ZxAnEthIfHis1DayPerfEntry ::= SEQUENCE { zxAnEthIfH1DIntervalNo		Integer32, zxAnEthIfH1DIntervalValidData	TruthValue, zxAnEthIfH1DDateTime			DateAndTime,

zxAnEthIfH1DInOctets	Counter64,

zxAnEthIfH1DInPkts	Counter64,

zxAnEthIfH1DInUcastPkts	Counter64,

zxAnEthIfH1DInMcastPkts	Counter64,

zxAnEthIfH1DInBcastPkts		Counter64, zxAnEthIfH1DInOversizedPkts	Counter64, zxAnEthIfH1DInUndersizedPkts	Counter64,

zxAnEthIfH1DOutOctets	Counter64,

zxAnEthIfH1DOutPkts	Counter64,

zxAnEthIfH1DOutUcastPkts	Counter64,

Confidential▼

zxAnEthIfH1DOutMcastPkts	Counter64,

zxAnEthIfH1DOutBcastPkts	Counter64,

zxAnEthIfH1DOutPausePkts	Counter64,

zxAnEthIfH1DOutDiscardPkts	Counter64,

zxAnEthIfH1DOutCollisions		Counter64, zxAnEthIfH1DSingleCollisions	Counter64,

zxAnEthIfH1DMultipleCollisions	Counter64, zxAnEthIfH1DFcsErrors		Counter64,

zxAnEthIfH1DAlignmentErrors	Counter64

}

    - Performance Threshold Alarm Profile

OID Specification

zxAnEthIfAlmProfileTable’s OID is .1.3.6.1.4.1.3902.1015.3.12.2.8. zxAnEthIfAlmProfileConfPolhe’s OID is .1.3.6.1.4.1.3902.1015.3.12.2.6.

Index Specification

{ zxAnEthIfAlmProfileName, zxAnEthIfPerfVariable }

MIB Specification

Refer to ZTE-AN-INTERFACE-ETH-PERF-MIB.mib.

Type 1 platform composite index.

zxAnEthIfAlmProfileTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnEthIfAlmProfileEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The performance alarm threshold profile table of the ethernet interfaces.It can be used to delete all of the performance alarm

threshold profiles which has the same name configured in

‒zxOjEtbImOhgVramiheIajmPolhe’."

::= { zxAnEthIfPerfObjects 8 }

zxAnEthIfAlmProfileEntry	OBJECT-TYPE SYNTAX	ZxAnEthIfAlmProfileEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The performance alarm threshold profile entry of the ethernet

interfaces."

INDEX	{ zxAnEthIfAlmProfileName}

::= { zxAnEthIfAlmProfileTable 1 }

Confidential▼

ZxAnEthIfAlmProfileEntry ::= SEQUENCE { zxAnEthIfAlmProfileRowStatus	RowStatus

}

zxAnEthIfAlmProfileConfTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnEthIfAlmProfileConfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The performance alarm threshold profile table of the ethernet interfaces."

::= { zxAnEthIfPerfObjects 6 }

zxAnEthIfAlmProfileConfEntry	OBJECT-TYPE SYNTAX	ZxAnEthIfAlmProfileConfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The performance alarm threshold profile entry of the ethernet interfaces.These variables in the profile configuration is optional, if not configured, it means the correspongding alarm is not reported, but the alarm and clear alarm threshold configuration is needed to bind."

INDEX	{ zxAnEthIfAlmProfileName, zxAnEthIfPerfVariable }

::= { zxAnEthIfAlmProfileConfTable 1 }

ZxAnEthIfAlmProfileConfEntry ::= SEQUENCE { zxAnEthIfAlmProfileName	DisplayString,

zxAnEthIfPerfVariable	OBJECT IDENTIFIER, zxAnEthIfRiseAlmThresh			HCPerfCurrentCount, zxAnEthIfClrRiseAlmThresh		HCPerfCurrentCount,

zxAnEthIfRiseWarnThresh			 HCPerfCurrentCount, zxAnEthIfClrRiseWarnThresh		 HCPerfCurrentCount, zxAnEthIfFallWarnThresh		 HCPerfCurrentCount, zxAnEthIfClrFallWarnThresh	 HCPerfCurrentCount, zxAnEthIfFallAlmThresh		HCPerfCurrentCount, zxAnEthIfClrFallAlmThresh	HCPerfCurrentCount, zxAnEthIfThreshUsed			BITS, zxAnEthIfAlmProfileConfRowStatus			RowStatus

}

Confidential▼

    - Performance Alarm Profile Loading

OID Specification

zxAnEthIfAlmProfileApplyTable’s OID is .1.3.6.1.4.1.3902.1015.3.12.2.7.

Index Specification

{ ifIndex } , Type 1 platform composite index.

MIB Specification

Refer to ZTE-AN-INTERFACE-ETH-PERF-MIB.mib.

zxAnEthIfAlmProfileApplyTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnEthIfAlmProfileApplyEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The performance alarm threshold profile configuration table of

the ethernet interfaces."

::= { zxAnEthIfPerfObjects 7 }

zxAnEthIfAlmProfileApplyEntry	OBJECT-TYPE SYNTAX	ZxAnEthIfAlmProfileApplyEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The performance alarm threshold profile configuration entry of the ethernet interfaces."

INDEX	{ ifIndex }

::= { zxAnEthIfAlmProfileApplyTable 1 }

ZxAnEthIfAlmProfileApplyEntry ::= SEQUENCE { zxAnEthIfAlmPrf	DisplayString, zxAnEthIfAlmPrfApplyRowStatus		RowStatus

}

  - VPORT Performance (V1.2.3)

    - Starting and Ending

OID Specification

zxGponVportCurrPerfTable’s OID is .1.3.6.1.4.1.3902.1012.3.34.2.

Confidential▼

Index Specification

{	zxGponMgmtPonOltId,	zxGponMgmtPonOnuId, zxGponMgmtPonOnuBpIndex }, of them, zxGponMgmtPonOltId is PON Type 1 composite index, while others are regular index.

MIB Specification

Refer to zxGponVportCurrPerfTable defined in zxGponService.mib.

By default, the system won’t perform VPORT performance statistics. If needed, you can start or end a certain VPORT performance statistics via operating zxGponVportCurrRowStatus.

zxGponVportCurrPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponVportCurrPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"Enable/disable the interface GPON vport Statisic."

::= { zxAnGponVportPerfMgmt 2 }

zxGponVportCurrPerfEntry OBJECT-TYPE SYNTAX	ZxGponVportCurrPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"It is entry in the table zxGponVportCurrPerfTable." INDEX	{ zxGponMgmtPonOltId, zxGponMgmtPonOnuId,

zxGponMgmtPonOnuBpIndex }

::= { zxGponVportCurrPerfTable 1 }

ZxGponVportCurrPerfEntry ::= SEQUENCE {

zxGponVportCurrRowStatus	RowStatus

}

    - Real-time Performance

OID Specification

zxAnBrgIfCurrStats Table’s OID is .1.3.6.1.4.1.3902.1015.3.11.2.3.

Index Specification

{ ifIndex } , Type 4 or 10 platform composite index.

MIB Specification

Confidential▼

Refer to ZTE-AN-INTERFACE-STATS-MIB.mib.

zxAnBrgIfCurrStatsTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnBrgIfCurrStatsEntry MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"Current performance data table of bridge interfaces."

::= { zxAnIfPerfObjects 3 }

zxAnBrgIfCurrStatsEntry	OBJECT-TYPE SYNTAX	ZxAnBrgIfCurrStatsEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"Current performance data entry of bridge interfaces." INDEX	{ ifIndex}

::= { zxAnBrgIfCurrStatsTable 1 }

ZxAnBrgIfCurrStatsEntry ::= SEQUENCE { zxAnBrgIfInOctets	Counter64,

zxAnBrgIfInUcastPkts	Counter64,

zxAnBrgIfInMcastPkts	Counter64,

zxAnBrgIfInBcastPkts	Counter64,

zxAnBrgIfOutOctets	Counter64,

zxAnBrgIfOutUcastPkts	Counter64,

zxAnBrgIfOutMcastPkts	Counter64,

zxAnBrgIfOutBcastPkts	Counter64,

zxAnBrgIfReset	INTEGER

}

    - 15-Minute Real-time Performance

OID Specification

zxAnBrgIfCurr15MinPerf Table’s OID is .1.3.6.1.4.1.3902.1015.3.13.2.2.

Index Specification

{ ifIndex } , Type 4 or 10 platform composite index.

Confidential▼

zxAnBrgIfCurr15MinPerfTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnBrgIfCurr15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 15 minutes interval performance data table of the bridge

interfaces."

::= { zxAnBrgIfPerfObjects 2 }

zxAnBrgIfCurr15MinPerfEntry	OBJECT-TYPE SYNTAX	ZxAnBrgIfCurr15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 15 minutes interval performance data entry of the bridge interfaces."

INDEX	{ ifIndex}

::= { zxAnBrgIfCurr15MinPerfTable 1 }

ZxAnBrgIfCurr15MinPerfEntry ::= SEQUENCE { zxAnBrgIfC15MTimeElapsed	Gauge32,

zxAnBrgIfC15MInOctets	Counter64,

zxAnBrgIfC15MInUcastPkts	Counter64,

zxAnBrgIfC15MInMcastPkts	Counter64,

zxAnBrgIfC15MInBcastPkts	Counter64,

zxAnBrgIfC15MOutOctets	Counter64,

zxAnBrgIfC15MOutUcastPkts	Counter64,

zxAnBrgIfC15MOutMcastPkts	Counter64,

zxAnBrgIfC15MOutBcastPkts	Counter64,

zxAnBrgIfC15MReset	INTEGER

}

    - 24-Hour Real-time Performance

OID Specification

zxAnBrgIfCurr1DayPerf Table’s OID is .1.3.6.1.4.1.3902.1015.3.13.2.2.

Index Specification

{ ifIndex } , Type 4 or 10 platform composite index.

Confidential▼

zxAnBrgIfCurr1DayPerfTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnBrgIfCurr1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 1 day interval performance data table of the bridge

interfaces."

::= { zxAnBrgIfPerfObjects 3 }

zxAnBrgIfCurr1DayPerfEntry	OBJECT-TYPE SYNTAX	ZxAnBrgIfCurr1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 1 day interval performance data entry of the bridge interfaces."

INDEX	{ ifIndex}

::= { zxAnBrgIfCurr1DayPerfTable 1 }

ZxAnBrgIfCurr1DayPerfEntry ::= SEQUENCE { zxAnBrgIfC1DTimeElapsed	Gauge32,

zxAnBrgIfC1DInOctets	Counter64,

zxAnBrgIfC1DInUcastPkts	Counter64,

zxAnBrgIfC1DInMcastPkts	Counter64,

zxAnBrgIfC1DInBcastPkts	Counter64,

zxAnBrgIfC1DOutOctets	Counter64,

zxAnBrgIfC1DOutUcastPkts	Counter64,

zxAnBrgIfC1DOutMcastPkts	Counter64,

zxAnBrgIfC1DOutBcastPkts	Counter64,

zxAnBrgIfC1DReset	INTEGER

}

    - 15-Minute Historical Performance

OID Specification

zxAnBrgIfHis15MinPerfPolhe—s OID is .1.3.6.1.4.1.3902.1015.3.13.2.4.

Index Specification

{ ifIndex } , Type 4 or 10 platform composite index.

MIB Specification

Refer to ZTE-AN-INTERFACE-BRG-PERF-MIB.mib.

zxAnBrgIfHis15MinPerfTable	OBJECT-TYPE

Confidential▼

SYNTAX	SEQUENCE OF ZxAnBrgIfHis15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 15 minutes interval history performance data table of the bridge interfaces."

::= { zxAnBrgIfPerfObjects 4 }

zxAnBrgIfHis15MinPerfEntry	OBJECT-TYPE SYNTAX	ZxAnBrgIfHis15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 15 minutes interval performance data entry of the bridge interfaces."

INDEX	{ ifIndex,zxAnBrgIfH15MIntervalNo }

::= { zxAnBrgIfHis15MinPerfTable 1 }

ZxAnBrgIfHis15MinPerfEntry ::= SEQUENCE { zxAnBrgIfH15MIntervalNo		Integer32, zxAnBrgIfH15MIntervalValidData	TruthValue,

zxAnBrgIfH15MDateTime	DateAndTime,

zxAnBrgIfH15MInOctets	Counter64,

zxAnBrgIfH15MInUcastPkts	Counter64,

zxAnBrgIfH15MInMcastPkts	Counter64,

zxAnBrgIfH15MInBcastPkts	Counter64,

zxAnBrgIfH15MOutOctets	Counter64,

zxAnBrgIfH15MOutUcastPkts	Counter64,

zxAnBrgIfH15MOutMcastPkts	Counter64,

zxAnBrgIfH15MOutBcastPkts	Counter64

}

    - 24-Hour Historical Performance

OID Specification

zxAnBrgIfHis1DayPerf Polhe—s OID is .1.3.6.1.4.1.3902.1015.3.13.2.5.

Index Specification

{ ifIndex } , Type 4 or 10 platform composite index.

MIB Specification

Refer to ZTE-AN-INTERFACE-BRG-PERF-MIB.mib.

Confidential▼

zxAnBrgIfHis1DayPerfTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnBrgIfHis1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 1 day interval history performance data table of the bridge

interfaces."

::= { zxAnBrgIfPerfObjects 5 }

zxAnBrgIfHis1DayPerfEntry	OBJECT-TYPE SYNTAX	ZxAnBrgIfHis1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 1 day interval performance data entry of the bridge interfaces."

INDEX	{ ifIndex,zxAnBrgIfH1DIntervalNo }

::= { zxAnBrgIfHis1DayPerfTable 1 }

ZxAnBrgIfHis1DayPerfEntry ::= SEQUENCE { zxAnBrgIfH1DIntervalNo		Integer32, zxAnBrgIfH1DIntervalValidData	TruthValue, zxAnBrgIfH1DDateTime			DateAndTime,

zxAnBrgIfH1DInOctets	Counter64,

zxAnBrgIfH1DInUcastPkts	Counter64,

zxAnBrgIfH1DInMcastPkts	Counter64,

zxAnBrgIfH1DInBcastPkts	Counter64,

zxAnBrgIfH1DOutOctets	Counter64,

zxAnBrgIfH1DOutUcastPkts	Counter64,

zxAnBrgIfH1DOutMcastPkts	Counter64,

zxAnBrgIfH1DOutBcastPkts	Counter64

}

    - Performance Threshold Alarm Profile

OID Specification

zxAnBrgIfAlmProfileTable's OID is .1.3.6.1.4.1.3902.1015.3.13.2.8. zxAnBrgIfAlmProfileConfTable's OID is .1.3.6.1.4.1.3902.1015.3.13.2.6.

Index Specification

{ ifIndex } , Type 4 or 10 platform composite index.

MIB Specification

Refer to ZTE-AN-INTERFACE-BRG-PERF-MIB.mib.

Confidential▼

zxAnBrgIfAlmProfileTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnBrgIfAlmProfileEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The performance alarm threshold profile table of the bridge interfaces. It can be used to delete all of the performance alarm

threshold profiles which has the same name configured in

‒zxOjLrcImOlgVramileIajmTable'."

::= { zxAnBrgIfPerfObjects 8 }

zxAnBrgIfAlmProfileEntry	OBJECT-TYPE SYNTAX	ZxAnBrgIfAlmProfileEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The performance alarm threshold profile entry of the bridge interfaces."

INDEX	{ zxAnBrgIfAlmProfileName}

::= { zxAnBrgIfAlmProfileTable 1 }

ZxAnBrgIfAlmProfileEntry ::= SEQUENCE { zxAnBrgIfAlmProfileRowStatus	RowStatus

}

zxAnBrgIfAlmProfileConfTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnBrgIfAlmProfileConfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The performance alarm threshold profile table of the bridge interfaces."

::= { zxAnBrgIfPerfObjects 6 }

zxAnBrgIfAlmProfileConfEntry	OBJECT-TYPE SYNTAX	ZxAnBrgIfAlmProfileConfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The performance alarm threshold profile table of the bridge interfaces.These variables in the profile configuration is optional, if not configured, it means the correspongding alarm is not reported, but the alarm and clear alarm threshold configuration is needed to bind. "

Confidential▼

INDEX	{ zxAnBrgIfAlmProfileName, zxAnBrgIfPerfVariable }

::= { zxAnBrgIfAlmProfileConfTable 1 }

ZxAnBrgIfAlmProfileConfEntry ::= SEQUENCE { zxAnBrgIfAlmProfileName		DisplayString, zxAnBrgIfPerfVariable	OBJECT IDENTIFIER,

zxAnBrgIfRiseAlmThresh		HCPerfCurrentCount, zxAnBrgIfClrRiseAlmThresh	HCPerfCurrentCount,

zxAnBrgIfRiseWarnThresh			 HCPerfCurrentCount, zxAnBrgIfClrRiseWarnThresh		 HCPerfCurrentCount, zxAnBrgIfFallWarnThresh		 HCPerfCurrentCount, zxAnBrgIfClrFallWarnThresh	 HCPerfCurrentCount, zxAnBrgIfFallAlmThresh		HCPerfCurrentCount, zxAnBrgIfClrFallAlmThresh	HCPerfCurrentCount, zxAnBrgIfThreshUsed			BITS, zxAnBrgIfAlmProfileConfRowStatus			RowStatus

}

    - Performance Alarm Profile Loading

OID Specification

zxAnBrgIfAlmProfileApplyTable's OID is .1.3.6.1.4.1.3902.1015.3.13.2.7.

Index Specification

{ ifIndex } , Type 4 or 10 platform composite index.

MIB Specification

Refer to ZTE-AN-INTERFACE-BRG-PERF-MIB.mib.

zxAnBrgIfAlmProfileApplyTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnBrgIfAlmProfileApplyEntry

MAX-ACCESS		not-accessible STATUS	current DESCRIPTION

"The performance alarm threshold profile configuration table of the bridge interfaces."

::= { zxAnBrgIfPerfObjects 7 }

zxAnBrgIfAlmProfileApplyEntry	OBJECT-TYPE SYNTAX	ZxAnBrgIfAlmProfileApplyEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The performance alarm threshold profile configuration entry of

Confidential▼

the bridge interfaces." INDEX	{ ifIndex }

::= { zxAnBrgIfAlmProfileApplyTable 1 }

ZxAnBrgIfAlmProfileApplyEntry ::= SEQUENCE { zxAnBrgIfAlmPrf	DisplayString,

zxAnBrgIfAlmPrfApplyRowStatus	RowStatus

}

  - OLT Ethernet Performance (V1.2.3)

The performance is started by default.

    - Real-time Performance

OID Specification

zxAnPonOltIfCurrPerfTable's OID is .1.3.6.1.4.1.3902.1015.1010.5.55.1.

Index Specification

{ zxAnXponOltIfIndex } , Type 1 PON composite index.

MIB Specification

Refer to zxAnXpon.mib.

zxAnPonOltIfCurrPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnPonOltIfCurrPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The current interface performance data table of XPON OLT."

::= { zxAnPonOltPerfMgmt 1 }

zxAnPonOltIfCurrPerfEntry OBJECT-TYPE SYNTAX	ZxAnPonOltIfCurrPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"An entry in the OLT current interface performance data table." INDEX	{ zxAnXponOltIfIndex }

::= { zxAnPonOltIfCurrPerfTable 1 }

ZxAnPonOltIfCurrPerfEntry ::=

Confidential▼

SEQUENCE {

zxAnPonOltIfDropEvents	Counter64,

zxAnPonOltIfRxOctets	Counter64,

zxAnPonOltIfTxOctets	Counter64,

zxAnPonOltIfRxPkts	Counter64,

zxAnPonOltIfTxPkts	Counter64,

zxAnPonOltIfRxBroadPkts	Counter64,

zxAnPonOltIfTxBroadPkts	Counter64,

zxAnPonOltIfRxMultiPkts	Counter64,

zxAnPonOltIfTxMultiPkts	Counter64,

zxAnPonOltIfRxCRCErrors	Counter64,

zxAnPonOltIfTxCRCErrors	Counter64,

zxAnPonOltIfRxUnderPkts	Counter64,

zxAnPonOltIfTxUnderPkts	Counter64,

zxAnPonOltIfRxOverPkts	Counter64,

zxAnPonOltIfTxOverPkts	Counter64,

zxAnPonOltIfRxFragments	Counter64,

zxAnPonOltIfTxFragments	Counter64,

zxAnPonOltIfRxJabbers	Counter64,

zxAnPonOltIfTxJabbers	Counter64,

zxAnPonOltIfCollisions	Counter64, zxAnPonOltIfRx64Octs		Counter64,

zxAnPonOltIfRx65To127Octs	Counter64,

zxAnPonOltIfRx128To255Octs	Counter64,

zxAnPonOltIfRx256To511Octs	Counter64,

zxAnPonOltIfRx512To1023Octs	Counter64,

zxAnPonOltIfRx1024To1518Octs	Counter64,

zxAnPonOltIfTx64Octs	Counter64,

zxAnPonOltIfTx65To127Octs	Counter64,

zxAnPonOltIfTx128To255Octs	Counter64,

zxAnPonOltIfTx256To511Octs	Counter64,

zxAnPonOltIfTx512To1023Octs	Counter64,

zxAnPonOltIfTx1024To1518Octs		Counter64, zxAnPonOltIfCurrPerfReset	INTEGER

}

    - 15-Minute Real-time Performance

OID Specification

zxAnPonOltIfCurr15MinPerfTable 's OID is .1.3.6.1.4.1.3902.1015.1010.5.55.2.

Index Specification

Confidential▼

{ zxAnXponOltIfIndex } , Type 1 PON composite index.

MIB Specification

Refer to zxAnXpon.mib.

zxAnPonOltIfCurr15MinPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnPonOltIfCurr15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The current 15 minutes performance data table of XPON OLT."

::= { zxAnPonOltPerfMgmt 2 }

zxAnPonOltIfCurr15MinPerfEntry OBJECT-TYPE SYNTAX	ZxAnPonOltIfCurr15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"An entry in the OLT current 15 minutes performance data table." INDEX	{ zxAnXponOltIfIndex }

::= { zxAnPonOltIfCurr15MinPerfTable 1 }

ZxAnPonOltIfCurr15MinPerfEntry ::= SEQUENCE {

zxAnPonOltIfC15MTimeElapsed	Gauge32,

zxAnPonOltIfC15MDropEvents	Counter64,

zxAnPonOltIfC15MRxOctets	Counter64,

zxAnPonOltIfC15MTxOctets	Counter64,

zxAnPonOltIfC15MRxPkts	Counter64,

zxAnPonOltIfC15MTxPkts	Counter64,

zxAnPonOltIfC15MRxBroadPkts	Counter64,

zxAnPonOltIfC15MTxBroadPkts	Counter64,

zxAnPonOltIfC15MRxMultiPkts	Counter64,

zxAnPonOltIfC15MTxMultiPkts	Counter64,

zxAnPonOltIfC15MRxCRCErrors	Counter64,

zxAnPonOltIfC15MTxCRCErrors	Counter64,

zxAnPonOltIfC15MRxUnderPkts	Counter64,

zxAnPonOltIfC15MTxUnderPkts	Counter64,

zxAnPonOltIfC15MRxOverPkts	Counter64,

zxAnPonOltIfC15MTxOverPkts	Counter64,

zxAnPonOltIfC15MRxFragments	Counter64,

zxAnPonOltIfC15MTxFragments	Counter64,

zxAnPonOltIfC15MRxJabbers	Counter64,

zxAnPonOltIfC15MTxJabbers	Counter64,

Confidential▼

zxAnPonOltIfC15MCollisions	Counter64, zxAnPonOltIfC15MRx64Octs		Counter64,

zxAnPonOltIfC15MRx65To127Octs	Counter64,

zxAnPonOltIfC15MRx128To255Octs	Counter64,

zxAnPonOltIfC15MRx256To511Octs	Counter64,

zxAnPonOltIfC15MRx512To1023Octs	Counter64,

zxAnPonOltIfC15MRx1024To1518Octs	Counter64,

zxAnPonOltIfC15MTx64Octs	Counter64,

zxAnPonOltIfC15MTx65To127Octs	Counter64,

zxAnPonOltIfC15MTx128To255Octs	Counter64,

zxAnPonOltIfC15MTx256To511Octs	Counter64,

zxAnPonOltIfC15MTx512To1023Octs	Counter64,

zxAnPonOltIfC15MTx1024To1518Octs	Counter64,

zxAnPonOltIfC15MPerfReset	INTEGER

}

    - 24-Minute Real-time Performance

OID Specification

zxAnPonOltIfCurr1DayPerfTable 's OID is .1.3.6.1.4.1.3902.1015.1010.5.55.3.

Index Specification

{ zxAnXponOltIfIndex } , Type 1 PON composite index.

MIB Specification

Refer to zxAnXpon.mib.

zxAnPonOltIfCurr1DayPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnPonOltIfCurr1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The current 1 day performance data table of XPON OLT."

::= { zxAnPonOltPerfMgmt 3 }

zxAnPonOltIfCurr1DayPerfEntry OBJECT-TYPE SYNTAX	ZxAnPonOltIfCurr1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"An entry in the OLT current 1 day performance data table." INDEX	{ zxAnXponOltIfIndex }

::= { zxAnPonOltIfCurr1DayPerfTable 1 }

Confidential▼

ZxAnPonOltIfCurr1DayPerfEntry ::= SEQUENCE {

zxAnPonOltIfC1DTimeElapsed	Gauge32,

zxAnPonOltIfC1DDropEvents	Counter64,

zxAnPonOltIfC1DRxOctets	Counter64,

zxAnPonOltIfC1DTxOctets	Counter64,

zxAnPonOltIfC1DRxPkts	Counter64,

zxAnPonOltIfC1DTxPkts	Counter64,

zxAnPonOltIfC1DRxBroadPkts	Counter64,

zxAnPonOltIfC1DTxBroadPkts	Counter64,

zxAnPonOltIfC1DRxMultiPkts	Counter64,

zxAnPonOltIfC1DTxMultiPkts	Counter64,

zxAnPonOltIfC1DRxCRCErrors	Counter64,

zxAnPonOltIfC1DTxCRCErrors	Counter64,

zxAnPonOltIfC1DRxUnderPkts	Counter64,

zxAnPonOltIfC1DTxUnderPkts	Counter64,

zxAnPonOltIfC1DRxOverPkts	Counter64,

zxAnPonOltIfC1DTxOverPkts	Counter64,

zxAnPonOltIfC1DRxFragments	Counter64,

zxAnPonOltIfC1DTxFragments	Counter64,

zxAnPonOltIfC1DRxJabbers	Counter64,

zxAnPonOltIfC1DTxJabbers	Counter64,

zxAnPonOltIfC1DCollisions	Counter64, zxAnPonOltIfC1DRx64Octs		Counter64,

zxAnPonOltIfC1DRx65To127Octs	Counter64,

zxAnPonOltIfC1DRx128To255Octs	Counter64,

zxAnPonOltIfC1DRx256To511Octs	Counter64,

zxAnPonOltIfC1DRx512To1023Octs	Counter64,

zxAnPonOltIfC1DRx1024To1518Octs	Counter64,

zxAnPonOltIfC1DTx64Octs	Counter64,

zxAnPonOltIfC1DTx65To127Octs	Counter64,

zxAnPonOltIfC1DTx128To255Octs	Counter64,

zxAnPonOltIfC1DTx256To511Octs	Counter64,

zxAnPonOltIfC1DTx512To1023Octs	Counter64,

zxAnPonOltIfC1DTx1024To1518Octs	Counter64,

zxAnPonOltIfC1DPerfReset	INTEGER

}

    - 15-Minute Historical Performance

OID Specification

zxAnPonOltIfHis15MinPerfTable 's OID is .1.3.6.1.4.1.3902.1015.1010.5.55.4.

Confidential▼

Index Specification

{ zxAnXponOltIfIndex } , Type 1 PON composite index.

MIB Specification

Refer to zxAnXpon.mib.

zxAnPonOltIfHis15MinPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnPonOltIfHis15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 15 minutes interval performance data table of XPON OLT."

::= { zxAnPonOltPerfMgmt 4 }

zxAnPonOltIfHis15MinPerfEntry OBJECT-TYPE SYNTAX	ZxAnPonOltIfHis15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"An entry in the OLT 15 minutes interval performance data table." INDEX	{ zxAnXponOltIfIndex,zxAnPonOltIfH15MIntervalNumber }

::= { zxAnPonOltIfHis15MinPerfTable 1 }

ZxAnPonOltIfHis15MinPerfEntry ::= SEQUENCE {

zxAnPonOltIfH15MIntervalNumber	Integer32,

zxAnPonOltIfH15MDateAndTime	DateAndTime,

zxAnPonOltIfH15MDropEvents	Counter64,

zxAnPonOltIfH15MRxOctets	Counter64,

zxAnPonOltIfH15MTxOctets	Counter64,

zxAnPonOltIfH15MRxPkts	Counter64,

zxAnPonOltIfH15MTxPkts	Counter64,

zxAnPonOltIfH15MRxBroadPkts	Counter64,

zxAnPonOltIfH15MTxBroadPkts	Counter64,

zxAnPonOltIfH15MRxMultiPkts	Counter64,

zxAnPonOltIfH15MTxMultiPkts	Counter64,

zxAnPonOltIfH15MRxCRCErrors	Counter64,

zxAnPonOltIfH15MTxCRCErrors	Counter64,

zxAnPonOltIfH15MRxUnderPkts	Counter64,

zxAnPonOltIfH15MTxUnderPkts	Counter64,

zxAnPonOltIfH15MRxOverPkts	Counter64,

zxAnPonOltIfH15MTxOverPkts	Counter64,

zxAnPonOltIfH15MRxFragments	Counter64,

Confidential▼

zxAnPonOltIfH15MTxFragments	Counter64,

zxAnPonOltIfH15MRxJabbers	Counter64,

zxAnPonOltIfH15MTxJabbers	Counter64,

zxAnPonOltIfH15MCollisions	Counter64, zxAnPonOltIfH15MRx64Octs		Counter64,

zxAnPonOltIfH15MRx65To127Octs	Counter64,

zxAnPonOltIfH15MRx128To255Octs	Counter64,

zxAnPonOltIfH15MRx256To511Octs	Counter64,

zxAnPonOltIfH15MRx512To1023Octs	Counter64,

zxAnPonOltIfH15MRx1024To1518Octs	Counter64,

zxAnPonOltIfH15MTx64Octs	Counter64,

zxAnPonOltIfH15MTx65To127Octs	Counter64,

zxAnPonOltIfH15MTx128To255Octs	Counter64,

zxAnPonOltIfH15MTx256To511Octs	Counter64,

zxAnPonOltIfH15MTx512To1023Octs	Counter64,

zxAnPonOltIfH15MTx1024To1518Octs		Counter64, zxAnPonOltIfH15MValidData	TruthValue

}

    - 24-Hour Historical Performance

OID Specification

zxAnPonOltIfHis1DayPerfTable 's OID is .1.3.6.1.4.1.3902.1015.1010.5.55.5.

Index Specification

{ zxAnXponOltIfIndex } , Type 1 PON composite index.

MIB Specification

Refer to zxAnXpon.mib.

zxAnPonOltIfHis1DayPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnPonOltIfHis1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 1 day interval performance data table of XPON OLT."

::= { zxAnPonOltPerfMgmt 5 }

zxAnPonOltIfHis1DayPerfEntry OBJECT-TYPE SYNTAX	ZxAnPonOltIfHis1DayPerfEntry

MAX-ACCESS		not-accessible STATUS	current

DESCRIPTION

Confidential▼

"The 1 day interval performance data entry of interfaces."

INDEX	{ zxAnXponOltIfIndex,zxAnPonOltIfH1DIntervalNumber }

::= { zxAnPonOltIfHis1DayPerfTable 1 }

ZxAnPonOltIfHis1DayPerfEntry ::= SEQUENCE {

zxAnPonOltIfH1DIntervalNumber	Integer32, zxAnPonOltIfH1DDateAndTime		DateAndTime,

zxAnPonOltIfH1DDropEvents	Counter64,

zxAnPonOltIfH1DRxOctets	Counter64,

zxAnPonOltIfH1DTxOctets	Counter64,

zxAnPonOltIfH1DRxPkts	Counter64,

zxAnPonOltIfH1DTxPkts	Counter64,

zxAnPonOltIfH1DRxBroadPkts	Counter64,

zxAnPonOltIfH1DTxBroadPkts	Counter64,

zxAnPonOltIfH1DRxMultiPkts	Counter64,

zxAnPonOltIfH1DTxMultiPkts	Counter64,

zxAnPonOltIfH1DRxCRCErrors	Counter64,

zxAnPonOltIfH1DTxCRCErrors	Counter64,

zxAnPonOltIfH1DRxUnderPkts	Counter64,

zxAnPonOltIfH1DTxUnderPkts	Counter64,

zxAnPonOltIfH1DRxOverPkts	Counter64,

zxAnPonOltIfH1DTxOverPkts	Counter64,

zxAnPonOltIfH1DRxFragments	Counter64,

zxAnPonOltIfH1DTxFragments	Counter64,

zxAnPonOltIfH1DRxJabbers	Counter64,

zxAnPonOltIfH1DTxJabbers	Counter64,

zxAnPonOltIfH1DCollisions	Counter64, zxAnPonOltIfH1DRx64Octs		Counter64,

zxAnPonOltIfH1DRx65To127Octs	Counter64,

zxAnPonOltIfH1DRx128To255Octs	Counter64,

zxAnPonOltIfH1DRx256To511Octs	Counter64,

zxAnPonOltIfH1DRx512To1023Octs	Counter64,

zxAnPonOltIfH1DRx1024To1518Octs	Counter64,

zxAnPonOltIfH1DTx64Octs	Counter64,

zxAnPonOltIfH1DTx65To127Octs	Counter64,

zxAnPonOltIfH1DTx128To255Octs	Counter64,

zxAnPonOltIfH1DTx256To511Octs	Counter64,

zxAnPonOltIfH1DTx512To1023Octs	Counter64,

zxAnPonOltIfH1DTx1024To1518Octs		Counter64, zxAnPonOltIfH1DValidData	TruthValue

}

Confidential▼

  - ONU Ethernet Performance (V1.2.3)

The performance is started by default.

    - Real-time Performance

OID Specification

zxAnPonOnuIfCurrPerfTable's OID is .1.3.6.1.4.1.3902.1015.1010.5.56.1.

Index Specification

{ zxAnXponOnuIfIndex },Type 3 or 9 PON composite index.

MIB Specification

Refer to zxAnXpon.mib.

zxAnPonOnuIfCurrPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnPonOnuIfCurrPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The current interface performance data table of XPON ONU."

::= { zxAnPonOnuPerfMgmt 1 }

zxAnPonOnuIfCurrPerfEntry OBJECT-TYPE SYNTAX	ZxAnPonOnuIfCurrPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"An entry in the ONU current interface performance data table." INDEX	{ zxAnXponOnuIfIndex }

::= { zxAnPonOnuIfCurrPerfTable 1 }

ZxAnPonOnuIfCurrPerfEntry ::= SEQUENCE {

zxAnPonOnuIfDropEvents	Counter64,

zxAnPonOnuIfRxOctets	Counter64,

zxAnPonOnuIfTxOctets	Counter64,

zxAnPonOnuIfRxPkts	Counter64,

zxAnPonOnuIfTxPkts	Counter64,

zxAnPonOnuIfRxBroadPkts	Counter64,

zxAnPonOnuIfTxBroadPkts	Counter64,

zxAnPonOnuIfRxMultiPkts	Counter64,

zxAnPonOnuIfTxMultiPkts	Counter64,

Confidential▼

zxAnPonOnuIfRxCRCErrors	Counter64,

zxAnPonOnuIfTxCRCErrors	Counter64,

zxAnPonOnuIfRxUnderPkts	Counter64,

zxAnPonOnuIfTxUnderPkts	Counter64,

zxAnPonOnuIfRxOverPkts	Counter64,

zxAnPonOnuIfTxOverPkts	Counter64,

zxAnPonOnuIfRxFragments	Counter64,

zxAnPonOnuIfTxFragments	Counter64,

zxAnPonOnuIfRxJabbers	Counter64,

zxAnPonOnuIfTxJabbers	Counter64,

zxAnPonOnuIfCollisions	Counter64, zxAnPonOnuIfRx64Octs		Counter64,

zxAnPonOnuIfRx65To127Octs	Counter64,

zxAnPonOnuIfRx128To255Octs	Counter64,

zxAnPonOnuIfRx256To511Octs	Counter64,

zxAnPonOnuIfRx512To1023Octs	Counter64,

zxAnPonOnuIfRx1024To1518Octs	Counter64,

zxAnPonOnuIfTx64Octs	Counter64,

zxAnPonOnuIfTx65To127Octs	Counter64,

zxAnPonOnuIfTx128To255Octs	Counter64,

zxAnPonOnuIfTx256To511Octs	Counter64,

zxAnPonOnuIfTx512To1023Octs	Counter64,

zxAnPonOnuIfTx1024To1518Octs	Counter64,

zxAnPonOnuIfCurrPerfReset	INTEGER

}

    - 15-Minute Real-time Performance

OID Specification

zxAnPonOnuIfCurr15MinPerfTable 's OID is .1.3.6.1.4.1.3902.1015.1010.5.56.2.

Index Specification

{ zxAnXponOnuIfIndex },Type 3 or 9 PON composite index.

MIB Specification

Refer to zxAnXpon.mib.

zxAnPonOnuIfCurr15MinPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnPonOnuIfCurr15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current

DESCRIPTION

"The current 15 minutes performance data table of XPON ONU."

::= { zxAnPonOnuPerfMgmt 2 }

Confidential▼

zxAnPonOnuIfCurr15MinPerfEntry OBJECT-TYPE SYNTAX	ZxAnPonOnuIfCurr15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"An entry in the ONU current 15 minutes performance data table." INDEX	{ zxAnXponOnuIfIndex }

::= { zxAnPonOnuIfCurr15MinPerfTable 1 }

ZxAnPonOnuIfCurr15MinPerfEntry ::= SEQUENCE {

zxAnPonOnuIfC15MTimeElapsed	Gauge32,

zxAnPonOnuIfC15MDropEvents	Counter64,

zxAnPonOnuIfC15MRxOctets	Counter64,

zxAnPonOnuIfC15MTxOctets	Counter64,

zxAnPonOnuIfC15MRxPkts	Counter64,

zxAnPonOnuIfC15MTxPkts	Counter64,

zxAnPonOnuIfC15MRxBroadPkts	Counter64,

zxAnPonOnuIfC15MTxBroadPkts	Counter64,

zxAnPonOnuIfC15MRxMultiPkts	Counter64,

zxAnPonOnuIfC15MTxMultiPkts	Counter64,

zxAnPonOnuIfC15MRxCRCErrors	Counter64,

zxAnPonOnuIfC15MTxCRCErrors	Counter64,

zxAnPonOnuIfC15MRxUnderPkts	Counter64,

zxAnPonOnuIfC15MTxUnderPkts	Counter64,

zxAnPonOnuIfC15MRxOverPkts	Counter64,

zxAnPonOnuIfC15MTxOverPkts	Counter64,

zxAnPonOnuIfC15MRxFragments	Counter64,

zxAnPonOnuIfC15MTxFragments	Counter64,

zxAnPonOnuIfC15MRxJabbers	Counter64,

zxAnPonOnuIfC15MTxJabbers	Counter64,

zxAnPonOnuIfC15MCollisions	Counter64, zxAnPonOnuIfC15MRx64Octs		Counter64,

zxAnPonOnuIfC15MRx65To127Octs	Counter64,

zxAnPonOnuIfC15MRx128To255Octs	Counter64,

zxAnPonOnuIfC15MRx256To511Octs	Counter64,

zxAnPonOnuIfC15MRx512To1023Octs	Counter64,

zxAnPonOnuIfC15MRx1024To1518Octs	Counter64,

zxAnPonOnuIfC15MTx64Octs	Counter64,

zxAnPonOnuIfC15MTx65To127Octs	Counter64,

zxAnPonOnuIfC15MTx128To255Octs	Counter64,

zxAnPonOnuIfC15MTx256To511Octs	Counter64,

zxAnPonOnuIfC15MTx512To1023Octs	Counter64,

Confidential▼

zxAnPonOnuIfC15MTx1024To1518Octs		Counter64, zxAnPonOnuIfC15MPerfReset	INTEGER

}

    - 24-Minute Real-time Performance

OID Specification

.zxAnPonOnuIfCurr1DayPerfTable 's OID is .1.3.6.1.4.1.3902.1015.1010.5.56.3.

Index Specification

{ zxAnXponOnuIfIndex },Type 3 or 9 PON composite index.

MIB Specification

Refer to zxAnXpon.mib.

zxAnPonOnuIfCurr1DayPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnPonOnuIfCurr1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The current 1 day performance data table of XPON ONU."

::= { zxAnPonOnuPerfMgmt 3 }

zxAnPonOnuIfCurr1DayPerfEntry OBJECT-TYPE SYNTAX	ZxAnPonOnuIfCurr1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"An entry in the ONU current 1 day performance data table." INDEX	{ zxAnXponOnuIfIndex }

::= { zxAnPonOnuIfCurr1DayPerfTable 1 }

ZxAnPonOnuIfCurr1DayPerfEntry ::= SEQUENCE {

zxAnPonOnuIfC1DTimeElapsed	Gauge32,

zxAnPonOnuIfC1DDropEvents	Counter64,

zxAnPonOnuIfC1DRxOctets	Counter64,

zxAnPonOnuIfC1DTxOctets	Counter64,

zxAnPonOnuIfC1DRxPkts	Counter64,

zxAnPonOnuIfC1DTxPkts	Counter64,

zxAnPonOnuIfC1DRxBroadPkts	Counter64,

zxAnPonOnuIfC1DTxBroadPkts	Counter64,

zxAnPonOnuIfC1DRxMultiPkts	Counter64,

Confidential▼

zxAnPonOnuIfC1DTxMultiPkts	Counter64,

zxAnPonOnuIfC1DRxCRCErrors	Counter64,

zxAnPonOnuIfC1DTxCRCErrors	Counter64,

zxAnPonOnuIfC1DRxUnderPkts	Counter64,

zxAnPonOnuIfC1DTxUnderPkts	Counter64,

zxAnPonOnuIfC1DRxOverPkts	Counter64,

zxAnPonOnuIfC1DTxOverPkts	Counter64,

zxAnPonOnuIfC1DRxFragments	Counter64,

zxAnPonOnuIfC1DTxFragments	Counter64,

zxAnPonOnuIfC1DRxJabbers	Counter64,

zxAnPonOnuIfC1DTxJabbers	Counter64,

zxAnPonOnuIfC1DCollisions	Counter64, zxAnPonOnuIfC1DRx64Octs		Counter64,

zxAnPonOnuIfC1DRx65To127Octs	Counter64,

zxAnPonOnuIfC1DRx128To255Octs	Counter64,

zxAnPonOnuIfC1DRx256To511Octs	Counter64,

zxAnPonOnuIfC1DRx512To1023Octs	Counter64,

zxAnPonOnuIfC1DRx1024To1518Octs	Counter64,

zxAnPonOnuIfC1DTx64Octs	Counter64,

zxAnPonOnuIfC1DTx65To127Octs	Counter64,

zxAnPonOnuIfC1DTx128To255Octs	Counter64,

zxAnPonOnuIfC1DTx256To511Octs	Counter64,

zxAnPonOnuIfC1DTx512To1023Octs	Counter64,

zxAnPonOnuIfC1DTx1024To1518Octs	Counter64,

zxAnPonOnuIfC1DPerfReset	INTEGER

}

    - 15-Minute Historical Performance

OID Specification

zxAnPonOnuIfHis15MinPerfTable 's OID is .1.3.6.1.4.1.3902.1015.1010.5.56.4.

Index Specification

{ zxAnXponOnuIfIndex },Type 3 or 9 PON composite index.

MIB Specification

Refer to zxAnXpon.mib.

zxAnPonOnuIfHis15MinPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnPonOnuIfHis15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 15 minutes interval performance data table of XPON ONU."

Confidential▼

::= { zxAnPonOnuPerfMgmt 4 }

zxAnPonOnuIfHis15MinPerfEntry OBJECT-TYPE SYNTAX	ZxAnPonOnuIfHis15MinPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"An entry in the ONU 15 minutes interval performance data table."

INDEX	{ zxAnXponOnuIfIndex, zxAnPonOnuIfH15MIntervalNumber}

::= { zxAnPonOnuIfHis15MinPerfTable 1 }

ZxAnPonOnuIfHis15MinPerfEntry ::= SEQUENCE {

zxAnPonOnuIfH15MIntervalNumber	Integer32, zxAnPonOnuIfH15MDateAndTime		DateAndTime,

zxAnPonOnuIfH15MDropEvents	Counter64,

zxAnPonOnuIfH15MRxOctets	Counter64,

zxAnPonOnuIfH15MTxOctets	Counter64,

zxAnPonOnuIfH15MRxPkts	Counter64,

zxAnPonOnuIfH15MTxPkts	Counter64,

zxAnPonOnuIfH15MRxBroadPkts	Counter64,

zxAnPonOnuIfH15MTxBroadPkts	Counter64,

zxAnPonOnuIfH15MRxMultiPkts	Counter64,

zxAnPonOnuIfH15MTxMultiPkts	Counter64,

zxAnPonOnuIfH15MRxCRCErrors	Counter64,

zxAnPonOnuIfH15MTxCRCErrors	Counter64,

zxAnPonOnuIfH15MRxUnderPkts	Counter64,

zxAnPonOnuIfH15MTxUnderPkts	Counter64,

zxAnPonOnuIfH15MRxOverPkts	Counter64,

zxAnPonOnuIfH15MTxOverPkts	Counter64,

zxAnPonOnuIfH15MRxFragments	Counter64,

zxAnPonOnuIfH15MTxFragments	Counter64,

zxAnPonOnuIfH15MRxJabbers	Counter64,

zxAnPonOnuIfH15MTxJabbers	Counter64,

zxAnPonOnuIfH15MCollisions	Counter64, zxAnPonOnuIfH15MRx64Octs		Counter64,

zxAnPonOnuIfH15MRx65To127Octs	Counter64,

zxAnPonOnuIfH15MRx128To255Octs	Counter64,

zxAnPonOnuIfH15MRx256To511Octs	Counter64,

zxAnPonOnuIfH15MRx512To1023Octs	Counter64,

zxAnPonOnuIfH15MRx1024To1518Octs	Counter64,

zxAnPonOnuIfH15MTx64Octs	Counter64,

zxAnPonOnuIfH15MTx65To127Octs	Counter64,

zxAnPonOnuIfH15MTx128To255Octs	Counter64,

Confidential▼

zxAnPonOnuIfH15MTx256To511Octs	Counter64,

zxAnPonOnuIfH15MTx512To1023Octs	Counter64,

zxAnPonOnuIfH15MTx1024To1518Octs		Counter64, zxAnPonOnuIfH15MValidData	TruthValue}

    - 24-Hour Historical Performance

OID Specification

zxAnPonOnuIfHis1DayPerfTable's OID is .1.3.6.1.4.1.3902.1015.1010.5.56.5.

Index Specification

{ zxAnXponOnuIfIndex },Type 3 or 9 PON composite index.

MIB Specification

Refer to zxAnXpon.mib.

zxAnPonOnuIfHis1DayPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxAnPonOnuIfHis1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 1 day interval performance data table of XPON ONU."

::= { zxAnPonOnuPerfMgmt 5 }

zxAnPonOnuIfHis1DayPerfEntry OBJECT-TYPE SYNTAX	ZxAnPonOnuIfHis1DayPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 1 day interval performance data entry of interfaces."

INDEX	{ zxAnXponOnuIfIndex,zxAnPonOnuIfH1DIntervalNumber }

::= { zxAnPonOnuIfHis1DayPerfTable 1 }

ZxAnPonOnuIfHis1DayPerfEntry ::= SEQUENCE {

zxAnPonOnuIfH1DIntervalNumber	Integer32, zxAnPonOnuIfH1DDateAndTime		DateAndTime,

zxAnPonOnuIfH1DDropEvents	Counter64,

zxAnPonOnuIfH1DRxOctets	Counter64,

zxAnPonOnuIfH1DTxOctets	Counter64,

zxAnPonOnuIfH1DRxPkts	Counter64,

zxAnPonOnuIfH1DTxPkts	Counter64,

zxAnPonOnuIfH1DRxBroadPkts	Counter64,

zxAnPonOnuIfH1DTxBroadPkts	Counter64,

Confidential▼

zxAnPonOnuIfH1DRxMultiPkts	Counter64,

zxAnPonOnuIfH1DTxMultiPkts	Counter64,

zxAnPonOnuIfH1DRxCRCErrors	Counter64,

zxAnPonOnuIfH1DTxCRCErrors	Counter64,

zxAnPonOnuIfH1DRxUnderPkts	Counter64,

zxAnPonOnuIfH1DTxUnderPkts	Counter64,

zxAnPonOnuIfH1DRxOverPkts	Counter64,

zxAnPonOnuIfH1DTxOverPkts	Counter64,

zxAnPonOnuIfH1DRxFragments	Counter64,

zxAnPonOnuIfH1DTxFragments	Counter64,

zxAnPonOnuIfH1DRxJabbers	Counter64,

zxAnPonOnuIfH1DTxJabbers	Counter64,

zxAnPonOnuIfH1DCollisions	Counter64, zxAnPonOnuIfH1DRx64Octs		Counter64,

zxAnPonOnuIfH1DRx65To127Octs	Counter64,

zxAnPonOnuIfH1DRx128To255Octs	Counter64,

zxAnPonOnuIfH1DRx256To511Octs	Counter64,

zxAnPonOnuIfH1DRx512To1023Octs	Counter64,

zxAnPonOnuIfH1DRx1024To1518Octs	Counter64,

zxAnPonOnuIfH1DTx64Octs	Counter64,

zxAnPonOnuIfH1DTx65To127Octs	Counter64,

zxAnPonOnuIfH1DTx128To255Octs	Counter64,

zxAnPonOnuIfH1DTx256To511Octs	Counter64,

zxAnPonOnuIfH1DTx512To1023Octs	Counter64,

zxAnPonOnuIfH1DTx1024To1518Octs		Counter64, zxAnPonOnuIfH1DValidData	TruthValue

}

  - GPON OLT PON-Layer Performance (V1.2.3)

    - Real-time Performance

OID Specification

zxGponOltCurrPerfTable’s OID is .1.3.6.1.4.1.3902.1012.3.31.1.

Index Specification

zxGponMgmtPonOltId , Type 1 PON composite index.

MIB Specification

Refer to zxGponService.mib.

zxGponOltCurrPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponOltCurrPerfEntry

Confidential▼

MAX-ACCESS		not-accessible STATUS	current DESCRIPTION

"The current performance data table of GPON OLT."

::= { zxAnGponOltPerfMgmt 1 }

zxGponOltCurrPerfEntry OBJECT-TYPE SYNTAX	ZxGponOltCurrPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"An entry in the OLT current performance data table." INDEX	{ zxGponMgmtPonOltId }

::= { zxGponOltCurrPerfTable 1 }

ZxGponOltCurrPerfEntry ::= SEQUENCE {

zxGponOltCorrNonIdleGemFrmUp						 Counter64, zxGponOltCorrIdleGemFrmUp		Counter64, zxGponOltErroredGemFrmUp						Counter64, zxGponOltGemPayloadBytesUp					Counter64, zxGponOltCorrEtherFrmUp	Counter64, zxGponOltErroredEtherFrmUp				Counter64, zxGponOltPerfErr			Counter64,

zxGponOltPerfRei	Counter64, zxGponOltValidEtherPktDown		Counter64, zxGponOltPloamDown			Counter64,

zxGponOltErroredPloamUp	Counter64,

zxGponOltPloamUp							Counter64, zxGponOltFecCorrBytes		Counter64, zxGponOltFecCorrWords			Counter64, zxGponOltFecUnCorrWords				Counter64, zxGponOltFecTotalRxWords						Counter64, zxGponOltBipCorrBits	Counter64, zxGponOltCrcErrPkts					Counter64,

zxGponOltCurrReset	INTEGER

}

    - 15-Minute Real-time Performance

OID Specification

zxGponOltCurrPerf15MinTable ’s OID is .1.3.6.1.4.1.3902.1012.3.31.2.

Confidential▼

Index Specification

zxGponMgmtPonOltId , Type 1 PON composite index.

MIB Specification

Refer to zxGponService.mib.

zxGponOltCurrPerf15MinTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponOltCurrPerf15MinEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The current 15 minutes performance data table of GPON OLT."

::= { zxAnGponOltPerfMgmt 2 }

zxGponOltCurrPerf15MinEntry OBJECT-TYPE SYNTAX	ZxGponOltCurrPerf15MinEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"An entry in the OLT current 15 minutes performance data table." INDEX	{ zxGponMgmtPonOltId }

::= { zxGponOltCurrPerf15MinTable 1 }

ZxGponOltCurrPerf15MinEntry ::= SEQUENCE {

zxGponOltC15MTimeElapsed				Gauge32, zxGponOltC15MCorrNonIdleGemFrmUp						 Counter64, zxGponOltC15MCorrIdleGemFrmUp		Counter64, zxGponOltC15MErroredGemFrmUp						Counter64, zxGponOltC15MGemPayloadBytesUp					Counter64, zxGponOltC15MCorrEtherFrmUp	Counter64, zxGponOltC15MErroredEtherFrmUp			Counter64, zxGponOltC15MErr				 Counter64,

zxGponOltC15MRei							 Counter64, zxGponOltC15MValidEtherPktDown						Counter64, zxGponOltC15MPloamDown									Counter64, zxGponOltC15MErroredPloamUp							Counter64, zxGponOltC15MPloamUp								Counter64, zxGponOltC15MFecCorrBytes		Counter64, zxGponOltC15MFecCorrWords			Counter64, zxGponOltC15MFecUnCorrWords				Counter64, zxGponOltC15MFecTotalRxWords							 Counter64, zxGponOltC15MBipCorrBits	Counter64, zxGponOltC15MCrcErrPkts					Counter64,

Confidential▼

zxGponOltC15MReset	INTEGER

}

    - 24-Hour Real-time Performance

OID Specification

zxGponOltCurrPerf1DayTable ’s OID is .1.3.6.1.4.1.3902.1012.3.31.3.

Index Specification

zxGponMgmtPonOltId , Type 1 PON composite index.

MIB Specification

Refer to zxGponService.mib.

zxGponOltCurrPerf1DayTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponOltCurrPerf1DayEntry

MAX-ACCESS		not-accessible STATUS	current

DESCRIPTION

"The current 1 day performance data table of GPON OLT."

::= { zxAnGponOltPerfMgmt 3 }

zxGponOltCurrPerf1DayEntry OBJECT-TYPE SYNTAX	ZxGponOltCurrPerf1DayEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"An entry in the OLT current 1 day performance data table."

INDEX	{ zxGponMgmtPonOltId }

::= { zxGponOltCurrPerf1DayTable 1 }

ZxGponOltCurrPerf1DayEntry ::= SEQUENCE {

zxGponOltC1DTimeElapsed				Gauge32, zxGponOltC1DCorrNonIdleGemFrmUp						 Counter64, zxGponOltC1DCorrIdleGemFrmUp		Counter64, zxGponOltC1DErroredGemFrmUp						Counter64, zxGponOltC1DGemPayloadBytesUp					Counter64, zxGponOltC1DCorrEtherFrmUp	Counter64, zxGponOltC1DErroredEtherFrmUp			Counter64,

zxGponOltC1DErr	Counter64,

zxGponOltC1DValidEtherPktDown	Counter64,

zxGponOltC1DRei	Counter64,

Confidential▼

zxGponOltC1DPloamDown								Counter64, zxGponOltC1DErroredPloamUp						Counter64, zxGponOltC1DPloamUp							Counter64, zxGponOltC1DFecCorrBytes		Counter64, zxGponOltC1DFecCorrWords			Counter64, zxGponOltC1DFecUnCorrWords				Counter64, zxGponOltC1DFecTotalRxWords						 Counter64, zxGponOltC1DBipCorrBits	Counter64, zxGponOltC1DCrcErrPkts					Counter64,

zxGponOltC1DReset	INTEGER

}

    - 15-Minute Historical Performance

OID Specification

zxGponOltPerf15MinIntervalTable ’s OID is .1.3.6.1.4.1.3902.1012.3.31.4.

Index Specification

zxGponMgmtPonOltId , Type 1 PON composite index.

MIB Specification

Refer to zxGponService.mib.

zxGponOltPerf15MinIntervalTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponOltPerf15MinIntervalEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 15 minutes interval performance data table of GPON OLT."

::= { zxAnGponOltPerfMgmt 4 }

zxGponOltPerf15MinIntervalEntry OBJECT-TYPE SYNTAX	ZxGponOltPerf15MinIntervalEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"An entry in the OLT 15 minutes interval performance data table." INDEX	{ zxGponMgmtPonOltId, zxGponOltH15MIntervalNumber	}

::= { zxGponOltPerf15MinIntervalTable 1 }

zxGponOltH15MIntervalNumber	Integer32,

ZxGponOltPerf15MinIntervalEntry ::= SEQUENCE {

Confidential▼

zxGponOltH15MDateAndTime					 DateAndTime, zxGponOltH15MCorrNonIdleGemFrmUp						 Counter64, zxGponOltH15MCorrIdleGemFrmUp		Counter64, zxGponOltH15MErroredGemFrmUp						Counter64, zxGponOltH15MGemPayloadBytesUp					Counter64, zxGponOltH15MCorrEtherFrmUp	Counter64, zxGponOltH15MErroredEtherFrmUp			Counter64, zxGponOltH15MErr				Counter64,

zxGponOltH15MRei						 Counter64, zxGponOltH15MValidEtherPktDown					Counter64, zxGponOltH15MPloamDown								Counter64, zxGponOltH15MErroredPloamUp						Counter64, zxGponOltH15MPloamUp							 Counter64, zxGponOltH15MFecCorrBytes		Counter64, zxGponOltH15MFecCorrWords			Counter64, zxGponOltH15MFecUnCorrWords							Counter64, zxGponOltH15MFecTotalRxWords						 Counter64, zxGponOltH15MBipCorrBits	Counter64, zxGponOltH15MCrcErrPkts				Counter64,

zxGponOltH15MValidData	TruthValue

}

    - 24-Hour Historical Performance

OID Specification

zxGponOltPerf1DayIntervalTable ’s OID is .1.3.6.1.4.1.3902.1012.3.31.5.

Index Specification

zxGponMgmtPonOltId , Type 1 PON composite index.

MIB Specification

Refer to zxGponService.mib.

zxGponOltPerf1DayIntervalTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponOltPerf1DayIntervalEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 1 day interval performance data table of interfaces."

::= { zxAnGponOltPerfMgmt 5 }

SYNTAX	ZxGponOltPerf1DayIntervalEntry

zxGponOltPerf1DayIntervalEntry	OBJECT-TYPE

Confidential▼

MAX-ACCESS		not-accessible STATUS	current DESCRIPTION

"The 1 day interval performance data entry of interfaces."

INDEX	{ zxGponMgmtPonOltId, zxGponOltH1DIntervalNumber }

::= { zxGponOltPerf1DayIntervalTable 1 }

ZxGponOltPerf1DayIntervalEntry ::= SEQUENCE {

zxGponOltH1DIntervalNumber	 Integer32, zxGponOltH1DDateAndTime			DateAndTime, zxGponOltH1DCorrNonIdleGemFrmUp						 Counter64, zxGponOltH1DCorrIdleGemFrmUp		Counter64, zxGponOltH1DErroredGemFrmUp						Counter64, zxGponOltH1DGemPayloadBytesUp					Counter64, zxGponOltH1DCorrEtherFrmUp	Counter64, zxGponOltH1DErroredEtherFrmUp				Counter64, zxGponOltH1DErr		 Counter64,

zxGponOltH1DRei	Counter64, zxGponOltH1DValidEtherPktDown		Counter64, zxGponOltH1DPloamDown			Counter64,

zxGponOltH1DErroredPloamUp	Counter64,

zxGponOltH1DPloamUp							Counter64, zxGponOltH1DFecCorrBytes		Counter64, zxGponOltH1DFecCorrWords			Counter64, zxGponOltH1DFecUnCorrWords				Counter64, zxGponOltH1DFecTotalRxWords						Counter64, zxGponOltH1DBipCorrBits	Counter64, zxGponOltH1DCrcErrPkts					Counter64,

zxGponOltH1DValidData	TruthValue

}

  - GPON ONU PON-Layer Performance (V1.2.3)

    - Real-time Performance

OID Specification

zxGponOntCurrPerfTable’s OID is .1.3.6.1.4.1.3902.1012.3.32.1.

Index Specification

{ zxGponMgmtPonOltId,	zxGponMgmtPonOnuId }, of them, zxGponMgmtPonOltId is PON

Type 1 composite index, while others are regular index.

Confidential▼

MIB Specification

Refer to zxGponService.mib.

zxGponOntCurrPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponOntCurrPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The GPON ONU current performance data table"

::= { zxAnGponOntPerfMgmt 1 }

zxGponOntCurrPerfEntry OBJECT-TYPE SYNTAX	ZxGponOntCurrPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The entry in the ONU current performance data table." INDEX	{ zxGponMgmtPonOltId, zxGponMgmtPonOnuId }

::= { zxGponOntCurrPerfTable 1 }

ZxGponOntCurrPerfEntry ::= SEQUENCE {

zxGponOntCorrNonIdleGemFrmUp	Counter64,

zxGponOntGemPayloadBytesUp	Counter64,

zxGponOntCorrEtherFrmUp	Counter64,

zxGponOntErroredEtherFrmUp	Counter64,

zxGponOntTotlOmciFrmUp		Counter64, zxGponOntErri	Counter64,

zxGponOntReii	Counter64,

zxGponOntUnrxBurstsUp	Counter64,

zxGponOntBipErrorUp	Counter64,

zxGponOntCorrBitsUp	Counter64,

zxGponOntNotCorrWordsUp	Counter64,

zxGponOntCorrNonIdleGemFrmDn	Counter64,

zxGponOntTxOmciFrames	Counter64,

zxGponOntTxPloamFrames	Counter64,

zxGponOntRxPloamFrames	Counter64,

zxGponOntLofiAlms	Counter64,

zxGponOntFecCorrBytes	Counter64,

zxGponOntFecCorrWords	Counter64,

zxGponOntFecUnCorrWords	Counter64, zxGponOntFecTotalRxWords		Counter64,

zxGponOntCrcErrPkts	Counter64,

Confidential▼

zxGponOntCurrReset	INTEGER

}

    - 15-Minute Real-time Performance

OID Specification

zxGponOntCurrPerf15MinTable ’s OID is .1.3.6.1.4.1.3902.1012.3.32.1.

Index Specification

{ zxGponMgmtPonOltId, zxGponMgmtPonOnuId }, of them, zxGponMgmtPonOltId is PON Type 1 composite index, while others are regular index.

MIB Specification

Refer to zxGponService.mib.

zxGponOntCurrPerf15MinTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponOntCurrPerf15MinEntry

MAX-ACCESS		not-accessible STATUS	current

DESCRIPTION

"The current 15 minutes performance data table of GPON ONU."

::= { zxAnGponOntPerfMgmt 2 }

zxGponOntCurrPerf15MinEntry OBJECT-TYPE SYNTAX	ZxGponOntCurrPerf15MinEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"An entry in the ONU current 15 minutes performance data table."

INDEX	{ zxGponMgmtPonOltId, zxGponMgmtPonOnuId	}

::= { zxGponOntCurrPerf15MinTable 1 }

ZxGponOntCurrPerf15MinEntry ::= SEQUENCE {

zxGponOntC15MTimeElapsed	Gauge32, zxGponOntC15MCorrNonIdleGemFrmUp			Counter64, zxGponOntC15MGemPayloadBytesUp		Counter64,

zxGponOntC15MCorrEtherFrmUp	Counter64,

zxGponOntC15MErroredEtherFrmUp	Counter64,

zxGponOntC15MTotlOmciFrmUp	Counter64,

zxGponOntC15MReii	Counter64

zxGponOntC15MErri	Counter64,

Confidential▼

zxGponOntC15MUnrxBurstsUp	Counter64,

zxGponOntC15MBipErrorUp	Counter64,

zxGponOntC15MCorrBitsUp	Counter64,

zxGponOntC15MNotCorrWordsUp	Counter64, zxGponOntC15MCorrNonIdleGemFrmDn			Counter64, zxGponOntC15MTxOmciFrms		Counter64,

zxGponOntC15MTxPloamFrms	Counter64,

zxGponOntC15MRxPloamFrms	Counter64,

zxGponOntC15MLofiAlarms		Counter64, zxGponOntC15MFecCorrBytes	Counter64,

zxGponOntC15MFecCorrWords	Counter64, zxGponOntC15MFecUnCorrWords		Counter64,

zxGponOntC15MFecTotalRxWords	Counter64,

zxGponOntC15MCrcErrPkts	Counter64,

zxGponOntC15MReset	INTEGER

}

    - 24-Hour Real-time Performance

OID Specification

zxGponOntCurrPerf1DayTable ’s OID is .1.3.6.1.4.1.3902.1012.3.32.3.

Index Specification

{ zxGponMgmtPonOltId, zxGponMgmtPonOnuId }, of them, zxGponMgmtPonOltId is PON Type 1 composite index, while others are regular index.

MIB Specification

Refer to zxGponService.mib.

zxGponOntCurrPerf1DayTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponOntCurrPerf1DayEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The current 1 day performance data table of GPON ONU."

::= { zxAnGponOntPerfMgmt 3 }

zxGponOntCurrPerf1DayEntry OBJECT-TYPE SYNTAX	ZxGponOntCurrPerf1DayEntry MAX-ACCESS		not-accessible

"An entry in the ONU current 1 day

STATUS	current DESCRIPTION

Confidential▼

performance data table."

INDEX	{ zxGponMgmtPonOltId, zxGponMgmtPonOnuId	}

::= { zxGponOntCurrPerf1DayTable 1 }

ZxGponOntCurrPerf1DayEntry ::= SEQUENCE {

zxGponOntC1DTimeElapsed	Gauge32, zxGponOntC1DCorrNonIdleGemFrmUp			Counter64, zxGponOntC1DGemPayloadBytesUp		Counter64,

zxGponOntC1DCorrEtherFrmUp	Counter64,

zxGponOntC1DErroredEtherFrmUp	Counter64,

zxGponOntC1DTotlOmciFrmUp	Counter64,

zxGponOntC1DErri	Counter64,

zxGponOntC1DReii	Counter64,

zxGponOntC1DUnrxBurstsUp	Counter64,

zxGponOntC1DBipErrorUp	Counter64,

zxGponOntC1DCorrBitsUp	Counter64,

zxGponOntC1DNotCorrWordsUp	Counter64, zxGponOntC1DCorrNonIdleGemFrmDn			Counter64, zxGponOntC1DTxOmciFrms		Counter64,

zxGponOntC1DTxPloamFrms	Counter64,

zxGponOntC1DRxPloamFrms	Counter64,

zxGponOntC1DLofiAlarms		Counter64, zxGponOntC1DFecCorrBytes	Counter64,

zxGponOntC1DFecCorrWords	Counter64, zxGponOntC1DFecUnCorrWords		Counter64,

zxGponOntC1DFecTotalRxWords	Counter64,

zxGponOntC1DCrcErrPkts	Counter64,

zxGponOntC1DReset	INTEGER

}

    - 15-Minute Historical Performance

OID Specification

zxGponOntPerf15MinIntervalTable ’s OID is .1.3.6.1.4.1.3902.1012.3.32.4.

Index Specification

{ zxGponMgmtPonOltId, zxGponMgmtPonOnuId }, of them, zxGponMgmtPonOltId is PON Type 1 composite index, while others are regular index.

MIB Specification

zxGponOntPerf15MinIntervalTable OBJECT-TYPE

Refer to zxGponService.mib.

Confidential▼

SYNTAX	SEQUENCE OF ZxGponOntPerf15MinIntervalEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 15 minutes interval performance data table of GPON ONT."

::= { zxAnGponOntPerfMgmt 4 }

zxGponOntPerf15MinIntervalEntry OBJECT-TYPE SYNTAX	ZxGponOntPerf15MinIntervalEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The entry in the ONU 15 minutes interval performance data table."

INDEX		{	zxGponMgmtPonOltId,	zxGponMgmtPonOnuId, zxGponOntH15MIntervalNumber	}

::= { zxGponOntPerf15MinIntervalTable 1 }

ZxGponOntPerf15MinIntervalEntry ::= SEQUENCE {

zxGponOntH15MIntervalNumber	Integer32,

zxGponOntH15MDateAndTime	DateAndTime, zxGponOntH15MCorrNonIdleGemFrmUp		Counter64, zxGponOntH15MGemPayloadBytesUp	 Counter64,

zxGponOntH15MCorrEtherFrmUp	Counter64,

zxGponOntH15MErroredEtherFrmUp	Counter64,

zxGponOntH15MTotlOmciFrmUp	Counter64,

zxGponOntH15MErri	Counter64,

zxGponOntH15MReii	Counter64,

zxGponOntH15MUnrxBurstsUp	Counter64,

zxGponOntH15MBipErrorUp	Counter64,

zxGponOntH15MCorrBitsUp	Counter64,

zxGponOntH15MNotCorrWordsUp	Counter64, zxGponOntH15MCorrNonIdleGemFrmDn			Counter64, zxGponOntH15MTxOmciFrms		Counter64,

zxGponOntH15MTxPloamFrms	Counter64,

zxGponOntH15MRxPloamFrms	Counter64,

zxGponOntH15MLofiAlarms		Counter64, zxGponOntH15MFecCorrBytes	Counter64,

zxGponOntH15MFecCorrWords	Counter64, zxGponOntH15MFecUnCorrWords		Counter64,

zxGponOntH15MFecTotalRxWords	Counter64,

zxGponOntH15MCrcErrPkts	Counter64,

zxGponOntH15MValidData	TruthValue

Confidential▼

}

    - 24-Hour Historical Performance

OID Specification

zxGponOntPerf1DayIntervalTable ’s OID is .1.3.6.1.4.1.3902.1012.3.32.5.

Index Specification

{ zxGponMgmtPonOltId, zxGponMgmtPonOnuId }, of them, zxGponMgmtPonOltId is PON Type 1 composite index, while others are regular index.

MIB Specification

Refer to zxGponService.mib.

zxGponOntPerf1DayIntervalTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponOntPerf1DayIntervalEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The 1 day interval performance data table of GPON ONT."

::= { zxAnGponOntPerfMgmt 5 }

zxGponOntPerf1DayIntervalEntry OBJECT-TYPE SYNTAX	ZxGponOntPerf1DayIntervalEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The entry in the ONU 1 day interval performance data table."

INDEX		{	zxGponMgmtPonOltId,	zxGponMgmtPonOnuId, zxGponOntH1DIntervalNumber	}

::= { zxGponOntPerf1DayIntervalTable 1 }

ZxGponOntPerf1DayIntervalEntry ::= SEQUENCE {

zxGponOntH1DIntervalNumber	Integer32,

zxGponOntH1DDateAndTime	DateAndTime, zxGponOntH1DCorrNonIdleGemFrmUp		Counter64, zxGponOntH1DGemPayloadBytesUp	 Counter64,

zxGponOntH1DCorrEtherFrmUp	Counter64,

zxGponOntH1DErroredEtherFrmUp	Counter64,

zxGponOntH1DTotlOmciFrmUp	Counter64,

zxGponOntH1DErri	Counter64,

zxGponOntH1DReii	Counter64,

All Rights reserved, No Spreading abroad without Permission of ZTE	Page 133

Confidential▼

zxGponOntH1DUnrxBurstsUp	Counter64,

zxGponOntH1DBipErrorUp	Counter64,

zxGponOntH1DCorrBitsUp	Counter64,

zxGponOntH1DNotCorrWordsUp	Counter64, zxGponOntH1DCorrNonIdleGemFrmDn			Counter64, zxGponOntH1DTxOmciFrms		Counter64,

zxGponOntH1DTxPloamFrms	Counter64,

zxGponOntH1DRxPloamFrms	Counter64,

zxGponOntH1DLofiAlarms		Counter64, zxGponOntH1DFecCorrBytes	Counter64,

zxGponOntH1DFecCorrWords	Counter64, zxGponOntH1DFecUnCorrWords		Counter64,

zxGponOntH1DFecTotalRxWords	Counter64,

zxGponOntH1DCrcErrPkts	Counter64,

zxGponOntH1DValidData	TruthValue

}

  - GPON GEMPORT Real-time Statistics (V1.2.3)

    - Real-time Performance

OID Specification

zxAnGponGemportPerfMgmt’s OID is .1.3.6.1.4.1.3902.1012.3.33.2.

Index Specification

{	zxGponMgmtPonOltId,	zxGponMgmtPonOnuId,	zxGponGemPortIdx	},	of	them, zxGponMgmtPonOltId is PON Type 1 composite index, while others are regular index.

MIB Specification

Refer to zxGponService.mib.

By default, the system won’t perform Gemport performance statistics. If needed, you need to create the performance point first via operating zxGponGemPortCurrRowStatus.

zxGponGemPortCurrPerfTable OBJECT-TYPE

SYNTAX	SEQUENCE OF ZxGponGemPortCurrPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The GPON GEM port current performance data table"

zxGponGemPortCurrPerfEntry OBJECT-TYPE

::= { zxAnGponGemportPerfMgmt 2 }

Confidential▼

SYNTAX	ZxGponGemPortCurrPerfEntry MAX-ACCESS		not-accessible

STATUS	current DESCRIPTION

"The entry in the GEM port current performance data table."

INDEX	{ zxGponMgmtPonOltId, zxGponMgmtPonOnuId, zxGponGemPortIdx }

::= { zxGponGemPortCurrPerfTable 1 }

ZxGponGemPortCurrPerfEntry ::= SEQUENCE {

zxGponGemPortRxOctets	Counter64,

zxGponGemPortRxUcastPkts	Counter64,

zxGponGemPortRxBroadcastPkts	Counter64,

zxGponGemPortRxMulticastPkts	Counter64,

zxGponGemPortRxDiscards	Counter64,

zxGponGemPortTxOctets	Counter64,

zxGponGemPortTxUcastPkts	Counter64,

zxGponGemPortTxBroadcastPkts	Counter64,

zxGponGemPortTxMulticastPkts	Counter64,

zxGponGemPortTxDiscards	Counter64,

zxGponGemPortCurrReset	INTEGER,

zxGponGemPortCurrRowStatus	RowStatus

}

Confidential▼

## System Control Alarms & Notifications

ZTE-AN-EQUIP-MIB.mib

  - Notification of System Cold Starting

TripOID

Variable Carried

.1.3.6.1.6.3.1.1.5.1

Null

Notification

  - Notification of Active / Standby Changeover

TripOID

.1.3.6.1.4.1.3902.1015.2.2.1.2

Notification

Variable

Carried

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.5

zxAnCardOperStatus

The  card  is  in

running status

  - Notification of Failing to Synchronize Active / Standby Data

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.2.2.1.3

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.5

zxAnCardOperStatus

Notification

The card is in running status

  - Alarms that the Card Works Abnormally

TripOID

.1.3.6.1.4.1.3902.1015.2.2.2.2

Major Alarm

Variable

Carried

Alarms

| .1.3.6.1.2.1.2.2.1.1 | ifIndex | ifIndex |
| --- | --- | --- |
| .1.3.6.1.4.1.3902.1015.2.1.1.3.1.2 | zxAnCardCfgMainType | Configured card type |
| .1.3.6.1.4.1.3902.1015.2.1.1.3.1.3 | zxAnCardActMainType | Actual card type |
| .1.3.6.1.4.1.3902.1015.2.1.1.3.1.4 | zxAnCardActType | Card name |

Confidential▼

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.6

zxAnCardAdminStatus	Administration

status

card

of

the

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.5

zxAnCardOperStatus

Running	status

of the card

Note: Of them, for the structure coding rules inside ifIndex, refer to ZTE EPON Equipment Composite Index Definitions.

The running status values of the card (.1.3.6.1.4.1.3902.1015.2.1.1.3.1.5):

1: The card is working normally 2: The card can not be used

3: The card’s software isn’t running 4: The card is off-line

5: The card is being configured

6: Configuration of the card failed 7: Card types aren’t matched

8: The card isn’t activated

9: The card is working abnormally 10: The card can not be used

    - Alarm Recovery

TripOID

.1.3.6.1.4.1.3902.1015.2.2.2.1

.1.3.6.1.2.1.2.2.1.1

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.2

ifIndex

zxAnCardCfgMainType

Variable

Carried

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.3

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.4

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.6

Alarm Recovery

ifIndex Configured card type

zxAnCardActMainType	Actual card type zxAnCardActType	Card name

zxAnCardAdminStatus	Administration

status

card

of

the

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.5

zxAnCardOperStatus

Running	status

of the card

  - Alarm of Failing to Detect / Scan Cards

TripOID	.1.3.6.1.4.1.3902.1015.2.2.2.3

Minor Alarm

Variable

Carried

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.3

zxAnCardActMainType

Actual

type

card

Alarm

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.4

zxAnCardActType

Card name

All Rights reserved, No Spreading abroad without Permission of ZTE	Page 137

Confidential▼

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.13	zxAnCardStandbyStatus

Active

/

standby status

of the card

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.2.2.2.4

Alarm

Recovery

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.3

zxAnCardActMainType

Actual

type

card

Variable

Carried

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.4	zxAnCardActType	Card name

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.13	zxAnCardStandbyStatus	Active	/

standby status of the card

  - Alarm of Wrong Communication of Card Services

    - Alarm

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.2.2.2.12

.1.3.6.1.2.1.2.2.1.1

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.2

ifIndex

zxAnCardCfgMainType

Minor Alarm

ifIndex

Configured card type

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.3

zxAnCardActMainType

Actual

type

card

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.4

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.6

zxAnCardActType

zxAnCardAdminStatus

Card name

Administration

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.5

zxAnCardOperStatus

status	of	the

card

Running status of the card

TripOID	.1.3.6.1.4.1.3902.1015.2.2.2.13

Variable

Carried

.1.3.6.1.2.1.2.2.1.1

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.2

ifIndex

zxAnCardCfgMainType

Alarm

Recovery

ifIndex Configured

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.3

zxAnCardActMainType

card type

Actual	card

Alarm Recovery

type

All Rights reserved, No Spreading abroad without Permission of ZTE	Page 138

|  |  | Confidential▼ |
| --- | --- | --- |
| .1.3.6.1.4.1.3902.1015.2.1.1.3.1.4 | zxAnCardActType | Card name |
| .1.3.6.1.4.1.3902.1015.2.1.1.3.1.6 | zxAnCardAdminStatus | Administration status of the card |
| .1.3.6.1.4.1.3902.1015.2.1.1.3.1.5 | zxAnCardOperStatus | Running status of the card |

  - Alarm of Memory Overloading

    - Alarm

TripOID	.1.3.6.1.4.1.3902.1015.2.2.2.3

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.11	zxAnCardMemUsage

Variable

Carried

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.12

Minor

Alarm Memory usage rate

zxAnCardMemUsageThreshold	Memory

usage

threshold

TripOID	.1.3.6.1.4.1.3902.1015.2.2.2.8

Alarm

Variable

Carried

Alarm Recovery

|  |  | Recovery |
| --- | --- | --- |
| .1.3.6.1.4.1.3902.1015.2.1.1.3.1.11 | zxAnCardMemUsage | Memory usage rate |
| .1.3.6.1.4.1.3902.1015.2.1.1.3.1.12 | zxAnCardMemUsageThreshold | Memory usage |
|  |  | threshold |

  - Alarm of CPU Overloading

TripOID	.1.3.6.1.4.1.3902.1015.2.2.2.6

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.9

zxAnCardCpuLoad

Variable

Carried

Minor

Alarm Current CPU

utilization

rate

Alarm

Confidential▼

.1.3.6.1.4.1.3902.1015.2.1.1.3.1.109	zxAnCardCpuLoadThreshold	CPU

utilization rate threshold

    - Alarm Recovery

| TripOID | .1.3.6.1.4.1.3902.1015.2.2.2.7 .1.3.6.1.4.1.3902.1015.2.1.1.3.1.9 | zxAnCardCpuLoad | Alarm Recovery Current |
| --- | --- | --- | --- |
|  |  |  | CPU utilization |
| Variable |  |  | rate |
| Carried | .1.3.6.1.4.1.3902.1015.2.1.1.3.1.109 | zxAnCardCpuLoadThreshold | CPU |
|  |  |  | utilization rate threshold |

  - Alarm that the Card Fails to Update Version

    - Alarm

TripOID	.1.3.6.1.4.1.3902.1015.2.2.2.10

.1.3.6.1.4.1.3902.1015.2.1.2.5.1.2

zxAnVerUpdateStatus

Common

Alarm

Version update

Variable Carried

.1.3.6.1.4.1.3902.1015.2.1.2.5.1.3

zxAnVerUpdateReason

status

Version update

reason

    - Alarm Recovery

TripOID

.1.3.6.1.4.1.3902.1015.2.2.2.11

.1.3.6.1.4.1.3902.1015.2.1.2.5.1.2

zxAnVerUpdateStatus

Variable

Carried	.1.3.6.1.4.1.3902.1015.2.1.2.5.1.3	zxAnVerUpdateReason

Alarm

Recovery Version update status

Version update

reason

Confidential▼

  - Alarm of Abnormal Power Supply

    - Alarm

TripOID	.1.3.6.1.4.1.3902.1015.2.2.3.12

Variable	.1.3.6.1.4.1.3902.1015.2.1.3.9.2.1.2

Carried

Common

Alarm zxAnPowerSupplyOperState	Operation

state

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.2.2.3.13

Variable	.1.3.6.1.4.1.3902.1015.2.1.3.9.2.1.2

Carried

Alarm

Recovery zxAnPowerSupplyOperState	Operation

state

  - High Temperature Alarm

    - Alarm

TripOID	.1.3.6.1.4.1.3902.1015.2.2.3.1

.1.3.6.1.4.1.3902.1015.2.1.3.2	zxAnEnvTemperature

Variable

Carried	.1.3.6.1.4.1.3902.1015.2.1.3.3	zxAnEnvTemperatureAlarmThreshold

Common

Alarm Actual Temperature Threshold

Temperature

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.2.2.3.2

.1.3.6.1.4.1.3902.1015.2.1.3.2	zxAnEnvTemperature

Variable

Carried	.1.3.6.1.4.1.3902.1015.2.1.3.3	zxAnEnvTemperatureAlarmThreshold

Alarm

Recovery Actual Temperature Threshold

Temperature

Confidential▼

  - Alarm that the Fan is Off-line

    - Alarm

TripOID	.1.3.6.1.4.1.3902.1015.2.2.3.5

Variable

Carried

.1.3.6.1.4.1.3902.1015.2.1.3.10.10.10.1.6

Common

Alarm

zxAnEnvFanOnlineStatus	Fan status

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.2.2.3.6

Variable

Carried

.1.3.6.1.4.1.3902.1015.2.1.3.10.10.10.1.6

Alarm

Recovery

zxAnEnvFanOnlineStatus	Fan status

  - Alarm that the Fan Operates Abnormally

    - Alarm

TripOID	.1.3.6.1.4.1.3902.1015.2.2.2.8

.1.3.6.1.4.1.3902.1015.2.1.3.10.10.10.1.5	zxAnEnvFanOperStatus

Variable

Carried

Common

Alarm Fan operation status

up(1),

down(2), unknown(3)

TripOID	.1.3.6.1.4.1.3902.1015.2.2.2.9

.1.3.6.1.4.1.3902.1015.2.1.3.10.10.10.1.5	zxAnEnvFanOperStatus

Variable

Carried

Alarm

Recovery Fan operation status

up(1),

down(2), unknown(3)

Alarm Recovery

Confidential▼

  - Alarm of Abnormal Dust Cover

    - Alarm

TripOID	.1.3.6.1.4.1.3902.1015.2.2.3.10

Common

Alarm

Variable

Carried

Null

    - Alarm Recovery

TripOID

.1.3.6.1.4.1.3902.1015.2.2.3.11

Alarm

Recovery

Variable

Carried

Null

  - Notification of Damaged Sensor

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.2.2.3.7

Null

Notification

  - Alarm	of	Link	Down	of	Environment	Monitoring Interface

    - Alarm

TripOID	.1.3.6.1.4.1.3902.1015.2.2.3.3

Common

Alarm

Variable

Carried

Null

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.2.2.3.4

Alarm

All Rights reserved, No Spreading abroad without Permission of ZTE	Page 143

Confidential▼

Recovery

Variable

Carried

Null

Confidential▼

## Platform Alarms & Notifications

  - LinkDown Alarm of Uplink Port

    - Alarm

TripOID	.1.3.6.1.4.1.3902.1015.3.2.2

.1.3.6.1.2.1.2.2.1.7

ifAdminStatus

Variable

Carried

.1.3.6.1.2.1.2.2.1.8

ifOperStatus

.1.3.6.1.2.1.2.2.1.3

ifType

Major Alarm

Administration status Operation status

Interface type

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.3.2.1

.1.3.6.1.2.1.2.2.1.7

ifAdminStatus

Variable

Carried

.1.3.6.1.2.1.2.2.1.8

ifOperStatus

.1.3.6.1.2.1.2.2.1.3

ifType

Alarm

Recovery Administration status Operation status

Interface type

  - Dos Attack Alarm

    - Alarm

TripOID	.1.3.6.1.4.1.3902.1015.11.2.1

Variable

Carried

.1.3.6.1.2.1.2.2.1.1

.1.3.6.1.4.1.3902.1015. 1.1.1.1.6

ifIndex

zxAnServiceAntiDosSourceMac

.1.3.6.1.4.1.3902.1015. 1.1.1.1.7

zxAnServiceAntiDosPortVlan

Common

Alarm IfIndex Source MAC

address

Port VLAN

Confidential▼

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.2.2.2.4

.1.3.6.1.2.1.2.2.1.1

.1.3.6.1.4.1.3902.1015. 1.1.1.1.6

ifIndex

zxAnServiceAntiDosSourceMac

Variable

Carried

.1.3.6.1.4.1.3902.1015.11.1.1.1.7

zxAnServiceAntiDosPortVlan

Alarm

Recovery IfIndex Source MAC

address

Port VLAN

Variable

Carried

.1.3.6.1.2.1.2.2.1.8

ifOperStatus

.1.3.6.1.2.1.2.2.1.3

ifType

status

Operation state

Interface type

LinkUp Notification of User Port

| TripOID | .1.3.6.1.4.1.3902.1015.2.2.2.9 |  | Notification |
| --- | --- | --- | --- |
|  | .1.3.6.1.2.1.2.2.1.7 | ifAdminStatus | Administration |

  - LinkDown Notification of User Port

TripOID	.1.3.6.1.4.1.3902.1015.2.2.2.4

.1.3.6.1.2.1.2.2.1.7

ifAdminStatus

Variable

Carried

.1.3.6.1.2.1.2.2.1.8

ifOperStatus

Notification

Administration status

Operation

.1.3.6.1.2.1.2.2.1.3

ifType

state

Interface type

  - Notification of Logging on Serial Port or Administration Port

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.1.2.6.1

.1.3.6.1.4.1.3902.1015.1.1.3.4

Notification

zxAnSysLatestLogonCrftTerminalType	Logon type

Confidential▼

  - Notification	of	Logging	out	Serial	Port	or Administration Port

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.1.2.6.2

.1.3.6.1.4.1.3902.1015.1.1.3.4

Notification

zxAnSysLatestLogonCrftTerminalType	Logon type

Confidential▼

## EPON Alarms & Notifications

ZXEPON-TRAP-MIB.mib

  - Notification of PON Port Resetting

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.1010.1.10.3.2

.1.3.6.1.2.1.2.2.1.1

ifIndex

Notification

ifIndex

  - Notification that Unknown ONUs are Online

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.1010.1.10.3.9

.1.3.6.1.2.1.2.2.1.1

ifIndex

Notification

ifIndex

.1.3.6.1.4.1.3902.1015.1010.1.10.3.1	zxAnEponTrapEventString	Additional

information

  - Notification of Refusing Registration of Unknown ONUs

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.1010.1.10.3.8

.1.3.6.1.2.1.2.2.1.1

ifIndex

Notification

ifIndex

  - Notification of Failing to Find Extended OAM

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.1010.1.10.3.5

.1.3.6.1.2.1.2.2.1.1

ifIndex

Notification

ifIndex

.1.3.6.1.4.1.3902.1015.1010.1.10.3.1	zxAnEponTrapEventString	Additional

information

Confidential▼

  - Notification that Unknown ONUs are Offline

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.3.7

.1.3.6.1.2.1.2.2.1.1	ifIndex

Variable

.1.3.6.1.4.1.3902.1015.1010.1.10.3.1	zxAnEponTrapEventString

Carried

Notification

ifIndex

Additional information

  - Notification of Power-down of ONUs

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.1010.1.10.3.12

.1.3.6.1.2.1.2.2.1.1

ifIndex

Notification

ifIndex

  - ONU Offline Alarm

TripOID

Variable

Carried

Alarm

| .1.3.6.1.4.1.3902.1015.1010.1.10.1.15 .1.3.6.1.2.1.2.2.1.1 | ifIndex | Major Alarm ifIndex |
| --- | --- | --- |
| .1.3.6.1.4.1.3902.1015.1010.1.10.2.8 | zxAnEponTrapOnuOffLineReason | Offline reason |

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.1.16

.1.3.6.1.2.1.2.2.1.1

Variable

.1.3.6.1.4.1.3902.1015.1010.1.10.2.8

Carried

Alarm

Recovery

ifIndex	ifIndex

zxAnEponTrapOnuOffLineReason	Offline

reason

Confidential▼

  - Alarm of Losing PON Port Signals

TripOID

Variable

Carried

Alarm

| .1.3.6.1.4.1.3902.1015.1010.1.10.1.13 |  | Major |
| --- | --- | --- |
|  |  | Alarm |
| .1.3.6.1.2.1.2.2.1.1 | ifIndex | ifIndex |
| .1.3.6.1.4.1.3902.1015.1010.1.10.2.7 | zxAnEponTrapPonLosReason | Alarm reason |

Values of LOS reasons (.1.3.6.1.4.1.3902.1015.1010.1.10.2.7):

1- The fiber is disconnected. 2 - Unknown reasons.

- - The ONU is powered down.

- - The ONU has never been online.

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.1.14

.1.3.6.1.2.1.2.2.1.1

Variable

.1.3.6.1.4.1.3902.1015.1010.1.10.2.7

Carried

Alarm

Recovery

ifIndex	ifIndex

zxAnEponTrapPonLosReason	Alarm

reason

  - Alarm of PON Port Bit Error Rate

    - Alarm

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.1010.1.10.1. 1

.1.3.6.1.2.1.2.2.1.1

ifIndex

Major Alarm

ifIndex

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.1.12

Variable

Carried

.1.3.6.1.2.1.2.2.1.1

ifIndex

Alarm

Recovery

ifIndex

Alarm Recovery

Confidential▼

  - Alarm of Exceeding Threshold of ONU Optical Power

    - Alarm

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.1.1	Major Alarm

.1.3.6.1.2.1.2.2.1.1	ifIndex	ifIndex

Variable

.1.3.6.1.4.1.3902.1015.1010.1.10.3.1	zxAnEponTrapEventString	Threshold

Carried

information

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.1.2

Alarm

Recovery

Variable

Carried

.1.3.6.1.2.1.2.2.1.1

.1.3.6.1.4.1.3902.1015.1010.1.10.3.1

ifIndex	ifIndex

zxAnEponTrapEventString	Threshold

information

  - Alarm of Interval Event of ONU—s Error Symbols

    - Alarm

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.1010.1.10.1.17

.1.3.6.1.2.1.2.2.1.1

ifIndex

Major Alarm

ifIndex

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.1.18

Variable

Carried

.1.3.6.1.2.1.2.2.1.1

ifIndex

Alarm

Recovery ifIndex

Confidential▼

    - ONU Error Frame Event

    - Alarm

TripOID

Variable

Carried

.1.3.6.1.4.1.3902.1015.1010.1.10.1.19

.1.3.6.1.2.1.2.2.1.1

ifIndex

Major Alarm

ifIndex

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.1.20

Variable

Carried

.1.3.6.1.2.1.2.2.1.1

ifIndex

Alarm

Recovery

ifIndex

  - Alarm of Interval Event of ONU—s Error Frames

    - Alarm

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.1010.1.10.1.21

.1.3.6.1.2.1.2.2.1.1

ifIndex

Major Alarm

ifIndex

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.1.22

Variable

Carried

.1.3.6.1.2.1.2.2.1.1

ifIndex

Alarm

Recovery

ifIndex

  - Alarm of Total Number of Seconds Event of ONU—s Error Frames

    - Alarm

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.1.23

Major Alarm

Confidential▼

Variable

Carried

.1.3.6.1.2.1.2.2.1.1

ifIndex

ifIndex

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.1.24

Variable

Carried

.1.3.6.1.2.1.2.2.1.1

ifIndex

Alarm

Recovery

ifIndex

  - Alarm of ONU Uplink—s Link Bit Error

    - Alarm

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.1.25

Major Alarm

Variable

Carried

.1.3.6.1.2.1.2.2.1.1

ifIndex

ifIndex

    - Alarm Recovery

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.1.26

Variable

Carried

.1.3.6.1.2.1.2.2.1.1

ifIndex

Alarm

Recovery

ifIndex

  - Alarm of ONU Uplink—s Link Frame Error

    - Alarm

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.1010.1.10.1.27

.1.3.6.1.2.1.2.2.1.1

ifIndex

Major Alarm

ifIndex

    - Alarm Recovery

| TripOID | .1.3.6.1.4.1.3902.1015.1010.1.10.1.28 |  | Alarm |
| --- | --- | --- | --- |
| Variable | .1.3.6.1.2.1.2.2.1.1 | ifIndex | Recovery ifIndex |

Carried

All Rights reserved, No Spreading abroad without Permission of ZTE	Page 153

Confidential▼

  - Notification of Failing to Configure Local ONUs

TripOID

Variable Carried

.1.3.6.1.4.1.3902.1015.1010.1.10.3.6

.1.3.6.1.2.1.2.2.1.1

ifIndex

Notification

ifIndex

  - Notification of Extended OAM Event

TripOID	.1.3.6.1.4.1.3902.1015.1010.1.10.3. 1	Notification

.1.3.6.1.2.1.2.2.1.1	ifIndex	ifIndex

Variable

.1.3.6.1.4.1.3902.1015.1010.1.10.3.1	zxAnEponTrapEventString	OAM

Carried

information

All Rights reserved, No Spreading abroad without Permission of ZTE	Page 154

Confidential▼

## 16	GPON Alarms & Notifications

{"zxGponOltSDi"	,	"1.3.6.1.4.1.3902.1012.3.45.109"} ,

{"zxGponOltSDiRestore"	,	"1.3.6.1.4.1.3902.1012.3.45.110"} ,

{"zxGponOltTxPowerAbnormalInform",	"1.3.6.1.4.1.3902.1012.3.45.181"} ,

{"zxGponOnuOMCITrap"	,	"1.3.6.1.4.1.3902.1012.3.50.7.1"} ,