# ZXA10	C300C320V2.x C300MC350MV4.x	MIB

# Specifications

ZXA10	C300C320V2.x	C300MC350MV4.x	MIB

Specifications

| Version | Date | Author | Reviewer | Notes |
| --- | --- | --- | --- | --- |
| V1.0 | 2013-11-6 | Zeng Tao |  | Not open to the third party |
|  | 2014-1-22 | Jing Yu Dan |  |  |

###### TABLE OF CONTENT

© 2020 ZTE Corporation. All rights reserved.

ZTE CONFIDENTIAL: This document contains proprietary information of ZTE and is not to be disclosed or used without the prior written permission of ZTE.

Due to update and improvement of ZTE products and technologies, information in this document is subjected to change without notice.

S

TABLE OF CONTENTS	2

- Overview	9

- Composite Indices	10

  - IfIndex	10

    - IfIndex Type 1	10

    - IfIndex Type 2	10

  - ZxAnSubIfIndex	11

    - ZxAnSubIfIndex Type 1	11

    - ZxAnSubIfIndex Type 2	11

    - ZxAnSubIfIndex Type 3	11

    - ZxAnSubIfIndex Type 4	11

    - ZxAnSubIfIndex Type 5	12

    - ZxAnSubIfIndex Type 6	12

    - ZxAnSubIfIndex Type 7	12

    - ZxAnSubIfIndex Type 8	12

    - ZxAnSubIfIndex Type 9	12

- System Control	13

  - NE Information	13

  - Shelf Information	13

  - Card Information	16

  - Version Information	22

  - Manually Updated Information	24

  - Auto Update Check	25

  - Auto Update Information	26

  - Auto Backup Information	27

  - File Operation and Configuration Information	29

  - File Server Configuration Information	30

  - Wachdog Configuration	31

  - Environmental Monitoring	32

    - Temperature	32

    - Emergency Power Saving	34

    - Fan	36

  - Environmental Device	38

    - CPE Information	38

    - Dynamic Patch	41

- Basic Services	44

  - VLAN	44

    - Switchport Configuration	44

    - Serviceport Configuration	44

  - Multicast	46

    - Global Management	46

    - Multicast Protocol Global Data	46

    - Multicast Management Global Data	46

    - Multicast Protocol Port Management	47

    - Multicast Protocol MVLAN Management	47

    - Multicast MVLAN SNOOPING Information	49

    - Multicast MVLAN Proxy Information	49

    - Multicast Management MVLAN Table	51

    - Multicast Management Receiving Port List	51

    - Multicast Management Source Port List	51

    - Multicast MVLAN Receiving Port List	52

    - Multicast Group Filter Table	52

    - Multicast Group Static Port List	52

    - Multicast Management Source Filter Mode Table	53

    - Multicast Group Forwarding Table	53

    - Multicast Group User Table	54

    - Multicast Group Prejoin Table	54

    - Multicast Group Configuration Table	55

    - Multicast Packet Statistics Table	55

    - Multicast Port Statistics Table	57

    - Multicast MVLAN Statistics Table	57

  - Port Positioning	58

    - DHCP Option82 Global Swtich	58

    - DHCP Option82 Port Switch	58

    - DHCP Option82 Port Trust Rule	59

    - DHCP Option82 Port Trust Measure	59

    - DHCP Option18 Global Swtich	60

    - DHCP Option18 Port Switch	60

    - DHCP Option18 Port Trust Status	61

    - DHCP Option18 Port Trust Measure	61

    - PPPoE Global Switch	62

    - PPPoE Port Swtich	62

    - PPPoE Port Trust Status	63

    - PPPoE Port Trust Measure	63

    - ND Global Switch	64

    - ND Port Switch	64

    - ND Port Trust Status	65

    - ND Port Trust Measure	65

    - NE Access Identifier Configuration	66

    - User-Defined NE Access Identifier Configuration	66

    - NE Rack ID Configuration	66

    - NE Shelf ID Configuration	67

    - DHCP Option82 Port Rate Configuration	67

    - PPPoE Port Rate Configuration	68

    - Port Positioning Format Configuration	69

    - Port Positioning Format Profile Configuration	70

    - Port Positioning access-loop-tag Switch	70

    - Port Positioning Remote ID Switch	71

    - Port Positioning Remote ID Protocol Swtich	71

    - Port Positioning RemotID Content	72

    - Port Positioning RemotID Format Profile	72

    - Port Positioning Mode Configuration	73

    - Operator Profile Global Application	73

    - Format Profile Configuration	73

    - Format Profile Content Configuration	74

    - Operator Profile Configuration	76

    - Operator Profile DHCP Option82 Configuration	77

    - Operator Profile DHCP Option18 Configuration	78

    - Operator Profile PPPoE Configuration	79

    - Operator Profile DHCP Option82 Port Rate Configuration	80

    - Operator Profile PPPoE Port Rate Configuration	81

    - Operator Profile Format Configuration	82

    - Operator Profile Port Application Configuration	83

    - Operator Profile VLAN Application Mode	83

  - DHCP SNOOPING	84

    - Global Management	84

    - DHCPv4 VLAN Switch	85

    - DHCPv4 Interface Swtich	85

    - DHCPv4 Dynamic Binding	86

    - Maximum IPv4 Binding	87

    - DHCPv4 Session Limit	88

    - DHCPv4 Packet Statistics	88

    - IP Source Guard Global Configuration	90

    - IP Source Guard Interface Switch	91

    - IPv4 Static Binding Table	92

  - L3 Interface	93

    - VLAN Interface Creation	93

    - VLAN Interface IP Configuration	93

- PON Services	95

  - Public PON Port Management	95

    - IP Address Pool	95

  - GPON OLT Management	96

    - ONU Management	96

    - ONU Status	99

    - T-CONT Profile	99

    - GEM Port Traffic Profile	101

    - T-CONT	102

    - GEM Port	103

    - GEM Port Rate Limit	104

    - VLAN (Service Port)	104

    - Unconfigured ONU	105

    - Traffic-profile	106

  - Remote GPON ONU Management	108

    - Service	108

    - LAN Interface	109

    - LAN Interface VLAN	110

    - Multicast VLAN	115

    - VoIP	116

    - POTS Interface	123

    - Remote Operation	123

    - MAC Address Table Query	124

    - WIFI Configuration	125

    - VEIP Configuration	131

    - Ethernet Port Performance Statistics	135

  - Configuration Process Examples	152

    - GPON HIS Service	152

    - GPON VOIP	155

- DSL Services	163

  - ADSL Configuration	163

    - Create Line Profile	163

    - Create Alarm Profile	172

    - Modify Line Profile	182

    - Modify Alarm Profile	183

    - Delete Line Profile	183

    - Delete Alarm Profile	184

    - Bind ADSL Line Profile To Port	184

    - Bind ADSL Alarm Profile To Port	185

  - SHDSL Configuration	185

    - Create Line Profile	185

    - Create Alarm Profile	189

    - Modify Line Profile	192

    - Modify Alarm Profile	193

    - Delete Line Profile	194

    - Delete Alarm Profile	194

    - Bind SHDSL Line Profile To Port	195

    - Bind SHDSL Alarm Profile To Port	195

- Optical Module and Optical Power	196

  - Optical Module	196

    - Information Query	196

    - Threshold Configuration	198

    - Threshold Loading	199

    - Als Config	199

  - Receiving Optical Power on the OLT Side	201

  - Optical Power on the GPON ONU Side	201

- Performance Statistics	203

  - Ethernet Port Realtime Statistics	203

    - ifTable Realtime Statistics	203

    - ifXTable Realtime Statistics	204

  - OLT Ethernet Realtime Statistics	206

    - Realtime Statistics	206

  - ONU Ethernet Realtime Statistics	209

    - Realtime Performance	209

  - GPON OLT PON Layer Realtime Statistics	212

    - Realtime Performance	212

  - GPON ONU PON Layer Realtime Statistics	213

    - Realtime Performance	213

  - Ethernet Interface Performance	215

    - Realtime Performance	215

    - 15-Minute Realtime Performance	216

    - 24-Hour Realtime Performance	217

    - 15-Minute History Performance	219

    - 24-Hour History Performance	220

    - Performance Threshold Alarm Profile	222

    - Performance Alarm Profile Loading	223

  - VPORT Performance	223

    - Startup and Stop	223

    - Realtime Performance	224

    - 15-Minute Realtime Performance	226

    - 24-Hour Realtime Performance	228

    - 15-Minute History Performance	230

    - 24-Hour History Performance	232

    - Performance Threshold Alarm Profile	235

    - Performance Alarm Profile Loading	239

  - OLT Ethernet Performance	240

    - Realtime Performance	240

    - 15-Minute Realtime Performance	243

    - 24-Hour Realtime Performance	246

    - 15-Minute History Performance	248

    - 24-Hour History Performance	250

  - ONU Ethernet Performance	253

    - Realtime Performance	253

    - 15-Minute Realtime Performance	256

    - 24-Hour Realtime Performance	258

    - 15-Mintue History Performance	261

    - 24-Hour History Performance	263

  - GPON OLT PON Layer Performance	266

    - Realtime Performance	266

    - 15-Minute Realtime Performance	267

    - 24-Hour Realtime Performance	268

    - 15-Minute History Performance	269

    - 24-Hour History Performance	271

  - GPON ONU PON Layer Performance	272

    - Realtime Performance	272

    - 15-Minute Realtime Performance	273

    - 24-Hour Realtime Performance	274

    - 15-Minute History Performance	276

    - 24-Hour History Performance	277

  - GPON GEMPORT Realtime Statistics	278

    - Realtime Performance	278

- Alarms and Notifications	280

  - System Control Alarm and Notification	280

    - System Cold Start Notification	280

    - Active and Standby Switchover Notification	280

    - Active and Standby Data Synchronization Failure Notification	280

    - Card Abnormal Operating Alarm	280

    - Memory Overload Alarm	282

    - CPU Overload Alarm	283

    - Card Version Update Failure Alarm	283

    - Card Auto Update Alarm	284

    - Card Version Download Alarm	284

    - Card Backup Alarm	284

    - High Ambient Temperature Alarm	285

    - Sensor Abnormal Alarm	285

    - Ambient Temperature Danger Alarm	285

    - Low Ambient Temperature Alarm	286

    - Card Offline Alarm	286

    - Environmental Monitoring Interface Down Alarm	287

    - Power Abnormal Alarm	288

    - Power Supply Overvoltage Alarm	288

    - Power Supply Undervoltage Alarm	288

    - Power Down Alarm	289

    - Power Supply Emergency Alarm	289

    - Fan Offline Alarm	290

    - Fan Card Offline Alarm	290

    - Fan Abnormal Alarm	291

    - Environmetal Monitoring Device Abnormal Alarm	291

    - Version Update Alarm	292

  - Common Alarm and Notification	293

    - Uplink Interface Linkdown Alarm	293

    - DoS Attack Alarm	293

    - User Interface LinkUp Notification	294

    - User Interface LinkDown Notification	294

    - Serial Interface or Management Interface Login Notification	295

    - Serial Interface or Management Interface Logout Notification	295

  - PON Alarm and Notification	296

    - PON Public Alarm and Notification	297

    - GPON Alarm and Notification	298

    - GPON ONU Remote Alarm and Notification	301

##### 10	304

# Overview

This document is applicable to ZTE PON OLT product ZXA10 C300C320V2.x C300MC350MV4.x.

The majority of the MIB object information is available from the corresponding MIB files, which are the ultimate and most reliable source of all MIB objects. This document only lists the key information and the related MIB files.

Unless otherwise stipulated, the MIBs remain unchanged between different versions and are applicable to all released versions of the OLT.

# Composite Indices

The ZXA10 C3xx v2.0 Interface Indices are composed of two four-bit indices. The first is IfIndex and the second is ZxAnSubIfIndex.

## IfIndex

For ZXA10 C300C320V2.x C300MC350MV4.x, IfTable includes all physical and logical ports. IfTable uses structural composite indices to identify different physical and logical ports. The IfTable indices are defined in ZTE-AN-TC-MIB.mib.

IfIndex defines two interface types. The first four bits indicate the interface type.

#### IfIndex Type 1

IfIndex Type 1 indicates a physical port.

| (MSB)Type(4bits) | 4bits | 8bits | 8bits | 8bits(LSB) |
| --- | --- | --- | --- | --- |
| 1 | rack | shelf | slot | port |

#### IfIndex Type 2

IfIndex Type 2 indicates a logical port.

| (MSB)Type(4bits) | subType(4bits) | 8bits | 16bits(LSB) |
| --- | --- | --- | --- |
| 2 | 1 | 0 | smartgroup ID |
| 2 | 2 | 0 | L3 interface VLAN ID |
| 2 | 3 | 0 | L3 interface Super VLAN ID |
| 2 | 4 | 0 | L3 interface Loopback ID |
| 2 | 5 | 0 | L3 interface Inband VLAN ID |
| 2 | 6 | 0 | Pseudo-wire No. |
| 2 | 7 | 0 | L3 mng interface No. |
| 2 | 8 | 0 | CIP interface No.for MPLS access circuit |

| (MSB)Type(4bits) | subType(4bits) | 8bits | 16bits(LSB) |
| --- | --- | --- | --- |
| 2 | 9 | 0 | CPU(1) |

## ZxAnSubIfIndex

ZxAnSubIfIndex indicates the sub-interfaces under one physical port. ZxAnSubIfIndex must be used together with ifIndex. ZxAnSubIfIndex uses complicated bit-coding to identify the sub-interfaces. The definitions of each bit field are stipulated in ZTE-AN-TC- MIB.mib.

ZxAnSubIfIndex defines nine interface types. The first five bits indicate an interface type.

#### ZxAnSubIfIndex Type 1

ZxAnSubIfIndex Type 1 indicates an ONT.

| (MSB)Type(5bits) | 11bits | 16bits (LSB) |
| --- | --- | --- |
| 1 | ONT ID | 0 |

#### ZxAnSubIfIndex Type 2

ZxAnSubIfIndex Type 2 indicates the ONT bridge port.

| (MSB)Type(5bits) | 11bits | 8bits | 8bits(LSB) |
| --- | --- | --- | --- |
| 2 | ONT ID | bridgeportID | 0 |

#### ZxAnSubIfIndex Type 3

ZxAnSubIfIndex Type 3 indicates an ONT service port.

| (MSB)Type(5bits) | 11bits | 8bits | 8bits(LSB) |
| --- | --- | --- | --- |
| 3 | ONT ID | Serviceport ID | 0 |

#### ZxAnSubIfIndex Type 4

ZxAnSubIfIndex Type 4 indicates an ONT gemport.

| (MSB)Type(5bits) | 11bits | 8bits | 8bits(LSB) |
| --- | --- | --- | --- |
| 4 | ONT ID | Gemport ID | 0 |

#### ZxAnSubIfIndex Type 5

ZxAnSubIfIndex Type 5 indicates an ONT UNI port.

| (MSB)Type(5bits) | 11bits | 8bits | 8bits(LSB) |
| --- | --- | --- | --- |
| 5 | ONT ID | Slot | UNI port ID |

#### ZxAnSubIfIndex Type 6

ZxAnSubIfIndex Type 6 indicates a DSL PVC.

| (MSB)Type(5bits) | 11bits | 8bits | 8bits(LSB) |
| --- | --- | --- | --- |
| 6 | 0 | 0 | PVC |

#### ZxAnSubIfIndex Type 7

ZxAnSubIfIndex Type 7 indicates an ATM/IMA PVC.

| (MSB)Type(5bits) | 11bits | 16bits(LSB) |
| --- | --- | --- |
| 7 | 0 | PVC |

#### ZxAnSubIfIndex Type 8

ZxAnSubIfIndex Type 8 indicates an ONT CVLAN and SVLAN pair.

| (MSB)Type(5bits) | 1bit | 12bits | 12bits(LSB) |
| --- | --- | --- | --- |
| 8 | 0 | SVLAN ID | CVLAN ID |

#### ZxAnSubIfIndex Type 9

ZxAnSubIfIndex Type 9 indicates a P2P or smartgroup serviceport.

| (MSB)Type(5bits) | 19bits | 8bits(LSB) |
| --- | --- | --- |

| 9 | 0 | Serviceport ID |
| --- | --- | --- |

# System Control

## NE Information

[Function description]:

This MIB is used for obtaining the basic NE information including Object ID, description information, system operating time, system location, etc.

[MIB file]:

SNMPv2-MIB.mib

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| sysDescr | .1.3.6.1.2.1.1.1 |  | Read only |
| sysObjectID | .1.3.6.1.2.1.1.2 |  | Read only |
| sysUpTime | .1.3.6.1.2.1.1.3 |  | Read only |
| sysContact | .1.3.6.1.2.1.1.4 |  |  |
| sysName | .1.3.6.1.2.1.1.5 |  |  |
| sysLocation | .1.3.6.1.2.1.1.6 |  |  |
| sysServices | .1.3.6.1.2.1.1.7 |  | Read only |
| sysORLastChange | .1.3.6.1.2.1.1.8 |  | Read only |
| sysORID | .1.3.6.1.2.1.1.9.1.2 |  | Read only |
| sysORDescr | .1.3.6.1.2.1.1.9.1.3 |  | Read only |
| sysORUpTime | .1.3.6.1.2.1.1.9.1.4 |  | Read only |

[Index description]: Global index; index .0 [MIB variable description]:

## Shelf Information

[Function description]:

This MIB is used for obtaining the actual shelf type and configured shelf type of an NE and for configuring the shelf type for an NE.

[MIB file]:

ZTE-AN-CHASSIS-MIB.mib

[Index description]:

The index consists of zxAnRack (rack ID) and zxAnShelf (shelf ID), both of which are numbered from 0.

[MIB variable description]:

| MIB Variable | OID | MIB Valu e | Descriptio n |
| --- | --- | --- | --- |
| zxAnChassisSysReboot | .1.3.6.1.4.1.3902.1082.10.1.1.1 |  |  |
| zxAnChassisSysLastRe bootReason | .1.3.6.1.4.1.3902.1082.10.1.1.2 |  | Read only |
| zxAnChassisSysLastSw apReason | .1.3.6.1.4.1.3902.1082.10.1.1.3 |  | Read only |
| zxAnChassisPnpMode | .1.3.6.1.4.1.3902.1082.10.1.1.4 |  |  |
| zxAnRackActualType | . 1.3.6.1.4.1.3902.1082.10.1.2.2.1. 2 |  | Read only |
| zxAnRackConfType | . 1.3.6.1.4.1.3902.1082.10.1.2.2.1. 3 |  |  |
| zxAnRackInvSn | . 1.3.6.1.4.1.3902.1082.10.1.2.2.1. 4 |  |  |
| zxAnRackRowStatus | . 1.3.6.1.4.1.3902.1082.10.1.2.2.1. 50 |  |  |
| zxAnShelfHardwareVersi on | . 1.3.6.1.4.1.3902.1082.10.1.2.3.1. 2 |  | Read only |
| zxAnShelfActualType | . 1.3.6.1.4.1.3902.1082.10.1.2.3.1. 3 |  | Read only |
| zxAnShelfConfType | . |  |  |

| MIB Variable | OID | MIB Valu e | Descriptio n |
| --- | --- | --- | --- |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.3.1. 4 |  |  |
| zxAnShelfInvSn | . 1.3.6.1.4.1.3902.1082.10.1.2.3.1. 5 |  |  |
| zxAnShelfCleiCode | . 1.3.6.1.4.1.3902.1082.10.1.2.3.1. 6 |  |  |
| zxAnLogicShelfNo | . 1.3.6.1.4.1.3902.1082.10.1.2.3.1. 7 |  |  |
| zxAnShelfRowStatus | . 1.3.6.1.4.1.3902.1082.10.1.2.3.1. 50 |  |  |
| zxAnCardAdminStatus | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 6 |  |  |
| zxAnCardPortNums | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 7 |  | Read only |
| zxAnCardActivePortNum s | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 8 |  | Read only |
| zxAnCardCpuLoad | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 9 |  | Read only |
| zxAnCardCpuLoadThres hold | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 10 |  |  |
| zxAnCardMemUsage | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 11 |  | Read only |
| zxAnCardMemUsageThr eshold | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 12 |  |  |
| zxAnCardStandbyStatus | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. |  | Read only |

| MIB Variable | OID | MIB Valu e | Descriptio n |
| --- | --- | --- | --- |
|  | 13 |  |  |
| zxAnCardInvSn | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 14 |  |  |
| zxAnCardCleiCode | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 15 |  | Read only |
| zxAnCardAccessoriesTy pe | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 16 |  | Read only |
| zxAnCardAccessoriesOp erStatus | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 17 |  | Read only |
| zxAnCardMemSize | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 19 |  | Read only |
| zxAnCardAvailableStora geSize | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 21 |  | Read only |
| zxAnCardTotalStorageSi ze | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 22 |  | Read only |
| zxAnCardHardwareVersi on | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 23 |  | Read only |
| zxAnCardEnergySaving Enable | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 24 |  |  |

Remark:

The actual shelf type zxAnShelfActualMIB and the configured shelf type zxAnShelfConfMIB are defined as follows:

0x0009 0101	19-inch shelf 0x0009 0102	21-inch shelf

0x0009 0103	C320 shelf 0x0009 0104	C350 shelf

## Card Information

[Function description]:

This MIB is used for obtaining the card configuration list of an NE. [MIB file]:

ZTE-AN-CHASSIS-MIB.mib

[Index description]:

The index consists of zxAnRack (rack ID), zxAnShelf (shelf ID), and zxAnSlot (slot ID). zxAnRack (rack ID) and zxAnShelf (shelf ID) are numbered from 0. zxAnSlot (slot ID) is numbered from 1.

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnCardConfMainTyp | . |  | Configured |
| e | 1.3.6.1.4.1.3902.1082. |  | card type |
|  | 10.1.2.4.1.2 |  |  |
| zxAnCardActualMainTy | . |  | Actual card |
| pe | 1.3.6.1.4.1.3902.1082. |  | type (read only) |
|  | 10.1.2.4.1.3 |  |  |
| zxAnCardActualType | . |  | Card name |
|  | 1.3.6.1.4.1.3902.1082. |  | (read only) |
|  | 10.1.2.4.1.4 |  |  |
| zxAnCardOperStatus | . |  | Card status |
|  | 1.3.6.1.4.1.3902.1082. |  | (read only) |
|  | 10.1.2.4.1.5 |  |  |
| zxAnSubcardAdminSta | . |  |  |
| tus | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.1.2.5.1.6 |  |  |
| zxAnSubcardPortNums | . |  | read only |
|  | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.1.2.5.1.7 |  |  |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnSubcardActivePor | . |  | read only |
| tNums | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.1.2.5.1.8 |  |  |
| zxAnCardCpuLoad | . |  | CPU load (read |
|  | 1.3.6.1.4.1.3902.1082. |  | only) |
|  | 10.1.2.4.1.9 |  |  |
| zxAnSubcardMemUsa | . |  | Read only |
| ge | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.1.2.5.1.10 |  |  |
| zxAnCardMemUsage | . |  | Memory usage |
|  | 1.3.6.1.4.1.3902.1082. |  | percentage (%) |
|  | 10.1.2.4.1.11 |  | (read only） |
| zxAnCardStandbyStatu | . |  | Card active and |
| s | 1.3.6.1.4.1.3902.1082. |  | standby status |
|  | 10.1.2.4.1.13 |  | (read only) |
| zxAnCardRowStatus | . |  |  |
|  | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.1.2.4.1.50 |  |  |

Remark:

The zxAnCardCfgMainMIB value and the zxAnCardActMainMIB value are defined as follows:

/*Main control card*/

| SCGL | (0x00090101) |
| --- | --- |
| SCGM | (0x00090102) |
| SCXL | (0x00090103) |
| SCXM | (0x00090104) |
| SCGS | (0x00090105) |
| SCXMC | (0x00090106) |
| SCXLC | (0x00090108) |
| SCTM | (0x00090109) |

SMXA	(0x0009010a)

SCXN	(0x0009010b) SCTMB		(0x0009010c)

/*Uplink card*/

| GCFD | (0x00091501) |
| --- | --- |
| GCTD | (0x00091502) |
| GUSQ | (0x00091503) |
| GUFQ | (0x00091504) |
| GCFDF | (0x00091505) |
| GUFQF | (0x00091506) |
| GUSQF | (0x00091507) |
| XUTQ | (0x00091508) |
| TUCDT | (0x00091509) |
| TUGQS | (0x0009150A) |
| XDTD | (0x0009150B) |
| XDTS | (0x0009150C) |
| HUTQ | (0x0009150D) |
| HUGQ | (0x0009150E) |
| XUXQ | (0x0009150F) |

/*Extended uplink card*/ GUTQ	(0x00091f01)

HUXQ	(0x00091f04)

HUVQ	(0x00091f06)

XUVQ	(0x00091f07)

HUKQ	(0x00091f08)

XDTO	(0x00091f09)

###### /* ADS line card */ ASWVC	 (0x00092001) APWVF	(0x00092003) AMWVF		(0x00092004) ANWVF	 (0x00092005)

###### /* SHDSL line card */ SSWGF	 (0x00092101) SMWGF		(0x00092102) SLWGF	(0x00092103)

###### /* VDSL line card */

| VMWKC | (0x00092204) |
| --- | --- |
| VMWVC | (0x00092206) |
| VMWVS | (0x00092209) |
| VIWVC | (0x0009220a) |
| VMWGC | (0x00092207) |
| VHWKC | (0x0009220b) |
| VPWVC | (0x0009220c) |

/*EPON line card*/

| ETGO | (0x00092301) |
| --- | --- |
| ETXD | (0x00092302) |
| ETTD | (0x00092303) |
| ETTS | (0x00092304) |
| WTBS | (0x00092305) |
| ETTQ | (0x00092306) |
| ETGH | (0x00092307) |

ETTO	(0x00092308)

/*GPON line card*/

| GTGO | (0x00092401) |
| --- | --- |
| GTGQ | (0x00092402) |
| GTXD | (0x000924003) |
| GTXT | (0x00092404) |
| GTHQ | (0x00092408) |
| GTXQ | (0x00092407) |
| GTGH | (0x00092409) |
| GTXO | (0x0009240C) |
| GTGOF | (0x0009240f) |

/*Power card*/

PRWG	(0x00094002)

PRAM	(0x00094003)

/*P2P interface card*/

| FTGH | (0x00091601) |
| --- | --- |
| GDFO | (0x00091602) |
| HDTT | (0x00091603) |
| GDFOU | (0x00091604) |
| FTGHU | (0x00091605) |
| HDTTU | (0x00091606) |
| FTGK | (0x00091607) |
| FTGKU | (0x00091608) |

FTGKN	(0x0009160a)

/*Public interface card*/ CICT	(0x0009A001)

CICG	(0x0009A002)

CICK	(0x0009A003)

/*CES interface card*/

| CTLA | (0x00092901) |
| --- | --- |
| CTEBA | (0x00092902) |
| CTEUA | (0x00092903) |
| CTTB | (0x00092904) |
| CTBB | (0x00092905) |
| CTUB | (0x00092906) |

Card status:

1: inService

2: notInService

3: hwOnline

4: hwOffline

5: configuring

6: configFailed

7: MIB value mismatch 8: deactived

9: faulty

10: invalid

11: noPower

Card active and standby status:

1: Active

2: Standby

15: Unknown

## Version Information

[Function description]:

This MIB is used for viewing the current running version of an NE including the main version, boot version, and fireware version.

[MIB file]:

ZTE-AN-SOFTWARE-MIB.mib

[Index description]:

The index consists of zxAnRack (rack ID), zxAnShelf (shelf ID), and zxAnSlot (slot ID). zxAnRack (rack ID) and zxAnShelf (shelf ID) are numbered from 0. zxAnSlot (slot ID) is numbered from 1.

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnSwCardFileNa | . |  | Main version |
| me | 1.3.6.1.4.1.3902.1082.20.30.2. |  | name |
|  | 2.2.1.5 |  |  |
| zxAnSwCardFileTy | . |  | Main version |
| pe | 1.3.6.1.4.1.3902.1082.20.30.2. |  | type |
|  | 2.2.1.6 |  |  |
| zxAnSwCardVersio | . |  | Main version |
| n | 1.3.6.1.4.1.3902.1082.20.30.2. |  | ID |
|  | 2.2.1.7 |  |  |

| zxAnSwCardFileLe | . |  | Main version |
| --- | --- | --- | --- |
| n | 1.3.6.1.4.1.3902.1082.20.30.2. |  | file length |
|  | 2.2.1.8 |  |  |
| zxAnSwCardBuildTi | . |  | Main version |
| me | 1.3.6.1.4.1.3902.1082.20.30.2. |  | build time |
|  | 2.2.1.9 |  |  |
| zxAnSwCardBootw | . |  | Boot version |
| areFileName | 1.3.6.1.4.1.3902.1082.20.30.2. |  | name |
|  | 2.2.1.10 |  |  |
| zxAnSwCardBootw | . |  | Boot version |
| areFileType | 1.3.6.1.4.1.3902.1082.20.30.2. |  | type |
|  | 2.2.1.11 |  |  |
| zxAnSwCardBootw | . |  | Boot version |
| areVersion | 1.3.6.1.4.1.3902.1082.20.30.2. |  | ID |
|  | 2.2.1.12 |  |  |
| zxAnSwCardBootw | . |  | Boot version |
| areFileLen | 1.3.6.1.4.1.3902.1082.20.30.2. |  | file length |
|  | 2.2.1.13 |  |  |
| zxAnSwCardBootw | . |  | Boot version |
| areBuildTime | 1.3.6.1.4.1.3902.1082.20.30.2. |  | build time |
|  | 2.2.1.14 |  |  |
| zxAnSwCardFirmw | . |  | Firmware |
| are1FileName | 1.3.6.1.4.1.3902.1082.20.30.2. |  | version name |
|  | 2.2.1.15 |  |  |
| zxAnSwCardFirmw | . |  | Firmware |
| are1FileType | 1.3.6.1.4.1.3902.1082.20.30.2. |  | version type |
|  | 2.2.1.16 |  |  |
| zxAnSwCardFirmw | . |  | Firmware |
| are1Version | 1.3.6.1.4.1.3902.1082.20.30.2. |  | version ID |
|  | 2.2.1.17 |  |  |
| zxAnSwCardFirmw | . |  | Firmware |
| are1FileLen | 1.3.6.1.4.1.3902.1082.20.30.2. |  | version file |
|  | 2.2.1.18 |  | length |
| zxAnSwCardFirmw | . |  | Firmware |
| are1BuildTime | 1.3.6.1.4.1.3902.1082.20.30.2. |  | version build |
|  | 2.2.1.19 |  | time |
| zxAnSwCardFirmw | . |  |  |
| are2FileName | 1.3.6.1.4.1.3902.1082.20.30.2. |  |  |
|  | 2.2.1.20 |  |  |
| zxAnSwCardFirmw | . |  |  |

| are2FileType | 1.3.6.1.4.1.3902.1082.20.30.2. 2.2.1.21 |  |  |
| --- | --- | --- | --- |
| zxAnSwCardFirmw are2Version | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.2.1.22 |  |  |
| zxAnSwCardFirmw are2FileLen | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.2.1.23 |  |  |
| zxAnSwCardFirmw are2BuildTime | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.2.1.24 |  |  |
| zxAnSwCardFirmw are3FileName | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.2.1.25 |  |  |
| zxAnSwCardFirmw are3FileType | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.2.1.26 |  |  |
| zxAnSwCardFirmw are3Version | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.2.1.27 |  |  |
| zxAnSwCardFirmw are3FileLen | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.2.1.28 |  |  |
| zxAnSwCardFirmw are3BuildTime | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.2.1.29 |  |  |
| zxAnSwSubcardFil eName | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.3 |  |  |
| zxAnSwSubcardFil eType | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.4 |  |  |
| zxAnSwSubcardVe rsion | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.5 |  |  |
| zxAnSwSubcardFil eLen | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.6 |  |  |
| zxAnSwSubcardBui ldTime | . 1.3.6.1.4.1.3902.1082.20.30.2. |  |  |

|  | 2.3.1.7 |  |  |
| --- | --- | --- | --- |
| zxAnSwSubcardBo otwareFileName | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.8 |  |  |
| zxAnSwSubcardBo otwareFileType | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.9 |  |  |
| zxAnSwSubcardBo otwareVersion | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.10 |  |  |
| zxAnSwSubcardBo otwareFileLen | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.11 |  |  |
| zxAnSwSubcardBo otwareBuildTime | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.12 |  |  |
| zxAnSwSubcardFir mwareFileName | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.13 |  |  |
| zxAnSwSubcardFir mwareFileType | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.14 |  |  |
| zxAnSwSubcardFir mwareVersion | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.15 |  |  |
| zxAnSwSubcardFir mwareFileLen | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.16 |  |  |
| zxAnSwSubcardFir mwareBuildTime | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.3.1.17 |  |  |
| zxAnSwImageFileT ype | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.4.1.2 |  |  |
| zxAnSwImageVersi on | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.4.1.3 |  |  |
| zxAnSwImageFileL en | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.4.1.4 |  |  |

| zxAnSwImageBuild Time | . 1.3.6.1.4.1.3902.1082.20.30.2. 2.4.1.5 |  |  |
| --- | --- | --- | --- |
| zxAnSwImageActiv | . |  |  |
| eStatus | 1.3.6.1.4.1.3902.1082.20.30.2. |  |  |
|  | 2.4.1.6 |  |  |

## Manually Updated Information

[Function description]:

This MIB is used for viewing the manually updated information. [MIB file]:

ZTE-AN-SOFTWARE-MIB.mib

[Index description]:

The index consists of zxAnRack (rack ID) and zxAnShelf (shelf ID), both of which are numbered from 0.

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnSwManualUp | . |  | Manually |
| dateRack | 1.3.6.1.4.1.3902.1082.20.30.2.3. |  | updated rack |
|  | 2.1.1 |  | ID |
| zxAnSwManualUp | . |  | Manually |
| dateShelf | 1.3.6.1.4.1.3902.1082.20.30.2.3. |  | updated |
|  | 2.1.2 |  | shelf ID |
| zxAnSwManualUp | . |  | Manually |
| dateSlotList | 1.3.6.1.4.1.3902.1082.20.30.2.3. |  | updated slot |
|  | 2.1.3 |  | ID |
| zxAnSwManualUp | . |  | Manually |
| dateSwType | 1.3.6.1.4.1.3902.1082.20.30.2.3. |  | updated file |
|  | 2.1.4 |  | type |
| zxAnSwManualUp | . |  |  |
| dateStatus | 1.3.6.1.4.1.3902.1082.20.30.2.3. |  |  |
|  | 2.5.1.2 |  |  |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnSwManualFai | . |  |  |
| ledReason | 1.3.6.1.4.1.3902.1082.20.30.2.3. |  |  |
|  | 2.5.1.3 |  |  |

## Auto Update Check

[Function description]:

This MIB is used for enabling the auto update check function. [MIB file]:

ZTE-AN-SOFTWARE-MIB.mib

[Index description]:

The index consists of zxAnRack (rack ID) and zxAnShelf (shelf ID), both of which are numbered from 0.

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnSwAutoUpdateC | . |  | Auto update |
| hkEnable | 1.3.6.1.4.1.3902.1082.20.3 |  | check enabled |
|  | 0.2.3.3.1.1.1 |  |  |
| zxAnSwAutoUpdateC | . |  | Auto update |
| hkStartTime | 1.3.6.1.4.1.3902.1082.20.3 |  | check start |
|  | 0.2.3.3.1.1.2 |  | time |
| zxAnSwAutoUpdateC | . |  | Auto update |
| hkInterval | 1.3.6.1.4.1.3902.1082.20.3 |  | check interval |
|  | 0.2.3.3.1.1.3 |  |  |
| zxAnSwAutoUpdateC | . |  | Current auto |
| urrChkStartTime | 1.3.6.1.4.1.3902.1082.20.3 |  | update check |
|  | 0.2.3.3.1.1.4 |  | start time |
| zxAnSwAutoUpdateC | . |  | Auto update |
| hkDifferFiles | 1.3.6.1.4.1.3902.1082.20.3 |  | check |
|  | 0.2.3.3.1.1.5 |  | inconsistent file |
| zxAnSwAutoUpdateC | . |  | Auto update |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| hkStatus | 1.3.6.1.4.1.3902.1082.20.3 0.2.3.3.1.1.6 |  | check status |
| zxAnSwAutoUpdateC hkFailedReason | . 1.3.6.1.4.1.3902.1082.20.3 0.2.3.3.1.1.7 |  | Auto update check failure reason |

## Auto Update Information

[MIB file]:

ZTE-AN-SOFTWARE-MIB.mib

[Index description]:

The index consists of zxAnRack (rack ID) and zxAnShelf (shelf ID), both of which are numbered from 0.

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnSwAutoUpdateActio | . |  | Auto update |
| n | 1.3.6.1.4.1.3902.1082.20. |  | action |
|  | 30.2.3.3.1.2.1 |  |  |
| zxAnSwAutoUpdateActiv | . |  | Auto update |
| eEnable | 1.3.6.1.4.1.3902.1082.20. |  | enabled |
|  | 30.2.3.3.1.2.2 |  |  |
| zxAnSwAutoUpdateSwB | . |  | Auto update |
| ackupEnable | 1.3.6.1.4.1.3902.1082.20. |  | backup |
|  | 30.2.3.3.1.2.3 |  | enabled |
| zxAnSwAutoUpdateStat | . |  | Auto update |
| us | 1.3.6.1.4.1.3902.1082.20. |  | status |
|  | 30.2.3.3.1.2.20 |  |  |
| zxAnSwAutoUpdateCurr | . |  | Current |
| FileName | 1.3.6.1.4.1.3902.1082.20. |  | update file |
|  | 30.2.3.3.1.2.21 |  | name |
| zxAnSwAutoUpdateCurr | . |  | Size of the |
| FileSize | 1.3.6.1.4.1.3902.1082.20. |  | current |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 30.2.3.3.1.2.22 |  | update file |
| zxAnSwAutoUpdateCurr | . |  | File update |
| FileProgress | 1.3.6.1.4.1.3902.1082.20. |  | progress |
|  | 30.2.3.3.1.2.23 |  |  |
| zxAnSwAutoUpdateTotal | . |  | Total |
| Files | 1.3.6.1.4.1.3902.1082.20. |  | number of |
|  | 30.2.3.3.1.2.24 |  | update files |
| zxAnSwAutoUpdateSucc | . |  | Total |
| essFiles | 1.3.6.1.4.1.3902.1082.20. |  | number of |
|  | 30.2.3.3.1.2.25 |  | successfully |
|  |  |  | updated files |
| zxAnSwAutoUpdateFaile | . |  | Update |
| dReason | 1.3.6.1.4.1.3902.1082.20. |  | failure |
|  | 30.2.3.3.1.2.26 |  | reason |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnSwSwapRack | . |  |  |
|  | 1.3.6.1.4.1.3902.1082.20. |  |  |
|  | 30.2.4.1.1 |  |  |
| zxAnSwSwapShelf | . |  |  |
|  | 1.3.6.1.4.1.3902.1082.20. |  |  |
|  | 30.2.4.1.2 |  |  |
| zxAnSwSwapSlot | . |  |  |
|  | 1.3.6.1.4.1.3902.1082.20. |  |  |
|  | 30.2.4.1.3 |  |  |

## Auto Backup Information

[MIB file]:

ZTE-AN-DATA-BACKUP-MIB.mib

[Index description]:

The index consists of zxAnRack (rack ID) and zxAnShelf (shelf ID), both of which are numbered from 0.

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnDataAutoBackupE | . |  | Auto backup |
| nable | 1.3.6.1.4.1.3902.1082.2 |  | enabled |
|  | 0.45.2.5.1.2 |  |  |
| zxAnDataAutoBackupS | . |  | Auto backup |
| tartTime | 1.3.6.1.4.1.3902.1082.2 |  | start time |
|  | 0.45.2.5.1.3 |  |  |
| zxAnDataAutoBackupIn | . |  | Auto backup |
| terval | 1.3.6.1.4.1.3902.1082.2 |  | interval |
|  | 0.45.2.5.1.4 |  |  |
| zxAnDataAutoBackupH | . |  | Auto backup |
| oldOffTime | 1.3.6.1.4.1.3902.1082.2 |  | hold off time |
|  | 0.45.2.5.1.5 |  |  |
| zxAnDataAutoBackupM | . |  | Auto backup |
| axHoldOffTime | 1.3.6.1.4.1.3902.1082.2 |  | maximum |
|  | 0.45.2.5.1.6 |  | hold off time |
| zxAnDataManualBackup | . |  | Auto backup |
| Action | 1.3.6.1.4.1.3902.1082.2 |  | started |
|  | 0.45.2.6.1.2 |  |  |
| zxAnDataBackupCurrSta | . |  | Current auto |
| rtTime | 1.3.6.1.4.1.3902.1082.2 |  | backup start |
|  | 0.45.2.7.1.2 |  | time |
| zxAnDataBackupCurrFil | . |  | Name of the |
| eName | 1.3.6.1.4.1.3902.1082.2 |  | current |
|  | 0.45.2.7.1.3 |  | backup file |
| zxAnDataBackupCurrFil | . |  | Size of the |
| eSize | 1.3.6.1.4.1.3902.1082.2 |  | current |
|  | 0.45.2.7.1.4 |  | backup file |
| zxAnDataBackupCurrFil | . |  | Current file |
| eProgress | 1.3.6.1.4.1.3902.1082.2 |  | backup |
|  | 0.45.2.7.1.5 |  | progress |
| zxAnDataBackupTotalFil | . |  | Total number |
| es | 1.3.6.1.4.1.3902.1082.2 |  | of backup files |
|  | 0.45.2.7.1.6 |  |  |
| zxAnDataBackupSucces | . |  | Number of |
| sFiles | 1.3.6.1.4.1.3902.1082.2 |  | successful |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 0.45.2.7.1.7 |  | backup files |
| zxAnDataBackupStatus | . |  | Backup status |
|  | 1.3.6.1.4.1.3902.1082.2 |  |  |
|  | 0.45.2.7.1.8 |  |  |
| zxAnDataBackupFailedR | . |  | Backup failure |
| eason | 1.3.6.1.4.1.3902.1082.2 |  | reason |
|  | 0.45.2.7.1.9 |  |  |

## File Operation and Configuration Information

[MIB file]:

ZTE-AN-FILE-MIB.mib

[Index description]:

The index consists of zxAnRack (rack ID) and zxAnShelf (shelf ID), both of which are numbered from 0.

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |  |
| --- | --- | --- | --- | --- |
| zxAnFileLoadDefault | . |  | Load default |  |
| Configuration | 1.3.6.1.4.1.3902.1082.20.50. |  | configurations |  |
|  | 1.2 |  |  |  |
| zxAnFileFtpFileType | . |  | File type |  |
|  | 1.3.6.1.4.1.3902.1082.20.50. |  |  |  |
|  | 2.1.1.1 |  |  |  |
| zxAnFileFtpOperType | . |  | File operation |  |
|  | 1.3.6.1.4.1.3902.1082.20.50. |  | type |  |
|  | 2.1.1.2 |  |  |  |
| zxAnFileFtpAction | . |  | File status |  |
|  | 1.3.6.1.4.1.3902.1082.20.50. |  | information |  |
|  | 2.1.1.3 |  |  |  |
| zxAnFileFtpSvrIpAddr | . |  | Server |  |
| essType | 1.3.6.1.4.1.3902.1082.20.50. |  | address type |  |
|  | 2.1.1.4 |  |  |  |
| zxAnFileFtpSvrIpAddr | . |  | File | FTP |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| ess | 1.3.6.1.4.1.3902.1082.20.50. |  | server IP |
|  | 2.1.1.5 |  | address |
| zxAnFileFtpSvrUserN | . |  | File FTP |
| ame | 1.3.6.1.4.1.3902.1082.20.50. |  | server user |
|  | 2.1.1.6 |  | name |
| zxAnFileFtpSvrUserP | . |  | File FTP |
| wd | 1.3.6.1.4.1.3902.1082.20.50. |  | server user |
|  | 2.1.1.7 |  | password |
| zxAnFileFtpSvrProtoc | . |  | File FTP |
| olType | 1.3.6.1.4.1.3902.1082.20.50. |  | server |
|  | 2.1.1.8 |  | protocol type |
| zxAnFileFtpSvrFilePa | . |  | File FTP |
| th | 1.3.6.1.4.1.3902.1082.20.50. |  | server path |
|  | 2.1.1.9 |  |  |
| zxAnFileFtpSvrFileNa | . |  | File name |
| me | 1.3.6.1.4.1.3902.1082.20.50. |  |  |
|  | 2.1.1.10 |  |  |
| zxAnFileFtpFileRetrie | . |  | File retrieve |
| veStartTime | 1.3.6.1.4.1.3902.1082.20.50. |  | start time |
|  | 2.1.1.21 |  |  |
| zxAnFileFtpFileRetrie | . |  | File retrieve |
| veEndTime | 1.3.6.1.4.1.3902.1082.20.50. |  | end time |
|  | 2.1.1.22 |  |  |
| zxAnFileFtpPerfLogT | . |  |  |
| ype | 1.3.6.1.4.1.3902.1082.20.50. |  |  |
|  | 2.1.1.30 |  |  |
| zxAnFileFtpOperStat | . |  |  |
| us | 1.3.6.1.4.1.3902.1082.20.50. |  |  |
|  | 2.1.1.50 |  |  |
| zxAnFileFtpCurrFileN | . |  |  |
| ame | 1.3.6.1.4.1.3902.1082.20.50. |  |  |
|  | 2.1.1.51 |  |  |
| zxAnFileFtpCurrFileSi | . |  |  |
| ze | 1.3.6.1.4.1.3902.1082.20.50. |  |  |
|  | 2.1.1.52 |  |  |
| zxAnFileFtpCurrFileP | . |  |  |
| rogress | 1.3.6.1.4.1.3902.1082.20.50. |  |  |
|  | 2.1.1.53 |  |  |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnFileFtpTotalFiles | . |  |  |
|  | 1.3.6.1.4.1.3902.1082.20.50. |  |  |
|  | 2.1.1.54 |  |  |
| zxAnFileFtpSuccessF | . |  |  |
| iles | 1.3.6.1.4.1.3902.1082.20.50. |  |  |
|  | 2.1.1.55 |  |  |
| zxAnFileFtpFailedRe | . |  |  |
| ason | 1.3.6.1.4.1.3902.1082.20.50. |  |  |
|  | 2.1.1.56 |  |  |
| zxAnFileSize | . |  | File size |
|  | 1.3.6.1.4.1.3902.1082.20.50. |  |  |
|  | 2.2.2.1.7 |  |  |
| zxAnFileModifyTime | . |  | File |
|  | 1.3.6.1.4.1.3902.1082.20.50. |  | modification |
|  | 2.2.2.1.8 |  | time |
| zxAnFileStorageDevT | . |  | File storage |
| ype | 1.3.6.1.4.1.3902.1082.20.50. |  | device type |
|  | 2.2.2.1.9 |  |  |
| zxAnFileOperation | . |  | Active and |
|  | 1.3.6.1.4.1.3902.1082.20.50. |  | standby |
|  | 2.2.2.1.15 |  | operation type |

## File Server Configuration Information

[MIB file]:

ZTE-AN-FILE-SERVER-MIB.mib

[Index description]:

The index consists of zxAnRack (rack ID) and zxAnShelf (shelf ID), both of which are numbered from 0.

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnFileServerGroupWork | . |  | File server |
| Mode | 1.3.6.1.4.1.3902.1082. |  | work mode |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 20.55.2.2.1.2 |  |  |
| zxAnFileServerIpAddressTy | . |  | File server |
| pe | 1.3.6.1.4.1.3902.1082. |  | IP address |
|  | 20.55.2.3.1.2 |  | type |
| zxAnFileServerIpAddress | . |  | File server |
|  | 1.3.6.1.4.1.3902.1082. |  | IP address |
|  | 20.55.2.3.1.3 |  |  |
| zxAnFileServerProtocolTyp | . |  | File server |
| e | 1.3.6.1.4.1.3902.1082. |  | protocol type |
|  | 20.55.2.3.1.4 |  |  |
| zxAnFileServerUserName | . |  | File server |
|  | 1.3.6.1.4.1.3902.1082. |  | user name |
|  | 20.55.2.3.1.5 |  |  |
| zxAnFileServerUserPwd | . |  | File server |
|  | 1.3.6.1.4.1.3902.1082. |  | password |
|  | 20.55.2.3.1.6 |  |  |
| zxAnFileServerPath | . |  | File server |
|  | 1.3.6.1.4.1.3902.1082. |  | path |
|  | 20.55.2.3.1.7 |  |  |

## Wachdog Configuration

[MIB file]:

ZTE-AN-WATCHDOG-MIB.mib

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnCardHardware | . | INTEGER | Hardware |
| WatchdogEnable | 1.3.6.1.4.1.3902.10 | {enabled (1), | watchdog |
|  | 82.10.20.2.1.2.1.1 | disabled ( 2 ) } | enabled |
| zxAnCardTaskSusp | . | INTEGER | Task suspended |
| endCardResetMod | 1.3.6.1.4.1.3902.10 | {notReset(1), | and reset |
| e | 82.10.20.2.1.2.1.2 | allTask |  |
|  |  | (2),criticalTask |  |
|  |  | (3)} |  |
| zxAnCardSoftware | . | INTEGER | Software |
| WatchdogEnable | 1.3.6.1.4.1.3902.10 | {enabled(1),disa | watchdog |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 82.10.20.2.1.2.1.3 | bled (2)} | enabled |
| zxAnCardTaskDurat | . | Integer32 (1 .. | Wait duration |
| ionThreshold | 1.3.6.1.4.1.3902.10 | 60) | threshold of |
|  | 82.10.20.2.1.2.1.4 |  | restart after task |
|  |  |  | endless loop |
| zxAnCardTaskCpu | . | Integer32(1 .. | CPU occupation |
| UsageThreshold | 1.3.6.1.4.1.3902.10 | 100) | threshold during |
|  | 82.10.20.2.1.2.1.5 |  | task endless |
|  |  |  | loop |

## Environmental Monitoring

[MIB file]:

ZTE-AN-ENVMON-MIB.mib

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnEnvMonCapabi lities | . 1.3.6.1.4.1.3902.10 82.10.10.1.1 |  | Read only |

3.12.1

Temperature

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnEnvCardShutdo | . |  | Card offline |
| wnReason | 1.3.6.1.4.1.3902.1082.10. |  | reason |
|  | 10.2.1.1.1 |  |  |
| zxAnEnvTemp | . |  | Current |
|  | 1.3.6.1.4.1.3902.1082.10. |  | ambient |
|  | 10.2.1.5.1.3 |  | temperature |
| zxAnEnvTempHighAl | . |  | Current high |
| mThreshold | 1.3.6.1.4.1.3902.1082.10. |  | temperature |
|  | 10.2.1.5.1.4 |  | threshold |
| zxAnEnvTempCritical | . |  | Current high |
| AlmThreshold | 1.3.6.1.4.1.3902.1082.10. |  | temperature |
|  | 10.2.1.5.1.5 |  | critical alarm |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | threshold |
| zxAnEnvTempLowAl | . |  | Current low |
| mThreshold | 1.3.6.1.4.1.3902.1082.10. |  | temperature |
|  | 10.2.1.5.1.6 |  | threshold |
| zxAnCardTemp | . |  | Current card |
|  | 1.3.6.1.4.1.3902.1082.10. |  | temperature |
|  | 10.2.1.6.1.2 |  | (some cards do |
|  |  |  | not support this |
|  |  |  | function) |
| zxAnCardPowerCons | . |  | Current power |
| umption | 1.3.6.1.4.1.3902.1082.10. |  | consumption of |
|  | 10.2.1.6.1.3 |  | all cards |
| zxAnEnvOverheatPro | . |  | Overheat |
| tectEnable | 1.3.6.1.4.1.3902.1082.10. |  | protection |
|  | 10.2.1.13.1.1 |  | enabled |
| zxAnEnvOverheatPro | . |  | Interval |
| tectDelay | 1.3.6.1.4.1.3902.1082.10. |  | between heat |
|  | 10.2.1.13.1.2 |  | protection |
|  |  |  | strategy 1 and |
|  |  |  | strategy 2 of |
|  |  |  | the overheat |
|  |  |  | area |
| zxAnEnvOverheatPro | . |  | Overheat |
| tectFirstStep | 1.3.6.1.4.1.3902.1082.10. |  | protection |
|  | 10.2.1.13.1.3 |  | strategy 1 |
| zxAnEnvOverheatPro | . |  | Overheat |
| tectSecondStep | 1.3.6.1.4.1.3902.1082.10. |  | protection |
|  | 10.2.1.13.1.4 |  | strategy 2 |
| zxAnCardEnvHis15Mi | . |  |  |
| nDateTime | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.7.1.2 |  |  |
| zxAnCardEnvHis15Mi | . |  | Average |
| nTemp | 1.3.6.1.4.1.3902.1082.10. |  | temperature in |
|  | 10.2.1.7.1.3 |  | 15 minute |
| zxAnCardEnvHis1Da | . |  |  |
| yDateTime | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.8.1.2 |  |  |
| zxAnCardEnvHis1Da | . |  | Average |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| yTemp | 1.3.6.1.4.1.3902.1082.10. |  | temperature in |
|  | 10.2.1.8.1.3 |  | 1 day |
| zxAnCardEnvRiseAl | . |  |  |
| mThresh | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.9.1.3 |  |  |
| zxAnCardEnvClrRise | . |  |  |
| AlmThresh | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.9.1.4 |  |  |
| zxAnCardEnvRiseWa | . |  |  |
| rnThresh | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.9.1.5 |  |  |
| zxAnCardEnvClrRise | . |  |  |
| WarnThresh | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.9.1.6 |  |  |
| zxAnCardEnvFallWar | . |  |  |
| nThresh | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.9.1.7 |  |  |
| zxAnCardEnvClrFall | . |  |  |
| WarnThresh | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.9.1.8 |  |  |
| zxAnCardEnvFallAlm | . |  |  |
| Thresh | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.9.1.9 |  |  |
| zxAnCardEnvClrFallA | . |  |  |
| lmThresh | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.9.1.10 |  |  |
| zxAnCardEnvAlmPrf | . |  |  |
| ConfRowStatus | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.9.1.50 |  |  |
| zxAnCardEnvAlmPrf | . |  |  |
|  | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.10.1.1 |  |  |
| zxAnCardEnvAlmPrf | . |  |  |
| ApplyRowStatus | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.10.1.50 |  |  |
| zxAnCardEnvAlmPrf | . |  |  |
| RowStatus | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.11.1.50 |  |  |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnEnvOverheatPro | . |  |  |
| tectionEnable | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.12.1.1 |  |  |
| zxAnEnvOverheatTe | . |  |  |
| mpThreshold | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.12.1.2 |  |  |
| zxAnEnvOverheatAut | . |  |  |
| oRecoveryTime | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.1.12.1.3 |  |  |
| zxAnEnvMonInterfac | . |  |  |
| eUsage | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.2.5.1.1 |  |  |
| zxAnEnvEpmConnect | . |  |  |
| Port | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.2.5.1.2 |  |  |
| zxAnEnvBackplaneInt | . |  |  |
| erfaceUsage | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.2.5.1.3 |  |  |
| zxAnPowerSupplyOp | . |  |  |
| erStatus | 1.3.6.1.4.1.3902.1082.10. |  |  |
|  | 10.2.3.11.1.1 |  |  |

#### Emergency Power Saving

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnEnvEmergencyPo | . |  | Emergency |
| werSaveEnable | 1.3.6.1.4.1.3902.1082.1 |  | power saving |
|  | 0.10.2.3.1.1 |  | enabled |
| zxAnEnvEmergencyPo | . |  | Emergency |
| werSaveDelay | 1.3.6.1.4.1.3902.1082.1 |  | power saving |
|  | 0.10.2.3.1.2 |  | effective time |
|  |  |  | after the AC |
|  |  |  | power is off |
| zxAnEnvEmergencyPo | . |  | Emergency |
| werSaveRecover | 1.3.6.1.4.1.3902.1082.1 |  | power saving |
|  | 0.10.2.3.1.3 |  | ineffective time |
|  |  |  | when the AC |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | power is restored |
| zxAnEnvEmergencyPo | . |  | Service type |
| werSaveSvcType | 1.3.6.1.4.1.3902.1082.1 |  | under |
|  | 0.10.2.3.1.4 |  | emergency |
|  |  |  | power saving |
| zxAnEnvPowerMode | . |  | Current power |
|  | 1.3.6.1.4.1.3902.1082.1 |  | supply mode |
|  | 0.10.2.3.1.5 |  |  |
| zxAnPowerSupplyMax | . |  | Maximum |
| PowerNum | 1.3.6.1.4.1.3902.1082.1 |  | number of power |
|  | 0.10.2.3.10.1.1 |  | supply |
| zxAnPowerSupplyInVol | . |  | Actual current |
| tage | 1.3.6.1.4.1.3902.1082.1 |  | input voltage |
|  | 0.10.2.3.11.1.2 |  |  |
| zxAnPowerSupplyInVol | . |  | Current voltage |
| tageStatus | 1.3.6.1.4.1.3902.1082.1 |  | status |
|  | 0.10.2.3.11.1.3 |  |  |
| zxAnPowerInVoltageU | . |  | Maximum input |
| pperThresh | 1.3.6.1.4.1.3902.1082.1 |  | voltage value. |
|  | 0.10.2.3.11.1.4 |  | Alarms will be |
|  |  |  | generated when |
|  |  |  | the input voltage |
|  |  |  | exceeds the |
|  |  |  | maximum |
|  |  |  | threshold. |
| zxAnPowerInVoltageLo | . |  | Minimum input |
| werThresh | 1.3.6.1.4.1.3902.1082.1 |  | voltage value. |
|  | 0.10.2.3.11.1.5 |  | Alarms will be |
|  |  |  | generated when |
|  |  |  | the input voltage |
|  |  |  | lowers than the |
|  |  |  | minimum |
|  |  |  | threshold. |
| zxAnPowerSupplyOper | . |  |  |
| Status | 1.3.6.1.4.1.3902.1082.1 |  |  |
|  | 0.10.2.3.11.1.1 |  |  |
| zxAnPowerSupplyInCu | . |  |  |
| rrent | 1.3.6.1.4.1.3902.1082.1 |  |  |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 0.10.2.3.11.1.6 |  |  |
| zxAnPowerInCurrentTh | . |  |  |
| resh | 1.3.6.1.4.1.3902.1082.1 |  |  |
|  | 0.10.2.3.11.1.7 |  |  |

#### Fan

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnFanTrayAlarmBee | . |  | Fan beep status |
| pEnable | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.10.2.4.10.1.1 |  |  |
| zxAnFanTrayAutoSwitc | . |  |  |
| hByCardUp | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.10.2.4.10.1.2 |  |  |
| zxAnFanTrayHardware | . |  |  |
| Version | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.10.2.4.10.1.3 |  |  |
| zxAnFanTraySoftware | . |  |  |
| Version | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.10.2.4.10.1.4 |  |  |
| zxAnFanTraySpeedCtrl | . |  | Fan mode (fixed |
| Mode | 1.3.6.1.4.1.3902.1082. |  | rotation speed or |
|  | 10.10.2.4.10.1.5 |  | temperature |
|  |  |  | control) |
| zxAnFanTrayLowSpee | . |  |  |
| d | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.10.2.4.10.1.6 |  |  |
| zxAnFanTrayStdSpeed | . |  |  |
|  | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.10.2.4.10.1.7 |  |  |
| zxAnFanTrayHighSpee | . |  |  |
| d | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.10.2.4.10.1.8 |  |  |
| zxAnFanTraySuperSpe | . |  |  |
| ed | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.10.2.4.10.1.9 |  |  |
| zxAnFanTrayLowSpee | . |  | Temperature that |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| dShiftTemp | 1.3.6.1.4.1.3902.1082. |  | triggers low |
|  | 10.10.2.4.10.1.10 |  | rotation speed |
| zxAnFanTrayStdSpeed | . |  | Temperature that |
| ShiftTemp | 1.3.6.1.4.1.3902.1082. |  | triggers middle |
|  | 10.10.2.4.10.1.11 |  | rotation speed |
| zxAnFanTrayHighSpee | . |  | Temperature that |
| dShiftTemp | 1.3.6.1.4.1.3902.1082. |  | triggers high |
|  | 10.10.2.4.10.1.12 |  | rotation speed |
| zxAnFanTraySuperSpe | . |  | Temperature that |
| edShiftTemp | 1.3.6.1.4.1.3902.1082. |  | triggers full |
|  | 10.10.2.4.10.1.13 |  | rotation speed |
| zxAnFanTrayLowSpee | . |  | Duty cycle in low |
| dPercent | 1.3.6.1.4.1.3902.1082. |  | rotation speed |
|  | 10.10.2.4.10.1.14 |  |  |
| zxAnFanTrayStdSpeed | . |  | Duty cycle in |
| Percent | 1.3.6.1.4.1.3902.1082. |  | middle rotation |
|  | 10.10.2.4.10.1.15 |  | speed |
| zxAnFanTrayHighSpee | . |  | Duty cycle in |
| dPercent | 1.3.6.1.4.1.3902.1082. |  | high rotation |
|  | 10.10.2.4.10.1.16 |  | speed |
| zxAnFanTraySuperSpe | . |  | Duty cycle in full |
| edPercent | 1.3.6.1.4.1.3902.1082. |  | rotation speed |
|  | 10.10.2.4.10.1.17 |  |  |
| zxAnFanTrayAdminSta | . |  | Current status of |
| tus | 1.3.6.1.4.1.3902.1082. |  | the fan card |
|  | 10.10.2.4.10.1.18 |  |  |
| zxAnFanTrayConfSpee | . |  | Current fan |
| dLevel | 1.3.6.1.4.1.3902.1082. |  | rotation speed |
|  | 10.10.2.4.10.1.19 |  | level |
| zxAnEnvFanConfSpee | . |  | Configured fan |
| dLevel | 1.3.6.1.4.1.3902.1082. |  | rotation speed |
|  | 10.10.2.4.11.1.2 |  | level |
| zxAnEnvFanActualSpe | . |  | Actual fan |
| edLevel | 1.3.6.1.4.1.3902.1082. |  | rotation speed |
|  | 10.10.2.4.11.1.3 |  | level |
| zxAnEnvFanAdminStat | . |  | Current fan |
| us | 1.3.6.1.4.1.3902.1082. |  | status switch (up |
|  | 10.10.2.4.11.1.4 |  | or down) |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnEnvFanOperStatu | . |  | Fan operating |
| s | 1.3.6.1.4.1.3902.1082. |  | status |
|  | 10.10.2.4.11.1.5 |  |  |
| zxAnEnvFanOnlineStat | . |  | Fan online status |
| us | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 10.10.2.4.11.1.6 |  |  |
| zxAnEnvFanActualSpe | . |  | Actual fan |
| ed | 1.3.6.1.4.1.3902.1082. |  | rotation speed |
|  | 10.10.2.4.11.1.7 |  |  |

## Environmental Device

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnEnvDeviceName | . |  | Environmental |
|  | 1.3.6.1.4.1.3902.1082. |  | device name |
|  | 10.10.2.5.2.1.2 |  |  |
| zxAnEnvDeviceRowStatu | . |  | Environmental |
| s | 1.3.6.1.4.1.3902.1082. |  | device row |
|  | 10.10.2.5.2.1.50 |  | status |
| zxAnEnvDevMonSwitchD | . |  | Device ID |
| eviceId | 1.3.6.1.4.1.3902.1082. |  | associated with |
|  | 10.10.2.5.3.1.2 |  | the volume |
|  |  |  | switch |
| zxAnEnvDevMonSwitchTr | . |  | Environmental |
| apEnable | 1.3.6.1.4.1.3902.1082. |  | device normal |
|  | 10.10.2.5.3.1.3 |  | or abnormal |
|  |  |  | alarm |
| zxAnEnvDevMonSwitchN | . |  | Normal status of |
| ormalStatus | 1.3.6.1.4.1.3902.1082. |  | the |
|  | 10.10.2.5.3.1.4 |  | environmental |
|  |  |  | device |
| zxAnEnvDevMonSwitchC | . |  | Current |
| urrStatus | 1.3.6.1.4.1.3902.1082. |  | environmental |
|  | 10.10.2.5.3.1.5 |  | device status |

#### CPE Information

[MIB file]:

ZTE-AN-REMOTE-UNIT-SW-MIB.mib

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnRuSwManualUpdat | . |  | Remote update |
| eList | 1.3.6.1.4.1.3902.1082.2 |  | list |
|  | 0.40.1.1.1 |  |  |
| zxAnRuSwManualUpdat | . |  | Software version |
| eAction | 1.3.6.1.4.1.3902.1082.2 |  | update strategy |
|  | 0.40.1.1.6 |  |  |
| zxAnRuSwManualUpdat | . |  | Software version |
| eFileName | 1.3.6.1.4.1.3902.1082.2 |  | file name |
|  | 0.40.1.1.7 |  |  |
| zxAnRuSwUpdatingTask | . |  | Task description |
| Desc | 1.3.6.1.4.1.3902.1082.2 |  |  |
|  | 0.40.2.2.1.2 |  |  |
| zxAnRuSwUpdatingTask | . |  | Task update |
| Mode | 1.3.6.1.4.1.3902.1082.2 |  | mode |
|  | 0.40.2.2.1.3 |  |  |
| zxAnRuSwUpdatingTask | . |  | Remote service |
| ServiceType | 1.3.6.1.4.1.3902.1082.2 |  | type: |
|  | 0.40.2.2.1.4 |  | GPON, |
|  |  |  | EPON…… |
| zxAnRuSwUpdatingTask | . |  | CPE vendor |
| Vendor | 1.3.6.1.4.1.3902.1082.2 |  | information |
|  | 0.40.2.2.1.5 |  |  |
| zxAnRuSwUpdatingTask | . |  | Remote update |
| EquipType | 1.3.6.1.4.1.3902.1082.2 |  | device type |
|  | 0.40.2.2.1.6 |  |  |
| zxAnRuSwUpdatingTask | . |  | Update Crtrn |
| CrtrnType | 1.3.6.1.4.1.3902.1082.2 |  | type |
|  | 0.40.2.2.1.7 |  |  |
| zxAnRuSwUpdatingTask | . |  | Compare with |
| CrtrnVer | 1.3.6.1.4.1.3902.1082.2 |  | the remote |
|  | 0.40.2.2.1.8 |  | update version. |
|  |  |  | The maximum |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | EPON byte is 16 and GPON byte is 14. |
| zxAnRuSwUpdatingTask OperObjType | . 1.3.6.1.4.1.3902.1082.2 0.40.2.2.1.9 |  | Task operation object type |
| zxAnRuSwUpdatingTask OperObjList | . 1.3.6.1.4.1.3902.1082.2 0.40.2.2.1.10 |  | Remote operation object list |
| zxAnRuSwUpdatingTask Action | . 1.3.6.1.4.1.3902.1082.2 0.40.2.2.1.11 |  | Task strategy |
| zxAnRuSwUpdatingTask FileName | . 1.3.6.1.4.1.3902.1082.2 0.40.2.2.1.12 |  | Name of the version to be updated |
| zxAnRuSwUpdatingTask Status | . 1.3.6.1.4.1.3902.1082.2 0.40.2.2.1.14 |  | Current task status |
| zxAnRuSwUpdatingTask RuAction | . 1.3.6.1.4.1.3902.1082.2 0.40.2.2.1.15 |  | Software version remote update strategy |
| zxAnRuSwUpdatingTask RowStatus | . 1.3.6.1.4.1.3902.1082.2 0.40.2.2.1.50 |  | Update task row status |
| zxAnRuSwTaskStatsSuc cesses | . 1.3.6.1.4.1.3902.1082.2 0.40.2.3.1.1 |  | Successfully updated part |
| zxAnRuSwTaskStatsFail ures | . 1.3.6.1.4.1.3902.1082.2 0.40.2.3.1.2 |  | Remote update failure part |
| zxAnRuSwTaskStatsUp datings | . 1.3.6.1.4.1.3902.1082.2 0.40.2.3.1.3 |  | Number of running remote part |
| zxAnRuSwUpdateStatus ServiceType | . 1.3.6.1.4.1.3902.1082.2 0.40.2.4.1.6 |  | Remote part type |
| zxAnRuSwUpdateStatus EquipType | . 1.3.6.1.4.1.3902.1082.2 |  | Remote update device type |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 0.40.2.4.1.7 |  |  |
| zxAnRuSwUpdateStatus FileName | . 1.3.6.1.4.1.3902.1082.2 0.40.2.4.1.8 |  | Update status file name |
| zxAnRuSwUpdateStatus Result | . 1.3.6.1.4.1.3902.1082.2 0.40.2.4.1.9 |  | Version update result |
| zxAnRuSwUpdateStatus FailReason | . 1.3.6.1.4.1.3902.1082.2 0.40.2.4.1.10 |  | Version update failure reason |
| zxAnRuSwUpdateStatus Progress | . 1.3.6.1.4.1.3902.1082.2 0.40.2.4.1.11 |  | Version update percentage |
| zxAnRuSwUpdateStatus Source | . 1.3.6.1.4.1.3902.1082.2 0.40.2.4.1.12 |  | Version update mode |
| zxAnRuSwUpdateStatus TaskName | . 1.3.6.1.4.1.3902.1082.2 0.40.2.4.1.13 |  | Update task name |
| zxAnRuSwUpdateStatus ModifyTime | . 1.3.6.1.4.1.3902.1082.2 0.40.2.4.1.14 |  | Ultimate modification time |
| zxAnRuSwUpda teStatusCurrVer sion | . 1.3.6.1.4.1.3902.1082.2 0.40.2.4.1.15 |  | Current software version attribute |
| zxAnRuSwUpdateStatus LastVersion | . 1.3.6.1.4.1.3902.1082.2 0.40.2.4.1.16 |  | Early software version attribute |
| zxAnRuSwImageVersion | . 1.3.6.1.4.1.3902.1082.2 0.40.2.5.1.2 |  | Software version attribute |
| zxAnRuSwImageStatus | . 1.3.6.1.4.1.3902.1082.2 0.40.2.5.1.3 |  | Image status |

[MIB file]:

ZTE-AN-REMOTE-UNIT-CHANNEL-MIB.mib

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnRuChannelTcpPort | . |  |  |
| AutoAllocEn | 1.3.6.1.4.1.3902.1082 |  |  |
|  | .20.41.2.2.1.1 |  |  |
| zxAnRuChannelTcpPort | . |  |  |
| No | 1.3.6.1.4.1.3902.1082 |  |  |
|  | .20.41.2.2.2.1.2 |  |  |
| zxAnRuChannelTcpPort | . |  |  |
| RowStatus | 1.3.6.1.4.1.3902.1082 |  |  |
|  | .20.41.2.2.2.1.50 |  |  |

#### Dynamic Patch

[MIB file]:

ZTE-AN-SOFTWARE-MIB.mib

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnSwSavedPat | . |  | This table includes |
| chTable | 1.3.6.1.4.1.3902.1082.20. |  | the saved patch |
|  | 30.2.5.2 |  | information. |
| zxAnSwSavedPat | . |  | An entry in |
| chEntry | 1.3.6.1.4.1.3902.1082.20. |  | zxAnSwSavedPat |
|  | 30.2.5.2.1 |  | chTable. |
| zxAnSwPatchRac | . |  | The rack number. |
| k | 1.3.6.1.4.1.3902.1082.20. |  |  |
|  | 30.2.5.2.1.1 |  |  |
| zxAnSwPatchShel | . |  | The shelf number. |
| f | 1.3.6.1.4.1.3902.1082.20. |  |  |
|  | 30.2.5.2.1.2 |  |  |
| zxAnSwPatchSlot | . |  | The slot number. |
|  | 1.3.6.1.4.1.3902.1082.20. |  |  |
|  | 30.2.5.2.1.3 |  |  |
| zxAnSwPatchNam | . |  | The patch file |
| e | 1.3.6.1.4.1.3902.1082.20. |  | name. |
|  | 30.2.5.2.1.4 |  |  |
| zxAnSwPatchOwn | . |  | The software |
| erSwVersion | 1.3.6.1.4.1.3902.1082.20. |  | version of the |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 30.2.5.2.1.5 |  | patch belonging to. |
| zxAnSwPatchVersi | . |  | The version tag of |
| on | 1.3.6.1.4.1.3902.1082.20. |  | the patch. |
|  | 30.2.5.2.1.6 |  |  |
| zxAnSwPatchSize | . |  | The patch file |
|  | 1.3.6.1.4.1.3902.1082.20. |  | length. |
|  | 30.2.5.2.1.7 |  |  |
| zxAnSwPatchBuild | . |  | The time of patch |
| Time | 1.3.6.1.4.1.3902.1082.20. |  | build. For |
|  | 30.2.5.2.1.8 |  | example: yyyy- |
|  |  |  | mm-dd |
|  |  |  | HH:MM:SS.(2001- |
|  |  |  | 01-01 02:22:33). |
| zxAnSwPatchConf | . |  | The configured |
| ActiveStatus | 1.3.6.1.4.1.3902.1082.20. |  | active status of the |
|  | 30.2.5.2.1.9 |  | patch file. |
|  |  |  | activated -- to |
|  |  |  | activate the patch |
|  |  |  | file. deactivated -- |
|  |  |  | to deactivate the |
|  |  |  | patch file. |
| zxAnSwPatchDes | . |  | The patch |
| cription | 1.3.6.1.4.1.3902.1082.20. |  | description. |
|  | 30.2.5.2.1.10 |  |  |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnSwCardRunni | . |  | This table includes |
| ngPatchTable | 1.3.6.1.4.1.3902.1082. |  | the card running |
|  | 20.30.2.5.3 |  | patch information. |
| zxAnSwCardRunni | . |  | An entry in |
| ngPatchEntry | 1.3.6.1.4.1.3902.1082. |  | zxAnSwCardRunni |
|  | 20.30.2.5.3.1 |  | ngPatchTable. |
| zxAnSwCardPatch | . |  | The patch file |
| Name | 1.3.6.1.4.1.3902.1082. |  | name of the card. |
|  | 20.30.2.5.3.1.1 |  |  |
| zxAnSwCardPatch | . |  | The software |
| OwnerSwVersion | 1.3.6.1.4.1.3902.1082. |  | version of the |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 20.30.2.5.3.1.2 |  | patch belonging to. |
| zxAnSwCardPatch Version | . 1.3.6.1.4.1.3902.1082. 20.30.2.5.3.1.3 |  | The version tag of the patch. |
| zxAnSwCardPatch Size | . 1.3.6.1.4.1.3902.1082. 20.30.2.5.3.1.4 |  | The patch file length. |
| zxAnSwCardPatch BuildTime | . 1.3.6.1.4.1.3902.1082. 20.30.2.5.3.1.5 |  | The time of patch build. For example: yyyy- mm-dd HH:MM:SS.(2001- 01-01 02:22:33). |
| zxAnSwCardPatch ActivatedTime | . 1.3.6.1.4.1.3902.1082. 20.30.2.5.3.1.6 |  | The time of the patch activated. For example: yyyy-mm-dd HH:MM:SS.(2001- 01-01 02:22:33). |
| zxAnSwCardPatch Description | . 1.3.6.1.4.1.3902.1082. 20.30.2.5.3.1.7 |  | The patch description. |

| MIB Variable | OID | MIB Value | Description |  |
| --- | --- | --- | --- | --- |
| zxAnSwSubcardR | . |  | This table includes |  |
| unningPatchTable | 1.3.6.1.4.1.3902.1082.20. |  | the subcard patch |  |
|  | 30.2.5.4 |  | information. |  |
| zxAnSwSubcardR | . |  | An entry in |  |
| unningPatchEntry | 1.3.6.1.4.1.3902.1082.20. |  | zxAnSwSubcardR |  |
|  | 30.2.5.4.1 |  | unningPatchTable. |  |
| zxAnSwPatchSub | . |  | The slot number of |  |
| cardSlot | 1.3.6.1.4.1.3902.1082.20. |  | the subcard. |  |
|  | 30.2.5.4.1.1 |  |  |  |
| zxAnSwSubcardP | . |  | The file name of |  |
| atchName | 1.3.6.1.4.1.3902.1082.20. |  | the patch |  |
|  | 30.2.5.4.1.2 |  |  |  |
| zxAnSwSubcardP | . |  | The | software |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| atchOwnerSwVers ion | 1.3.6.1.4.1.3902.1082.20. 30.2.5.4.1.3 |  | version of the patch belonging to. |
| zxAnSwSubcardP atchVersion | . 1.3.6.1.4.1.3902.1082.20. 30.2.5.4.1.4 |  | The version tag of the patch. |
| zxAnSwSubcardP atchSize | . 1.3.6.1.4.1.3902.1082.20. 30.2.5.4.1.5 |  | The patch file length. |
| zxAnSwSubcardP atchBuildTime | . 1.3.6.1.4.1.3902.1082.20. 30.2.5.4.1.6 |  | The time of patch build. For example: yyyy- mm-dd HH:MM:SS.(2001- 01-01 02:22:33). |
| zxAnSwSubcardP atchActivatedTime | . 1.3.6.1.4.1.3902.1082.20. 30.2.5.4.1.7 |  | The time of the patch activated. For example: yyyy-mm-dd HH:MM:SS.(2001- 01-01 02:22:33). |
| zxAnSwSubcardP atchDescription | . 1.3.6.1.4.1.3902.1082.20. 30.2.5.4.1.8 |  | The patch Description. |

# Basic Services

## VLAN

#### Switchport Configuration

[MIB file]:

ZTE-AN-VLAN-MIB.mib

{"zxAnVlanIfConfVlanCmdTable" ,	"1.3.6.1.4.1.3902.1082.40.50.2.1.5"} ,

{"zxAnVlanIfConfVlanCmdEntry" ,	"1.3.6.1.4.1.3902.1082.40.50.2.1.5.1"} ,

{"zxAnVlanIfConfVlanCmd"	, "1.3.6.1.4.1.3902.1082.40.50.2.1.5.1.1"} ,

{"zxAnVlanIfConfVlanList"	,	"1.3.6.1.4.1.3902.1082.40.50.2.1.5.1.2"} ,

#### Serviceport Configuration

[MIB file]:

ZTE-AN-SERVICE-PORT-MIB.mib

{"zxAnServicePort"	,"1.3.6.1.4.1.3902.1082.110.5"} ,

{"zxAnServicePortObjects"	, "1.3.6.1.4.1.3902.1082.110.5.2"} ,

{"zxAnServicePortConfTable"	,	"1.3.6.1.4.1.3902.1082.110.5.2.2"} ,

{"zxAnServicePortConfEntry"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1"} ,

| {"zxAnSrvPortDesc" | , | "1.3.6.1.4.1.3902.1082.110.5.2.2.1.1"}, |
| --- | --- | --- |
| {"zxAnSrvPortAdminStatus" |  | , "1.3.6.1.4.1.3902.1082.110.5.2.2.1.2"}, |
| {"zxAnSrvPortServiceMode" |  | ,"1.3.6.1.4.1.3902.1082.110.5.2.2.1.4"} , |
| {"zxAnSrvPortBrgIfId" | , | "1.3.6.1.4.1.3902.1082.110.5.2.2.1.5"} , |

| {"zxAnSrvPortOnuSlot" | , |  | "1.3.6.1.4.1.3902.1082.110.5.2.2.1.6"}, |
| --- | --- | --- | --- |
| {"zxAnSrvPortOnuUni" | , |  | "1.3.6.1.4.1.3902.1082.110.5.2.2.1.7"} , |
| {"zxAnSrvPortUserVid" | , |  | "1.3.6.1.4.1.3902.1082.110.5.2.2.1.8"} , |
| {"zxAnSrvPortUserSVid" |  | , | "1.3.6.1.4.1.3902.1082.110.5.2.2.1.9"} , |

{"zxAnSrvPortUserCos"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.10"} ,

{"zxAnSrvPortUserStagCos"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.11"} ,

{"zxAnSrvPortStartUserVid"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.12"} ,

{"zxAnSrvPortEndUserVid"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.13"} ,

{"zxAnSrvPortUserEthType"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.14"} ,

{"zxAnSrvPortUserDefinedEthType" , "1.3.6.1.4.1.3902.1082.110.5.2.2.1.15"} ,

{"zxAnSrvPortAllowedEthType" , "1.3.6.1.4.1.3902.1082.110.5.2.2.1.16"} ,

{"zxAnSrvPortAllowedUserDefEthType","1.3.6.1.4.1.3902.1082.110.5.2.2.1.17"},

{"zxAnSrvPortCVid"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.18"} ,

{"zxAnSrvPortSVid"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.19"} ,

{"zxAnSrvPortCtagCos"	,	"1.3.6.1.4.1.3902.1082.110.5.2.2.1.20"} ,

{"zxAnSrvPortStagCos"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.21"} ,

{"zxAnSrvPortTlsSVid"	, " 1.3.6.1.4.1.3902.1082.110.5.2.2.1.22"} ,

{"zxAnSrvPortVlanMerge"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.23"} ,

{"zxAnSrvPortMfdfPrf"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.24"} ,

{"zxAnSrvPortProfileType"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.25"} ,

{"zxAnSrvPortProfileName"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.26"} ,

{"zxAnSrvPortIngressTrafficPrf" , "1.3.6.1.4.1.3902.1082.110.5.2.2.1.27"} ,

{"zxAnSrvPortEgressTrafficPrf" , "1.3.6.1.4.1.3902.1082.110.5.2.2.1.28"} ,

{"zxAnSrvPortQueueId"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.29"} ,

{"zxAnSrvPortQueueMapPrf"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.30"} ,

{"zxAnSrvPortQueueBlockPrf" , "1.3.6.1.4.1.3902.1082.110.5.2.2.1.31"},

{"zxAnSrvPortQueueShapingPrf" , "1.3.6.1.4.1.3902.1082.110.5.2.2.1.32"} ,

{"zxAnSrvPortRowStatus"	, "1.3.6.1.4.1.3902.1082.110.5.2.2.1.50"} ,

## Multicast

[MIB file]:

ZTE-AN-IGMP-MIB.mib

#### Global Management

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpLogEn | 1.3.6.1.4.1.3902.10 | INTEGER | The administrative |
| able | 82.60.1.1.1 | [enabled(1), | status of IGMP log. |
|  |  | disabled(2) ] | Default: disabled |

#### Multicast Protocol Global Data

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpEna | 1.3.6.1.4.1.3902.108 | INTEGER | The administrative |
| ble | 2.60.1.2.1.2.1.1 | [enabled(1),disable | status of global |
|  |  | d(2) ] | IGMP service. |
|  |  |  | Default: enabled |

#### Multicast Management Global Data

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpUserTrackE | 1.3.6.1.4.1.390 | INTEGER | The administrative |
| nable | 2.1082.60.1.2.2 | [enabled(1), | status of IGMP explicit |
|  | .1.2 | disabled(2)] | host tracking. |
|  |  |  | Default: disabled |
| zxAnIgmpBandwidthC | 1.3.6.1.4.1.390 | INTEGER | Enable/disable the |
| trlEnable | 2.1082.60.1.2.2 | [enabled(1), | multicast band-width |
|  | .1.3 | disabled(2)] | control. |
|  |  |  | Default: disabled |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpGroupPrejoi nInterval | 1.3.6.1.4.1.390 2.1082.60.1.2.2 .1.4 | Integer32 [60-300] | The interval of IGMP requests sent by the multicast group which the prejoin is enabled. (Def:120) |
| zxAnIgmpSpanVlanE nable | 1.3.6.1.4.1.390 2.1082.60.1.2.2 .1.5 | INTEGER [enabled(1), disabled(2)] | Enable/disable the Span-VLAN multicast. Default: disabled |

#### Multicast Protocol Port Management

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1 | InterfaceInde |  |
|  | .1 | x |  |
| zxAnSubIfIndex | 1.3.6.1.4.1.3902 | ZxAnSubIfInd | Sub-interface index at |
|  | .1082.30.5.5.1.1 | ex | this interface. |
|  | .2 |  |  |
| zxAnIgmpIfAdminSt | 1.3.6.1.4.1.3902 | INTEGER | The administrative |
| atus | .1082.60.1.2.1.2 | [enabled(1), | status of port's IGMP |
|  | .2.1.1 | drop(3) ] | service. |
|  |  |  | Default: enabled |
| zxAnIgmpIfProtoVer | 1.3.6.1.4.1.3902 | INTEGER | The max version of |
| sion | .1082.60.1.2.1.2 | [igmpv1(1),ig | IGMP protocol which is |
|  | .2.1.2 | mpv2(2),igm | supported on this port. |
|  |  | pv3(3)] | Default: igmpv3 |
| zxAnIgmpIfFastLea | 1.3.6.1.4.1.3902 | INTEGER | The administrative |
| veEnable | .1082.60.1.2.1.2 | [enabled(1), | status of port's fast- |
|  | .2.1.3 | disabled(2) ] | leave property |
|  |  |  | Default: enabled |

#### Multicast Protocol MVLAN Management

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVid | 1.3.6.1.4.1.3902.10 | Integer3 | Multicast VLAN ID. |
|  | 82.60.1.2.1.2.3.1.1 | 2 |  |
|  |  | [1-4094] |  |
| zxAnIgmpMVlanIgm | 1.3.6.1.4.1.3902.10 | INTEGE | The administrative status |
| pAdminStatus | 82.60.1.2.1.2.3.1.2 | R | of IGMP service in this |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  | [enable d(1), drop(3)] | MVLAN. Default: enabled |
| zxAnIgmpMVlanIgm pV1OperMode | 1.3.6.1.4.1.3902.10 82.60.1.2.1.2.3.1.3 | INTEGE R [accept( 1), drop(3)] | The operational mode of IGMP v1 packet. Default: accept |
| zxAnIgmpMVlanIgm pV2OperMode | 1.3.6.1.4.1.3902.10 82.60.1.2.1.2.3.1.4 | INTEGE R[accep t(1), drop(3)] | The operational mode of IGMP v2 packet. Default: accept |
| zxAnIgmpMVlanIgm pV3OperMode | 1.3.6.1.4.1.3902.10 82.60.1.2.1.2.3.1.5 | INTEGE R [accept( 1), drop(3) ] | The operational mode of IGMP v3 packet. Default: accept |
| zxAnIgmpMVlanIgm pWorkMode | 1.3.6.1.4.1.3902.10 82.60.1.2.1.2.3.1.6 | INTEGE R [snoopi ng (1),spr( 2),proxy (3)] | The IGMP work mode in this MVLAN. Default:proxy |
| zxAnIgmpMVlanIgm pSsmCtrlMode | 1.3.6.1.4.1.3902.10 82.60.1.2.1.2.3.1.7 | INTEGE R [asm (1), ssm(2), AsmAn dSsm (3)] | The ASM/SSM service control model. Default: asmAndSsm |
| zxAnIgmpMVlanIgm pSsmIpAddr | 1.3.6.1.4.1.3902.10 82.60.1.2.1.2.3.1.8 | IpAddre ss | The SSM IP address in this MVLAN. All SSM(S, G) request must be in the same IP network segment as the |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | configued SSM IP address and netmask. Default : 'E8000000'H |
| zxAnIgmpMVlanIgm | 1.3.6.1.4.1.3902.10 | IpAddre | The mask of SSM IP |
| pSsmIpAddrMask | 82.60.1.2.1.2.3.1.9 | ss | address in this MVLAN. |
|  |  |  | All SSM(S, G) request |
|  |  |  | must be in the same IP |
|  |  |  | network segment as the |
|  |  |  | configued SSM IP |
|  |  |  | address and netmask. |
|  |  |  | Default: 'FF000000'H |

#### Multicast MVLAN SNOOPING Information

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpSnoopin | 1.3.6.1.4.1.3902.10 | Integer3 | The timeout period in |
| gMVlanAgingTime | 82.60.1.2.1.3.2.1.1 | 2 | seconds for aging out |
|  |  | [30- | multicast group |
|  |  | 3600] | memberships dynamically |
|  |  |  | learned with IGMP. |
|  |  |  | Note that it only takes |
|  |  |  | effect in Snooping mode. |
|  |  |  | Default:300 |

#### Multicast MVLAN Proxy Information

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVlanRobustn essVariable | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.1 | Integer32 [1-7] | IGMP robustness variable in the MVLAN. Default:2 |
| zxAnIgmpMVlanHostVer sion | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.2 | INTEGER auto(1) igmpv2(2) igmpv3(3) | The administrative version of IGMP packet sent on the host interface. Default: auto |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVlanHostIpA ddr | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.3 | IpAddress | The source IP address of IGMP packet sent on the host interface. Default: 'C0A8020E'H |
| zxAnIgmpMVlanUnsolRe portInterval | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.4 | Integer32 [1-300] | IGMP unsolicited report interval in the MVLAN. Def: 1 |
| zxAnIgmpMVlanHostQue rier | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.5 | IpAddress | The IP address of the IGMP Querier attached on the host interface which is learned from received IGMP general query. |
| zxAnIgmpMVlanHostAct ualVersion | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.6 | INTEGER [igmpv1(1), igmpv2(2), igmpv3(3)] | The running version of IGMP packet sent on the host interface. Def: igmpv3 |
| zxAnIgmpMVlanHostV1 QuerierTimer | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.7 | TimeTicks | The time remaining until the host assumes that there are no IGMPv1 routers present on the interface. Def: 0 |
| zxAnIgmpMVlanHostV2 QuerierTimer | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.8 | TimeTicks | The time remaining until the host assumes that there are no IGMPv2 routers present on the interface. Def: 0 |
| zxAnIgmpMVlanRouterIp Addr | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.9 | IpAddress | The source IP address of IGMP query packet sent on the router interface. Def: 'C0A8020E'H |
| zxAnIgmpMVlanQueryInt erval | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.10 | Integer32 [60-300] | IGMP general query interval in the MVLAN. Def: 125 |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVlanQueryRe spInterval | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.11 | Integer32 [10-250] | IGMP query max response time in the MVLAN. Def:100 |
| zxAnIgmpMVlanLastMe mberQryIntvl | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.12 | Integer32 [1-255] | IGMP last member query interval in the MVLAN. Def:10 |
| zxAnIgmpMVlanLastMe mberQryCount | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.13 | Integer32 [2-5] | IGMP last member query count in the MVLAN. Def: 2 |
| zxAnIgmpMVlanStartUp QryInterval | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.14 | Integer32 [1-100] | IGMP startup query interval in the MVLAN. When system is start up or port is up. Def:30 |
| zxAnIgmpMVlanStartUp QryCount | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.15 | Integer32 [0-10] | IGMP startup query count in the MVLAN. Def:2 |
| zxAnIgmpMVlanIgmpPac ketPriority | 1.3.6.1.4.1.39 02.1082.60.1. 2.1.4.2.1.16 | Integer32 [0-7] | 802.1p priority of IGMP packet sent by the device.Note that it only takes effect in proxy and SPR mode. Def:0 |

#### Multicast Management MVLAN Table

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVlanGr | 1.3.6.1.4.1.3902.108 | INTEGER | The administrative |
| oupFilterEnable | 2.60.1.2.2.2.1.1 | [enabled( | status of group filter in |
|  |  | 1), | the MVLAN. |
|  |  | disabled( | Def:2 |
|  |  | 2)] |  |
| zxAnIgmpMVlanRo | 1.3.6.1.4.1.3902.108 | RowStatu |  |
| wStatus | 2.60.1.2.2.2.1.50 | s |  |

#### Multicast Management Receiving Port List

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMgmtMVl | 1.3.6.1.4.1.3902.108 | RowStatu |  |
| anRecvIfRowStatus | 2.60.1.2.2.3.1.50 | s |  |

#### Multicast Management Source Port List

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVlanSrc | 1.3.6.1.4.1.3902.108 | RowStatu |  |
| IfRowStatus | 2.60.1.2.2.4.1.50 | s |  |

#### Multicast MVLAN Receiving Port List

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVid | 1.3.6.1.4.1.3902. | Integer32 | Multicast VLAN ID. |
|  | 1082.60.1.2.1.2. | [1-4094] |  |
|  | 3.1.1 |  |  |
| zxAnIgmpMVlanPor | 1.3.6.1.4.1.3902. | Integer32 | The shelf that contains |
| tListShelf | 1082.60.1.2.2.5. |  | the receive port list. |
|  | 1.1 |  |  |
| zxAnIgmpMVlanPor | 1.3.6.1.4.1.3902. | Integer32 | The slot that contains |
| tListSlot | 1082.60.1.2.2.5. |  | the receive port list. |
|  | 1.2 |  |  |
| zxAnIgmpMVlanRe | 1.3.6.1.4.1.3902. | ZxAnPortList | The receive port list in |
| cvPortList | 1082.60.1.2.2.5. |  | the MVLAN. |
|  | 1.3 |  |  |

#### Multicast Group Filter Table

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVid | 1.3.6.1.4.1.3902.108 | Integer32 | Multicast VLAN ID. |
|  | 2.60.1.2.1.2.3.1.1 | [1-4094] |  |
| zxAnIgmpGroupFilt | 1.3.6.1.4.1.3902.108 | IpAddress | Start group ip |
| erStartGrpIp | 2.60.1.2.2.6.1.1 |  | address of a group |
|  |  |  | segment. |
| zxAnIgmpGroupFilt | 1.3.6.1.4.1.3902.108 | IpAddress | End group ip |
| erEndGrpIp | 2.60.1.2.2.6.1.2 |  | address of a group |
|  |  |  | segment. |
| zxAnIgmpGroupFilt | 1.3.6.1.4.1.3902.108 | RowStatus |  |
| erRowStatus | 2.60.1.2.2.6.1.50 |  |  |

#### Multicast Group Static Port List

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVid | 1.3.6.1.4.1.3902.10 82.60.1.2.1.2.3.1.1 | Integer32 | Multicast VLAN ID. [1-4094] |
| zxAnIgmpGroupIpAd dr | 1.3.6.1.4.1.3902.10 82.60.1.2.2.9.1.1 | IpAddress | Multicast group IP Address. |
| zxAnIgmpSourceIpAd dr | 1.3.6.1.4.1.3902.10 82.60.1.2.2.9.1.2 | IpAddress | Multicast source IP Address. |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | InterfaceInd ex |  |
| zxAnSubIfIndex | 1.3.6.1.4.1.3902.10 82.30.5.5.1.1.2 | ZxAnSubIfI ndex |  |
| zxAnIgmpGroupStati cIfRowStatus | 1.3.6.1.4.1.3902.10 82.60.1.2.2.7.1.50 | RowStatus |  |

#### Multicast Management Source Filter Mode Table

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVid | 1.3.6.1.4.1.3902.10 | Integer32 | Multicast VLAN ID. |
|  | 82.60.1.2.1.2.3.1.1 |  | [1-4094] |
| zxAnIgmpGroupIpAd | 1.3.6.1.4.1.3902.10 | IpAddress | Multicast group IP |
| dr | 82.60.1.2.2.9.1.1 |  | Address. |
| zxAnIgmpSourceIpAd | 1.3.6.1.4.1.3902.10 | IpAddress | Multicast source IP |
| dr | 82.60.1.2.2.9.1.2 |  | Address. |
| zxAnIgmpGroupSrcFi | 1.3.6.1.4.1.3902.10 | INTEGER | IGMPv3 source filter |
| lterMode | 82.60.1.2.2.8.1.1 | [include | mode of the group. |
|  |  | ( 1 ) , |  |
|  |  | exclude |  |
|  |  | ( 2 )] |  |

#### Multicast Group Forwarding Table

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVid | 1.3.6.1.4.1.3902.1082.6 0.1.2.1.2.3.1.1 | Integer32 [1-4094] | Multicast VLAN ID. |
| zxAnIgmpGroupI pAddr | 1.3.6.1.4.1.3902.1082.6 0.1.2.2.9.1.1 | IpAddress | Multicast group IP Address. |
| zxAnIgmpSource IpAddr | 1.3.6.1.4.1.3902.1082.6 0.1.2.2.9.1.2 | IpAddress | Multicast source IP Address. |
| zxAnIgmpShelf | 1.3.6.1.4.1.3902.1082.6 | Integer32 | The shelf that |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 0.1.2.2.9.1.3 |  | contains the port list. |
| zxAnIgmpSlot | 1.3.6.1.4.1.3902.1082.6 0.1.2.2.9.1.4 | Integer32 | The slot that contains the port list. |
| zxAnIgmpGroup ForwardingPortLi st | 1.3.6.1.4.1.3902.1082.6 0.1.2.2.9.1.6 | ZxAnPortLi st | The forwarding port list on a line card. In GPON, the forwarding port is PON interface. |

#### Multicast Group User Table

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVid | 1.3.6.1.4.1.3902.10 82.60.1.2.1.2.3.1.1 | Integer32 [1-4094] | Multicast VLAN ID. |
| zxAnIgmpGroupIpA ddr | 1.3.6.1.4.1.3902.10 82.60.1.2.2.9.1.1 | IpAddress | Multicast group IP Address. |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | InterfaceInd ex |  |
| zxAnSubIfIndex | 1.3.6.1.4.1.3902.10 82.30.5.5.1.1.2 | ZxAnSubIfIn dex |  |
| zxAnIgmpSourceIp Addr | 1.3.6.1.4.1.3902.10 82.60.1.2.2.9.1.2 | IpAddress | Multicast source IP Address. |
| zxAnIgmpUserMac Addr | 1.3.6.1.4.1.3902.10 82.60.1.2.2.10.1.1 | MacAddres s | The user MAC address which is tracked from the IGMP request packets sent by user device. Note that |
| zxAnIgmpUserIpAd dr | 1.3.6.1.4.1.3902.10 82.60.1.2.2.10.1.2 | Integer32 | The slot that contains the port list. |
| zxAnIgmpGrpUserS rcFilterMode | 1.3.6.1.4.1.3902.10 82.60.1.2.2.10.1.3 | INTEGER [include ( 1 ) , exclude ( 2 )] | IGMPv3 source filter mode of the group user |

#### Multicast Group Prejoin Table

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVid | 1.3.6.1.4.1.3902.108 2.60.1.2.1.2.3.1.1 | Integer32 | Multicast VLAN ID. [1-4094] |
| zxAnIgmpGroupIpAd dr | 1.3.6.1.4.1.3902.108 2.60.1.2.2.9.1.1 | IpAddress | Multicast group IP Address. |
| zxAnIgmpSourceIpA ddr | 1.3.6.1.4.1.3902.108 2.60.1.2.2.9.1.2 | IpAddress | Multicast source IP Address. |
| zxAnIgmpGroupPrej oinRowStatus | 1.3.6.1.4.1.3902.108 2.60.1.2.2.11.1.50 | RowStatu s |  |

#### Multicast Group Configuration Table

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVid | 1.3.6.1.4.1.3902.108 | Integer3 | Multicast VLAN ID. |
|  | 2.60.1.2.1.2.3.1.1 | 2 | [1-4094] |
| zxAnIgmpGroupIpAd | 1.3.6.1.4.1.3902.108 | IpAddre | Multicast group IP |
| dr | 2.60.1.2.2.9.1.1 | ss | Address. |
| zxAnIgmpSourceIpA | 1.3.6.1.4.1.3902.108 | IpAddre | Multicast source IP |
| ddr | 2.60.1.2.2.9.1.2 | ss | Address. |
| zxAnIgmpGroupBan | 1.3.6.1.4.1.3902.108 | Integer3 | Multicast group |
| dwidth | 2.60.1.2.2.12.1.1 | 2 | bandwidth. |
|  |  |  | [100-65535] Def:2048 |
| zxAnIgmpGroupConf | 1.3.6.1.4.1.3902.108 | RowStat |  |
| RowStatus | 2.60.1.2.2.12.1.50 | us |  |

#### Multicast Packet Statistics Table

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpPktStatsRxC | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| ommQry | 082.60.1.2.50.2.1. | 32 | general query message |
|  | 1 |  | received. |
| zxAnIgmpPktStatsRxS | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| pecialQry | 082.60.1.2.50.2.1. | 32 | special query message |
|  | 2 |  | received. |
| zxAnIgmpPktStatsRxV | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| 1Report | 082.60.1.2.50.2.1. | 32 | v1 report message |
|  | 3 |  | received. |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpPktStatsRxV | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| 2Report | 082.60.1.2.50.2.1. | 32 | v2 report message |
|  | 4 |  | received. |
| zxAnIgmpPktStatsRxV | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| 3Report | 082.60.1.2.50.2.1. | 32 | v3 report message |
|  | 5 |  | received. |
| zxAnIgmpPktStatsRxLe | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| ave | 082.60.1.2.50.2.1. | 32 | leave message |
|  | 6 |  | received. |
| zxAnIgmpPktStatsRxEr | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| ror | 082.60.1.2.50.2.1. | 32 | error message |
|  | 7 |  | received. |
| zxAnIgmpPktStatsRxU | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| nknown | 082.60.1.2.50.2.1. | 32 | unknown message |
|  | 8 |  | received. |
| zxAnIgmpPktStatsTxC | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| ommQry | 082.60.1.2.50.2.1. | 32 | general query message |
|  | 9 |  | transmitted. |
| zxAnIgmpPktStatsTxSp | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| ecialQry | 082.60.1.2.50.2.1. | 32 | special query message |
|  | 10 |  | transmitted. |
| zxAnIgmpPktStatsTxV1 | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| Report | 082.60.1.2.50.2.1. | 32 | v1 report message |
|  | 11 |  | transmitted. |
| zxAnIgmpPktStatsTxV2 | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| Report | 082.60.1.2.50.2.1. | 32 | v2 report message |
|  | 12 |  | transmitted. |
| zxAnIgmpPktStatsTxV3 | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| Report | 082.60.1.2.50.2.1. | 32 | v3 report message |
|  | 13 |  | transmitted. |
| zxAnIgmpPktStatsTxLe | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| ave | 082.60.1.2.50.2.1. | 32 | leave message |
|  | 14 |  | transmitted. |
| zxAnIgmpPktStatsDrop | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| RxCommQry | 082.60.1.2.50.2.1. | 32 | general query message |
|  | 15 |  | dropped. |
| zxAnIgmpPktStatsDrop | 1.3.6.1.4.1.3902.1 | Counter | The counter of IGMP |
| RxSpeQry | 082.60.1.2.50.2.1. | 32 | special query message |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 16 |  | dropped. |
| zxAnIgmpPktStatsDrop RxV1Report | 1.3.6.1.4.1.3902.1 082.60.1.2.50.2.1. 17 | Counter 32 | The counter of IGMP v1 report message dropped. |
| zxAnIgmpPktStatsDrop RxV2Report | 1.3.6.1.4.1.3902.1 082.60.1.2.50.2.1. 18 | Counter 32 | The counter of IGMP v2 report message dropped. |
| zxAnIgmpPktStatsDrop RxV3Report | 1.3.6.1.4.1.3902.1 082.60.1.2.50.2.1. 19 | Counter 32 | The counter of IGMP v3 report message dropped. |
| zxAnIgmpPktStatsDrop RxLeave | 1.3.6.1.4.1.3902.1 082.60.1.2.50.2.1. 20 | Counter 32 | The counter of IGMP leave message dropped. |
| zxAnIgmpPktStatsDrop RxUnknown | 1.3.6.1.4.1.3902.1 082.60.1.2.50.2.1. 21 | Counter 32 | The counter of IGMP unknown message dropped. |
| zxAnIgmpPktStatsJoin Success | 1.3.6.1.4.1.3902.1 082.60.1.2.50.2.1. 22 | Counter 32 | The counter of successful IGMP joins. |
| zxAnIgmpPktStatsJoin Failure | 1.3.6.1.4.1.3902.1 082.60.1.2.50.2.1. 23 | Counter 32 | The counter of failure IGMP joins. |
| zxAnIgmpPktStatsRese t | 1.3.6.1.4.1.3902.1 082.60.1.2.50.2.1. 100 | Integer3 2 | The action object to reset the statistics values. When reset the statistics, it must be set to '1'. When be queried, it always returns '0'. |

#### Multicast Port Statistics Table

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1. 1 | InterfaceInde x |  |
| zxAnSubIfIndex | 1.3.6.1.4.1.3902. 1082.30.5.5.1.1.2 | ZxAnSubIfInd ex |  |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpIfCurrActGro | 1.3.6.1.4.1.3902. | Gauge32 | The active |
| ups | 1082.60.1.2.50.3. |  | multicast group |
|  | 1.1 |  | number on this |
|  |  |  | interface. |

#### Multicast MVLAN Statistics Table

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIgmpMVlanCurrA | 1.3.6.1.4.1.3902. | Gauge32 | The active |
| ctGroups | 1082.60.1.2.50.4. |  | multicast group |
|  | 1.1 |  | number in the |
|  |  |  | MVLAN. |

## Port Positioning

[MIB file]:

ZTE-AN-PORT-IDENTIFICATION-MIB.mib

ZTE-AN-PORT-IDENTIFICATION-PROFILE-MIB.mib ZTE-AN-DHCP-L2-RELAY-AGENT-MIB.mib

ZTE-AN-NDP-LIO-MIB.mib

ZTE-AN-PPPOE-INTERMEDIATE-AGENT-MIB.mib ZTE-AN-PORT-IDENTIFICATION-PROFILE-MIB.mib

#### DHCP Option82 Global Swtich

[Function description]:

This MIB enables and disables the DHCP Option82 packet extraction function. If it is disabled, the driver will not extract DHCP Option82 packets. If it is enabled, the driver will extract DHCP Option82 packets.

[Index description]:

[MIB variable description]:

| MIB Variable | OID | MIB Value | Descriptio n |
| --- | --- | --- | --- |
| zxAnDhcpV4L2RA Enable | 1.3.6.1.4.1.3902.1082.7 0.60.2.1.1.1 | INTEGER {enable ( 1 ) , disable ( 2 )} | Default value: 2 |

#### DHCP Option82 Port Switch

[Function description]:

This MIB enables and disables the DHCP Option82 port swtich. When the swtich is disabled, the port will not process the port positioning information contained in DHCP Option82 packets. When the swtich is enabled, the port will process the port positioning information contained in DHCP Option82 packets.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnDhcpV4L2R | 1.3.6.1.4.1.3902.108 | INTEGER | Default value: 2 |
| AIfConfEnable | 2.70.60.2.1.3.1.1 | {enable ( 1 ) , |  |
|  |  | disable ( 2 ) } |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### DHCP Option82 Port Trust Rule

[Function description]:

This MIB defines the port trust status. “True” indicates that the port positioning information contained in the upstream packets is trusted. “False” indicates that the port positioning information contained in the upstream packets is not trusted.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnDhcpV4L2RAIfC | 1.3.6.1.4.1.3902.1082.70. | INTEGER | Default |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| onfTrust | 60.2.1.3.1.2 | {true ( 1 ) , false ( 2 ) } | value: 2 |

#### DHCP Option82 Port Trust Measure

[Function description]:

This MIB defines the DHCP Option82 port trust measures. When the trust status is “True”, the trust measures can be configured as “Keep” or “Replace”. When the trust status is “False”, the trust measures can be configured as “Discard” or “Add”. “Keep” indicates keeping the port positioning information contained in the upstream packets. “Replace” indicates replacing the port positioning information contained in the upstream packets. “Discard” indicates dropping the upstream packets. “Add” indicates adding other port positioning information.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnDhcpV4L2RAIfC | 1.3.6.1.4.1.3902.1082. | INTEGER | Default |
| onfPolicy | 70.60.2.1.3.1.3 | { keep ( 1 ) , replace | value: 4 |
|  |  | ( 2 ) , discard ( 3 ) , |  |
|  |  | add ( 4 ) } |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### DHCP Option18 Global Swtich

[Function description]:

This MIB enables and disables global DHCPv6-option18 packet extraction function. When it is disabled, the driver will not extract DHCPv6-option18 packets. When it is enabled, the driver will extract DHCPv6-option18 packets.

[Index description]:

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnDhcpV6L2R | 1.3.6.1.4.1.3902.10 | INTEGER { enable | Default value: 2 |

| AEnable | 82.70.60.2.2.1.1 | ( 1 ) , disable ( 2 ) } |  |
| --- | --- | --- | --- |

#### DHCP Option18 Port Switch

[Function description]:

This MIB enables and disables the DHCPv6 port switch. When it is disabled, the port will not process the port positioning information in the DHCPv6 packets. When it is enabled, the port will process the port positioning information in the DHCPv6 packets.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnDhcpV6L2RAIf | 1.3.6.1.4.1.3902.10 | INTEGER | Default value: 2 |
| ConfEnable | 82.70.60.2.2.3.1.1 | { enable ( 1 ) , |  |
|  |  | disable ( 2 ) } |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### DHCP Option18 Port Trust Status

[Function description]:

This MIB defines the port trust status. “True” indicates that the port positioning information contained in the upstream packets is trusted. “False” indicates that the port positioning information contained in the upstream packets is not trusted.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnDhcpV6L2R | 1.3.6.1.4.1.3902.10 | INTEGER | Default value: 2 |
| AIfConfTrust | 82.70.60.2.2.3.1.2 | {true ( 1 ) , false |  |
|  |  | ( 2 ) } |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### DHCP Option18 Port Trust Measure

[Function description]:

This MIB defines the DHCP Option18 port trust measures. When the trust status is “True”, the trust measures can be configured as “Keep” or “Replace”. When the trust status is “False”, the trust measures can be configured as “Discard” or “Add”. “Keep” indicates keeping the port positioning information contained in the upstream packets. “Replace” indicates replacing the port positioning information contained in the upstream packets. “Discard” indicates dropping the upstream packets. “Add” indicates adding other port positioning information.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnDhcpV6L2RAIf | 1.3.6.1.4.1.3902.10 | INTEGER | Default value: 4 |
| ConfPolicy | 82.70.60.2.2.3.1.3 | { keep ( 1 ) , |  |
|  |  | replace ( 2 ) , |  |
|  |  | discard ( 3 ) , add ( |  |
|  |  | 4 ) } |  |

Platform composite index type 4. [MIB variable description]:

#### PPPoE Global Switch

[Function description]:

This MIB enables and disables the global PPPoE packet extraction function. If it is disabled, the driver will not extract PPPoE packets. If it is enabled, the driver will extract PPPoE packets.

[Index description]:

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPppoeIAEnabl | 1.3.6.1.4.1.3902.1082.70.55. | INTEGER | Default value: |
| e | 1.1 | { enable ( 1 ) , | 2 |
|  |  | disable ( 2 ) } |  |

#### PPPoE Port Swtich

[Function description]:

This MIB enables and disables the PPPoE port swtich. When the switch is disabled, the port will not process the port positioning information contained in PPPoE packets. When the switch is enabled, the port will process the port positioning information contained in PPPoE packets.

[Index description]:

| MIB Variable | OID | MIB Value | Descripti on |
| --- | --- | --- | --- |
| zxAnPppoeIAIfConfEnab | 1.3.6.1.4.1.3902.1082.7 | INTEGER | Default |
| le | 0.55.2.2.1.1 | {enable (1), | value: 2 |
|  |  | disable ( 2 ) } |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### PPPoE Port Trust Status

[Function description]:

This MIB defines the PPPoE port trust status. “True” indicates that the port positioning information contained in the upstream packets is trusted. “False” indicates that the port positioning information contained in the upstream packets is not trusted.

[Index description]:

| MIB Variable | OID | MIB Value | Descripti on |
| --- | --- | --- | --- |
| zxAnPppoeIAIfConfT | 1.3.6.1.4.1.3902.1082.7 | INTEGER | Default |
| rust | 0.55.2.2.1.2 | { true (1) , false (2) | value: 2 |
|  |  | } |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### PPPoE Port Trust Measure

[Function description]:

This MIB defines the PPPoE port trust measures. When the trust status is “True”, the trust measures can be configured as “Keep” or “Replace”. When the trust status is

“False”, the trust measures can be configured as “Discard” or “Add”. “Keep” indicates keeping the port positioning information contained in the upstream packets. “Replace” indicates replacing the port positioning information contained in the upstream packets. “Discard” indicates dropping the upstream packets. “Add” indicates adding other port positioning information.

[Index description]:

| MIB Variable | OID | MIB Value | Descripti on |
| --- | --- | --- | --- |
| zxAnPppoeIAIfConfPol | 1.3.6.1.4.1.3902.1082. | INTEGER | Default |
| icy | 70.55.2.2.1.3 | {keep (1), replace | value: 4 |
|  |  | (2) , discard (3) , |  |
|  |  | add (4) } |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### ND Global Switch

[Function description]:

This MIB enables and disables the global ND packet extraction switch. When the switch is disabled, the driver will not extract ND packets. When the switch is enabled, the driver will extract ND packets.

[Index description]:

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnNdpLioEnable | 1.3.6.1.4.1.3902.1082.70.65.1. | INTEGER | Default |
|  | 1 | { enable ( 1 ) , | value: 2 |
|  |  | disable ( 2 ) } |  |

#### ND Port Switch

[Function description]:

This MIB enables and disables the ND port switch. When the switch is disabled, the port will not process the port positioning information in ND packets. When the switch is enabled, the port will process the port positioning information in ND packets.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnNdpLioIfConfEnabl | 1.3.6.1.4.1.3902.1082.70. | INTEGER | Default value: |
| e | 65.2.2.1.1 | {enable(1) , | 2 |
|  |  | disable(2) } |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### ND Port Trust Status

[Function description]:

This MIB defines the ND port trust status. “True” indicates that the port positioning information contained in the upstream packets is trusted. “False” indicates that the port positioning information contained in the upstream packets is not trusted.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnNdpLioIfConfTru | 1.3.6.1.4.1.3902.108 | INTEGER | Default value: |
| st | 2.70.65.2.2.1.2 | {true(1),false(2) | 2 |
|  |  | } |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### ND Port Trust Measure

[Function description]:

This MIB defines the ND port trust measures. When the trust status is “True”, the trust measures can be configured as “Keep” or “Replace”. When the trust status is “False”, the trust measures can be configured as “Discard” or “Add”. “Keep” indicates keeping the port positioning information contained in the upstream packets. “Replace” indicates replacing the port positioning information contained in the upstream packets. “Discard” indicates dropping the upstream packets. “Add” indicates adding other port positioning information.

[Index description]:

| MIB Variable | OID | MIB Value | Descriptio n |
| --- | --- | --- | --- |
| zxAnNdpLioIfConfPolic | 1.3.6.1.4.1.3902.1082.7 | INTEGER | Default |
| y | 0.65.2.2.1.3 | {keep ( 1 ) , | value: 4 |
|  |  | replace ( 2 ) , |  |
|  |  | discard ( 3 ) , add |  |
|  |  | ( 4 ) } |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### NE Access Identifier Configuration

[Function description]:

This MIB configures the NE access type. When the NE access type is inband Mac, the NE access identifier uses the NE inband MAC address. When the NE access type is accessNodeName, the NE access identifier uses the custom string.

[Index description]:

[MIB variable description]:

| MIB Variable | OID | MIB Value | Descripti on |
| --- | --- | --- | --- |
| zxAnPortIdAccessNo | 1.3.6.1.4.1.3902.1082.7 | INTEGER | Default v |
| deIdType | 0.50.1.2 | { inbandMac(1),a | alue: 1 |
|  |  | ccessNodeName( |  |
|  |  | 2)} |  |

#### User-Defined NE Access Identifier Configuration

[Function description]:

This MIB defines string-type access identifier for an NE. [Index description]:

[MIB variable description]:

| MIB Variable | OID | MIB Value | Descripti on |
| --- | --- | --- | --- |
| zxAnPortIdAccessNode | 1.3.6.1.4.1.3902.1082.70.5 | DisplayStrin |  |
| Name | 0.1.1 | g |  |
|  |  | (SIZE(0.. 50) |  |
|  |  | ) |  |

#### NE Rack ID Configuration

[Function description]:

This MIB configures the NE rack ID in the port positioning information. [Index description]:

[MIB variable description]:

| MIB Variable | OID | MIB Value | Descriptio n |
| --- | --- | --- | --- |
| zxAnPortIdRac | 1.3.6.1.4.1.3902.1082.70.50. | INTEGER{0…… |  |
| k | 1.4 | 63} |  |

#### NE Shelf ID Configuration

[Function description]:

This MIB configures the NE shelf ID in the port positioning information. [Index description]:

[MIB variable description]:

| MIB Variable | OID | MIB Value | Descriptio n |
| --- | --- | --- | --- |
| zxAnPortIdShelf | 1.3.6.1.4.1.3902.1082.70.5 | INTEGER{0…… |  |
|  | 0.1.5 | 31} |  |

#### DHCP Option82 Port Rate Configuration

[Function description]:

This MIB configures the DHCP Option82 port rate from subopt81 to subopt8e.

subopt81 Actual upstream rate subopt82 Actual downstream rate subopt83 Minimum upstream rate subopt84 Minimum downstream rate subopt85 Attainable upstream rate subopt86 Attainable downstream rate subopt87 Maximum upstream rate subopt88 Maximum downstream rate

subopt89 Minimum upstream rate in low power consumption mode subopt8a Minimum downstream rate in low power consumption mode subopt8b Maximum interleaving upstream delay

subopt8c	Actual interleaving upstream delay subopt8d Maximum interleaving downstream delay subopt8e Actual interleaving downstream delay [Index description]:

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdDhcpV4 | 1.3.6.1.4.1.3902.10 | BITS |  |
| AccessLoopChar | 82.70.50.1.6 | {suboption81(0), |  |
|  |  | suboption82(1), |  |
|  |  | suboption83(2), |  |
|  |  | suboption84(3), |  |
|  |  | suboption85(4), |  |
|  |  | suboption86(5), |  |
|  |  | suboption87(6), |  |
|  |  | suboption88(7), |  |
|  |  | suboption89(8), |  |
|  |  | suboption8A(9), |  |
|  |  | suboption8B(10), |  |

|  |  | suboption8C(11), suboption8D(12), suboption8E(13) } |  |
| --- | --- | --- | --- |

#### PPPoE Port Rate Configuration

[Function description]:

This MIB configures the DHCP Option82 port rate from subopt81 to subopt8e. subopt81 Actual upstream rate

subopt82 Actual downstream rate subopt83 Minimum upstream rate subopt84 Minimum downstream rate subopt85 Attainable upstream rate subopt86 Attainable downstream rate subopt87 Maximum upstream rate subopt88 Maximum downstream rate

subopt89 Minimum upstream rate in low power consumption mode subopt8a Minimum downstream rate in low power comsumption mode subopt8b Maximum interleaving upstream delay

subopt8c	Actual interleaving upstream delay subopt8d Maximum interleaving downstream delay subopt8e Actual interleaving downstream delay [Index description]:

[MIB variable description]:

| MIB Variable | OID | MIB Value | Descriptio n |
| --- | --- | --- | --- |
| zxAnPortIdPppoeA | 1.3.6.1.4.1.3902.1082.7 | BITS |  |

| MIB Variable | OID | MIB Value | Descriptio n |
| --- | --- | --- | --- |
| ccessLoopChar | 0.50.1.7 | {suboption81(0), suboption82(1), suboption83(2), suboption84(3), suboption85(4), suboption86(5), suboption87(6), suboption88(7), suboption89(8), suboption8A(9), suboption8B(10), suboption8C(11), suboption8D(12), suboption8E(13) } |  |

#### Port Positioning Format Configuration

[Function description]:

This MIB configures the port positioning format. [Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdIfConfFor | 1.3.6.1.4.1.39 | INTEGER |  |
| mat | 02.1082.70.5 | {chinaTel(1),dslForum(2), |  |
|  | 0.2.1.1.1 | chinaNet(3),flexibleSynta |  |
|  |  | x(8)} |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### Port Positioning Format Profile Configuration

[Function description]:

This MIB configures the port positioning format profile. [Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdIfConfForm | 1.3.6.1.4.1.3902. | DisplayString |  |
| atProfile | 1082.70.50.2.1.1. | ( SIZE ( 1 .. 32 ) |  |
|  | 6 | ) |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### Port Positioning access-loop-tag Switch

[Function description]:

This MIB enables and disables the acess-loop-tag switch in the port positioning information. When the switch is disabled, the port link rate information cannot be addeded in the packets. When the switch id enabled, the port link rate information can be added in the packets.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdIfConfAlcEna | 1.3.6.1.4.1.3902.10 | INTEGER |  |
| ble | 82.70.50.2.1.1.8 | { enable ( 1 ) |  |
|  |  | , disable ( 2 ) |  |
|  |  | } |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### Port Positioning Remote ID Switch

[Function description]:

This MIB enables and disables the port positioning RID switch. When the switch is diabled, the RID cannot be inserted in the packets. When the switch is enabled, the RID can be inserted in the packets. While, when the RID protocol switch is disabled, the RID cannot be added in the RID protocol but in other protocols.

[Index description]:

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2).

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdIfConfRidEna | 1.3.6.1.4.1.3902.1 | INTEGER |  |
| ble | 082.70.50.2.1.1.2 | { enable ( 1 ) , |  |
|  |  | disable ( 2 ) } |  |

#### Port Positioning Remote ID Protocol Swtich

[Function description]:

This MIB enables and disables the RID protocol switch. When the RID is disabled, the RID cannot be cannot be inserted in the packets. When the RID is enabled and the corresponding protocol switch is enabled, the RID can be inserted in the packets. While, when the corresponding protocol switch is disabled, the RID cannot be inserted in the protocol.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdIfConfRidProtoc | 1.3.6.1.4.1.3902. | BITS |  |
| ol | 1082.70.50.2.1.1. | {pppoe(0), |  |
|  | 4 | dhcpv4(1), |  |
|  |  | dhcpv6(2)} |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### Port Positioning RemotID Content

[Function description]:

This MIB defines the port RID composed of one to 64 characters. The priority level of the port RID format profile is higher than the port RID content. If both of them are configured, the port RID format profile can be used as the RID content.

[Index description]:

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdIfConfR | 1.3.6.1.4.1.3902.108 | DisplayString |  |
| id | 2.70.50.2.1.1.3 | ( SIZE ( 0 .. 64 ) ) |  |

#### Port Positioning RemotID Format Profile

[Function description]:

This MIB configures the specific RID content. Each RID corresponds to one format profile and the priority level of the port RID format profile is higher than the port RID content. If both of them are configured, the port RID format profile can be used as the RID content.

[Index description]:

| MIB Variable | OID | MIB Value | Descr iption |
| --- | --- | --- | --- |
| zxAnPortIdIfConfRidF | 1.3.6.1.4.1.3902.1082.70.5 | DisplayString |  |
| ormatProfile | 0.2.1.1.7 | ( SIZE ( 0 .. |  |
|  |  | 32 ) ) |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### Port Positioning Mode Configuration

[Function description]:

This MIB configures whether to use operator’s mode or common mode for port positioning. By default, the port positioning mode is common mode.

[Index description]:

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdSystemConfMo | 1.3.6.1.4.1.3902.1082. | INTEGER |  |
| de | 70.67.1.1 | { 1: normal(1) |  |
|  |  | 2: profile(2)} |  |

#### Operator Profile Global Application

[Function description]:

This MIB configures the global application of the opeator’s profile. [Index description]:

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdGlobalOperat | 1.3.6.1.4.1.3902.10 | DisplayString |  |
| orPrf | 82.70.67.1.2 | ( SIZE ( 0 .. |  |
|  |  | 32 ) ) |  |

#### Format Profile Configuration

[Function description]:

This MIB views, creates, or deletes a format profile, which is a configuration profile used for configuring the port positioning information.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdFormatPrfNam e | 1.3.6.1.4.1.3902.10 82.70.67.2.2.1.1 | DisplayString ( SIZE ( 1 .. 32 ) ) |  |
| zxAnPortIdFormatPrfRow Status | 1.3.6.1.4.1.3902.10 82.70.67.2.2.1.50 | RowStatus |  |

It is a custom index composed of 1 to 32 characters. [MIB variable description]:

#### Format Profile Content Configuration

[Function description]:

This MIB configures the format profile contents. It adds separator, variable, and character string. The component type is determined by “zxAnPortIdCidComponentType”, among which, “standardVar” and “extendedVar” are variables. “zxAnPortIdCidComponentId” defines the variable type. “separator” is the separator and can only be added with spaces. “userDefinedString” is character string.

The variables corresponding to zxAnPortIdCidComponentId are as follows: access-mode	User access mode

access-node-id	User identifier. It can be the inband MAC or a self-defined by the user.

access-node-Type User access mode chassis	Value: 0

cvlan	User access internal VLAN ethernet-priority	User access link priority level frame	User access physical frame ID

gem-port	User access PON port in GPON access mode

loid	Identifier sent by the ONU

loidpw	Identifier sent by the ONU

onu-id	ONU ID in PON access mode

onu-info	ONU configuration information in PON access mode

onu-number	ONU ID in PON access mode

port	User access port ID

q-vid	VLAN before conversion in user access

rack	User access rack ID

slot	User access slot ID

sn	ONU SN value in PON access mode

sub-slot	Value: 0

svlan	User access external vlan

vci	Link VCI in ATM access mode

vpi	Link VPI in ATM access mode

vport	Link VPORT in PON access mode

xci	For ATM access mode, it is VCI; For ETH access mode, it is 0.

xpi	For ATM access mode, it is VPI; For ETH access mode, it is 0. [Index description]:

This index includes zxAnPortIdFormatPrfName and zxAnPortIdCidComponentIndex. There is no empty component between the componenets.

| MIB Variable | OID | MIB Value | Descripti on |
| --- | --- | --- | --- |
| zxAnPortIdCidCompon entIndex | 1.3.6.1.4.1.3902.108 2.70.67.2.3.1.1 | INTEGER { 1……30} |  |
| zxAnPortIdCidCompon entType | 1.3.6.1.4.1.3902.108 2.70.67.2.3.1.2 | INTEGER {1: standardVar(1) 2: extendedVar(2) 3: separator(3) 4: userDefinedString(4)} |  |
| zxAnPortIdCidCompon entId | 1.3.6.1.4.1.3902.108 2.70.67.2.3.1.3 | INTEGER { access-mode(21) access-node-id(1) access-node- type(22) chassis(2) cvlan(14) ethernet-priority(11) frame(4) gem-port(13) loid(17) loidpw(18) onu-id(12) onu-info(20) onu-number(19) port(7) q-vid(10) |  |

User-Defined two-level index. [MIB variable description]:

| MIB Variable | OID | MIB Value | Descripti on |
| --- | --- | --- | --- |
|  |  | rack(3) slot(5) sn(16) sub-slot(6) svlan(15) vci(9) vpi(8) vport(27) xci(24) xpi(23)} |  |
| zxAnPortIdCidCompon entWidth | 1.3.6.1.4.1.3902.108 2.70.67.2.3.1.4 | INTEGER { 0……5} |  |
| zxAnPortIdCidCompon entString | 1.3.6.1.4.1.3902.108 2.70.67.2.3.1.5 | DisplayString ( SIZE ( 0 .. 50 ) ) |  |
| zxAnPortIdCidCompon entRowStatus | 1.3.6.1.4.1.3902.108 2.70.67.2.3.1.50 | RowStatus |  |

#### Operator Profile Configuration

[Function description]:

This MIB views, creates, or deletes an operator profile. [Index description]:

| MIB Variable | OID | MIB Value | Descriptio n |
| --- | --- | --- | --- |
| zxAnPortIdOperPrfName | 1.3.6.1.4.1.3902.1082. 70.67.2.4.1.1 | DisplayString (SIZE (1.. 32) ) |  |
| zxAnPortIdOperPrfRowSt atus | 1.3.6.1.4.1.3902.1082. 70.67.2.4.1.100 | RowStatus |  |

It is a self-defined index composed of 1 to 32 characters. [MIB variable description]:

#### Operator Profile DHCP Option82 Configuration

[Function description]:

Operator profile DHCP Option82 configuration:

DHCP Option82 operator profile switch: It enables and disables the DHCP Option82 of the operator’s profile. When the switch is disabled, the port will not process the port positioning information in DHCP Option82 packets. When the switch is enabled, the port will process the port positioning information in DHCP Option82 packets.

When the trust status is “True”, the trust measures can be configured as “Keep” or “Replace”. When the trust status is “False”, the trust measures can be configured as “Discard” or “Add”. “Keep” indicates keeping the port positioning information contained in the upstream packets. “Replace” indicates replacing the port positioning information contained in the upstream packets. “Discard” indicates dropping the upstream packets. “Add” indicates adding other port positioning information.

[Index description]:

| MIB Variable | OID | MIB Value | Descripti on |
| --- | --- | --- | --- |
| zxAnPortIdOperPrfName | 1.3.6.1.4.1.3902.1082. 70.67.2.4.1.1 | DisplayString (SIZE ( 1 .. 32 ) ) |  |
| zxAnPortIdOperDhcpV4L 2RAEnable | 1.3.6.1.4.1.3902.1082. 70.67.2.4.1.11 | INTEGER {enable ( 1 ) , disable ( 2 ) } | Default value: 2 |
| zxAnPortIdOperDhcpV4L 2RATrust | 1.3.6.1.4.1.3902.1082. 70.67.2.4.1.12 | INTEGER { true ( 1 ) , false ( 2 ) } | Default value: 2 |
| zxAnPortIdOperDhcpV4L 2RAPolicy | 1.3.6.1.4.1.3902.1082. 70.67.2.4.1.13 | INTEGER { keep ( 1 ) , replace ( 2 ) , discard ( 3 ) , add ( 4 ) } | Default value: 4 |

It is a self-defined index composed of 1 to 32 characters. [MIB variable description]:

#### Operator Profile DHCP Option18 Configuration

[Function description]:

Operator profile DHCP Option18 configuration:

DHCP Option18 operator profile switch: It enables and disables the DHCP Option18 of the operator’s profile. When the switch is disabled, the port will not process the port positioning information in DHCP Option18 packets. When the switch is enabled, the port will process the port positioning information in DHCP Option18 packets.

When the trust status is “True”, the trust measures can be configured as “Keep” or “Replace”. When the trust status is “False”, the trust measures can be configured as “Discard” or “Add”. “Keep” indicates keeping the port positioning information contained in the upstream packets. “Replace” indicates replacing the port positioning information contained in the upstream packets. “Discard” indicates dropping the upstream packets. “Add” indicates adding other port positioning information.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdOperPrfName | 1.3.6.1.4.1.3902.108 | DisplayString |  |
|  | 2.70.67.2.4.1.1 | ( SIZE ( 1 .. 32 ) ) |  |
| zxAnPortIdOperDhcpV6L | 1.3.6.1.4.1.3902.108 | INTEGER | Default |
| 2RAEnable | 2.70.67.2.4.1.21 | { enable ( 1 ) , | value: 2 |
|  |  | disable ( 2 ) } |  |
| zxAnPortIdOperDhcpV6L | 1.3.6.1.4.1.3902.108 | INTEGER | Default |
| 2RATrust | 2.70.67.2.4.1.22 | { true ( 1 ) , false | value: 2 |
|  |  | ( 2 ) } |  |
| zxAnPortIdOperDhcpV6L | 1.3.6.1.4.1.3902.108 | INTEGER | Default |
| 2RAPolicy | 2.70.67.2.4.1.23 | { keep ( 1 ) , | value: 4 |
|  |  | replace ( 2 ) , |  |
|  |  | discard ( 3 ) , add |  |
|  |  | ( 4 ) } |  |

It is a self-defined index composed of 1 to 32 characters. [MIB variable description]:

#### Operator Profile PPPoE Configuration

[Function description]:

Operator profile PPPoE configuration:

PPPoE operator profile switch: It enables and disables the PPPoE of the operator’s profile. When the switch is disabled, the port will not process the port positioning

information in PPPoE packets. When the switch is enabled, the port will process the port positioning information in PPPoE packets.

When the trust status is “True”, the trust measures can be configured as “Keep” or “Replace”. When the trust status is “False”, the trust measures can be configured as “Discard” or “Add”. “Keep” indicates keeping the port positioning information contained in the upstream packets. “Replace” indicates replacing the port positioning information contained in the upstream packets. “Discard” indicates dropping the upstream packets. “Add” indicates adding other port positioning information.

[Index description]:

| MIB Variable | OID | MIB Value | Descripti on |
| --- | --- | --- | --- |
| zxAnPortIdOperPrfN | 1.3.6.1.4.1.3902.1082. | DisplayString |  |
| ame | 70.67.2.4.1.1 | ( SIZE ( 1 .. 32 ) ) |  |
| zxAnPortIdOperPppo | 1.3.6.1.4.1.3902.1082. | INTEGER | Default |
| eIAEnable | 70.67.2.4.1.31 | { enable ( 1 ) , disable ( | value: 2 |
|  |  | 2 ) }。。 |  |
| zxAnPortIdOperPppo | 1.3.6.1.4.1.3902.1082. | INTEGER | Default |
| eIATrust | 70.67.2.4.1.32 | { true ( 1 ) , false ( 2 ) } | value: 2 |
| zxAnPortIdOperPppo | 1.3.6.1.4.1.3902.1082. | INTEGER | Default |
| eIAPolicy | 70.67.2.4.1.33 | { keep ( 1 ) , replace | value: 4 |
|  |  | ( 2 ) , discard ( 3 ) , |  |
|  |  | add ( 4 ) } |  |

It is a self-defined index composed of 1 to 32 characters. [MIB variable description]:

#### Operator Profile DHCP Option82 Port Rate Configuration

[Function description]:

This MIB configures the DHCP Option82 port rate from subopt81 to subopt8e. subopt81 Actual upstream rate

subopt82 Actual downstream rate subopt83 Minimum upstream rate subopt84 Minimum downstream rate

subopt85 Attainable upstream rate subopt86 Attainable downstream rate subopt87 Maximum upstream rate subopt88 Maximum downstream rate

subopt89 Minimum upstream rate in low power consumption mode subopt8a Minimum downstream rate in low power comsumption mode subopt8b Maximum interleaving upstream delay

subopt8c	Actual interleaving upstream delay subopt8d Maximum interleaving downstream delay subopt8e Actual interleaving downstream delay [Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdOperPrf | 1.3.6.1.4.1.3902.1082. | DisplayString |  |
| Name | 70.67.2.4.1.1 | ( SIZE ( 1 .. 32 ) ) |  |
| zxAnPortIdOperDh | 1.3.6.1.4.1.3902.1082. | BITS |  |
| cpAlcTag | 70.67.2.4.1.14 | {suboption81(0), |  |
|  |  | suboption82(1), |  |
|  |  | suboption83(2), |  |
|  |  | suboption84(3), |  |
|  |  | suboption85(4), |  |
|  |  | suboption86(5), |  |
|  |  | suboption87(6), |  |
|  |  | suboption88(7), |  |
|  |  | suboption89(8), |  |
|  |  | suboption8A(9), |  |
|  |  | suboption8B(10), |  |
|  |  | suboption8C(11), |  |
|  |  | suboption8D(12), |  |
|  |  | suboption8E(13) } |  |

It is a self-defined index composed of 1 to 32 characters. [MIB variable description]:

#### Operator Profile PPPoE Port Rate Configuration

[Function description]:

This MIB configures the PPPoE port rate from subopt81 to subopt8e. subopt81 Actual upstream rate

subopt82 Actual downstream rate subopt83 Minimum upstream rate subopt84 Minimum downstream rate subopt85 Attainable upstream rate subopt86 Attainable downstream rate subopt87 Maximum upstream rate subopt88 Maximum downstream rate

subopt89 Minimum upstream rate in low power consumption mode subopt8a Minimum downstream rate in low power comsumption mode subopt8b Maximum interleaving upstream delay

subopt8c	Actual interleaving upstream delay subopt8d Maximum interleaving downstream delay subopt8e Actual interleaving downstream delay [Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdOperPrfNam | 1.3.6.1.4.1.3902.10 | DisplayString |  |
| e | 82.70.67.2.4.1.1 | ( SIZE ( 1 .. 32 ) ) |  |
| zxAnPortIdOperPppoe | 1.3.6.1.4.1.3902.10 | BITS |  |
| AlcTag | 82.70.67.2.4.1.34 | {suboption81(0), |  |
|  |  | suboption82(1), |  |

It is a self-defined index composed of 1 to 32 characters. [MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  | suboption83(2), suboption84(3), suboption85(4), suboption86(5), suboption87(6), suboption88(7), suboption89(8), suboption8A(9), suboption8B(10), suboption8C(11), suboption8D(12), suboption8E(13) } |  |

#### Operator Profile Format Configuration

[Function description]:

This MIB configures the format of the operator’s profile in different access mode. zxAnPortIdOperAtmFormatPrf is the format profile in ATM access mode. zxAnPortIdOperPtmEtherFormatPrf is the format profile in ETH access mode. zxAnPortIdOperPonFormatPrf is the format profile in PON access mode.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdOperPrfN | 1.3.6.1.4.1.3902.1082.7 | DisplayString |  |
| ame | 0.67.2.4.1.1 | ( SIZE ( 1 .. 32 ) |  |
|  |  | ) |  |
| zxAnPortIdOperAtm | 1.3.6.1.4.1.3902.1082.7 | DisplayString |  |
| FormatPrf | 0.67.2.4.1.2 | ( SIZE ( 1 .. 32 ) |  |
|  |  | ) |  |
| zxAnPortIdOperPtm | 1.3.6.1.4.1.3902.1082.7 | DisplayString |  |
| EtherFormatPrf | 0.67.2.4.1.3 | ( SIZE ( 1 .. 32 ) |  |
|  |  | ) |  |

It is a self-defined index composed of 1 to 32 characters. [MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdOperPon | 1.3.6.1.4.1.3902.1082.7 | DisplayString |  |
| FormatPrf | 0.67.2.4.1.4 | ( SIZE ( 1 .. 32 ) |  |
|  |  | ) |  |

#### Operator Profile Port Application Configuration

[Function description]:

This MIB applies the operator’s profile to a designated port. [Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdIfOperPrf | 1.3.6.1.4.1.3902.1082.7 | DisplayString |  |
|  | 0.67.2.6.1.1 | ( SIZE ( 1 .. 32 ) |  |
|  |  | ) |  |
| zxAnPortIdIfOperPrfR | 1.3.6.1.4.1.3902.1082.7 | RowStatus |  |
| owStatus | 0.67.2.6.1.50 |  |  |

The index includes ifIndex (type 2) and zxAnSubIfIndex (type 2). [MIB variable description]:

#### Operator Profile VLAN Application Mode

[Function description]:

This MIB applies the operator’s profile to a designated VLAN. [Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPortIdVid | 1.3.6.1.4.1.3902.108 2.70.67.2.5.1.1 | INTEGER {1……4094} |  |
| zxAnPortIdVlanOperP rf | 1.3.6.1.4.1.3902.108 2.70.67.2.5.1.2 | DisplayString ( SIZE ( 1 .. 32 ) ) |  |
| zxAnPortIdVlanOperP rfRowStatus | 1.3.6.1.4.1.3902.108 2.70.67.2.5.1.50 | RowStatus |  |

The “zxAnPortIdVid” index indicates the VLAN value of the operator’s profile. [MIB variable description]:

## DHCP SNOOPING

[MIB file]:

ZTE-AN-DHCP-SNOOPING-MIB.mib

#### Global Management

[Function description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnDhcpSnoo pingCapabilities | 1.3.6.1.4.1.39 02.1082.70.4 0.1.1 | supportIp SrcGuard IpBinding Table(0) | The management capabilities of DHCP snooping. The bit value 1 means the function represented by this bit is supported by this equipment. value 1 means supporting zxAnIpv4SrcGuardIpBindingTabl e and zxAnIpv6SrcGuardIpBindingTable , zxAnDsDhcpV4StaticIpBindingTa ble and zxAnDsDhcpV6StaticIpBindingTa ble was deprecated. |
|  |  |  | NOTE: No recommended to display it in NMS. |
| zxAnDsDhcpV4 | 1.3.6.1.4.1.39 |  | Enable or disable DHCPv4 |
| GlobalEnable | 02.1082.70.4 |  | snooping. |
|  | 0.2.1.1.1 |  |  |

This MIB enables and disables the DHCPv4 global switch. [MIB variable description]:

#### DHCPv4 VLAN Switch

[Function description]:

This MIB configures DHCPv4 Snooping VLAN switch. [Index description]:

Vlan ID is the index.

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnDsDhcpV 4Vid | 1.3.6.1.4.1.3902.1082.70.40. 2.1.2.1.1 |  | This object indicates the vlan number on which IPv4 DHCP Snooping feature is configured. |
| zxAnDsDhcpV 4VlanRowStatu s | 1.3.6.1.4.1.3902.1082.70.40. 2.1.2.1.50 | RowSta tus |  |

#### DHCPv4 Interface Swtich

[Function description]:

This MIB configures DHCPv4 Snooping interface switch. [Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| ifIndex | . 1.3.6.1.2.1.2.2.1.1 |  |  |
| zxAnSubIfIndex | . 1.3.6.1.4.1.3902.1 082.30.5.5.1.1.2 |  |  |
| zxAnDsDhcpV4IfE nable | . 1.3.6.1.4.1.3902.1 082.70.40.2.1.3.1. 1 |  | This object indicates whether DHCPv4 Snooping is enabled at this interface.If this object is set to 'enabled', |

Composite indx: ifIndex+ zxAnSubIfIndex [MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | DHCPv4 Snooping at this interface is enabled. If this object is set to 'disabled', DHCPv4 Snooping at this interface is disabled. |

#### DHCPv4 Dynamic Binding

[Function description]:

This MIB views or deletes DHCPv4 Snooping dynamic binding items. [Index description]:

Composite index: ifIndex+ zxAnSubIfIndex+ zxAnDsDhcpV4IpBindingMac+ zxAnDsDhcpV4IpBindingSVid

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| ifIndex | .1.3.6.1.2.1.2.2.1.1 |  |  |
| zxAnSubIfIndex | . 1.3.6.1.4.1.3902.1082.3 0.5.5.1.1.2 |  |  |
| zxAnDsDhcpV4Ip BindingMac | . 1.3.6.1.4.1.3902.1082.7 0.40.2.1.6.1.1 |  | This object indicates the MAC address of an IPv4 DHCP client host. |
| zxAnDsDhcpV4Ip BindingSVid | . 1.3.6.1.4.1.3902.1082.7 0.40.2.1.6.1.2 | 1 to 4094 | This object indicates the service vlan to which an IPv4 DHCP client host belongs. |
| zxAnDsDhcpV4Ip BindingIpAddress | . 1.3.6.1.4.1.3902.1082.7 |  | This object indicates the |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 0.40.2.1.6.1.3 |  | allocated IPv4 address of an IPv4 DHCP client host. |
| zxAnDsDhcpV4Ip BindingLeasedTi me | . 1.3.6.1.4.1.3902.1082.7 0.40.2.1.6.1.4 | The format is 2d-1d- 1d,1d:1d:1d. 1d. | This object indicates the leased time of this IPv4 DHCP bindings. 0. |
| zxAnDsDhcpV4Ip BindingUserVid | . 1.3.6.1.4.1.3902.1082.7 0.40.2.1.6.1.5 | 1 to 4094 | This object indicates the vlan to which an IPv4 DHCP client host belongs. |
| zxAnDsDhcpV4Ip BindingRowStatu s | . 1.3.6.1.4.1.3902.1082.7 0.40.2.1.6.1.50 | RowStatus |  |

#### Maximum IPv4 Binding

[Function description]:

This MIB configures the maximum IPv4 items in the port IP source guard. [Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| ifIndex | .1.3.6.1.2.1.2.2.1.1 |  |  |
| zxAnSubIfIndex | . 1.3.6.1.4.1.3902.1082.3 0.5.5.1.1.2 |  |  |
| zxAnDsDhcpV4 IfBindingLimit | . 1.3.6.1.4.1.3902.1082.7 0.40.2.1.8.1.1 | The total number of DHCPv4 and DHCPv6 bindings can not exceed 8 | This object specifies the maximum number of DHCPv4 bindings |

Composite index: ifIndex + zxAnSubIfIndex [MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | allowed on this interface. |

#### DHCPv4 Session Limit

[Function description]:

This MIB configures the maximum DHCPv4 Snooping items that can be monitored by an interface.

[Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| ifIndex | .1.3.6.1.2.1.2.2.1.1 |  |  |
| zxAnSubIfIndex | . 1.3.6.1.4.1.3902.1082.3 0.5.5.1.1.2 |  |  |
| zxAnDsDhcpV4If SessionLimit | . 1.3.6.1.4.1.3902.1082.7 0.40.2.1.9.1.1 |  | This object specifies the maximum number of DHCPv4 bindings allowed on this interface.when the value is 0, that means no limit in the user interface. |

Composite index: ifIndex + zxAnSubIfIndex [MIB variable description]:

#### DHCPv4 Packet Statistics

[Function description]:

This MIB makes statistics on or resets the DHCPv4 packets on an interface. [Index description]:

Composite index: fIndex + zxAnSubIfIndex [MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| ifIndex | .1.3.6.1.2.1.2.2.1.1 |  |  |
| zxAnSubIfIndex | . 1.3.6.1.4.1.3902.1082 .30.5.5.1.1.2 |  |  |
| zxAnDsDhcpV4IfStats DiscoverPkts | . 1.3.6.1.4.1.3902.1082 .70.40.2.1.50.1.1 |  | This object indicates the discover number of packets subjected to IPv4 DHCP Snooping is received. |
| zxAnDsDhcpV4IfStats OfferPkts | . 1.3.6.1.4.1.3902.1082 .70.40.2.1.50.1.2 |  | This object indicates the offer number of packets subjected to IPv4 DHCP Snooping is received. |
| zxAnDsDhcpV4IfStats RequestPkts | . 1.3.6.1.4.1.3902.1082 .70.40.2.1.50.1.3 |  | This object indicates the request number of packets subjected to IPv4 DHCP Snooping is received. |
| zxAnDsDhcpV4IfStats AckPkts | . 1.3.6.1.4.1.3902.1082 .70.40.2.1.50.1.4 |  | This object indicates the ack number of packets subjected to IPv4 DHCP Snooping is received. |
| zxAnDsDhcpV4IfStats NackPkts | . 1.3.6.1.4.1.3902.1082 .70.40.2.1.50.1.5 |  | This object indicates the nack number of packets subjected to IPv4 DHCP Snooping is received. |
| zxAnDsDhcpV4IfStats ReleasePkts | . 1.3.6.1.4.1.3902.1082 |  | This object indicates the release number |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | .70.40.2.1.50.1.6 |  | of packets subjected to IPv4 DHCP Snooping is received. |
| zxAnDsDhcpV4IfStats DeclinePkts | . 1.3.6.1.4.1.3902.1082 .70.40.2.1.50.1.7 |  | This object indicates the decline number of packets subjected to IPv4 DHCP Snooping is received. |
| zxAnDsDhcpV4IfStatsI nformPkts | . 1.3.6.1.4.1.3902.1082 .70.40.2.1.50.1.8 |  | This object indicates the inform number of packets subjected to IPv4 DHCP Snooping is received. |
| zxAnDsDhcpV4IfStats Reset | . 1.3.6.1.4.1.3902.1082 .70.40.2.1.50.1.100 |  | This object indicates whether IPv4 DHCP statistics is cleared at this interface. If this object is set to 'reset(1)', IPv4 dhcp statistics at this interface will be cleared. |

#### IP Source Guard Global Configuration

[Function description]:

This MIB configures the IP source guard switch and binding type.

[Index description]:

[MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnIpsgIpSpoofTr apRptInterval | . 1.3.6.1.4.1.3902.1082. 70.40.2.3.1.1.6 |  | The interval that the same IP spoofing trap report. |
| zxAnIpsgIpSpoofing BlockIfEnable | . 1.3.6.1.4.1.3902.1082. 70.40.2.3.1.1.7 |  | Enable or disable IP source guard blocking port function. |
| zxAnIpsgIpSpoofing BlockDuration | . 1.3.6.1.4.1.3902.1082. 70.40.2.3.1.1.8 |  | Indicates the port blocked duration. |
| zxAnIpSrcGuardEn able | . 1.3.6.1.4.1.3902.1082. 70.40.2.3.1.21 |  | Enable/disable IP source guard. |
| zxAnIpSrcGuardBin dType | . 1.3.6.1.4.1.3902.1082. 70.40.2.3.1.22 | ip(1) ipAndMa c(2) | IP source guard binding Type. ip(1) - IP source guard bind IP address only. ipAndMac(2) - IP source guard bind both IP address and MAC address. |

#### IP Source Guard Interface Switch

[Function description]:

This MIB configures ip source guard interface switch. [Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| ifIndex | . |  |  |

Composite index: ifIndex + zxAnSubIfIndex [MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 1.3.6.1.2.1.2.2.1. 1 |  |  |
| zxAnSubIfIndex | . 1.3.6.1.4.1.3902. 1082.30.5.5.1.1.2 |  |  |
| zxAnIpSrcGuardI fEnable | . 1.3.6.1.4.1.3902. 1082.70.40.2.3.2. 1.1 | 1-true,2- false | This object indicates whether IP Source Guard is enabled at this interface. If IP Source Guard at this interface and the VLAN number on which IP Source Guard at this interface are both enabled, Traffic coming to this interface will be forwarded if it is from the list of IP addresses obtained through DHCP Snooping or static configuration. Otherwise, it is denied. |

#### IPv4 Static Binding Table

[Function description]:

This MIB configures IPv4 static binding items. [Index description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| ifIndex | . 1.3.6.1.2.1.2.2.1 .1 |  |  |
| zxAnSubIfIndex | . 1.3.6.1.4.1.3902 |  |  |

Composite index: ifIndex + zxAnSubIfIndex [MIB variable description]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | .1082.30.5.5.1.1 .2 |  |  |
| zxAnIpv4SrcGuardIpB indIpAddress | . 1.3.6.1.4.1.3902 .1082.70.40.2.3. 20.2.1.1 |  | IP address. |
| zxAnIpv4SrcGuardIpB indMac | . 1.3.6.1.4.1.3902 .1082.70.40.2.3. 20.2.1.2 |  | MAC address. |
| zxAnIpv4SrcGuardIpB indType | . 1.3.6.1.4.1.3902 .1082.70.40.2.3. 20.2.1.3 |  | IP address binding Type. If zxAnIpv4SrcGuardIpBin dType is static(1), this item can't be inactive. |
| zxAnIpv4SrcGuardIpB indRowStatus | . 1.3.6.1.4.1.3902 .1082.70.40.2.3. 20.2.1.50 | RowStatu s |  |

## L3 Interface

#### VLAN Interface Creation

[MIB file]:

ZTE-AN-L3-IF-MIB.mib

{"zxAnL3IfTable"	, "1.3.6.1.4.1.3902.1082.90.10.2.1"} ,

| MIB Variable | OID | MIB Value | Descri ption |
| --- | --- | --- | --- |
| IfIndex | . 1.3.6.1.2.1.2.2.1.1 |  | index |
| zxAnL3IfName | . | STRING ( SIZE ( |  |
|  | 1.3.6.1.4.1.3902.1082.90.10. | 0 .. 64 ) ) |  |
|  | 2.1.1.1 |  |  |
| zxAnL3IfMngEnable | . | INTEGER |  |

| MIB Variable | OID | MIB Value | Descri ption |
| --- | --- | --- | --- |
|  | 1.3.6.1.4.1.3902.1082.90.10. | { enable ( 1 ) , |  |
|  | 2.1.1.2 | disable ( 2 ) } |  |
| zxAnL3IfArpProxyE | . | INTEGER |  |
| nable | 1.3.6.1.4.1.3902.1082.90.10. | { enable ( 1 ) , |  |
|  | 2.1.1.3 | disable ( 2 ) } |  |
| zxAnL3IfArpAgingTi | . | Integer32 ( 1 .. |  |
| me | 1.3.6.1.4.1.3902.1082.90.10. | 4294967 ) |  |
|  | 2.1.1.4 |  |  |
| zxAnL3IfIpv6Enable | . | INTEGER |  |
|  | 1.3.6.1.4.1.3902.1082.90.10. | { enable ( 1 ) , |  |
|  | 2.1.1.5 | disable ( 2 ) } |  |
| zxAnL3IfIpv6Mtu | . | Integer32 |  |
|  | 1.3.6.1.4.1.3902.1082.90.10. | ( 1280 .. 1500 ) |  |
|  | 2.1.1.6 |  |  |
| zxAnL3IfIpv6DadAtt | . | Integer32 ( 1 .. |  |
| emps | 1.3.6.1.4.1.3902.1082.90.10. | 60 ) |  |
|  | 2.1.1.7 |  |  |
| zxAnL3IfRowStatus | . | rowStatus |  |
|  | 1.3.6.1.4.1.3902.1082.90.10. |  |  |
|  | 2.1.1.50 |  |  |

#### VLAN Interface IP Configuration

[MIB file]:

ZTE-AN-L3-IF-MIB.mib

{"zxAnL3IfIpAddressTable"	," 1.3.6.1.4.1.3902.1082.90.10.2.2"} ,

| MIB Variable | OID | MIB Value | Descript ion |
| --- | --- | --- | --- |
| ifIndex | . 1.3.6.1.2.1.2.2.1.1 |  | index |
| zxAnL3IfIpAddress | . |  | index |
|  | 1.3.6.1.4.1.3902.1082.90.10.2.2 |  |  |
|  | .1.1 |  |  |
| zxAnL3IfIpAddressMa | . |  |  |
| sk | 1.3.6.1.4.1.3902.1082.90.10.2.2 |  |  |
|  | .1.2 |  |  |
| zxAnL3IfIpCatagory | . |  |  |

| MIB Variable | OID | MIB Value | Descript ion |
| --- | --- | --- | --- |
|  | 1.3.6.1.4.1.3902.1082.90.10.2.2 .1.3 |  |  |
| zxAnL3IfIpRowStatus | . 1.3.6.1.4.1.3902.1082.90.10.2.2 .1.50 | rowStatus |  |

# PON Services

## Public PON Port Management

#### IP Address Pool

[MIB file]:

ZTE-AN-PON-SERVICE-MIB.mib

{"zxAnPonOnuMgmtIpAddrType"	, "1.3.6.1.4.1.3902.1082.500.2.2.2.5.1.1"} ,

{"zxAnPonOnuMgmtIpAddr"	,	"1.3.6.1.4.1.3902.1082.500.2.2.2.5.1.2"} ,

{"zxAnPonOnuMgmtIpPfxLen"	, "1.3.6.1.4.1.3902.1082.500.2.2.2.5.1.3"} ,

{"zxAnPonOnuMgmtPri"	, "1.3.6.1.4.1.3902.1082.500.2.2.2.5.1.4"} ,

{"zxAnPonOnuMgmtCVid"	,	"1.3.6.1.4.1.3902.1082.500.2.2.2.5.1.5"} ,

{"zxAnPonOnuMgmtSVid"	, "1.3.6.1.4.1.3902.1082.500.2.2.2.5.1.6"} ,

{"zxAnPonOnuMgmtNetAddrType"	,	"1.3.6.1.4.1.3902.1082.500.2.2.2.5.1.7"} ,

{"zxAnPonOnuMgmtNetAddr" , "1.3.6.1.4.1.3902.1082.500.2.2.2.5.1.8"} ,

{"zxAnPonOnuMgmtNetPfxLen" ,	"1.3.6.1.4.1.3902.1082.500.2.2.2.5.1.9"} ,

{"zxAnPonOnuMgmtNetGatewayAddrType","1.3.6.1.4.1.3902.1082.500.2.2.2.5.1.10"} ,

{"zxAnPonOnuMgmtNetGatewayAddr",	"1.3.6.1.4.1.3902.1082.500.2.2.2.5.1.11"} ,

{"zxAnPonOnuMgmtIpEnable",	"1.3.6.1.4.1.3902.1082.500.2.2.2.5.1.12"} ,

## GPON OLT Management

#### ONU Management

[Function description]:

This MIB creates and deletes the ONU under a certain OLT, configures ONU authentication mode and authentication information, and manages the ONU.

[MIB file]:

Please refer to zxAnGponSrvOnuMgmtTable defined in ZTE-AN-GPON-SERVICE- MIB.mib.

When modifying authentication information, zxAnGponOnuMgmtRegMode and authentication information field corresponding to the authentication mode must be specified.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponOnuM gmtType | . 1.3.6.1.4.1.390 2.1082.500.10. 2.3.3.1.1 | DisplayString ( SIZE ( 1 .. 64 ) ) | The ONU Type name which should refer to the entry of zxAnPonOnuTypeTable . |
| zxAnGponOnuM gmtRegMode | . 1.3.6.1.4.1.390 2.1082.500.10. 2.3.3.1.5 | INTEGER { regModeSn (1) , regModePwd (2) , regModeSnPlusPwd( 3) , regModeRegisterId (4) , regModeRegisterIdPl us8021x (5) , regModeRegisterIdPl usMutual (6), regModeHexPwd (7), | This attribute indicates the ONU registration mode. regModeSn: GPON/XGPON - G.984 G.987 - 8bytes regModePwd: GPON - G.984 - 10Bytes regModeSnPlusPwd: GPON - G.984 - 8+10Bytes regModeRegisterId: |

zxAnGponOnuMgmtMIB, zxAnGponOnuMgmtRegMode and zxAnGponOnuMgmtRowStatus must be specified in ONU creation.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  | regModeSnPlusHex Pwd (8), regModeLoid ( 9 ) , regModeLoidPlusPw d ( 10 ) } | GPON/XGPON - G.984 G.987 - 36Bytes(XGPON) regModeRegisterIdPlus 8021x: XGPON - G.987 - 36Bytes regModeRegisterIdPlus Mutual: XGPON - G.987 - 36Bytes regModeHexPwd: GPON--Hex Password format - 7Bytes(hex,fixed) regModeSnPlusHexPw d: GPON - Hex SN+Password format - 8+7Bytes(hex,fixed) regModeLoid: GPON/XGPON - CTC format - 24Bytes regModeLoidPlusPwd: GPON/XGPON - CTC format - 24+12Bytes The attribute is also used to define the registration mode in 'zxAnGponOnuMgmtRe gistrationInfo'. |
| zxAnGponOnuM gmtSn | . 1.3.6.1.4.1.390 2.1082.500.10. 2.3.3.1.6 | OCTET STRING ( SIZE ( 8 ) ) | SN configed by OLT for the purpose of register. The serial number is unique for each ONU. It is defined in [ITU-T G.984.3] and [ITU-T G.987.3] and contains the vendor id and vendor specific serial |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | number. The first four bytes are an ASCII-encoded four- letter vendor ID. The second four bytes are a binary encoded serial number, under the control of the ONU vendor. This attribute is valid when 'zxAnGponOnuMgmtRe gMode' is following value: regModeSn(1) - 8 Bytes, regModeSnPlusHexPw d(8) - 8 Bytes |
| zxAnGponOnuM gmtPwd | . 1.3.6.1.4.1.390 2.1082.500.10. 2.3.3.1.7 | OCTET STRING ( SIZE ( 0 .. 12 ) ) | Password configed by OLT for the purpose of register. This attribute is valid when 'zxAnGponOnuMgmtRe gMode' is following value: regModePwd(2) - 10 Bytes, regModeSnPlusPwd(3) - 10 Bytes, regModeHexPwd(7) - 7 Bytes(hex), regModeSnPlusHexPw d(8) - 7 Bytes(hex), regModeLoidPlusPwd( 10) - 12 Bytes |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponOnuM gmtLoid | . 1.3.6.1.4.1.390 2.1082.500.10. 2.3.3.1.8 | DisplayString ( SIZE ( 0 .. 24 ) ) | LOID configed by OLT for the purpose of register. This attribute is valid when 'zxAnGponOnuMgmtRe gMode' is following value: regModeLoid(9) regModeLoidPlusPwd( 10) |
| zxAnGponOnuM gmtRegistrationI d | . 1.3.6.1.4.1.390 2.1082.500.10. 2.3.3.1.9 | DisplayString ( SIZE ( 0 .. 36 ) ) | Registration ID configed by OLT for the purpose of register. This attribute is valid when 'zxAnGponOnuMgmtRe gMode' is following value: regModeRegisterId(4) regModeRegisterIdPlus 8021x(5) regModeRegisterIdPlus Mutual(6) |
| zxAnGponOnuM gmtRowStatus | . 1.3.6.1.4.1.390 2.1082.500.10. 2.3.3.1.50 | RowStatus |  |

#### ONU Status

[MIB file]:

Please refer to zxAnGponSrvOnuStatusTable defined in ZTE-AN-GPON- SERVICE-MIB.mib.

| MIB Variable | OID | MIB Value | Descriptio n |
| --- | --- | --- | --- |
| zxAnGponSrvOnu | . | INTEGER |  |
| PhaseStatus | 1.3.6.1.4.1.3902.1082. | { logging ( 1 ) , los |  |
|  | 500.10.2.3.8.1.4 | ( 2 ) , syncMib ( 3 ) , |  |
|  |  | working ( 4 ) , |  |
|  |  | dyingGasp ( 5 ) , |  |
|  |  | authFailed ( 6 ) , |  |
|  |  | offline ( 7 ) } |  |

#### T-CONT Profile

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponSrvBwPrf Name | . 1.3.6.1.4.1.3902. 1082.500.10.2.1. 2.1.1 | DisplayString (SIZE(1.. 64 ) ) |  |
| zxAnGponSrvBwPrf Fixed | . 1.3.6.1.4.1.3902. 1082.500.10.2.1. 2.1.2 | Integer32 (64.. 488320 ) | The Fixed Bandwidth is entirely reserved and cyclically allocated in order to achieve a low traffic transfer delay. The unit is kbps. It may be created/read/modified . Modifying can be done only when the entry is not used by any other entry. Guaranteed Bandwidth is the summation of Assured Bandwidth and |

Please refer to zxAnGponSrvBandwidthPrfTable defined in ZTE-AN-GPON- SERVICE-MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | Fixed Bandwidth. |
| zxAnGponSrvBwPrf Assured | . 1.3.6.1.4.1.3902. 1082.500.10.2.1. 2.1.3 | Integer32 (64.. 2488320) kbps | The Assured Bandwidth is bandwidth that is always available to the ONU/ONT if the T-CONT buffer is expected to have traffic to transmit. If the T-CONT buffer does not have traffic to transmit, this bandwidth may be used by other T- CONTs. Guaranteed Bandwidth is the summation of Assured Bandwidth and Fixed Bandwidth. |
| zxAnGponSrvBwPrf Maximum | . 1.3.6.1.4.1.3902. 1082.500.10.2.1. 2.1.4 | Integer32 (64.. 488320 ) | The Maximum Bandwidth.The unit is kbps. It may be created/read/modified . Writing or modifying can be done when the entry is not used by any other entry. |
| zxAnGponSrvBwPrf Type | . 1.3.6.1.4.1.3902. 1082.500.10.2.1. 2.1.5 | INTEGER { Type1 ( 1 ) , Type 2 ( 2 ) , Type 3 ( 3 ) , Type 4 ( 4 ) , Type 5 ( 5 ) } | There are 5 kinds of bandwidth profile mode: Type 1 is characterized by Fixed bandwidth only; Type 2 is characterized by Assured bandwidth |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | only; Type 3 has Assured bandwidth and Non- assured bandwidth; Type 4 has Best- effort bandwidth only and does not have any Guaranteed bandwidth; Type 5 is the super set of all of Type s. when modify or create this entry, this value must be set. If the value is useless to the value of Type, it will be ignored and set to 0 in the system. |
| zxAnGponSrvBwPrf RowStatus | . 1.3.6.1.4.1.3902. 1082.500.10.2.1. 2.1.50 | RowStatus |  |

#### GEM Port Traffic Profile

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponSrvTraffic | . | DisplayString |  |
| PrfName | 1.3.6.1.4.1.3902. | (SIZE ( 1 .. 64 |  |
|  | 1082.500.10.2.1. | ) ) |  |
|  | 3.1.1 |  |  |
| zxAnGponSrvTraffic | . | Integer32 | The SIR is |
| PrfSir | 1.3.6.1.4.1.3902. | (0.. 10000000 | sustainable rate of |
|  | 1082.500.10.2.1. | ) | the traffic, modifying |
|  | 3.1.2 |  | can be done when |

Please refer to zxAnGponSrvTrafficPrfTable defined in ZTE-AN-GPON-SERVICE- MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | the entry is not used by any other entry. |
| zxAnGponSrvTraffic PrfPir | . 1.3.6.1.4.1.3902. 1082.500.10.2.1. 3.1.3 | Integer32 (64.. 10000000 ) | The PIR is peak rate of the traffic, modifying can be done when the entry is not used by any other entry. |
| zxAnGponSvrTraffic PrfCbs | . 1.3.6.1.4.1.3902. 1082.500.10.2.1. 3.1.4 | Integer32 ( 0 .. 1023 ) | This attribute specifies committed block size. The value 0 accepts the system default policy. |
| zxAnGponSvrTraffic PrfPbs | . 1.3.6.1.4.1.3902. 1082.500.10.2.1. 3.1.5 | Integer32 ( 0 .. 1023 ) | This attribute specifies peak block size. The value 0 accepts the system default policy. |
| zxAnGponSrvTraffic PrfRowStatus | . 1.3.6.1.4.1.3902. 1082.500.10.2.1. 3.1.50 | RowStatus |  |

#### T-CONT

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponSrvTcontI ndex | . 1.3.6.1.4.1.390 2.1082.500.10. 2.3.4.1.1 |  | An arbitrary(unique) integer for identifying an entry in the table, from 1 to upper limit supported T-CONT count of ONU Type. |

Please refer to zxAnGponSrvTcontTable defined in ZTE-AN-GPON-SERVICE- MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponSrvTcont BwPrfName | . 1.3.6.1.4.1.390 2.1082.500.10. 2.3.4.1.3 | DisplayString (SIZE ( 0 .. 64 ) ) | This attribute is the name of profile used by T-CONT bandwidth configuration. According to 'zxAnGponSrvBwPrf Name' in 'zxAnGponSrvBand widthPrfTable'. |
| zxAnGponSrvTcont RowStatus | . 1.3.6.1.4.1.390 2.1082.500.10. 2.3.4.1.50 | RowStatus |  |

#### GEM Port

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponSrvGemP ortIndex | . 1.3.6.1.4.1.3902.1 082.500.10.2.3.5.1 .1 |  | An arbitrary(unique) integer for identifying an entry in the table, from 1 to upper limit supported GEM port count of ONU Type. |
| zxAnGponSrvGemP ortName | . 1.3.6.1.4.1.3902.1 082.500.10.2.3.5.1 .2 | DisplayString ( SIZE ( 0 .. 64 ) ) | This attribute specifies the name of the GEM port. |
| zxAnGponSrvGemP ortTcontIndex | . 1.3.6.1.4.1.3902.1 082.500.10.2.3.5.1 .3 |  | It is equal to the 'zxAnGponSrvOnuTc ontIndex' of 'zxAnGponSrvOnuTc ontTable'. |

Please refer to zxAnGponSrvGemPortTable defined in ZTE-AN-GPON-SERVICE- MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponSrvGemP | . | RowStatus |  |
| ortRowStatus | 1.3.6.1.4.1.3902.1 |  |  |
|  | 082.500.10.2.3.5.1 |  |  |
|  | .50 |  |  |

#### GEM Port Rate Limit

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponSrvGem PortUsTrafficPrf | . 1.3.6.1.4.1.3902.1082. 500.10.2.3.5.1.10 | DisplayStrin g ( SIZE ( 0 .. 64 ) ) | Refers to 'zxAnGponSrvTraffi cPrfName', upstream traffic limit of GEM port at OLT side. |
| zxAnGponSrvGem PortDsTrafficPrf | . 1.3.6.1.4.1.3902.1082. 500.10.2.3.5.1.11 | DisplayStrin g ( SIZE ( 0 .. 64 ) ) | Refers to 'zxAnGponSrvTraffi cPrfName', downstream traffic limit of GEM port at OLT side. |

Please refer to zxAnGponSrvGemPortTable defined in ZTE-AN-GPON-SERVICE- MIB.mib.

#### VLAN (Service Port)

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnSrvPortServiceMo de | 1.3.6.1.4.1.3902.1082.11 0.5.2.2.1.4 | 1-12 |  |
| zxAnSrvPortUserVid | 1.3.6.1.4.1.3902.1082.11 0.5.2.2.1.8 | 0-4094 |  |
| zxAnSrvPortCVid | 1.3.6.1.4.1.3902.1082.11 | 0-4094 |  |

Please refer to zxAnServicePortConfTable defined in ZTE-AN-SERVICEPORT- MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 0.5.2.2.1.18 |  |  |
| zxAnSrvPortSVid | 1.3.6.1.4.1.3902.1082.11 0.5.2.2.1.19 | 0-4094 |  |
| zxAnSrvPortRowStatus | 1.3.6.1.4.1.3902.1082.11 0.5.2.2.1.50 |  |  |

#### Unconfigured ONU

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponSrvUnConf OnuIndex | . 1.3.6.1.4.1.3902.108 2.500.10.2.2.5.1.1 |  | The index of the unconfigured ONU. |
| zxAnGponSrvUnConf OnuSn | . 1.3.6.1.4.1.3902.108 2.500.10.2.2.5.1.2 | OCTET STRING ( SIZE ( 8 ) ) | The SN of the unconfigured ONU. The serial number is unique for each ONU. It is defined in [ITU-T G.984.3] and [ITU-T G.987.3] and contains the vendor id and vendor specific serial number. The first four bytes are an ASCII-encoded four-letter vendor ID. The second four bytes are a binary encoded serial |

Please refer to zxAnGponSrvUnConfOnuTable defined in ZTE-AN- SERVICEPORT-MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | number, under the control of the ONU vendor. |
| zxAnGponSrvUnConf OnuPwd | . 1.3.6.1.4.1.3902.108 2.500.10.2.2.5.1.3 | OCTET STRING (SIZE ( 10 ) ) | The password reported by the ONU. The format could be display string or HEX octets. |
| zxAnGponSrvUnConf OnuLoid | . 1.3.6.1.4.1.3902.108 2.500.10.2.2.5.1.4 | DisplayString ( SIZE ( 0 .. 24 ) ) | This attribute identifies the LOID of the ONU. |
| zxAnGponSrvUnConf OnuLoidPwd | . 1.3.6.1.4.1.3902.108 2.500.10.2.2.5.1.5 | OCTET STRING (SIZE ( 12 ) ) | This attribute identifies the LOID password of the ONU. The format could be display string or HEX octets. |
| zxAnGponSrvUnConf OnuRegId | . 1.3.6.1.4.1.3902.108 2.500.10.2.2.5.1.6 | DisplayString ( SIZE ( 0 .. 36 ) ) | This attribute identifies the registration ID of the ONU. |
| zxAnGponSrvUnConf OnuType | . 1.3.6.1.4.1.3902.108 2.500.10.2.2.5.1.7 | DisplayString ( SIZE ( 0 .. 14 ) ) | This attribute identifies the Type of the ONU. |
| zxAnGponSrvUnConf OnuSwVer | . 1.3.6.1.4.1.3902.108 2.500.10.2.2.5.1.8 | DisplayString ( SIZE ( 0 .. 14 ) ) | This attribute identifies the version of the ONU. |

#### Traffic-profile

[MIB file]:

Please refer to zxAnQos3TrafficConfigTable defined in ZTE-AN-QOS3-MIB.mib.

| MIB Variable | OID | MIB Value | Descrip tion |
| --- | --- | --- | --- |
| zxAnQosTrafficIfConfTab le | .1.3.6.1.4.1.3902.1082.50.2.2.2 |  |  |
| zxAnQosTrafficIfConfEnt ry | . 1.3.6.1.4.1.3902.1082.50.2.2.2. 1 |  |  |
| zxAnQosTrafficIfConfPrf | . 1.3.6.1.4.1.3902.1082.50.2.2.2. 1.2 |  |  |
| zxAnQosTrafficIfDirectio n | . 1.3.6.1.4.1.3902.1082.50.2.2.2. 1.1 |  |  |
| zxAnQosTrafficIfRowStat us | . 1.3.6.1.4.1.3902.1082.50.2.2.2. 1.50 |  |  |

The relationship between profile and ONU interface

| MIB Variable | OID | MIB Value | Descrip tion |
| --- | --- | --- | --- |
| ifIndex | .1.3.6.1.2.1.2.2.1.1 |  |  |
|  | . |  |  |
| zxAnSubIfIndex | 1.3.6.1.4.1.3902.1082.30.5.5.1. |  |  |
|  | 1.2 |  |  |

## Remote GPON ONU Management

#### Service

[Function description]:

This MIB creates the mapping from the user traffic to GEM port by creating service configurations.

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmServi ceName | . 1.3.6.1.4.1.3902. 1082.500.20.2.7. 2.1.1 | DisplayString ( SIZE ( 1 .. 32 ) ) | Name of service. |
| zxAnGponRmServi ceGemPort | . 1.3.6.1.4.1.3902. 1082.500.20.2.7. 2.1.2 | Integer32 ( 1 .. 255 ) | GEM port index of service. |
| zxAnGponRmServi ceMapType | . 1.3.6.1.4.1.3902. 1082.500.20.2.7. 2.1.4 | INTEGER { cos ( 1 ) , vlan (2) , cosAndVlan ( 3 ) , transparent ( 4) , untag ( 5 ) } | Map Type service. - cos: mapping to gemport by cos value - vlan: mapping to gemport by VLAN value - cosAndVlan: mapping to gemport by both cos and VLAN value - transparent: mapping to gemport transparently - untag: mapping to gemport for untag frames |

Please refer to zxAnGponRmServiceMapTable defined in ZTE-AN-GPON-REMOTE- ONU-MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmServi ceMapCos | . 1.3.6.1.4.1.3902. 1082.500.20.2.7. 2.1.5 | Integer32 ( 0 .. 255 ) | A bitfield defines the frames with the priority related with this service. MSB stands for priority 7, LSB stands for priority 0. A bit value of 0 specifies that frames with the associated priority are to be discarded. |
| zxAnGponRmServi ceMapVid | . 1.3.6.1.4.1.3902. 1082.500.20.2.7. 2.1.6 | OCTET STRING ( SIZE ( 24 ) ) | Each 2 bytes stand for the VLAN ID related with this service. The first N*2 bytes are valid, until VLAN ID=0. |
| zxAnGponRmServi ceRowStatus | . 1.3.6.1.4.1.3902. 1082.500.20.2.7. 2.1.50 | RowStatus |  |

#### LAN Interface

##### Interface Management

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmEthU niAdminState | . 1.3.6.1.4.1.3902.108 2.500.20.2.3.2.1.5 | INTEGER {unlocked (1 ) , locked ( 2 ) } | This attribute is used to activate (unlock: value 0x01) and deactivate(lock: value 0x02) the |

Please refer to zxAnGponRmEthUniTable defined in ZTE-AN-GPON-REMOTE-ONU- MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | functions performed by instances of this managed entity. |
| zxAnGponRmEthU niOperState | . 1.3.6.1.4.1.3902.108 2.500.20.2.3.2.1.6 | INTEGER { enabled ( 1 ) , disabled ( 2 ) } | This attribute indicates whether or not this managed entity is capable of performing its task. The operational state reflects the perceived ability to receive or to generate a valid signal. Valid values are enabled (0x01) and disabled (0x02). |

#### LAN Interface VLAN

[Function description]:

This MIB adds and deletes the ONU UNI VLAN. [MIB file]:

Please refer to the following tables defined in ZTE-AN-GPON-REMOTE-ONU-MIB.mib.

Configuring ONU user port VLAN usually involve two MIB tables. Please refer to the MIB document for the definition of each MIB variable.

{"zxAnGponRmVlanPortTable" , "1.3.6.1.4.1.3902.1082.500.20.2.4.63"} ,

{"zxAnGponRmVlanPortEntry" , "1.3.6.1.4.1.3902.1082.500.20.2.4.63.1"} ,

{"zxAnGponRmVlanPortMode", "1.3.6.1.4.1.3902.1082.500.20.2.4.63.1.3"} ,

{"zxAnGponRmVlanPortDefaultVid" , "1.3.6.1.4.1.3902.1082.500.20.2.4.63.1.4"} ,

{"zxAnGponRmVlanPortCmdTable", "1.3.6.1.4.1.3902.1082.500.20.2.4.64"} ,

{"zxAnGponRmVlanPortCmdEntry",	"1.3.6.1.4.1.3902.1082.500.20.2.4.64.1"} ,

{"zxAnGponRmVlanPortCmd"	, "1.3.6.1.4.1.3902.1082.500.20.2.4.64.1.1"} ,

{"zxAnGponRmVlanPortVid"	, "1.3.6.1.4.1.3902.1082.500.20.2.4.64.1.2"} ,

Configuration examples:

Take gpon-onu_1/3/1:1 eth_0/1 configuration for example:

- Transparent transmission mode:

ZXAN(gpon-onu-mng)#show onu running config gpon-onu_1/3/1:1 pon-onu-mng gpon-onu_1/3/1:1

vlan port eth_0/1 mode transprant

！

The corresponding MIB configurations are:

snmpset -v2c -c public ipaddr .1.3.6.1.4.1.3902.1082.500.20.2.4.63.1.3.285278977.1.1 i 4

- trunk mode:

ZXAN(gpon-onu-mng)#show onu running config gpon-onu_1/3/1:1 pon-onu-mng gpon-onu_1/3/1:1

vlan port eth_0/1 mode trunk vlan port eth_0/1 vlan 100

！

The corresponding MIB operations are as follows:

snmpset -v2c -c public ipaddr .1.3.6.1.4.1.3902.1082.500.20.2.4.63.1.3.285278977.1.1 i 2

snmpset -v2c -c public ipaddr .1.3.6.1.4.1.3902.1082.500.20.2.4.64.1.1.285278977.1.1 i 1 .1.3.6.1.4.1.3902.1082.500.20.2.4.64.1.2.285278977.1.1 i 100

Other VLAN processing mode is similar. Please configure as per the MIB description.

##### zxAnGponRmVlanPortTable

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmVlan PortMode | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 63.1.3 | INTEGER { access ( 1 ) , trunk ( 2 ) , hybrid ( 3 ) , transparent ( 4 ) , none ( 5 ) } | The access mode of the port. There are five Types: - access: access mode of port access - trunk: trunk mode of port access - hybrid: hybrid mode of port access - transparent: transparent mode of port access - none: delete all the VLAN configuration of the port, restore to ONU's internal policy. |
| zxAnGponRmVlan PortDefaultVid | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 63.1.4 | Integer32 ( 1 .. 4094 ) | The pvid of the port. |
| zxAnGponRmVlan PortDefaultCos | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 63.1.5 | Integer32 ( 0 .. 7 ) | The default priority of the port. |

##### zxAnGponRmVlanPortCmdTable

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmVlan | . | INTEGER | The command to |
| PortCmd | 1.3.6.1.4.1.3902.108 | {addVlan | operate VLAN list of |
|  | 2.500.20.2.4.64.1.1 | (1) , delVlan | 'zxAnGponRmVlanPo |
|  |  | ( 2 ) } | rtTable'. |
|  |  |  | The multi-variables |
|  |  |  | binding rule should be |
|  |  |  | complied with |
|  |  |  | when a manager |
|  |  |  | configures port VLAN |
|  |  |  | cmd. According to this |
|  |  |  | rule, |
|  |  |  | all the related mib |
|  |  |  | variables must be |
|  |  |  | included in one |
|  |  |  | SNMP |
|  |  |  | set operation. |
|  |  |  | There are two Types |
|  |  |  | of variables: |
|  |  |  | mandatory and |
|  |  |  | optional. |
|  |  |  | All of the mandatory |
|  |  |  | variables must be |
|  |  |  | included in one |
|  |  |  | SNMP |
|  |  |  | set operation. |
|  |  |  | Optional variables |
|  |  |  | may be included in |
|  |  |  | one |
|  |  |  | SNMP set operation |
|  |  |  | with the mandatory |
|  |  |  | variables, but must |
|  |  |  | not be used alone. |
|  |  |  | To configure VLAN, |
|  |  |  | the mandatory and |
|  |  |  | optional variables |
|  |  |  | are as follows: |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | mandatoty: zxAnGponRmVlanPor tCmd, zxAnGponRmVlanPor tVid optional : none |
| zxAnGponRmVlan PortVid | . 1.3.6.1.4.1.3902.108 2.500.20.2.4.64.1.2 | Integer32 ( 1 .. 4094 ) | This variable must comply with the multi- variables binding rule described in 'zxAnGponRmVlanPo rtCmd'. |

##### zxAnGponRmVlanTransTable

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmVlan TransUserVid | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 65.1.1 | Integer32 ( 1 .. 4094 ) | User VLAN ID for translation. |
| zxAnGponRmVlan TransUserCos | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 65.1.2 | Integer32 ( 0 .. 8 ) | User priority for translation, the range of priority is from 0 to 8, and '8' indicates no configuration. |
| zxAnGponRmVlan TransCVid | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 65.1.3 | Integer32 (0.. 4094 ) | Inner VLAN for VLAN translate,'0' indicates no translation. The multi-variables binding rule should be complied with when a manager configures port VLAN translate. According to this rule, all the related mib variables must be included in one SNMP set operation. |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | There are two Types of variables: mandatory and optional. All of the mandatory variables must be included in one SNMP set operation. Optional variables may be included in one SNMP set operation with the mandatory variables, but must not be used alone. To configure VLAN translation, the mandatory and optional variables are as follows: mandatoty: zxAnGponRmVlanTransS Vid optional : zxAnGponRmVlanTransC Vid, zxAnGponRmVlanTransCt agCos, zxAnGponRmVlanTransS tagCos |
| zxAnGponRmVlan TransCtagCos | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 65.1.4 | Integer32 ( 0 .. 8 ) | Inner VLAN priority for VLAN translate,'8' indicates no translation. This variable must comply with the multi- variables binding rule described in 'zxAnGponRmVlanTransC Vid'. |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmVlan TransSVid | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 65.1.5 | Integer32 (0.. 4094 ) | Outer VLAN for VLAN translate,'0' indicates no SVLAN. This variable must comply with the multi- variables binding rule described in 'zxAnGponRmVlanTransC Vid'. |
| zxAnGponRmVlan TransStagCos | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 65.1.6 | Integer32 ( 0 .. 8 ) | Outer VLAN priority for VLAN translate,'8' indicates copy priority from inner VLAN priority. This variable must comply with the multi- variables binding rule described in 'zxAnGponRmVlanTransC Vid'. |
| zxAnGponRmVlan TransRowStatus | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 65.1.50 | RowStatus | In particular, a newly created row cannot be created until the corresponding at least one of 'zxAnGponRmVlanTrans CVid zxAnGponRmVlanTransC tagCos zxAnGponRmVlanTransS Vid zxAnGponRmVlanTransS tagCos' have been set. |

#### Multicast VLAN

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmMVid | . | Integer32 | This attribute |
|  | 1.3.6.1.4.1.3902.1 | ( 1 .. 4094 ) | represents the VLAN |
|  | 082.500.20.2.4.30. |  | ID(1~4094). |
|  | 1.1 |  |  |
| zxAnGponRmMVla | . | RowStatus |  |
| nRowStatus | 1.3.6.1.4.1.3902.1 |  |  |
|  | 082.500.20.2.4.30. |  |  |
|  | 1.50 |  |  |

Please refer to zxAnGponRmMVlanTable defined in ZTE-AN-GPON-REMOTE-ONU- MIB.mib.

#### VoIP

##### IP Profile

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmIpPrf | . | DisplayString | The profile name. |
| Name | 1.3.6.1.4.1.3902.1 | ( SIZE ( 1 .. |  |
|  | 082.500.20.2.6.24. | 64 ) ) |  |
|  | 1.1 |  |  |
| zxAnGponRmIpGa | . | InetAddress | The default gateway |
| teway | 1.3.6.1.4.1.3902.1 |  | address used for IP |
|  | 082.500.20.2.6.24. |  | host services, |
|  | 1.3 |  | this attribute has |
|  |  |  | default value 0 (not |
|  |  |  | set). If this |
|  |  |  | value is set, it |
|  |  |  | overrides any values |
|  |  |  | returned in DHCP. |
|  |  |  | The address Type |
|  |  |  | (InetAddressType) |
|  |  |  | that relates to |
|  |  |  | this object is |

Please refer to zxAnGponRmIpPrfTable defined in ZTE-AN-GPON-REMOTE-ONU- MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | specified by the corresponding value of 'zxAnGponRmIpGate wayType'. |
| zxAnGponRmIpPrf RowStatus | . 1.3.6.1.4.1.3902.1 082.500.20.2.6.24. 1.50 | RowStatus |  |

##### VLAN Profile

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmVlan PrfName | . 1.3.6.1.4.1.3902.1 082.500.20.2.6.25. 1.1 | DisplayStrin g (SIZE(1.. 64)) | This object indicates the profile name. |
| zxAnGponRmVlan PrfTagMode | . 1.3.6.1.4.1.3902.1 082.500.20.2.6.25. 1.2 | INTEGER {tag (1), vlanStackin g ( 2 ) } | VLAN tag mode. If mode is tag, 'zxAnGponRmVlanPrfS Vid' need not to be configured. If mode is vlanStacking, 'zxAnGponRmVlanPrf SVid' should be configured. |
| zxAnGponRmVlan PrfCVid | . 1.3.6.1.4.1.3902.1 082.500.20.2.6.25. 1.3 | Integer32 ( 1 .. 4094 ) | CVLAN tag ID. |
| zxAnGponRmVlan PrfCtagCos | . 1.3.6.1.4.1.3902.1 082.500.20.2.6.25. | Integer32 ( 0 .. 7 ) | CVLAN tag priority. |

Please refer to zxAnGponRmVlanPrfTable defined in ZTE-AN-GPON-REMOTE-ONU- MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 1.4 |  |  |
| zxAnGponRmVlan | . | RowStatus |  |
| PrfRowStatus | 1.3.6.1.4.1.3902.1 |  |  |
|  | 082.500.20.2.6.25. |  |  |
|  | 1.50 |  |  |

##### VoIP Protocol Configuration

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmVoip SignalProtoUsed | . 1.3.6.1.4.1.390 2.1082.500.20. 2.15.3.1.2 | INTEGER { none ( 1 ) , sip ( 2 ) , h248 ( 3 ) , mgcp ( 4 ) } | This attribute identifies the Type of VoIP signaling protocol used for ONU. Only one Type of protocol is allowed. Valid values are: 0x01 = None 0x02 = SIP 0x03 = H.248 0x04 = MGCP |

Please refer to zxAnGponRmVoipConfTable defined in ZTE-AN-GPON-REMOTE-ONU- MIB.mib.

##### VoIP IP Configuration

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmSl otId | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 .6.1.1 | Integer32 ( 0 .. 16 ) | This attribute indicates the index of slot in ONU. If slot ID is zero, it means the ONU is |

Please refer to zxAnGponRmVoipIpConfTable defined in ZTE-AN-GPON-REMOTE- ONU-MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | box-Type, which can not plug card. |
| zxAnGponRmM ediaIndex | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 .6.1.2 | INTEGER {signalAndMedia ( 1 ) , media ( 2 ) } | This attribute indicates the whether the separation of media and signaling.If 'media' is configured,The media will use a separation IP address,otherwise media and signaling will share the same IP address. |
| zxAnGponRmV oipIpMode | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 .6.1.3 | INTEGER { static ( 1 ) , dhcp ( 2 ) , pppoe ( 3 ) } | IP address configuration mode. |
| zxAnGponRmV oipIpPrf | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 .6.1.4 | DisplayString ( SIZE ( 0 .. 64 ) ) | This attribute refers to 'zxAnGponRmIpPrfNa me'. If no profile name is indicated, this attribute is set to a null string. |
| zxAnGponRmV oipVlanPrf | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 .6.1.5 | DisplayString ( SIZE ( 1 .. 64 ) ) | This attribute refers to 'zxAnGponRmVlanPrf Name'. |
| zxAnGponRmV oipHostId | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 .6.1.6 | Integer32 ( 1 .. 255 ) | The ONU IP host ID, if this attribute is not specified, system will select an available IP host ID. |
| zxAnGponRmV oipIpAddr | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 | InetAddress | This attribute is relation to 'zxAnGponRmVoipIp |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | .6.1.9 |  | Mode'. This attribute is valid only when 'zxAnGponRmVoipIp Mode' is static. |
| zxAnGponRmV oipIpAddrPfxLe n | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 .6.1.10 | InetAddressPrefix Length | VoIP IP address prefix length. |
| zxAnGponRmV oipIpConfRowSt atus | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 .6.1.50 | RowStatus |  |

##### SIP Profile

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmSipP rfName | . 1.3.6.1.4.1.3902.1082. 500.20.2.6.22.1.1 | DisplayStrin g ( SIZE ( 1 .. 64 ) ) | This object indicates the profile name. |
| zxAnGponRmSipP roxyAddr | . 1.3.6.1.4.1.3902.1082. 500.20.2.6.22.1.2 | DisplayStrin g ( SIZE ( 0 .. 128 ) ) | This attribute points to a large string ME that contains the name (IP address or URI) of the SIP proxy server for SIP signalling messages. |
| zxAnGponRmSipP rfRowStatus | . 1.3.6.1.4.1.3902.1082. 500.20.2.6.22.1.50 | RowStatus |  |

Please refer to zxAnGponRmSipPrfTable defined in ZTE-AN-GPON-REMOTE-ONU- MIB.mib.

##### MGC Profile

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmMgc PrfName | . 1.3.6.1.4.1.3902.1082. 500.20.2.6.23.1.1 | DisplayStrin g ( SIZE ( 1 .. 64 ) ) | This object indicates the profile name. |
| zxAnGponRmMgc PrimarySvr | . 1.3.6.1.4.1.3902.1082. 500.20.2.6.23.1.2 | DisplayStrin g ( SIZE ( 0 .. 128 ) ) | This attribute points to a network address ME that contains the name(IP address or resolved name) of the primary MGC that controls the signalling messages. |
| zxAnGponRmMgc UserTidAssignPoli cy | . 1.3.6.1.4.1.3902.1082. 500.20.2.6.23.1.20 | INTEGER {default ( 1 ) , specified ( 2 ) } | Default value '1' indicates User TID is assigned by ONU. The value '2' indicates that User TID is produced by the config data. |
| zxAnGponRmMgc UserTidPrefix | . 1.3.6.1.4.1.3902.1082. 500.20.2.6.23.1.21 | DisplayStrin g (SIZE ( 0 .. 128 ) ) | User TID Prefix. |
| zxAnGponRmMgc UserTidDigitLen | . 1.3.6.1.4.1.3902.1082. 500.20.2.6.23.1.22 | Integer32 ( 0 .. 7 ) | Length of user TID postfix |
| zxAnGponRmMgc UserTidDigitStartN o | . 1.3.6.1.4.1.3902.1082. 500.20.2.6.23.1.23 | Integer32 ( 0 .. 9 ) | Start number of user TID postfix. |
| zxAnGponRmMgc RtpTidAssignPolicy | . 1.3.6.1.4.1.3902.1082. | INTEGER {default ( 1 ) | Default value '1' indicates RTP TID |

Please refer to zxAnGponRmMgcPrfTable defined in ZTE-AN-GPON-REMOTE-ONU- MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 500.20.2.6.23.1.24 | , specified ( 2 ) } | is assigned by ONU. The value '2' indicates that RTP TID is produced by the config data. |
| zxAnGponRmMgc RtpTidPrefix | . 1.3.6.1.4.1.3902.1082. 500.20.2.6.23.1.25 | DisplayStrin g (SIZE(0.. 8 ) ) | RTP TID prefix |
| zxAnGponRmMgc RtpTidDigitLen | . 1.3.6.1.4.1.3902.1082. 500.20.2.6.23.1.26 | Integer32 ( 0 .. 7 ) | Length of RTP TID postfix. |
| zxAnGponRmMgc RtpTidDigitStartNo | . 1.3.6.1.4.1.3902.1082. 500.20.2.6.23.1.27 | Integer32 ( 0 .. 9 ) | Start number of RTP TID postfix. |
| zxAnGponRmMgc PrfRowStatus | . 1.3.6.1.4.1.3902.1082. 500.20.2.6.23.1.100 | RowStatus |  |

##### SIP Service

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmSipP rf | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 .4.1.1 | DisplayString (SIZE(1.. 64 ) ) | This attribute refers to the name of table 'zxAnGponRmSipPr fName'. |
| zxAnGponRmSipU serId | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 .4.1.2 | DisplayString (SIZE(1.. 128 ) ) | This attribute points to a large string that contains the user identification part of the address of record. This can take the form of an |

Please refer to zxAnGponRmSipUserTable defined in ZTE-AN-GPON-REMOTE-ONU- MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | alphanumeric string or the subscriber's directory number. |
| zxAnGponRmSipU sername | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 .4.1.3 | DisplayString (SIZE ( 0 .. 50 ) ) | This string attribute is the user name. If the string is shorter than 50 bytes, it must be null terminated. |
| zxAnGponRmSipP assword | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 .4.1.4 | DisplayString (SIZE ( 0 .. 25 ) ) | This string attribute is the password. If the string is shorter than 25 bytes, it must be null terminated. |
| zxAnGponRmSipR owStatus | . 1.3.6.1.4.1.3902. 1082.500.20.2.15 .4.1.50 | RowStatus |  |

##### MGC Service

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmMgc Prf | . 1.3.6.1.4.1.3902.1 082.500.20.2.15.5. 1.1 | DisplayString ( SIZE ( 1 .. 64 ) ) | This attribute refers to the name of table 'zxAnGponRmMgcPrf Name'. |
| zxAnGponRmMgc RowStatus | . 1.3.6.1.4.1.3902.1 082.500.20.2.15.5. 1.50 | RowStatus |  |

Please refer to zxAnGponRmMgcUserTable defined in ZTE-AN-GPON-REMOTE-ONU- MIB.mib.

#### POTS Interface

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmPots | . | INTEGER |  |
| UniAdminState | 1.3.6.1.4.1.3902.1082. | {unlocked |  |
|  | 500.20.2.15.2.1.1 | (1 ) , locked ( |  |
|  |  | 2 ) } |  |
| zxAnGponRmPots | . | INTEGER |  |
| UniOperState | 1.3.6.1.4.1.3902.1082. | { enabled |  |
|  | 500.20.2.15.2.1.8 | ( 1 ) , |  |
|  |  | disabled ( 2 ) |  |
|  |  | } |  |

Please refer to zxAnGponRmPotsUniTable defined in ZTE-AN-GPON-REMOTE-ONU- MIB.mib.

#### Remote Operation

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmOn uReboot | . 1.3.6.1.4.1.3902.1 082.500.20.2.1.10. 1.1 | INTEGER {reboot ( 1 ) } | This attribute identifies the reboot action of the ONU. |
| zxAnGponRmOn uSyncTime | . 1.3.6.1.4.1.3902.1 082.500.20.2.1.10. 1.2 | INTEGER {syncTime ( 1 ) } | This attribute identifies the action of the ONU. Synchronize the start time of all performance monitoring managed entities of the ONU with the reference time of the OLT. |
| zxAnGponRmOn uRestore | . 1.3.6.1.4.1.3902.1 082.500.20.2.1.10. 1.3 | INTEGER {restore ( 1 ) } | This attribute identifies the action of the ONU. Reset the MIB data sync attribute to 0 |

Please refer to zxAnGponRmOnuOperTable defined in ZTE-AN-GPON-REMOTE-ONU- MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | and reset the MIB of the ONU to its default, then deliver the MIB data to ONU to bring ONU MIB into alignment with its own. |
| zxAnGponRmOn uRestoreFactory | . 1.3.6.1.4.1.3902.1 082.500.20.2.1.10. 1.4 | INTEGER {restore (1) } | This attribute identifies the action of the ONU, restores the ONU's factory configuration. |

#### MAC Address Table Query

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmBpM acAddrIndex | . 1.3.6.1.4.1.3902 .1082.500.20.2. 4.16.1.1 |  |  |
| zxAnGponRmBpM acAddrType | . 1.3.6.1.4.1.3902 .1082.500.20.2. 4.16.1.2 | INTEGER { static ( 1 ) , dynamic ( 2 ) } | The Type of addresses entries, the valied value is: static -- This entry is statically assigned. dynamic -- This entry is dynamically learned. |
| zxAnGponRmBpM acAddrFilterMode | . 1.3.6.1.4.1.3902 .1082.500.20.2. 4.16.1.3 | INTEGER { forward ( 1 ) , filter ( 2 ) } | The filter mode of addresses entries, indecate whether packets having them as destination addresses are filtered or forwarded. |

Please refer to zxAnGponRmBpMacAddrListTable defined in ZTE-AN-GPON- REMOTE-ONU-MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmBpM acAddrAge | . 1.3.6.1.4.1.3902 .1082.500.20.2. 4.16.1.4 | Integer32 ( 1 .. 4095 ) | The MAC address's age, unit in seconds. |
| zxAnGponRmBpM acAddr | . 1.3.6.1.4.1.3902 .1082.500.20.2. 4.16.1.5 | MacAddress | The MAC address. |

Operation instructions:

Only support getnext operations.

#### WIFI Configuration

[MIB file]:

Please refer to zxAnPonRmWifiUniTable defined in ZTE-AN-PON-REMOTE-ONU- MIB.mib.

###### zxAnPonRmWifiIfAdminStatus

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.3.1.1

###### INTEGER

###### { enabled ( 1 ) , disabled ( 2 ) }

MIB Variable OID

MIB Value Description

###### Enable or disable the WiFi interface function. zxAnPonRmWifiIfOperationalStatus

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.3.1.2

###### INTEGER

###### { enabled ( 1 ) , disabled ( 2 ) }

###### The operational status of WiFi interface. zxAnPonRmWifiIfRadioMeasEnable

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.3.1.3

###### INTEGER

###### { enabled ( 1 ) , disabled ( 2 ) }

###### Enable or disable the radio measurement function.

###### zxAnPonRmWifiIfIsolationEnable

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.3.1.4

###### INTEGER

###### { enabled ( 1 ) , disabled ( 2 ) }

###### Enable or disable the SSID isolation function. zxAnPonRmWifiIfSgiEnable

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.3.1.5

###### INTEGER

###### { enabled ( 1 ) , disabled ( 2 ) }

###### This attribute enable or disable the SGI function.

###### This attribute is valid when zxAnPonRmWifiIfWirelessWorkMode is '802.11n' and '802.11b-11g-11n'.

###### zxAnPonRmWifiIfWirelessWorkMode

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.3.1.6

###### BITS {ieee802for11b (0) , ieee802for11g (1) , ieee802for11n ( 2 ) } The wireless work mode of WiFi interface.

###### The attribute is a bit map, format as follows:

###### Bit 0 indicates '802.11b' Bit 1 indicates '802.11g' Bit 2 indicates '802.11n'.

###### Each bit,0 indicates not supported, 1 indicates supportd. zxAnPonRmWifiIfRegulatoryDomain

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.3.1.7

###### INTEGER { other ( 1 ) , usa ( 2 ) , canada ( 3 ) , etsi ( 4 ) , spain ( 5 ) , france ( 6 ) , mkk ( 7 ) , china ( 8 ) , russia ( 9 ) , brazil ( 10 ) , egypt ( 11 ) , germany ( 12 ) , greece ( 13 ) , hongkong ( 14 ) , italy ( 15 ) , korea ( 16 ) , uk ( 17 ) , portugal ( 18 ) , lithuania ( 19 ) }

###### The current regulatory domain.

###### The values 'mkk' and 'etsi' are only suported by EPON. The values only supported by GPON are as follow:

###### 'russia', 'brazil', 'egypt', 'germany', 'greece', 'hongkong',

###### 'italy', 'korea', 'uk', 'portugal', 'lithuania'.

###### The value 'brazil' is valid when zxAnPonRmWifiIfWirelessWorkMode is '802.11b'.

###### zxAnPonRmWifiIfChannel

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.3.1.8

###### INTEGER

###### {auto ( 1 ) , channel1 ( 2 ) , channel2 ( 3 ) , channel3 ( 4 ) , channel4 ( 5 ) , channel5 ( 6 ) , channel6 ( 7 ) , channel7 ( 8 ) , channel8 ( 9 ) , channel9 ( 10 ) , channel10 ( 11 ) , channel11 ( 12 ) , channel12 ( 13 ) , channel13 ( 14 ) }

###### This attribute defines the channel. This attribute can not be

###### assigned as 'channel12' and 'channel13' when zxAnPonRmWifiIfRegulatoryDomain is 'fcc' or 'doc'. zxAnPonRmWifiIfChannelBandwidth

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.3.1.9

###### INTEGER { for20Mhz ( 1 ) , for40Mhz ( 2 ) }

###### This attribute defines the channel bandwidth of WiFi interface. This attribute is valid when zxAnPonRmWifiIfWirelessWorkMode is '802.11n' and '802.11b-11g-11n'.

###### zxAnPonRmWifiIfQosType

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.3.1.10

###### INTEGER { disabled ( 1 ) , wmm ( 2 ) , ssid ( 3 ) } This attribute defines the QoS Type. zxAnPonRmWifiIfTxRate

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.3.1.11

###### INTEGER

###### { auto ( 1 ) , for1Mbps ( 2 ) , for2Mbps ( 3 ) , for5Dot5Mbps ( 4 ) , for6Mbps ( 5 ) , for9Mbps ( 6 ) , for11Mbps ( 7 ) , for12Mbps ( 8 ) , for18Mbps ( 9 ) , for24Mbps ( 10 ) , for36Mbps ( 11 ) , for48Mbps ( 12 ) , for54Mbps ( 13 ) }

###### This attribute defines the transmitting rate.

###### When zxAnPonRmWifiIfWirelessWorkMode is '802.11b', this attribute can only be assigned as 'auto', 'for1Mbps', 'for2Mbps', 'for5Dot5Mbps' and 'for11Mbps'.

###### When zxAnPonRmWifiIfWirelessWorkMode is '802.11n' and '802.11b-11g-11n', this attribute

###### can only be assigned as 'auto'. zxAnPonRmWifiIfTxPower

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.3.1.12

###### INTEGER { percent100 ( 1 ) , percent80 ( 2 ) , percent60 ( 3 ) , percent40 ( 4 ) , percent20 ( 5 ) } This attribute defines the transmitting power.

Please refer to zxAnPonRmWifiSsidTable defined in ZTE-AN-PON-REMOTE-ONU- MIB.mib.

###### zxAnPonRmWifiSsid

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.1

###### Integer32

###### MIB Variable OID

###### MIB Value Description

###### ( 1 .. 8 )

###### ONU WiFi SSID(Service Set Identifier). zxAnPonRmWifiSsidName

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.2

###### DisplayString ( SIZE ( 0 .. 32 ) ) The SSID name. zxAnPonRmWifiSsidAdminStatus

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.3

###### INTEGER { enabled ( 1 ) , disabled ( 2 ) } Enable or disable the SSID. zxAnPonRmWifiSsidHideEnable

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.4

###### INTEGER { enabled ( 1 ) , disabled ( 2 ) } Enable or disable the SSID hide function. zxAnPonRmWifiSsidIsolationEnable

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.5

###### INTEGER { enabled (1) , disabled ( 2 ) }

###### Enable or disable the SSID user isolation function. zxAnPonRmWifiSsidMaxUserNum

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.6

###### Integer32

###### ( 1 .. 65535 )

###### The maximum SSID user number. zxAnPonRmWifiSsidCurrentUserNum

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.7

###### Integer32

###### ( 0 .. 65535 )

###### The current SSID user number. zxAnPonRmWifiSsidMacAddress

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.8

###### MacAddress

###### This MAC address associated with this SSID. zxAnPonRmWifiSsidAuthAlgorithm

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.9

###### INTEGER

###### { openSystem ( 1 ) , sharedKey ( 2 ) , wpaPsk ( 3 ) , wpa2Psk ( 4 ) , wpaPskAndWpa2Psk ( 5 ) } The SSID authentication algorithms.

###### zxAnPonRmWifiSsidWepEnable

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.10

###### INTEGER

###### {enabled(1), disabled ( 2 ) }

###### Enable or disable the WEP encryption function.

###### The attribute is valid when zxAnPonRmWifiSsidAuthAlgorithm is openSystem or sharedKey.

###### zxAnPonRmWifiSsidWepKeyLen

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.11

###### INTEGER

###### { for64 ( 1 ) , for128 ( 2 ) }

###### The WEP encryption key bits length.

###### The attribute is valid when zxAnPonRmWifiSsidWepEncryEnable is enabled.

###### zxAnPonRmWifiSsidWepKeyFormat

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.12

###### INTEGER

###### { ascii ( 1 ) , hex ( 2 ) }

###### The WEP encryption key format.

###### The attribute is valid when zxAnPonRmWifiSsidWepEncryEnable is enabled.

###### zxAnPonRmWifiSsidWepKeyValue

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.13

###### DisplayString

###### ( SIZE ( 0 | 20 | 40 | 52 | 104 ) )

###### The WEP encryption key value that consists of four groups ASCII or hex value,format as follows:

###### for64 and ascii -- each group consists of 5 bytes string; for128 and ascii -- each group consists of 13 bytes string; for64 and hex -- each group consists of 10 bytes hex string; for128 and hex -- each group consists of 26 bytes hex string;

###### The attribute is valid when zxAnPonRmWifiSsidWepEncryEnable is enable.

###### It should be NULL when this entry used WAP authenticaton. zxAnPonRmWifiSsidWepDefKey

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.14

###### Integer32 ( 1 .. 4 )

###### The default WEP encryption key index.

###### The attribute is valid when zxAnPonRmWifiSsidWepEncryEnable is enabled.

###### zxAnPonRmWifiSsidWpaInterval

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.15

###### Integer32

###### ( 100 .. 1000 )

###### The WPA encryption key updating interal.

###### The attribute is valid when zxAnPonRmWifiSsidAuthAlgorithm is wpaPsk or wpa2Psk or wpaPskAndWpa2Psk. zxAnPonRmWifiSsidWpaAlgorithm

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.16

###### BITS

###### { aes ( 0 ) , tkip ( 1 ) , wep ( 2 ) } The WPA encryption Algorithm.

###### The attribute is valid when zxAnPonRmWifiSsidAuthAlgorithm is wpaPsk or wpa2Psk or wpaPskAndWpa2Psk.

###### The attribute is a bit map, format as follows:

###### Bit 0 indicates 'AES' Bit 1 indicates 'TKIP' Bit 2 indicates 'WEP'.

###### Each bit,0 indicates not supported, 1 indicates supportd. zxAnPonRmWifiSsidWpaKeyValue

###### .1.3.6.1.4.1.3902.1082.500.3.2.7.4.1.17

###### DisplayString

###### ( SIZE ( 0 | 8 .. 63 ) )

###### The WPA encryption key value that consists of 8-63 bytes string; The attribute is valid when zxAnPonRmWifiSsidAuthAlgorithm is wpaPsk or wpa2Psk or wpaPskAndWpa2Psk.

###### It should be NULL when this entry used WEP authenticaton.

#### VEIP Configuration

switchport-bind switch_0/1 veip 1

{"zxGponUNIMACBridgePortConfigTable", "1.3.6.1.4.1.3902.1012.3.50.15.3"} ,

{"zxGponUNIMACBridgePortConfigEntry", "1.3.6.1.4.1.3902.1012.3.50.15.3.1"} ,

{"zxGponUNIMACBridgePortConfigBridgeId","1.3.6.1.4.1.3902.1012.3.50.15.3.1.1"}  ,

{"zxGponUNIMACBridgePortConfigBridgePort","1.3.6.1.4.1.3902.1012.3.50.15.3.1.2"}  ,

{"zxGponUNIMACBridgePortConfigPortPrio","1.3.6.1.4.1.3902.1012.3.50.15.3.1.3"}  ,

{"zxGponUNIMACBridgePortConfigPortPathCost","1.3.6.1.4.1.3902.1012.3.50.15.3.1.4"

} ,

{"zxGponUNIMACBridgePortConfigPortSpanTreeInd","1.3.6.1.4.1.3902.1012.3.50.15.3. 1.5"} ,

{"zxGponUNIMACBridgePortConfigEncapMethod","1.3.6.1.4.1.3902.1012.3.50.15.3.1.6"

} ,

{"zxGponUNIMACBridgePortConfigLANFCSInd","1.3.6.1.4.1.3902.1012.3.50.15.3.1.7"}

,

{"zxGponUNIMACBridgePortConfigPortMACAddr","1.3.6.1.4.1.3902.1012.3.50.15.3.1.8"

} ,

{"zxGponUNIMACBridgePortConfigEntryStatus","1.3.6.1.4.1.3902.1012.3.50.15.3.1.9"}  ,

{"zxGponUNIMACBridgePortConfigOutboundTDPtr","1.3.6.1.4.1.3902.1012.3.50.15.3.1. 10"} ,

{"zxGponUNIMACBridgePortConfigInboundTDPtr","1.3.6.1.4.1.3902.1012.3.50.15.3.1.1 1"} ,

{"zxGponUniMacBridgePortAddrLearnDepth","1.3.6.1.4.1.3902.1012.3.50.15.3.1.12"}  ,

{"zxGponUniMacBPortOutboundTdPrf",	"1.3.6.1.4.1.3902.1012.3.50.15.3.1.13"} ,

{"zxGponUniMacBPortInboundTdPrf", "1.3.6.1.4.1.3902.1012.3.50.15.3.1.14"} ,

Third-level index: INDEX { zxGponOltIndex, zxGponONTIndex, zxGponUNIIndex } The code of zxGponUNIIndex is (2 << 16) | (UNI slot << 8) | UNI port

2 indicates the VEIP type.

When creating the VEIP bridge port, you need to specify zxGponUNIMACBridgePortConfigBridgeId.

##### VEIP Bridge Port Configuration

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmBpC onfBridgeId | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 15.1.1 | Integer32 ( 0 .. 65535 ) | This attribute is set- by-create, indicating the bridge when create an bridge port. The first byte stands for the slot ID, the second byte stands for the bridge ID.(2 bytes) |
| zxAnGponRmBpC onfPortPri | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 15.1.2 | Integer32 ( 0 .. 255 ) | This attribute denotes the priority of the port for use in (rapid) spanning tree algorithms. The range is 0..255. |
| zxAnGponRmBpC onfPortPathCost | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 15.1.4 | Integer32 ( 1 .. 65535 ) | This attribute provides the cost contribution of the port to the path cost towards the spanning tree root bridge. The range is 0x0001 to 0xFFFF (1 to 65535). |
| zxAnGponRmBpC onfPortStpEnable | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 15.1.5 | INTEGER { enabled ( 1 ) , disabled ( 2 ) } | This attribute indicates whether or not STP LAN topology change detection is enabled at this port. |
| zxAnGponRmBpC onfPortMacAddr | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. | MacAddress | This attribute indicates the physical MAC address used by |

Please refer to zxAnGponRmBpConfTable defined in ZTE-AN-GPON-REMOTE-ONU- MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  | 15.1.6 |  | the Port as defined by the TP pointer when the TP Type is set to LAN. |
| zxAnGponRmBpC onfOutPrfName | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 15.1.7 | DisplayString (SIZE ( 0 .. 64 ) ) | This attribute points to a GEM traffic descriptor that limits the traffic rate leaving the MAC bridge. |
| zxAnGponRmBpC onfInPrfName | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 15.1.8 | DisplayString ( SIZE ( 0 .. 64 ) ) | This attribute points to a GEM traffic descriptor that limits the traffic rate entering the MAC bridge. |
| zxAnGponRmBpC onfAddrLearnDept h | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 15.1.9 | Integer32 ( 0 .. 255 ) | This attribute specifies the maximum number of MAC addresses to be learned by this MAC bridge port. The default value 0 specifies that there is no administratively- imposed limit. |
| zxAnGponRmBpM acAddrListTotalNu m | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 15.1.10 |  | Total number of addresses entries related with this port. |
| zxAnGponRmBpC onfEntryRowStatus | . 1.3.6.1.4.1.3902. 1082.500.20.2.4. 15.1.50 | RowStatus |  |

zxAnPonRmOnuIfIndex sets the VEIP interface type.

##### VEIP VLAN Configuration

[MIB file]:

Please refer to zxAnGponRmVlanPortTable, zxAnGponRmVlanPortCmdTable and zxAnGponRmVlanTransTable defined in ZTE-AN-GPON-REMOTE-ONU-MIB.mib.

Remarks:

If you use VEIP VLAN configurations, the VEIP bridge ports will be created automatically.

#### Ethernet Port Performance Statistics

##### Enabling/Disabling Ethernet Port Performance Statistics

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmPerf Type | . 1.3.6.1.4.1.3902 .1082.500.23.2. 2.1.1 | INTEGER { fec ( 1 ) , gemport ( 2 ) , brg ( 3 ) , bp ( 4 ) , dot1x ( 5 ) , raidus ( 6 ) , eth ( 7 ) , mvlan ( 8 ) , vlan ( 9 ) , iphost ( 10 ) , pw ( 11 ) , rtp ( 12 ) , sip ( 13 ) , mgc ( 14 ) , callctrl ( 15 ) , voice ( 16 ) , uniVoice ( 17 ) } | zxAnGponRmPerfTyp e indentified the GPON remote ONU performance monitoring MIB table Type. |
| zxAnGponRmPerfI ndex | . 1.3.6.1.4.1.3902 .1082.500.23.2. 2.1.2 | OBJECT IDENTIFIER | The index can be referred from the related current 15 minutes MIB |

Please refer to zxAnGponRmPerfConfTable defined in ZTE-AN-GPON-REMOTE-ONU- PERF-MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | table. The related current 15 minutes MIB table is determined by 'zxAnGponRmPerfTy pe': fec -- zxAnGponRmFecC15 MPerfTable gemport -- zxAnGponRmGemPo rtC15MPerfTable brg -- zxAnGponRmBrgC15 MPerfTable bp -- zxAnGponRmBpC15 MPerfTable dot1x -- zxAnGponRmDot1xC 15MPerfTable raidus -- zxAnGponRmRadius C15MPerfTable eth -- zxAnGponRmEthC15 MPerfTable mvlan -- zxAnGponRmMVlanC 15MPerfTable vlan -- zxAnGponRmVlanC1 5MPerfTable iphost -- zxAnGponRmIphostC 15MPerfTable pw -- zxAnGponRmPwC15 |

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
|  |  |  | MPerfTable rtp -- zxAnGponRmRtpC15 MPerfTable sip -- zxAnGponRmSipC15 MPerfTable mgc -- zxAnGponRmMgcC1 5MPerfTable callctrl -- zxAnGponRmCallCtrl C15MPerfTable voice -- zxAnGponRmVoiceC 15MPerfTable uniVoice -- zxAnGponRmUniVoic eC15MPerfTable |
| zxAnGponRmPerf ThreshPrf | . 1.3.6.1.4.1.3902 .1082.500.23.2. 2.1.3 | DisplayString ( SIZE ( 0 .. 32 ) ) | The performance threshold profile. |
| zxAnGponRmPerf ConfRowStatus | . 1.3.6.1.4.1.3902 .1082.500.23.2. 2.1.50 | RowStatus |  |

The value of zxAnGponRmPerfMIB is set into eth(7). zxAnGponRmPerfIndex is set into the index of corresponding Ethernet port.

##### Querying Ethernet Port Performance Statistics

[Function description]:

Ethernet port performance statistics can only be queried after the Ethernt port performance statistics is enabled.

        - Ethernet Port Performance Statistics in the Current 15 Minutes

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponRmC15 MRxDropEvents | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 3 | This object counts the total number of events in which packets received were dropped due to lack of resources. This is not necessarily the number of packets dropped;it is the number of times this event was detected. |
| zxAnGponRmC15 MRxOctets | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 4 | This object counts the total number of octets received from the CPE, including those in bad packets, excluding framing bytes, but including FCS. |
| zxAnGponRmC15 MRxPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 5 | This object counts the total number of packets received, including bad packets,broadcast packets and multicast packets. |
| zxAnGponRmC15 MRxUcastPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 6 | This object counts the total number of unicast frames received. |
| zxAnGponRmC15 MRxBcastPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 7 | This object counts the total number of received good packets directed to the broadcast address. This does not include multicast packets. |
| zxAnGponRmC15 MRxMcastPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 8 | This object counts the total number of received good packets directed to a multicast address. This does not include broadcast packets. |
| zxAnGponRmC15 MRxCrcAlignError s | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 9 | The total number of upstream packets received that had a length (excluding framing bits, but including FCS octets) of between 64 and |

Please refer to zxAnGponRmEthC15MPerfTable defined in ZTE-AN-GPON-REMOTE- ONU-PERF-MIB.mib for the Ethernet port performance statistics in the current 15 minutes.

| MIB Variable | OID | Description |
| --- | --- | --- |
|  |  | 1518 octets,inclusive, but had either a bad frame check sequence (FCS) with an integral number of octets (FCS error) or a bad FCS with a non-integral number of octets (alignment error). |
| zxAnGponRmC15 MRxUndersizePkt s | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 10 | This object counts the total number of packets received that were less than 64 octets long but were otherwise well formed (excluding framing bits, but including FCS). |
| zxAnGponRmC15 MRxOversizePkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 11 | The total number of upstream packets received that were longer than 1518 octets (excluding framing bits, but including FCS) and were otherwise well formed. Note- If 2000-byte ethernet frames are supported,counts in this performance parameter are not necessarily errors. |
| zxAnGponRmC15 MRxDiscardPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 12 | This object counts the total number of discarded packets received. |
| zxAnGponRmC15 MRxPkts64Octets | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 13 | This object counts the total number of received packets (including bad packets) that were 64 octets long, excluding framing bits but including FCS. |
| zxAnGponRmC15 MRx65To127Octet s | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 14 | This object counts the total number of received packets (including bad packets) that were 65..127 octets long, excluding framing bits but including FCS. |
| zxAnGponRmC15 MRx128To255Oct ets | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 15 | This objecet counts the total number of packets (including bad packets) received that were 128..255 octets long, excluding framing |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  |  | bits but including FCS. |
| zxAnGponRmC15 MRx256To511Oct ets | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 16 | This object counts the total number of packets (including bad packets) received that were 256..511 octets long, excluding framing bits but including FCS. |
| zxAnGponRmC15 MRx512To1023Oc tets | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 17 | This object counts the total number of packets (including bad packets) received that were 512..1023 octets long, excluding framing bits but including FCS. |
| zxAnGponRmC15 MRx1024To1518O ctets | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 18 | This object counts The total number of packets (including bad packets) received that were 1024..1518 octets long, excluding framing bits but including FCS. |
| zxAnGponRmC15 MRxPauseFrms | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 19 | This object counts the total number of PAUSE frames received. |
| zxAnGponRmC15 MRxErrorOctets | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 20 | This object counts the total number of error packets bytes received. |
| zxAnGponRmC15 MRxRate | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 21 | Received bits per second. |
| zxAnGponRmC15 MRxMcastRate | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 22 | Received multicast bits per second. |
| zxAnGponRmC15 MTxDropEvents | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 23 | This object counts the total number of events in which packets transmitted were dropped due to lack of resources. This is not necessarily the number of packets dropped;it is the number of times this event was detected. |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponRmC15 MTxOctets | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 24 | This object counts the total number of octets transmitted from the CPE,including those in bad packets, excluding framing bytes, but including FCS. |
| zxAnGponRmC15 MTxPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 25 | This object counts the total number of packets transmitted, including bad packets,broadcast packets and multicast packets. |
| zxAnGponRmC15 MTxUcastPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 26 | This object counts the total number of unicast frames transmitted. |
| zxAnGponRmC15 MTxBcastPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 27 | This object counts the total number of transmitted good packets directed to the broadcast address. This does not include multicast packets. |
| zxAnGponRmC15 MTxMcastPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 28 | This object counts the total number of transmitted good packets directed to a multicast address. This does not include broadcast packets. |
| zxAnGponRmC15 MTxCrcAlignError s | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 29 | The total number of upstream packets transmitted that had a length (excluding framing bits, but including FCS octets) of between 64 and 1518 octets,inclusive, but had either a bad frame check sequence (FCS) with an integral number of octets (FCS error) or a bad FCS with a non-integral number of octets (alignment error). |
| zxAnGponRmC15 MTxUndersizePkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 30 | This object counts the total number of packets transmitted that were less than 64 octets long but were otherwise well formed (excluding framing bits, but including FCS). |
| zxAnGponRmC15 | . | The total number of upstream packets |

| MIB Variable | OID | Description |
| --- | --- | --- |
| MTxOversizePkts | 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 31 | transmitted that were longer than 1518 octets (excluding framing bits, but including FCS) and were otherwise well formed. Note- If 2000-byte ethernet frames are supported,counts in this performance parameter are not necessarily errors. |
| zxAnGponRmC15 MTxDiscardPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 32 | This object counts the total number of discarded packets transmitted. |
| zxAnGponRmC15 MTxPkts64Octets | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 33 | This object counts the total number of transmitted packets (including bad packets) that were 64 octets long, excluding framing bits but including FCS. |
| zxAnGponRmC15 MTx65To127Octet s | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 34 | This object counts the total number of transmitted packets (including bad packets) that were 65..127 octets long, excluding framing bits but including FCS. |
| zxAnGponRmC15 MTx128To255Oct ets | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 35 | This objecet counts the total number of packets (including bad packets) transmitted that were 128..255 octets long, excluding framing bits but including FCS. |
| zxAnGponRmC15 MTx256To511Octe ts | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 36 | This object counts the total number of packets (including bad packets) transmitted that were 256..511 octets long, excluding framing bits but including FCS. |
| zxAnGponRmC15 MTx512To1023Oc tets | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 37 | This object counts the total number of packets (including bad packets) transmitted that were 512..1023 octets long, excluding framing bits but including FCS. |
| zxAnGponRmC15 | . | This object counts The total number of |

| MIB Variable | OID | Description |
| --- | --- | --- |
| MTx1024To1518O ctets | 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 38 | packets (including bad packets) transmitted that were 1024..1518 octets long, excluding framing bits but including FCS. |
| zxAnGponRmC15 MTxPauseFrms | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 39 | This object counts the total number of PAUSE frames transmitted. |
| zxAnGponRmC15 MTxErrorOctets | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 40 | This object counts the total number of error packets bytes transmitted. |
| zxAnGponRmC15 MTxRate | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 41 | Transmitted bits per second. |
| zxAnGponRmC15 MTxMcastRate | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 42 | Transmitted multicast bits per second. |
| zxAnGponRmC15 MPppoeFilterFrms | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 43 | This object counts the number of frames discarded due to PPPoE filtering. |
| zxAnGponRmC15 MFragments | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 44 | This object counts the total number of packets received that were less than 64 octets long, excluding framing bits but including FCS octets, and had either a bad frame check sequence (FCS) with an integral number of octets (FCS error) or a bad FCS with a non-integral number of octets (alignment error). It is entirely normal for this attribute to increment. This is because it counts both runts (which are normal occurrences due to collisions) and noise hits. |
| zxAnGponRmC15 | . | This object counts the total number of |

| MIB Variable | OID | Description |
| --- | --- | --- |
| MJabbers | 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 45 | packets received that were longer than 1518 octets,excluding framing bits but including FCS octets, and had either a bad frame check sequence (FCS) with an integral number of octets (FCS error) or a bad FCS with a non-integral number of octets (alignment error). The range to detect jabber is between 20 ms and 150 ms. |
| zxAnGponRmC15 MCollisions | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 46 | This object counts the total number of collisions transmitted (half-duplex Mode). |
| zxAnGponRmC15 MAlignErrors | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 47 | This object counts received frames that were not an integral number of octets in length and did not pass the FCS check. |
| zxAnGponRmC15 MFcsErrors | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 48 | This object counts frames received on a particular interface that were an integral number of octets in length but failed the frame check sequence (FCS) check. |
| zxAnGponRmC15 MSingleCollisionFr m | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 49 | This object counts successfully transmitted frames whose transmission was delayed by exactly one collision. |
| zxAnGponRmC15 MMultiCollisionFr ms | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 50 | This object counts successfully transmitted frames whose transmission was delayed by more than one collision. |
| zxAnGponRmC15 MSqeTestErrors | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 51 | This object counts the number of times that the SQE test error message was generated by the PLS sublayer. |
| zxAnGponRmC15 MDeferredTxFrms | . 1.3.6.1.4.1.3902.1 | This object counts frames whose first transmission attempt was delayed |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | 082.500.23.2.36.1. 52 | because the medium was busy. The count does not include frames involved in collisions. |
| zxAnGponRmC15 MLateCollisions | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 53 | This object counts the number of times that a collision was detected later than 512 bit times into the transmission of a packet. |
| zxAnGponRmC15 MExcessCollisions | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 54 | This objecet counts frames whose transmission failed due to excessive collisions. |
| zxAnGponRmC15 MInternalMacTxEr rs | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 55 | This object counts frames whose transmission failed due to an internal MAC sublayer transmit error. |
| zxAnGponRmC15 MInternalMacRxEr rs | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 56 | This object counts frames whose reception failed due to an internal MAC sublayer receive error. |
| zxAnGponRmC15 MCarrierSenseErr ors | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 57 | This object counts the number of times that carrier sense was lost or never asserted when attempting to transmit a frame. |
| zxAnGponRmC15 MFrameTooLongs | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 58 | This object counts received frames that exceeded the maximum permitted frame size. |
| zxAnGponRmC15 MRxBufOverflows | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 59 | This object counts the number of times that the receive buffer overflowed. |
| zxAnGponRmC15 MTxBufOverflows | . 1.3.6.1.4.1.3902.1 082.500.23.2.36.1. 60 | This object counts the number of times that the transmit buffer overflowed. |

        - Ethernet Port Performance Statistics in the Last 15 Minutes

[Function description]:

This MIB only support querying Ethernet port performance statistics in the last 15 minutes.

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponRmH15 MRxDropEvents | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 5 | This object counts the total number of events in which packets received were dropped due to lack of resources. This is not necessarily the number of packets dropped;it is the number of times this event was detected. |
| zxAnGponRmH15 MRxOctets | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 6 | This object counts the total number of octets received from the CPE, including those in bad packets, excluding framing bytes, but including FCS. |
| zxAnGponRmH15 MRxPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 7 | This object counts the total number of packets received, including bad packets,broadcast packets and multicast packets. |
| zxAnGponRmH15 MRxUcastPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 8 | This object counts the total number of unicast frames received. |
| zxAnGponRmH15 MRxBcastPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 9 | This object counts the total number of received good packets directed to the broadcast address. This does not include multicast packets. |
| zxAnGponRmH15 MRxMcastPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 10 | This object counts the total number of received good packets directed to a multicast address. This does not include broadcast packets. |

Please refer to zxAnGponRmEthH15MPerfTable defined in ZTE-AN-GPON-REMOTE- ONU-PERF-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponRmH15 MRxCrcAlignErrors | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 11 | The total number of upstream packets received that had a length (excluding framing bits, but including FCS octets) of between 64 and 1518 octets,inclusive, but had either a bad frame check sequence (FCS) with an integral number of octets (FCS error) or a bad FCS with a non-integral number of octets (alignment error). |
| zxAnGponRmH15 MRxUndersizePkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 12 | This object counts the total number of packets received that were less than 64 octets long but were otherwise well formed (excluding framing bits, but including FCS). |
| zxAnGponRmH15 MRxOversizePkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 13 | The total number of upstream packets received that were longer than 1518 octets (excluding framing bits, but including FCS) and were otherwise well formed. Note- If 2000-byte ethernet frames are supported,counts in this performance parameter are not necessarily errors. |
| zxAnGponRmH15 MRxDiscardPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 14 | This object counts the total number of discarded packets received. |
| zxAnGponRmH15 MRxPkts64Octets | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 15 | This object counts the total number of received packets (including bad packets) that were 64 octets long, excluding framing bits but including FCS. |
| zxAnGponRmH15 MRx65To127Octet s | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 16 | This object counts the total number of received packets (including bad packets) that were 65..127 octets long, excluding framing bits but |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  |  | including FCS. |
| zxAnGponRmH15 MRx128To255Octe ts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 17 | This objecet counts the total number of packets (including bad packets) received that were 128..255 octets long, excluding framing bits but including FCS. |
| zxAnGponRmH15 MRx256To511Octe ts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 18 | This object counts the total number of packets (including bad packets) received that were 256..511 octets long, excluding framing bits but including FCS. |
| zxAnGponRmH15 MRx512To1023Oct ets | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 19 | This object counts the total number of packets (including bad packets) received that were 512..1023 octets long, excluding framing bits but including FCS. |
| zxAnGponRmH15 MRx1024To1518O ctets | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 20 | This object counts The total number of packets (including bad packets) received that were 1024..1518 octets long, excluding framing bits but including FCS. |
| zxAnGponRmH15 MRxPauseFrms | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 21 | This object counts the total number of PAUSE frames received. |
| zxAnGponRmH15 MRxErrorOctets | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 22 | This object counts the total number of error packets bytes received. |
| zxAnGponRmH15 MRxRate | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 23 | Received bits per second. |
| zxAnGponRmH15 MRxMcastRate | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 24 | Received multicast bits per second. |
| zxAnGponRmH15 | . | This object counts the total number of |

| MIB Variable | OID | Description |
| --- | --- | --- |
| MTxDropEvents | 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 25 | events in which packets transmitted were dropped due to lack of resources. This is not necessarily the number of packets dropped;it is the number of times this event was detected. |
| zxAnGponRmH15 MTxOctets | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 26 | This object counts the total number of octets transmitted from the CPE,including those in bad packets, excluding framing bytes, but including FCS. |
| zxAnGponRmH15 MTxPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 27 | This object counts the total number of packets transmitted, including bad packets,broadcast packets and multicast packets. |
| zxAnGponRmH15 MTxUcastPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 28 | This object counts the total number of unicast frames transmitted. |
| zxAnGponRmH15 MTxBcastPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 29 | This object counts the total number of transmitted good packets directed to the broadcast address. This does not include multicast packets. |
| zxAnGponRmH15 MTxMcastPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 30 | This object counts the total number of transmitted good packets directed to a multicast address. This does not include broadcast packets. |
| zxAnGponRmH15 MTxCrcAlignErrors | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 31 | The total number of upstream packets transmitted that had a length (excluding framing bits, but including FCS octets) of between 64 and 1518 octets,inclusive, but had either a bad frame check sequence (FCS) with an integral number of octets (FCS error) or a bad FCS with a non-integral number of octets |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  |  | (alignment error). |
| zxAnGponRmH15 MTxUndersizePkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 32 | This object counts the total number of packets transmitted that were less than 64 octets long but were otherwise well formed (excluding framing bits, but including FCS). |
| zxAnGponRmH15 MTxOversizePkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 33 | The total number of upstream packets transmitted that were longer than 1518 octets (excluding framing bits, but including FCS) and were otherwise well formed. Note- If 2000-byte ethernet frames are supported,counts in this performance parameter are not necessarily errors. |
| zxAnGponRmH15 MTxDiscardPkts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 34 | This object counts the total number of discarded packets transmitted. |
| zxAnGponRmH15 MTxPkts64Octets | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 35 | This object counts the total number of transmitted packets (including bad packets) that were 64 octets long, excluding framing bits but including FCS. |
| zxAnGponRmH15 MTx65To127Octet s | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 36 | This object counts the total number of transmitted packets (including bad packets) that were 65..127 octets long, excluding framing bits but including FCS. |
| zxAnGponRmH15 MTx128To255Octe ts | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 37 | This objecet counts the total number of packets (including bad packets) transmitted that were 128..255 octets long, excluding framing bits but including FCS. |
| zxAnGponRmH15 MTx256To511Octet s | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 38 | This object counts the total number of packets (including bad packets) transmitted that were 256..511 octets long, excluding |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  |  | framing bits but including FCS. |
| zxAnGponRmH15 MTx512To1023Oct ets | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 39 | This object counts the total number of packets (including bad packets) transmitted that were 512..1023 octets long, excluding framing bits but including FCS. |
| zxAnGponRmH15 MTx1024To1518O ctets | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 40 | This object counts The total number of packets (including bad packets) transmitted that were 1024..1518 octets long, excluding framing bits but including FCS. |
| zxAnGponRmH15 MTxPauseFrms | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 41 | This object counts the total number of PAUSE frames transmitted. |
| zxAnGponRmH15 MTxErrorOctets | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 42 | This object counts the total number of error packets bytes transmitted. |
| zxAnGponRmH15 MTxRate | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 43 | Transmitted bits per second. |
| zxAnGponRmH15 MTxMcastRate | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 44 | Transmitted multicast bits per second. |
| zxAnGponRmH15 MPppoeFilterFrms | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 45 | This object counts the number of frames discarded due to PPPoE filtering. |
| zxAnGponRmH15 MFragments | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 46 | This object counts the total number of packets received that were less than 64 octets long, excluding framing bits but including FCS octets, and had either a bad frame check sequence (FCS) with an |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  |  | integral number of octets (FCS error) or a bad FCS with a non-integral number of octets (alignment error). It is entirely normal for this attribute to increment. This is because it counts both runts (which are normal occurrences due to collisions) and noise hits. |
| zxAnGponRmH15 MJabbers | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 47 | This object counts the total number of packets received that were longer than 1518 octets,excluding framing bits but including FCS octets, and had either a bad frame check sequence (FCS) with an integral number of octets (FCS error) or a bad FCS with a non-integral number of octets (alignment error). The range to detect jabber is between 20 ms and 150 ms. |
| zxAnGponRmH15 MCollisions | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 48 | This object counts the total number of collisions transmitted (half-duplex Mode). |
| zxAnGponRmH15 MAlignErrors | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 49 | This object counts received frames that were not an integral number of octets in length and did not pass the FCS check. |
| zxAnGponRmH15 MFcsErrors | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 50 | This object counts frames received on a particular interface that were an integral number of octets in length but failed the frame check sequence (FCS) check. |
| zxAnGponRmH15 MSingleCollisionFr m | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 51 | This object counts successfully transmitted frames whose transmission was delayed by exactly one collision." |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponRmH15 MMultiCollisionFrm s | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 52 | "This object counts successfully transmitted frames whose transmission was delayed by more than one collision." |
| zxAnGponRmH15 MSqeTestErrors | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 53 | This object counts the number of times that the SQE test error message was generated by the PLS sublayer. |
| zxAnGponRmH15 MDeferredTxFrms | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 54 | This object counts frames whose first transmission attempt was delayed because the medium was busy. The count does not include frames involved in collisions. |
| zxAnGponRmH15 MLateCollisions | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 55 | This object counts the number of times that a collision was detected later than 512 bit times into the transmission of a packet. |
| zxAnGponRmH15 MExcessCollisions | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 56 | This objecet counts frames whose transmission failed due to excessive collisions. |
| zxAnGponRmH15 MInternalMacTxErr s | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 57 | This object counts frames whose transmission failed due to an internal MAC sublayer transmit error. |
| zxAnGponRmH15 MInternalMacRxErr s | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 58 | This object counts frames whose reception failed due to an internal MAC sublayer receive error. |
| zxAnGponRmH15 MCarrierSenseErro rs | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 59 | This object counts the number of times that carrier sense was lost or never asserted when attempting to transmit a frame. |
| zxAnGponRmH15 MFrameTooLongs | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 60 | This object counts received frames that exceeded the maximum permitted frame size. |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponRmH15 MRxBufOverflows | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 61 | This object counts the number of times that the receive buffer overflowed. |
| zxAnGponRmH15 MTxBufOverflows | . 1.3.6.1.4.1.3902.1 082.500.23.2.37.1. 62 | This object counts the number of times that the transmit buffer overflowed. |

## Configuration Process Examples

#### GPON HIS Service

##### Configuration Data

| Item | Data |
| --- | --- |
| T-CONT bandwidth profile | Name:T1-20M Type1 fixed 20M |
| ONU | Slot 3 olt 1 onu 1 Type ZTE-F622 register mode : sn ZTEG00000001 |
| T-CONT | Index 1 Link profile T1-20M |
| GEM-Port | Index 1 Link T-CONT 1 |
| Service mapping | Name: internet Link GEM-Port 1 ServiceMapType:VLAN ServiceMapVlan:100 |
| VLAN | vlanPortMode:hybrid untag add pvid at ethUni, then vlan 10 translate to vlan 100. |

##### Configuration Procedure

PC

MIB Browser

C300

Create T-CONT Profile

Create ONU

Create T-CONT

Create GEM-Port

Create service

Create vlan port

In CLI case:

ZXAN(config-gpon)#profile tcont T1-20M Type 1 fixed 20000 ZXAN(config-if)#onu 1 Type ZTE-F622 sn ZTEG00000001 ZXAN(config-if)#tcont 1 profile T1-20M

ZXAN(config-if)#gemport 1 tcont 1

ZXAN(gpon-onu-mng)#service internet gemport 1 vlan 100 ZXAN(gpon-onu-mng)#vlan port eth_0/1 mode hybrid def-vlan 10 ZXAN(gpon-onu-mng)#vlan port eth_0/1 vlan 10

ZXAN(gpon-onu-mng)#vlan port eth_0/1 translate vlan 10 svlan 100

| STEP | MIB Variable | Parameter |
| --- | --- | --- |
| Create T- CONT bandwidth profile | zxAnGponSrvBwPrfNam e | T1-20M profile name |
|  | zxAnGponSrvBwPrfFixe d | 20000 rate 20Mbps |

| STEP | MIB Variable | Parameter |
| --- | --- | --- |
|  | zxAnGponSrvBwPrfAss ured | 0 |
|  | zxAnGponSrvBwPrfMaxi mum | 0 |
|  | zxAnGponSrvBwPrfTyp e | 1 fixed bandwidth |
|  | zxAnGponSrvBwPrfRow Status | 4 Creation |
| Create ONU | zxAnGponOnuMgmtTyp eName | ZTE-F622 ONU Type |
|  | zxAnGponOnuMgmtReg Mode | 1 registration mode:SN |
|  | zxAnGponOnuMgmtSn | Hex format number for ONU SN,e.g.ZTEG00000001 means '5a:54:45:47:00:00:00:01' |
|  | zxAnGponOnuMgmtRo wStatus | 4 creation |
| Create T- CONT | zxAnGponSrvTcontInde x | 1 T-CONT ID |
|  | zxAnGponSrvTcontBwPr fName | T1-20M T-CONT bandwidth profile name |
|  | zxAnGponSrvTcontRow Status | 4 creation |
| Create GEM- Port | zxAnGponSrvGemPortI ndex | 1 GEM-Port ID |
|  | zxAnGponSrvGemPortT contIndex | 1 associated T-CONT ID |
|  | zxAnGponSrvGemPortR owStatus | 4 creation |
| Create service mapping | zxAnGponRmServiceNa me | internet |
|  | zxAnGponRmServiceGe mPort | 1 associated GEM- Port ID |
|  | zxAnGponRmServiceMa pType | 2 vlan |
|  | zxAnGponRmServiceMa pVid | 100 |
|  | zxAnGponRmServiceRo wStatus | 4 creation |

| STEP | MIB Variable | Parameter |
| --- | --- | --- |
| Create VLAN | zxAnGponRmVlanPortM ode | 3 hybrid |
|  | zxAnGponRmVlanPortD efaultVid | 10 |
|  | zxAnGponRmVlanPortC md | 1 addVlan |
|  | zxAnGponRmVlanPortVi d | 10 |
|  | zxAnGponRmVlanTrans UserVid | 10 |
|  | zxAnGponRmVlanTrans UserCos | 8 any Cos |
|  | zxAnGponRmVlanTrans SVid | 100 |
|  | zxAnGponRmVlanTrans StagCos | 8 copy from user Cos |
|  | zxAnGponRmVlanTrans RowStatus | 4 creation |

#### GPON VOIP

##### SIP

        - Configuration Data

| Item | Data |
| --- | --- |
| T-CONT bandwidth profile | Name:T1-20M Type1 fixed 20M |
| ONU | Slot 3 olt 1 onu 1 Type ZTE-F622 register mode : sn ZTEG00000001 |
| T-CONT | Index 1 Link profile T1-20M |
| GEM-Port | Index 1 Link T-CONT 1 |
| Service mapping | Name: voip Link GEM-Port 1 |

| Item | Data |
| --- | --- |
|  | ServiceMapType:VLAN ServiceMapVlan:200 |
| VoIP（SIP） | SignalProtocolUsed:SIP VLAN 200 ONT IP: 10.40.110.101, Mask: 255.255.255.0, Gateway：10.40.110.254 POTS1: AOR：66660050 Username：66660050 Password：123 POTS2: AOR：66660051 Username：66660051 Password：123 |

        - Configuration Procedure

PC

MIB Browser

C300

Create T-CONT Profile

Create ONU

Create T-CONT

Create GEM-Port

Create Service

Create IP Profile

Create Vlan Profile

Configure VoIP Protocol

Create VoIP IP

Create SIP Profile

Create SIP Service

In CLI case：

PON configuration is the same as 10.1.2.

ZXAN(gpon-onu-mng)#service voip gemport 1 vlan 200 ZXAN(config-gpon)#onu profile ip ipProfile gateway 10.40.110.254

ZXAN(config-gpon)#onu profile vlan vlanProfile tag-mode tag cvlan 200 ZXAN(gpon-onu-mng)#voip protocol sip

ZXAN(gpon-onu-mng)#voip-ip mode static ip-profile ipProfile ip-address 10.40.110.101 mask 255.255.255.0 vlan-profile vlanProfile

ZXAN(config-gpon)#onu profile sip sipProfile proxy 10.40.123.25

ZXAN(gpon-onu-mng)#sip-service pots_0/1 profile sipProfile userid 66660050 username 66660050 password 123

| Steps |  | MIB Variable | Parameter |  |
| --- | --- | --- | --- | --- |
| Create service |  | zxAnGponRmServiceName | voip |  |
|  |  | zxAnGponRmServiceGemPor t | 1 Port ID | associated GEM- |
|  |  | zxAnGponRmServiceMapTyp e | 2 | vlan |
|  |  | zxAnGponRmServiceMapVid | 200 |  |
|  |  | zxAnGponRmServiceRowStat us | 4 | creation |
| Create profile | IP | zxAnGponRmIpPrfName | ipProfile |  |
|  |  | zxAnGponRmIpGateway | ’0a:28:6e:fe’ 10.40.110.254 |  |
|  |  | zxAnGponRmIpPrfRowStatus | 4 | creation |
| Create VLAN profile |  | zxAnGponRmVlanPrfName | vlanProfile |  |
|  |  | zxAnGponRmVlanPrfTagMod e | 1 | tag |
|  |  | zxAnGponRmVlanPrfCVid | 200 |  |
|  |  | zxAnGponRmVlanPrfRowStat us | 4 | creation |
| Configure VoIP protocol |  | zxAnGponRmVoipSignalProto Used | 2 | sip |
| Create VoIP IP |  | zxAnGponRmVoipIpMode | 1 | static |
|  |  | zxAnGponRmVoipIpPrf | ipProfile |  |
|  |  | zxAnGponRmVoipVlanPrf | vlanProfile |  |
|  |  | zxAnGponRmVoipIpAddr | ’0a:28:6e:65’ 10.40.110.101 |  |
|  |  | zxAnGponRmVoipIpAddrPfxL en | 24 255.255.255.0 |  |
|  |  | zxAnGponRmVoipIpConfRow Status | 4 | creation |

ZXAN(gpon-onu-mng)#sip-service pots_0/2 profile sipProfile userid 66660051 username 66660051 password 123

| Steps | MIB Variable | Parameter |
| --- | --- | --- |
| Create SIP profile | zxAnGponRmSipPrfName | sipProfile |
|  | zxAnGponRmSipProxyAddr | ’0a:28:7b:19’ 10.40.123.25 |
|  | zxAnGponRmSipPrfRowStatu s | 4 creation |
| Create SIP service | zxAnPonRmOnuUniIndex | 1 (index) |
|  | zxAnGponRmSipPrf | sipProfile |
|  | zxAnGponRmSipUserId | 66660050 |
|  | zxAnGponRmSipUsername | 66660050 |
|  | zxAnGponRmSipPassword | 123 |
|  | zxAnGponRmSipRowStatus | 4 creation |
|  | zxAnPonRmOnuUniIndex | 2 (index) |
|  | zxAnGponRmSipPrf | sipProfile |
|  | zxAnGponRmSipUserId | 66660051 |
|  | zxAnGponRmSipUsername | 66660051 |
|  | zxAnGponRmSipPassword | 123 |
|  | zxAnGponRmSipRowStatus | 4 creation |

##### 5.4.2.2	H.248

        - Configuration Data

| Item | Data |
| --- | --- |
| T-CONT bandwidth profile | Name:T1-20M Type1 fixed 20M |
| ONU | Slot 3 olt 1 onu 1 Type ZTE-F622 register mode : sn ZTEG00000001 |
| T-CONT | Index 1 Link profile T1-20M |
| GEM-Port | Index 1 Link T-CONT 1 |
| Service | Name: voip Link GEM-Port 1 ServiceMapType:VLAN |

| Item | Data |
| --- | --- |
|  | ServiceMapVlan:200 |
| VoIP（H.248） | SignalProtocolUsed: H.248 VLAN: 200 ONT IP: 10.40.110.101, Mask: 255.255.255.0, Gateway：10.40.110.254 Server: 10.63.198.200 user-tid: AG589, postfix-len:5, postfix-start:1 rtp-tid：RTP, postfix-len:3, postfix-start:0 |

        - Configuration Procedure

PC

MIB Browser

C300

Create T-CONT Profile

Create ONU

Create T-CONT

Create GEM-Port

Create Service

Create IP Profile

Create Vlan Profile

Configure VoIP Protocol

Create VoIP IP

Create MGC Profile

Create MGC Service

In CLI case：

PON configuration is same as 10.1.2.

ZXAN(gpon-onu-mng)#service voip gemport 1 vlan 200 ZXAN(config-gpon)#onu profile ip ipProfile gateway 10.40.110.254

ZXAN(config-gpon)#onu profile vlan vlanProfile tag-mode tag cvlan 200 ZXAN(gpon-onu-mng)#voip protocol h248 domain domain1

ZXAN(gpon-onu-mng)#voip-ip mode static ip-profile ipProfile ip 10.40.110.101 mask 255.255.255.0 vlan-profile vlanProfile

ZXAN(config-gpon)#onu profile mgc mgcProfile server1 10.63.198.200

ZXAN(config-gpon)#onu profile mgc mgcProfile user-tid prefix AG589 postfix-len 5 postfix-start 1

ZXAN(config-gpon)#onu profile mgc mgcProfile rtp-tid prefix RTP postfix-len 3 postfix- start 0

ZXAN(gpon-onu-mng)#mgc-service pots_0/1 profile mgcProfile

| Steps |  | MIB Variable | Parameter |  |
| --- | --- | --- | --- | --- |
| Create service |  | zxAnGponRmServiceName | voip |  |
|  |  | zxAnGponRmServiceGemPor t | 1 Port ID | associated GEM- |
|  |  | zxAnGponRmServiceMapTyp e | 2 | vlan |
|  |  | zxAnGponRmServiceMapVid | 200 |  |
|  |  | zxAnGponRmServiceRowStat us | 4 | creation |
| Create profile | IP | zxAnGponRmIpPrfName | ipProfile |  |
|  |  | zxAnGponRmIpGateway | ’0a:28:6e:fe’ 10.40.110.254 |  |
|  |  | zxAnGponRmIpPrfRowStatus | 4 | creation |
| Create VLAN profile |  | zxAnGponRmVlanPrfName | vlanProfile |  |
|  |  | zxAnGponRmVlanPrfTagMod e | 1 | tag |
|  |  | zxAnGponRmVlanPrfCVid | 200 |  |
|  |  | zxAnGponRmVlanPrfRowStat us | 4 | creation |
| Configure VoIP protocol |  | zxAnGponRmVoipSignalProto Used | 3 | h248 |
|  |  | zxAnGponRmVoipDomainNa | domain1 |  |

| Steps | MIB Variable | Parameter |  |  |
| --- | --- | --- | --- | --- |
|  | me |  |  |  |
| Create VoIP IP | zxAnGponRmVoipIpMode | 1 |  | static |
|  | zxAnGponRmVoipIpPrf | ipProfile |  |  |
|  | zxAnGponRmVoipVlanPrf | vlanProfile |  |  |
|  | zxAnGponRmVoipIpAddr | ’0a:28:6e:65’ 10.40.110.101 |  |  |
|  | zxAnGponRmVoipIpAddrPfxL en | 24 255.255.255.0 |  |  |
|  | zxAnGponRmVoipIpConfRow Status | 4 |  | creation |
| Create MGC profile | zxAnGponRmMgcPrfName | mgcProfile |  |  |
|  | zxAnGponRmMgcPrimarySvr | ’0a:28:7b:19’ 10.40.123.25 |  |  |
|  | zxAnGponRmMgcUserTidAss ignPolicy | 2 |  | specified |
|  | zxAnGponRmMgcUserTidPref ix | AG589 |  |  |
|  | zxAnGponRmMgcUserTidDigi tLen | 5 |  |  |
|  | zxAnGponRmMgcUserTidDigi tStartNo | 1 |  |  |
|  | zxAnGponRmMgcRtpTidAssi gnPolicy | 2 |  | specified |
|  | zxAnGponRmMgcRtpTidPrefi x | RTP |  |  |
|  | zxAnGponRmMgcRtpTidDigit Len | 3 |  |  |
|  | zxAnGponRmMgcRtpTidDigit StartNo | 0 |  |  |
|  | zxAnGponRmMgcPrfRowStat us | 4 |  | creation |
| Create MGC service | zxAnPonRmOnuUniIndex | 1 | (index) |  |

# DSL Services

## ADSL Configuration

#### Create Line Profile

[Function]:

Create ADSL Line Profile [MIB description]:

ADSL-LINE-MIB.mib, ADSL-LINE-EXT-MIB.mib, ADSL-DMT-LINE-MIB.mib ZTE-AN- ADSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |  |
| --- | --- | --- | --- |
| adslLineConfProfileName | SnmpAdminStrin | Length is | 32, including |
| （index） | g | suffix(.prf), | and valid |
| 1.3.6.1.2.1.10.94.1.1.14.1.1 |  | characters | are numbers |
|  |  | and letters. |  |
| adslLineConfProfileRowStatus 1.3.6.1.2.1.10.94.1.1.14.1.30 | RowStatus | Set rowstatus value of 4. |  |
| adslAtucConfRateMode 1.3.6.1.2.1.10.94.1.1.14.1.2 | INTEGER | [fix(1),adaptAtStart(2), adaptAtRun(3)](Def:2) Defines what form of transmit rate adaptation is configured on this modem. |  |
| adslAtucConfRateChanRatio | INTEGER | [0-100](Def:0), take the |  |
| 1.3.6.1.2.1.10.94.1.1.14.1.3 |  | default value Configured |  |
|  |  | allocation ratio of excess |  |
|  |  | transmit bandwidth |  |
|  |  | between fast and |  |
|  |  | interleaved channels. Only |  |
|  |  | applies when two channel |  |

Create Adsl line profile, the big network to the network element set only adslLineConfProfileRowStatus. Set rowstatus value of 4, other mibs of the profiles will be assigned as default values.

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | mode and RADSL are supported. |
| adslAtucConfTargetSnrMgn | INTEGER | [0-310](Def:60) |
| 1.3.6.1.2.1.10.94.1.1.14.1.4 |  | take the default |
|  |  | value，Unit 0.1db |
|  |  | Configured atuc Target |
|  |  | Signal /Noise Margin. |
| adslAtucConfMaxSnrMgn 1.3.6.1.2.1.10.94.1.1.14.1.5 | INTEGER | [0-310](Def:310), take the default value Configured atuc Maximum acceptable Signal/Noise Margin. |
| adslAtucConfMinSnrMgn 1.3.6.1.2.1.10.94.1.1.14.1.6 | INTEGER | [0-310](Def:0) , take the default value atuc Minimum acceptable Signal/Noise Margin. |
| adslAtucConfDownshiftSnrMgn 1.3.6.1.2.1.10.94.1.1.14.1.7 | INTEGER | [0-310](Def:0) , take the default value atuc Signal/Noise Margin for rate downshift. |
| adslAtucConfUpshiftSnrMgn 1.3.6.1.2.1.10.94.1.1.14.1.8 | INTEGER | [0-310](Def:0) , take the default value atuc Signal/Noise Margin for rate upshift. |
| adslAtucConfMinUpshiftTime 1.3.6.1.2.1.10.94.1.1.14.1.9 | INTEGER | [0-16383](Def:0), take the default value atuc Minimum time that the current margin is above UpshiftSnrMgn before an upshift occurs. |
| adslAtucConfMinDownshiftTime 1.3.6.1.2.1.10.94.1.1.14.1.10 | INTEGER | [0-16383](Def:0) , take the default value atuc Minimum time that the current margin is below DownshiftSnrMgn before a downshift occurs. |
| adslAtucChanConfFastMinTxRa te 1.3.6.1.2.1.10.94.1.1.14.1.11 | Unsigned32 | [0-102400](Def:0) take the default value，Unit kbps |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | atuc Minimum Transmit rate for `Fast' channels. |
| adslAtucChanConfInterleaveMin | Unsigned32 | [0-102400](Def:0) |
| TxRate |  | take the default |
| 1.3.6.1.2.1.10.94.1.1.14.1.12 |  | value，Unit kbps |
|  |  | atuc Minimum Transmit |
|  |  | rate for `Interleave' |
|  |  | channels. |
| adslAtucChanConfFastMaxTxR | Unsigned32 | [0-102400](Def:1024) |
| ate |  | take the default |
| 1.3.6.1.2.1.10.94.1.1.14.1.13 |  | value，Unit kbps |
|  |  | atuc Maximum Transmit |
|  |  | rate for `Fast' channels. |
| adslAtucChanConfInterleaveMa | Unsigned32 | [0-102400](Def:1024) |
| xTxRate |  | take the default |
| 1.3.6.1.2.1.10.94.1.1.14.1.14 |  | value，Unit kbps |
|  |  | Atuc Maximum Transmit |
|  |  | rate for `Interleave' |
|  |  | channels. |
| adslAtucChanConfMaxInterleav eDelay 1.3.6.1.2.1.10.94.1.1.14.1.15 | INTEGER | [0-255](Def:8), take the default value atuc maximum Interleave Delay for this channel. |
| adslAturConfRateMode 1.3.6.1.2.1.10.94.1.1.14.1.16 | INTEGER | [fixed(1),adaptAtStartup(2) ,adaptAtRuntime(3)](Def:2) take the default value transmit rate adaptation is configured on this modem. |
| adslAturConfRateChanRatio | INTEGER | [0-100](Def:0) |
| 1.3.6.1.2.1.10.94.1.1.14.1.17 |  | allocation ratio of excess |
|  |  | transmit bandwidth |
|  |  | between fast and |
|  |  | interleaved channels. |
| adslAturConfTargetSnrMgn | INTEGER | [0-310](Def:60) |
| 1.3.6.1.2.1.10.94.1.1.14.1.18 |  | take the default |
|  |  | value，Unit 0.1db |
|  |  | atur Target Signal/Noise |
|  |  | Margin. |
| adslAturConfMaxSnrMgn | INTEGER | [0-310](Def:310) |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.10.94.1.1.14.1.19 |  | atur Maximum acceptable Signal/Noise Margin. |
| adslAturConfMinSnrMgn 1.3.6.1.2.1.10.94.1.1.14.1.20 | INTEGER | [0-310](Def:0) atur Minimum acceptable Signal/Noise Margin. |
| adslAturConfDownshiftSnrMgn 1.3.6.1.2.1.10.94.1.1.14.1.21 | INTEGER | [0-310](Def:0) atur Signal/Noise Margin for rate downshift. |
| adslAturConfUpshiftSnrMgn 1.3.6.1.2.1.10.94.1.1.14.1.22 | INTEGER | [0-310](Def:0) atur Signal/Noise Margin for rate upshift. |
| adslAturConfMinUpshiftTime 1.3.6.1.2.1.10.94.1.1.14.1.23 | INTEGER | [0-16383](Def:0) atur Minimum time that the current margin is above UpshiftSnrMgn before an upshift occurs. |
| adslAturConfMinDownshiftTime 1.3.6.1.2.1.10.94.1.1.14.1.24 | INTEGER | [0-16383](Def:0) atur Minimum Transmit rate for `Fast' channels. |
| adslAturChanConfFastMinTxRa te 1.3.6.1.2.1.10.94.1.1.14.1.25 | Unsigned32 | [0-10240](Def:0) Configured Minimum Transmit rate for `Fast' channels,in kbps. |
| adslAturChanConfInterleaveMin TxRate 1.3.6.1.2.1.10.94.1.1.14.1.26 | Unsigned32 | [0-10240](Def:0) Configured Minimum Transmit rate for `Interleave' channels, in kbps. |
| adslAturChanConfFastMaxTxR ate 1.3.6.1.2.1.10.94.1.1.14.1.27 | Unsigned32 | [0-10240](Def:512) Configured Maximum Transmit rate for `Fast' channels, in kbps. |
| adslAturChanConfInterleaveMa xTxRate 1.3.6.1.2.1.10.94.1.1.14.1.28 | Unsigned32 | [0-10240](Def:512) Configured Maximum Transmit rate for `Interleave' channels, in kbps. |
| adslAturChanConfMaxInterleav eDelay | INTEGER | [0-255](Def:8) Configured maximum |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.10.94.1.1.14.1.29 |  | Interleave Delay for this channel. |
| adslAtucDMTConfFreqBins 1.3.6.1.2.1.10.94.1.1.16.1.1.14. 1.1 | OCTET STRING | A string of 32 bits to enable/disable the incoming DMT frequency bins that the ATU-C receiver monitors. bit set to 1 disables reception of the specified sub-carrier frequency bin and a 0 enables reception. |
| adslAturDMTConfFreqBins 1.3.6.1.2.1.10.94.1.1.16.1.1.14. 1.2 | OCTET STRING | Full value of 0, 64 bytes. |
| adslLineDMTConfEOC 1.3.6.1.2.1.10.94.1.1.16.1.1.14. 1.3 | INTEGER | [byte(1),streaming(2)] (Def:1) Defines the embedded operations channel mode for the DMT ADSL line. |
| adslConfProfileLineType 1.3.6.1.2.1.10.94.3.1.22.1.1 | INTEGER | [noChannel(1),fastOnly(2),i nterleavedOnly(3),fastOrIn te rleaved(4),fastAndInterleav ed(5)](Def:2) This object is used to configure the ADSL physical line mode. |
| zxAnAdslLineConfTxStartBin 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.1 | INTEGER | [0-64]Specifies lowest bin number allowed for transmitted signal. Different line coding type has different default value. 0x20: default for Annex A only. 0x20:default for ADSL2/ADS L2 plus. 0x06: default for Annex A, G.Span and/or G.Span+ are also defined. |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | 0x40: default for Annex B. 0x06: default for Annex C/C.x. 0x06: default for G.Span. 0x06: default for G.Span+. Def:32 |
| zxAnAdslLineConfTxEndBin 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.2 | INTEGER | [0-512]Specifies highest bin number allowed for transmitted signal. 0x3f: default for all Annexes and ADSL2/ADSL2+. Def:63 |
| zxAnAdslLineConfRxStartBin 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.3 | INTEGER | [0-64]Specifies lowest bin number allowed for received signal. Different line coding type has different default value. 0x06: default for Annex A. 0x01: default for Annex B. 0x06: default for Annex C/C.x. 0x06: efault for G.SpanADSL+. 0x06: default for G.Span+. 0x06: default for ADSL2/ADLS2+. Def:6 |
| zxAnAdslLineConfRxEndBin 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.4 | INTEGER | [0-512]Specifies highest bin number allowed for received signal. 0x3f: default for all Annexes and ADSL2/ADLS2+. Def:63 |
| zxAnAdslLineConfUseCustomBi ns1.3.6.1.4.1.3902.1082.130.1. 2.2.2.1.5 | INTEGER | Enable(1)/disable(2) custom bin selection for upstream/downstream. This parameter is used in |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | conjunction with the adslAtucDMTConfFreqBin s and dslAturDMTConfFreqBins Def: disabled(2) |
| zxAnAdslLineConfPMMode 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.6 | BITS | This configuration parameter defines the line states the ATU-C may autonomously transition to on this line Bit 0 L3 state (Idle state) Bit 1 L1/L2 state (Low power state). |
| zxAnAdslLineConfPML2Rate 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.7 | INTEGER | [256-1024]PM configuration parameter, related to the L2 low power state. This parameter specifies the minimum net data rate during the low power state (L2). Def:512 |
| zxAnAdslLineConfMinProtection Ds1.3.6.1.4.1.3902.1082.130.1. 2.2.2.1.8 | INTEGER | Minimum impulse noise protection on Downstream direction. The minimum impulse noise protection for the bearer channel, expressed in symbols. autoAdaption(1), noProtection(2), halfSymbol(3), singleSymbol(4), twoSymbols(5), fourSymbols(6), eightSymbols(7), sixteenSymbols(8) Def: halfSymbol(3) |
| zxAnAdslLineConfMinProtection | INTEGER | Minimum Impulse Noise |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| Us1.3.6.1.4.1.3902.1082.130.1. 2.2.2.1.9 |  | Protection on Upstream direction. The minimum impulse noise protection for the bearer channel,expressed in symbols. autoAdaption(1), noProtection(2), halfSymbol(3), singleSymbol(4), twoSymbols(5), fourSymbols(6), eightSymbols(7), sixteenSymbols(8) Def: halfSymbol |
| zxAnAdslLineConfDMTConfTrel lis 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.10 | INTEGER | Enable(1)/disable(2) trellis coding.Trellis coding should always be enabled for its clear performance advantage. Trellis coding is mandatory for ADSL/ADSL2plus. The value enable(1) is default for all Annexes and ADSL2/ADSL2+. Def: enabled(1) |
| zxAnAdslLineConfMaxBitsPerBi n 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.11 | INTEGER | [0-15]Specifies maximum received number of bits per bin. Def:15 |
| zxAnAdslLineConfBitSwapDs 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.12 | INTEGER | Enable/disabledownstream bit swapping.Different line coding type has different default value. enabled(1): default for Annex A. disabled(2): default for ADSL2/ADLS2+, Annex B, Annex C/C.x, |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | G.Span/ADSL+ and G.Span+. Def: disabled(2) |
| zxAnAdslLineConfBitSwapUs 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.13 | INTEGER | Enable/disable upstream bit swapping. Different line coding type has different default value. enabled(1): default for Annex A. disabled(2): default for ADSL2/ADLS2+, Annex B, Annex C/C.x, G.Span/ADSL+ and G.Span+. Def: disabled |
| zxAnAdslLineConfReAdsl2Enab le 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.14 | INTEGER | Enable(1)/disable(2) downstream RE-ADSL2 operation should be forced by the ATU-C. Def: disabled(2) |
| zxAnAdslLineConfPmL0Time 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.15 | INTEGER | [0-255]PM configuration parameter, related to the L2 low power state. This parameter represents the minimum time between an exit from the L2 state and the next entry into the L2 state. Def:240 |
| zxAnAdslLineConfPmL2Time 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.16 | INTEGER | [0-255]PM configuration parameter,related to the L2 low power state. This parameter represents the minimum time between an entry into the L2 state and the first Power Trim in the L2 state and between two consecutive Power Trims in the L2 State. |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | Def:120 |
| zxAnAdslLineConfPmL2Atpr 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.17 | INTEGER | [0-31]PM configuration parameter,related to the L2 low power state. This parameter represents the Maximum Aggregate Transmit Power Reduction (in dB) that can be performed through a single Power Trim in the L2 state. It ranges from 0 dB to 31 dB. Def:3 |
| zxAnAdslLineConfPsdMaskSele ctUs 1.3.6.1.4.1.3902.1082.130.1.2.2 .2.1.18 | INTEGER | The selected upstream PSD mask. This parameter is used only for annexes J and M of G.992.3 and G.992.5, and the same selection is used for all relevant enabled bits in adslLineTransAtucConfig of ADSL-LINE-EXT-MIB. adlu32Eu32 (1) - ADLU-32 / EU-32 adlu36Eu36 (2) - ADLU-36 / EU-36 adlu40Eu40 (3) - ADLU-40 / EU-40 adlu44Eu44 (4) - ADLU-44 / EU-44 adlu48Eu48 (5) - ADLU-48 / EU-48 adlu52Eu52 (6) - ADLU-52 / EU-52 adlu56Eu56 (7) - ADLU-56 / EU-56 adlu60Eu60 (8) - ADLU-60 / EU-60 adlu64Eu64 (9) - ADLU-64 |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | / EU-61 Def: adlu32Eu32(1) |

### Create Alarm Profile

[Function]:

Create ADSL Alarm Profile [MIB description]:

ADSL-LINE-MIB.mib, ADSL-LINE-EXT-MIB.mib, ZTE-AN-ADSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| adslLineAlarmConfProfileName （index） 1.3.6.1.2.1.10.94.1.1.15.1.1 | SnmpAdminSt ring | Length is 32, including suffix(.prf), and valid characters are numbers and letters. |
| adslLineAlarmConfProfileRowSt atus 1.3.6.1.2.1.10.94.1.1.15.1.20 | RowStatus | Set rowstatus value of 4. |
| adslAtucThresh15MinLofs 1.3.6.1.2.1.10.94.1.1.15.1.2 | INTEGER | [0-900]The number of Loss of Frame Seconds encountered by an ADSL interface within any given 15 minutes performance data collection period, which causes the SNMP agent to send an adslAtucPerfLofsThreshTrap. One trap will be sent per interval per interface. A value of `0' will disable the trap. |
| adslAtucThresh15MinLoss 1.3.6.1.2.1.10.94.1.1.15.1.3 | INTEGER | [0-900]The number of Loss of Signal Seconds encountered |

Create Adsl alarm profile, the big network to the network element set only adslLineAlarmConfProfileRowStatus. Set rowstatus value of 4, other mibs of the profiles will be assigned as default values.

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | by an ADSL interface within any given 15 minutes performance data collection period, which causes the SNMP agent to send an adslAtucPerfLossThreshTrap. One trap will be sent per interval per interface. A value of `0' will disable the trap. |
| adslAtucThresh15MinLols 1.3.6.1.2.1.10.94.1.1.15.1.4 | INTEGER | [0-900]The number of Loss of Link Seconds encountered by an ADSL interface within any given 15 minutes performance data collection period, which causes the SNMP agent to send an adslAtucPerfLolsThreshTrap. One trap will be sent per interval per interface. A value of `0' will disable the trap. |
| adslAtucThresh15MinLprs 1.3.6.1.2.1.10.94.1.1.15.1.5 | INTEGER | [0-900]The number of Loss of Power Seconds encountered by an ADSL interface within any given 15 minutes performance data collection period, which causes the SNMP agent to send an adslAtucPerfLprsThreshTrap. One trap will be sent per interval per interface. A value of `0' will disable the trap. |
| adslAtucThresh15MinESs 1.3.6.1.2.1.10.94.1.1.15.1.6 | INTEGER | [0-900]The number of Errored Seconds encountered by an ADSL interface within any given 15 minutes performance data collection |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | period, which causes the SNMP agent to send an adslAtucPerfESsThreshTrap. One trap will be sent per interval per interface. A value of `0' will disable the trap. |
| adslAtucThreshFastRateUp 1.3.6.1.2.1.10.94.1.1.15.1.7 | Unsigned32 | [0-65535]Applies to `Fast' channels only. Configured change in rate causing an adslAtucRateChangeTrap. A trap is produced when: ChanCurrTxRate >= ChanPrevTxRate plus the value of this object. A value of `0' will disable the trap. |
| adslAtucThreshInterleaveRateU p 1.3.6.1.2.1.10.94.1.1.15.1.8 | Unsigned32 | [0-65535]Applies to `Interleave' channels only. Configured change in rate causing an adslAtucRateChangeTrap. A trap is produced when: ChanCurrTxRate >= ChanPrevTxRate plus the value of this object. A value of `0' will disable the trap. |
| adslAtucThreshFastRateDown 1.3.6.1.2.1.10.94.1.1.15.1.9 | Unsigned32 | [0-65535]Applies to `Fast' channels only. Configured change in rate causing an adslAtucRateChangeTrap. A trap is produced when: ChanCurrTxRate <= ChanPrevTxRate minus the value of this object. A value of `0' will disable the trap. |
| adslAtucThreshInterleaveRateD own 1.3.6.1.2.1.10.94.1.1.15.1.10 | Unsigned32 | [0-65535]Applies to `Interleave' channels only. Configured change in rate causing an |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | adslAtucRateChangeTrap. A trap is produced when: ChanCurrTxRate <= ChanPrevTxRate minus the value of this object. A value of `0' will disable the trap. |
| adslAtucInitFailureTrapEnable 1.3.6.1.2.1.10.94.1.1.15.1.11 | INTEGER | Enables(1) and disables(2) the InitFailureTrap.This object is defaulted disable(2). Def: disable(2) |
| adslAturThresh15MinLofs 1.3.6.1.2.1.10.94.1.1.15.1.12 | INTEGER | [0-900]The number of Loss of Frame Seconds encountered by an ADSL interface within any given 15 minutes performance data collection period, which causes the SNMP agent to send an adslAturPerfLofsThreshTrap. One trap will be sent per interval per interface. A value of `0' will disable the trap. |
| adslAturThresh15MinLoss 1.3.6.1.2.1.10.94.1.1.15.1.13 | INTEGER | [0-900]The number of Loss of Signal Seconds encountered by an ADSL interface within any given 15 minutes performance data collection period, which causes the SNMP agent to send an adslAturPerfLossThreshTrap. One trap will be sent per interval per interface. A value of `0' will disable the trap. |
| adslAturThresh15MinLprs 1.3.6.1.2.1.10.94.1.1.15.1.14 | INTEGER | [0-900]The number of Loss of Power Seconds encountered by an ADSL interface within any given 15 minutes performance data collection |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | period, which causes the SNMP agent to send an adslAturPerfLprsThreshTrap. One trap will be sent per interval per interface. A value of `0' will disable the trap. |
| adslAturThresh15MinESs 1.3.6.1.2.1.10.94.1.1.15.1.15 | INTEGER | [0-900]The number of Errored Seconds encountered by an ADSL interface within any given 15 minutes performance data collection period, which causes the SNMP agent to send an adslAturPerfESsThreshTrap. One trap will be sent per interval per interface. A value of `0' will disable the trap. |
| adslAturThreshFastRateUp 1.3.6.1.2.1.10.94.1.1.15.1.16 | Unsigned32 | [0-65535]Applies to `Fast' channels only. Configured change in rate causing an adslAturRateChangeTrap. A trap is produced when: ChanCurrTxRate >= ChanPrevTxRate plus the value of this object. A value of `0' will disable the trap. |
| adslAturThreshInterleaveRateU p 1.3.6.1.2.1.10.94.1.1.15.1.17 | Unsigned32 | [0-65535]Applies to `Interleave' channels only. configured change in rate causing an adslAturRateChangeTrap. A trap is produced when: ChanCurrTxRate >= ChanPrevTxRate plus the value of this object. A value of `0' will disable the trap. |
| adslAturThreshFastRateDown | Unsigned32 | [0-65535]Applies to `Fast' |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| 1.3.6.1.2.1.10.94.1.1.15.1.18 |  | channels only. Configured change in rate causing an adslAturRateChangeTrap. A trap is produced when: ChanCurrTxRate <= ChanPrevTxRate minus the value of this object. A value of `0' will disable the trap. |
| adslAturThreshInterleaveRateD own 1.3.6.1.2.1.10.94.1.1.15.1.19 | Unsigned32 | [0-65535]Applies to `Interleave' channels only. Configured change in rate causing an adslAturRateChangeTrap. A trap is produced when: ChanCurrTxRate <= ChanPrevTxRate minus the value of this object. A value of `0' will disable the trap. |
| zxAnAdslAtucConnRateToleran ce 1.3.6.1.4.1.3902.1082.130.1.2.2 .3.1.1 | INTEGER | [0-100]Tolerance between adslAtucChanConfInterleave MaxTxRate and adslAtucChanCurrTxRate. Value '0' will disable the trap. Def:0 |
| zxAnAdslAturConnRateToleranc e 1.3.6.1.4.1.3902.1082.130.1.2.2 .3.1.2 | INTEGER | [0-100]Tolerance between adslAturChanConfInterleave MaxTxRate and adslAturChanCurrTxRate. Value '0' will disable the trap. Def:0 |
| zxAnAdslAtucThreshConnRate 1.3.6.1.4.1.3902.1082.130.1.2.2 .3.1.3 | INTEGER | [0-100000]ATUC connection rate threshold. If dslAtucChanCurrTxRate is lower than zxAnAdslAtucThreshConnRat e, zxAnAdslAtucLowConnRateA lm will be sent. Value '0' will disable the trap. |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | Def:0 |
| zxAnAdslAturThreshConnRate 1.3.6.1.4.1.3902.1082.130.1.2.2 .3.1.4 | INTEGER | [0-100000]ATUR connection rate threshold. If adslAturChanCurrTxRate is lower than zxAnAdslAturThreshConnRat e, zxAnAdslAturLowConnRateAl m will be sent. Value '0' will disable the trap. Def:0 |
| zxAnAdslMaxAtucConnRateTol erance 1.3.6.1.4.1.3902.1082.130.1.2.2 .3.1.5 | INTEGER | [0-100]Tolerance between adslAtucChanConfInterleave MaxTxRate and adslAtucChanCurrTxRate. Value '0' will disable the trap. Def:0 |
| zxAnAdslMaxAturConnRateTole rance 1.3.6.1.4.1.3902.1082.130.1.2.2 .3.1.6 | INTEGER | [0-100]Tolerance between adslAturChanConfInterleave MaxTxRate and adslAturChanCurrTxRate. Value '0' will disable the trap. Def:0 |
| zxAnAdslMaxThreshAtucConnR ate 1.3.6.1.4.1.3902.1082.130.1.2.2 .3.1.7 | INTEGER | [0-100000]ATUC maximum connection rate threshold. If adslAtucChanCurrTxRate is higher than zxAnAdslMaxThreshAtucCon nRate, zxAnAdslAtucHighConnRate Alm will be sent. Value '0' will disable the trap. Def:0 |
| zxAnAdslMaxThreshAturConnR ate 1.3.6.1.4.1.3902.1082.130.1.2.2 .3.1.8 | INTEGER | [0-100000]ATUR maxmimum connection rate threshold. If adslAturChanCurrTxRate is higher than zxAnAdslMaxThreshAturCon nRate, |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | zxAnAdslAturHighConnRateA lm will be sent. Value '0' will disable the trap. Def:0 |
| zxAnAdslAturInitFailTrapEnable 1.3.6.1.4.1.3902.1082.130.1.2.2 .3.1.9 | BITS | Enable/disable the InitFailureTrap on the ATUR interface. unused1(0) - This bit is no use to enable or disable the the InitFailureTrap on the atur interface. lossOfFraming(1) - Value 0 means disable the loss of frame trap on the atur interface. lossOfSignal(2) - Value 0 means disable the loss of signal trap on the atur interface. lossOfPower(3) - Value 0 means disable the loss of power trap on the atur interface. unused2(4) - This bit is no use to enable or disable the InitFailureTrap on the atur interface. lossOfSignalQuality(5) - Value 0 means disable the loss of signal quality trap on the atur interface. |
| zxAnAdslAtucThreshInpLeftr 1.3.6.1.4.1.3902.1082.130.1.2.2 .3.1.10 | INTEGER | [0-100]G.INP configuration parameters of downstream. The fraction of NDR that shall be used as the threshold for leftr defects. Valid values are from 0 to 100 in increments of 0.01. |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | Def:0 |
| zxAnAdslAturThreshInpLeftr 1.3.6.1.4.1.3902.1082.130.1.2.2 .3.1.11 | INTEGER | [0-100]GINP configuration parameters of upstream. The fraction of NDR that shall be used as the threshold for leftr defects. Valid values are from 0 to 100 in increments of 0.01. Def:0 |
| adslAtucThreshold15MinFailedF astR | INTEGER | [0-900] The first time the value of the corresponding instance of adslAtucPerfCurr15MinFailed FastR reaches or exceeds this value within a given 15- minute performance data collection period,an adslAtucFailedFastRThreshTr ap notification will be generated. The value '0' will disable the notification. Def:0 |
| adslAtucThreshold15MinSesL | INTEGER | [0-900] The first time the value of the corresponding instance of adslAtucPerf15MinSesL reaches or exceeds this value within a given 15-minute performance data collection period, an adslAtucSesLThreshTrap notification will be generated. The value '0' will disable the notification. Def:0 |
| adslAtucThreshold15MinUasL | INTEGER | [0-900] The first time the value of the corresponding instance of adslAtucPerf15MinUasL |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | reaches or exceeds this value within a given 15-minute performance data collection period, an adslAtucUasLThreshTrap notification will be generated. The value '0' will disable the notification. Def:0 |
| adslAturThreshold15MinSesL | INTEGER | [0-900] The first time the value of the corresponding instance of adslAturPerf15MinSesL reaches or exceeds this value within a given 15-minute performance data collection period, an adslAturSesLThreshTrap notification will be generated. The value '0' will disable the notification. Def:0 |
| adslAturThreshold15MinUasL | INTEGER | [0-900] The first time the value of the corresponding instance of adslAturPerf15MinUasL reaches or exceeds this value within a given 15-minute performance data collection period, an adslAturUasLThreshTrap notification will be generated. The value '0' will disable the notification. Def:0 |

### Modify Line Profile

[Function]:

Modify ADSL Line Profile [MIB description]:

ADSL-LINE-MIB.mib, ADSL-LINE-EXT-MIB.mib, ADSL-DMT-LINE-MIB.mib ZTE-AN- ADSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| adslLineConfProfileName (index) 1.3.6.1.2.1.10.94.1.1.14.1.1 | SnmpAdminString | Length is 32, including suffix(.prf), and valid characters are numbers and letters |
| adslLineConfProfileRowStatus 1.3.6.1.2.1.10.94.1.1.14.1.30 | RowStatus | Set the value to 1 (ROWSTATUS_ACTIVE). |
| … | … | See the table of creating ADSL line profile. |

Modify adsl line profile, the network will need to set the adslLineConfProfileRowStatus value 1 (ACTIVE). Then set the mibs of line profile table to the network element. Line profile mib table description see table above the amount.

### Modify Alarm Profile

[Function]:

Modify ADSL Alarm Profile [MIB description]:

ADSL-LINE-MIB.mib, ADSL-LINE-EXT-MIB.mib, ZTE-AN-ADSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| adslLineAlarmConfProfileName | SnmpAdminStrin | Length is 32, including |

Modify adsl alarm profile, the network will need to set the adslLineAlarmConfProfileRowStatus value 1 (ACTIVE). Then set the mibs of alarm profile table to the network element. Alarm profile mib table description see table above the amount.

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| (index) 1.3.6.1.2.1.10.94.1.1.15.1.1 | g | suffix(.prf), and valid characters are numbers and letters |
| adslLineAlarmConfProfileRowStatu s 1.3.6.1.2.1.10.94.1.1.14.1.30 | RowStatus | Set the value to 1 (ROWSTATUS_ACTIVE ). |
| … | … | See the table of creating ADSL alarm profile. |

### Delete Line Profile

[Function]:

Delete ADSL Line Profile [MIB description]:

ADSL-LINE-MIB.mib, ADSL-LINE-EXT-MIB.mib, ADSL-DMT-LINE-MIB.mib ZTE-AN- ADSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| adslLineConfProfileName (index) 1.3.6.1.2.1.10.94.1.1.14.1.1 | SnmpAdminString | Length is 32, including suffix(.prf), and valid characters are numbers and letters |
| adslLineConfProfileRowStatu s 1.3.6.1.2.1.10.94.1.1.14.1.30 | RowStatus | Set the value to 6 (ROWSTATUS_DELETE). |

Delete adsl line profile, the network will need to set the adslLineConfProfileRowStatus value 6 (DELETE).

### Delete Alarm Profile

[Function]:

Delete ADSL Alarm Profile

[MIB description]:

ADSL-LINE-MIB.mib, ADSL-LINE-EXT-MIB.mib, ZTE-AN-ADSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| adslLineAlarmConfProfileName (index) 1.3.6.1.2.1.10.94.1.1.15.1.1 | SnmpAdminStrin g | Length is 32, including suffix(.prf), and valid characters are numbers and letters |
| adslLineAlarmConfProfileRowStat us 1.3.6.1.2.1.10.94.1.1.14.1.30 | RowStatus | Set the value to 6 (ROWSTATUS_DELETE ). |

Delete adsl alarm profile, the network will need to set the adslLineConfProfileRowStatus value 6 (DELETE).

### Bind ADSL Line Profile To Port

[Function]:

When the adsl profile is created and modified, it need to be applied to the port. Port will re-link under the new profile.

[MIB description]:

ADSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| ifIndex 1.3.6.1.2.1.2.2.1.1 | InterfaceIndex | NormalPort |
| adslLineConfProfile 1.3.6.1.2.1.10.94.1.1.1.1.4 | SnmpAdminString | String length is 32 (including the terminator), demand is setting the existed line profile name. |

### Bind ADSL Alarm Profile To Port

[Function]:

When the adsl alarm profile is created and modified, it need to be applied to the port. Port will re-link under the new profile.

[MIB description]:

ADSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| ifIndex 1.3.6.1.2.1.2.2.1.1 | InterfaceIndex | NormalPort |
| adslLineAlarmConfProfile 1.3.6.1.2.1.10.94.1.1.1.1.5 | SnmpAdminString | String length is 32 (including the terminator), demand is setting the existed alarm profile name. |

## SHDSL Configuration

### Create Line Profile

[Function]:

Create SHDSL Line Profile [MIB description]:

HDSL2-SHDSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| hdsl2ShdslSpanConfProfileNam | SnmpAdminSt | Length is 32, including |
| e（index） | ring | suffix(.prf), and valid |
| 1.3.6.1.2.1.10.48.1.10.1.1 |  | characters are numbers and |
|  |  | letters. |
| hdsl2ShdslSpanConfProfileRow Status 1.3.6.1.2.1.10.48.1.10.1.16 | RowStatus | Set rowstatus value of 4. |

Create Shdsl line profile, the big network to the network element set only hdsl2ShdslSpanConfProfileRowStatus. Set rowstatus value of 4, other mibs of the profiles will be assigned as default values.

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| hdsl2ShdslSpanConfWireInterfa ce 1.3.6.1.2.1.10.48.1.10.1.2 | INTEGER | This object configures the two-wire(1) or optional four- wire(2) operation for SHDSL Lines. Def: twoWire(1) |
| hdsl2ShdslSpanConfMinLineRa | Unsigned32 | This object configures the |
| te |  | minimum transmission rate |
| 1.3.6.1.2.1.10.48.1.10.1.3 |  | for |
|  |  | the associated SHDSL Line in |
|  |  | bits-per-second (bps). If the |
|  |  | minimum line rate equals the |
|  |  | maximum line rate |
|  |  | (hdsl2ShdslSpanMaxLineRat |
|  |  | e), the line rate is considered |
|  |  | 'fixed'. If the minimum line |
|  |  | rate is less than the |
|  |  | maximum line rate, the line |
|  |  | rate is considered 'rate- |
|  |  | adaptive'. |
|  |  | Def: 1552000 |
| hdsl2ShdslSpanConfMaxLineR | Unsigned32 | This object configures the |
| ate |  | maximum transmission rate |
| 1.3.6.1.2.1.10.48.1.10.1.4 |  | for |
|  |  | the associated SHDSL Line in |
|  |  | bits-per-second (bps). If the |
|  |  | minimum line rate equals the |
|  |  | maximum line rate |
|  |  | (hdsl2ShdslSpanMaxLineRat |
|  |  | e), the line rate is considered |
|  |  | 'fixed'. If the minimum line |
|  |  | rate is less than the |
|  |  | maximum line rate, the line |
|  |  | rate is considered rate- |
|  |  | adaptive'. |
|  |  | Def: 1552000 |
| hdsl2ShdslSpanConfPSD | INTEGER | This object configures use of |
| 1.3.6.1.2.1.10.48.1.10.1.5 |  | symmetric(1)/asymmetric(2) |
|  |  | PSD (Power Spectral |
|  |  | Density) Mask for the |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | associated SHDSL Line. Support for symmetric PSD is mandatory for all supported data rates. Support for asymmetric PSD is optional. Def: symmetric(1) |
| hdsl2ShdslSpanConfTransmissi onMode 1.3.6.1.2.1.10.48.1.10.1.6 | Hdsl2ShdslTr ansmissionMo deType | This object specifies the regional setting(region1(0)/ region2(1)) for the SHDSL line. Def: region2(1) |
| hdsl2ShdslSpanConfRemoteEn abled 1.3.6.1.2.1.10.48.1.10.1.7 | INTEGER | This object enables(1) /disables(2) support for remote management of the units in a SHDSL line from the STU-R via the EOC. Def: enabled(1) |
| hdsl2ShdslSpanConfPowerFee ding 1.3.6.1.2.1.10.48.1.10.1.8 | INTEGER | This object enables/disables support for optional power feeding in a SHDSL line. noPower(1), powerFeed(2), wettingCurrent(3) Def: noPower(1) |
| hdsl2ShdslSpanConfCurrCondT argetMarginDown 1.3.6.1.2.1.10.48.1.10.1.9 | INTEGER | This object specifies the downstream current condition target SNR margin for a SHDSL line. The SNR margin is the difference between the desired SNR and the actual SNR. Target SNR margin is the desired SNR margin for a unit. Def:0 |
| hdsl2ShdslSpanConfWorstCase TargetMarginDown 1.3.6.1.2.1.10.48.1.10.1.10 | INTEGER | This object specifies the downstream worst case target SNR margin for a SHDSL line. The SNR margin is the difference |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | between the desired SNR and the actual SNR. Target SNR margin is the desired SNR margin for a unit. Def:0 |
| hdsl2ShdslSpanConfCurrCondT argetMarginUp 1.3.6.1.2.1.10.48.1.10.1.11 | INTEGER | This object specifies the upstream current condition target SNR margin for a SHDSL line. The SNR margin is the difference between the desired SNR and the actual SNR. Target SNR margin is the desired SNR margin for a unit. Def:0 |
| hdsl2ShdslSpanConfWorstCase TargetMarginUp 1.3.6.1.2.1.10.48.1.10.1.12 | INTEGER | This object specifies the upstream worst case target SNR margin for a SHDSL line.The SNR margin is the difference between the desired SNR and the actual SNR. Target SNR margin is the desired SNR margin for a unit. Def:0 |
| hdsl2ShdslSpanConfUsedTarge tMargins 1.3.6.1.2.1.10.48.1.10.1.13 | BITS | Contains indicates whether a target SNR margin is enabled or disabled.This is a bit-map of possible settings. currCondDown(0), worstCaseDown(1), currCondUp(2), worstCaseUp(3) Def: currCondDown(0) |
| hdsl2ShdslSpanConfReference Clock 1.3.6.1.2.1.10.48.1.10.1.14 | Hdsl2ShdslCl ockReference Type | "This object configures the clock reference for the STU-C in a SHDSL Line. 1-LocalClk, |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | - NetworkClk, - DataOrNetworkClk, 4-DataClk Def: LocalClk |
| hdsl2ShdslSpanConfLineProbe | INTEGER | This object enables(2) |
| Enable |  | /disables(1) support for Line |
| 1.3.6.1.2.1.10.48.1.10.1.15 |  | Probe of the units in a |
|  |  | SHDSL line. When Line |
|  |  | Probe is enabled, the system |
|  |  | performs Line Probing to find |
|  |  | the best possible rate.If Line |
|  |  | probe is disabled, the rate |
|  |  | adaptation phase is skipped |
|  |  | to shorten set up time. |
|  |  | Def: disable(1) |

### Create Alarm Profile

[Function]:

Create SHDSL Alarm Profile [MIB description]:

HDSL2-SHDSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| hdsl2ShdslEndpointAlarmConfP rofileName （index） 1.3.6.1.2.1.10.48.1.11.1.1 | SnmpAdminSt ring | Length is 32, including suffix(.prf), and valid characters are numbers and letters. |
| hdsl2ShdslEndpointAlarmConfP rofileRowStatus 1.3.6.1.2.1.10.48.1.11.1.9 | RowStatus | Set rowstatus value of 4. |
| hdsl2ShdslEndpointThreshLoop | Integer | This object configures the |

Create Shdsl alarm profile, the big network to the network element set only hdsl2ShdslEndpointAlarmConfProfileRowStatus. Set rowstatus value of 4, other mibs of the profiles will be assigned as default values.

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| Attenuation 1.3.6.1.2.1.10.48.1.11.1.2 |  | loop attenuation alarm threshold. When the current value of hdsl2ShdslEndpointCurrAtn reaches or exceeds this threshold, a hdsl2ShdslLoopAttenCrossin g MAY be generated. Def:0 |
| hdsl2ShdslEndpointThreshSNR Margin 1.3.6.1.2.1.10.48.1.11.1.3 | Integer | This object configures the SNR margin alarm threshold. When the current value of hdsl2ShdslEndpointCurrSnrM gn reaches or drops below this threshold, a hdsl2ShdslSNRMarginCrossi ng MAY be generated. Def:0 |
| hdsl2ShdslEndpointThreshES 1.3.6.1.2.1.10.48.1.11.1.4 | Hdsl2ShdslPe rfIntervalThres hold | This object configures the threshold for the number of errored seconds (ES) within any given 15-minute performance data collection interval.If the value of errored seconds in a particular 15-minute collection interval reaches/ exceeds this value, a hdsl2ShdslPerfESThresh MAY be generated. At most one notification will be sent per interval per endpoint. Def:0 |
| hdsl2ShdslEndpointThreshSES 1.3.6.1.2.1.10.48.1.11.1.5 | Hdsl2ShdslPe rfIntervalThres hold | This object configures the threshold for the number of severely errored seconds |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | (SES) within any given 15- minute performance data collection interval.If the value of severely errored seconds in a particular 15-minute collection interval reaches/exceeds this value, a hdsl2ShdslPerfSESThresh MAY be generated. At most one notification will be sent per interval per endpoint. Def:0 |
| hdsl2ShdslEndpointThreshCRC anomalies 1.3.6.1.2.1.10.48.1.11.1.6 | Integer | This object configures the threshold for the number of CRC anomalies within any given 15-minute performance data collection interval. If the value of CRC anomalies in a particular 15-minute collection interval reaches/exceeds this value, a hdsl2ShdslPerfCRCanomalie sThresh MAY be generated. At most one notification will be sent per interval per endpoint. Def:0 |
| hdsl2ShdslEndpointThreshLOS WS 1.3.6.1.2.1.10.48.1.11.1.7 | Hdsl2ShdslPe rfIntervalThres hold | This object configures the threshold for the number of Loss of Sync Word (LOSW) Seconds within any given 15- minute performance data collection interval.If the value of LOSW in a particular 15- minute collection interval reaches/exceeds this value, a |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | hdsl2ShdslPerfLOSWSThres h MAY be generated. At most one notification will be sent per interval per endpoint. Def:0 |
| hdsl2ShdslEndpointThreshUAS | Hdsl2ShdslPe | This object configures the |
| 1.3.6.1.2.1.10.48.1.11.1.8 | rfIntervalThres | threshold for the number of |
|  | hold | unavailable seconds (UAS) |
|  |  | within any given 15-minute |
|  |  | performance data collection |
|  |  | interval. If the value of UAS |
|  |  | in a particular 15-minute |
|  |  | collection interval |
|  |  | reaches/exceeds this value, a |
|  |  | hdsl2ShdslPerfUASThresh |
|  |  | MAY be generated.At most |
|  |  | one notification will be sent |
|  |  | per interval per endpoint. |
|  |  | Def:0 |

### Modify Line Profile

[Function]:

Modify SHDSL Line Profile [MIB description]:

HDSL2-SHDSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| hdsl2ShdslSpanConfProfileName (index) 1.3.6.1.2.1.10.48.1.10.1.1 | SnmpAdminStrin g | Length is 32, including suffix(.prf), and valid characters are numbers |

Modify shdsl line profile, the network will need to set the hdsl2ShdslSpanConfProfileRowStatus value 1 (ACTIVE). Then set the mibs of line profile table to the network element. Line profile mib table description see table above the amount.

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | and letters |
| hdsl2ShdslSpanConfProfileRowStat us 1.3.6.1.2.1.10.48.1.10.1.16 | RowStatus | Set the value to 1 (ROWSTATUS_ACTIVE ). |
| … | … | See the table of creating SHDSL line profile. |

### Modify Alarm Profile

[Function]:

Modify SHDSL Alarm Profile [MIB description]:

HDSL2-SHDSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| hdsl2ShdslEndpointAlarmConfProfileNa me (index) 1.3.6.1.2.1.10.48.1.11.1.1 | SnmpAdminStr ing | Length is 32, including suffix(.prf), and valid characters are numbers and letters |
| hdsl2ShdslEndpointAlarmConfProfileRo wStatus 1.3.6.1.2.1.10.48.1.11.1.9 | RowStatus | Set the value to 1 (ROWSTATUS_ACTI VE). |
| … | … | See the table of creating SHDSL alarm profile. |

Modify SHDSL alarm profile, the network will need to set the hdsl2ShdslEndpointAlarmConfProfileRowStatus value 1 (ACTIVE). Then set the mibs of line profile table to the network element. Line profile mib table description see table above the amount.

### Delete Line Profile

[Function]:

Delete SHDSL Line Profile [MIB description]:

HDSL2-SHDSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| hdsl2ShdslSpanConfProfileName (index) 1.3.6.1.2.1.10.48.1.10.1.1 | SnmpAdminStrin g | Length is 32, including suffix(.prf), and valid characters are numbers and letters |
| hdsl2ShdslSpanConfProfileRowStat us 1.3.6.1.2.1.10.48.1.10.1.16 | RowStatus | Set the value to 6 (ROWSTATUS_DELET E). |

Delete shdsl line profile, the network will need to set the hdsl2ShdslSpanConfProfileRowStatus value 6 (DELETE).

### Delete Alarm Profile

[Function]:

Delete SHDSL Alarm Profile [MIB description]:

HDSL2-SHDSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| hdsl2ShdslEndpointAlarmConfProfileNa me (index) 1.3.6.1.2.1.10.48.1.11.1.1 | SnmpAdminStr ing | Length is 32, including suffix(.prf), and valid characters are numbers and letters |  |  |  |  |
| hdsl2ShdslEndpointAlarmConfProfileRo | RowStatus | Set | the | value | to | 6 |

Delete shdsl alarm profile, the network will need to set the hdsl2ShdslEndpointAlarmConfProfileRowStatus value 6 (DELETE).

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| wStatus 1.3.6.1.2.1.10.48.1.11.1.9 |  | (ROWSTATUS_DELE TE). |

### Bind SHDSL Line Profile To Port

[Function]:

When the shdsl profile is created and modified, it need to be applied to the port. Port will re-link under the new profile.

[MIB description]:

HDSL2-SHDSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| ifIndex 1.3.6.1.2.1.2.2.1.1 | InterfaceIndex | NormalPort |
| hdsl2ShdslSpanConfProfile 1.3.6.1.2.1.10.48.1.1.1.2 | SnmpAdminString | String length is 32 (including the terminator), demand is setting the existed line profile name. |

### Bind SHDSL Alarm Profile To Port

[Function]:

When the shdsl alarm profile is created and modified, it need to be applied to the port. Port will re-link under the new profile.

[MIB description]:

HDSL2-SHDSL-LINE-MIB.mib

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
| ifIndex 1.3.6.1.2.1.2.2.1.1 | InterfaceIndex | NormalPort |
| hdsl2ShdslSpanConfAlarmProfile 1.3.6.1.2.1.10.48.1.1.1.3 | SnmpAdminString | String length is 32 (including the |

| MIB Variable/ OID | Type | Description |
| --- | --- | --- |
|  |  | terminator), demand is setting the existed line profile name. |

# Optical Module and Optical Power

## Optical Module

[Function description]:

This MIB queries Ethernt port and PON port optical modules. [MIB file]:

ZTE-AN-OPTICAL-MODULE-MIB.mib

#### Information Query

[MIB variable description]:

The OID of zxAnOpticalModuleInfoTable is .1.3.6.1.4.1.3902.1082.30.40.2.4.

OLT transmitting optical power

zxAnOpticalIfRxPwrCurrValue: .1.3.6.1.4.1.3902.1082.30.40.2.4.1.2 The index is ifIndex.

[MIB file]:

Refer to ZTE-AN-OPTICAL-MODULE-MIB.mib for detailed descriptions. zxAnOpticalModuleInfoTable OBJECT-Type

SYNTAX	SEQUENCE OF ZxAnOpticalModuleInfoEntry MAX-ACCESS not-accessible

STATUS	current

DESCRIPTION "This table includes the optical module information."

::= { zxAnOpticalModuleObjects 4 }

zxAnOpticalModuleInfoEntry OBJECT-Type SYNTAX	ZxAnOpticalModuleInfoEntry MAX-ACCESS not-accessible

STATUS	current

DESCRIPTION "An entry in zxAnOpticalModuleInfoTable." INDEX	{ ifIndex }

::= { zxAnOpticalModuleInfoTable 1 }

ZxAnOpticalModuleInfoEntry ::= SEQUENCE { zxAnOpticalIfRxPwrCurrValue			 Integer32, zxAnOpticalIfTxPwrCurrValue			Integer32, zxAnOpticalIfRate	Integer32, zxAnOpticalBiasCurrent		Integer32,

zxAnOpticalSupplyVoltage	Integer32,

zxAnOpticalWavelength	Integer32,

zxAnOpticalTemperature	Integer32,

zxAnOpticalFiberType	INTEGER,

zxAnOpticalVersionLevel	DisplayString,

zxAnOpticalVendorPn	DisplayString,

zxAnOpticalVendorName	DisplayString,

zxAnOpticalVendorSn	DisplayString, zxAnOpticalProductionDate				DisplayString, zxAnOpticalModuleType			DisplayString, zxAnOpticalFiberInterfaceType						DisplayString, zxAnOpticalMaterialNumber					OCTET STRING, zxAnOpticalRegisterData		OCTET STRING

}

#### Threshold Configuration

[Index description]:

| MIB Variable | OID | Description |  |
| --- | --- | --- | --- |
| zxAnOpticalRxPwrLowerT | . | Minimum threshold | of |
| hresh | 1.3.6.1.4.1.3902.1082 | receiving optical power |  |
|  | .30.40.2.2.1.2 |  |  |
| zxAnOpticalRxPwrUpperT | . | Maximum threshold | of |
| hresh | 1.3.6.1.4.1.3902.1082 | receiving optical power |  |
|  | .30.40.2.2.1.3 |  |  |
| zxAnOpticalTxPwrLowerT | . | Minimum threshold | of |
| hresh | 1.3.6.1.4.1.3902.1082 | transmitting optical power |  |
|  | .30.40.2.2.1.4 |  |  |
| zxAnOpticalTxPwrUpperT | . | Maximum threshold | of |
| hresh | 1.3.6.1.4.1.3902.1082 | transmitting optical power |  |
|  | .30.40.2.2.1.5 |  |  |
| zxAnOpticalBiasCurrLowe | . | Minimum current threshold |  |
| rThresh | 1.3.6.1.4.1.3902.1082 |  |  |
|  | .30.40.2.2.1.6 |  |  |
| zxAnOpticalBiasCurrUppe | . | Maximum current threshold |  |
| rThresh | 1.3.6.1.4.1.3902.1082 |  |  |
|  | .30.40.2.2.1.7 |  |  |

Profile name, character string. [MIB variable description]:

| MIB Variable | OID | Description |  |
| --- | --- | --- | --- |
| zxAnOpticalVoltageLower | . | Minimum voltage threshold |  |
| Thresh | 1.3.6.1.4.1.3902.1082 |  |  |
|  | .30.40.2.2.1.8 |  |  |
| zxAnOpticalVoltageUpper | . | Maximum voltage threshold |  |
| Thresh | 1.3.6.1.4.1.3902.1082 |  |  |
|  | .30.40.2.2.1.9 |  |  |
| zxAnOpticalTempLowerTh | . | Minimum | temperature |
| resh | 1.3.6.1.4.1.3902.1082 | threshold |  |
|  | .30.40.2.2.1.10 |  |  |
| zxAnOpticalTempUpperTh | . | Maximum | temperature |
| resh | 1.3.6.1.4.1.3902.1082 | threshold |  |
|  | .30.40.2.2.1.11 |  |  |
| zxAnOpticalOfflineTrapEn | . | Online alarm swtich |  |
| able | 1.3.6.1.4.1.3902.1082 |  |  |
|  | .30.40.2.2.1.12 |  |  |
| zxAnOpticalModuleAlmPrf | . | Profile enabling |  |
| RowStatus | 1.3.6.1.4.1.3902.1082 |  |  |
|  | .30.40.2.2.1.50 |  |  |

#### Threshold Loading

[Index description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnOpticalIfAlmPrf | . | Profile |
|  | 1.3.6.1.4.1.3902.1082.30.40.2.3.1.2 | name |

Type 1 platform composite index. [MIB variable description]:

#### Als Config

[MIB variable description]:

The OID of zxAnOpticalAlsIfConfTable is .1.3.6.1.4.1.3902.1082.30.40.2.5

###### zxAnOptAlsEnable：.1.3.6.1.4.1.3902.1082.30.40.2.5.1.1

The index is ifIndex.

[MIB file]:

###### zxAnOpticalAlsIfConfTable OBJECT-TYPE

###### SYNTAX	SEQUENCE OF ZxAnOpticalAlsIfConfEntry MAX-ACCESS not-accessible

###### STATUS	current DESCRIPTION

###### "This table includes the ALS configuration of the optical module."

###### ::= { zxAnOpticalModuleObjects 5 }

###### zxAnOpticalAlsIfConfEntry OBJECT-TYPE SYNTAX	ZxAnOpticalAlsIfConfEntry MAX-ACCESS not-accessible

###### STATUS	current DESCRIPTION

###### "An entry in zxAnOpticalAlsIfConfTable." INDEX	{ ifIndex }

###### ::= { zxAnOpticalAlsIfConfTable 1 }

###### ZxAnOpticalAlsIfConfEntry ::= SEQUENCE { zxAnOptAlsEnable	INTEGER, zxAnOptAlsLaserLosDuration	Integer32, zxAnOptAlsLaserShutdownDuration Integer32, zxAnOptAlsLaserStartUpDuration Integer32

}

###### zxAnOptAlsEnable OBJECT-TYPE SYNTAX INTEGER{

###### enabled(1), disabled(2)

}

###### MAX-ACCESS read-write STATUS current DESCRIPTION

###### "Enable or disable the ALS function." DEFVAL { disabled }

###### ::= { zxAnOpticalAlsIfConfEntry 1 }

###### zxAnOptAlsLaserLosDuration OBJECT-TYPE SYNTAX Integer32(5..50)

###### UNITS "100ms"

###### MAX-ACCESS read-write

###### STATUS current DESCRIPTION

###### "This attribute specifies the value of the LOS duration.

###### The laser will be shutdown when detected the LOS duration longer than zxAnOptAlsLaserLosDuration value.

###### Setting this object will enable the ALS function." DEFVAL { 5 }

###### ::= { zxAnOpticalAlsIfConfEntry 2}

###### zxAnOptAlsLaserShutdownDuration OBJECT-TYPE SYNTAX Integer32(20..2000)

###### UNITS "100ms"

###### MAX-ACCESS read-write STATUS current DESCRIPTION

###### "This attribute specifies the value of the laser shutdown duration.

###### Setting this object will enable the ALS function." DEFVAL { 100 }

###### ::= { zxAnOpticalAlsIfConfEntry 3}

###### zxAnOptAlsLaserStartUpDuration OBJECT-TYPE SYNTAX	Integer32(50..20000)

###### UNITS	"100ms"

###### MAX-ACCESS read-write STATUS	current DESCRIPTION

###### "This attribute specifies the value of the laser re-open duration.

###### Setting this object will enable the ALS function." DEFVAL { 50 }

###### ::= { zxAnOpticalAlsIfConfEntry 4}

## Receiving Optical Power on the OLT Side

[MIB file]:

Please refer to zxAnPonRxOpticalPowerTable defined in ZTE-AN-PON-BASE-MIB.mib.

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnPonRxOptic alPower | . 1.3.6.1.4.1.3902 .1082.500.1.2.4. 2.1.2 | INTEGER {reboot ( 1 ) } | "Measured the received optical power. 65535000 means N/A, ONU is not in service. -80000 means no signal received." |

## Optical Power on the GPON ONU Side

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponRmAni RxOptLevel | . 1.3.6.1.4.1.3902 .1082.500.20.2. 2.2.1.10 |  | This attribute reports the current measurement of total downstream optical signal level, dBuW, 2s complement, 0.002 dB resolution. 0xFFFF means the optical signal level is unknown(N/A) when ONU is offline. |
| zxAnGponRmAni TxOptLevel | . 1.3.6.1.4.1.3902 .1082.500.20.2. 2.2.1.14 |  | This attribute reports the current measurement of mean optical launch power, dBuW, 2s complement, 0.002 dB resolution. 0xFFFF means the optical signal level is unknown(N/A) when ONU is offline. |

Please refer to zxAnGponRmAniTable defined in ZTE-AN-GPON-REMOTE-ONU- MIB.mib.

# Performance Statistics

## Ethernet Port Realtime Statistics

#### ifTable Realtime Statistics

[MIB file]:

Please refer to rfc2863.mib. [Index description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| ifDescr | 1.3.6.1.2.1.2.2.1.2 | DisplayString ( SIZE ( 0 .. 255 ) ) |
| ifType | 1.3.6.1.2.1.2.2.1.3 | IANAifMIB value |
| ifMtu | 1.3.6.1.2.1.2.2.1.4 | Integer32 |
| ifSpeed | 1.3.6.1.2.1.2.2.1.5 | Gauge32 |
| ifPhysAddres s | 1.3.6.1.2.1.2.2.1.6 | PhysAddress |
| ifAdminStatus | 1.3.6.1.2.1.2.2.1.7 | INTEGER { up ( 1 ) , down ( 2 ) , testing ( 3 ) } |
| ifOperStatus | 1.3.6.1.2.1.2.2.1.8 | INTEGER { up ( 1 ) , down ( 2 ) , testing ( 3 ) , unknown ( 4 ) , dormant ( 5 ) , notPresent ( 6 ) , lowerLayerDown ( 7 ) } |
| ifLastChange | 1.3.6.1.2.1.2.2.1.9 | TimeTicks |
| ifInOctets | 1.3.6.1.2.1.2.2.1.1 0 | Counter32 |
| ifInUcastPkts | 1.3.6.1.2.1.2.2.1.11 | Counter32 |
| ifInNUcastPkt s | 1.3.6.1.2.1.2.2.1.1 2 | Counter32 |

{ ifIndex }. Type 1 platform composite index. [MIB variable description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifInDiscards | 1.3.6.1.2.1.2.2.1.1 3 | Counter32 |
| ifInErrors | 1.3.6.1.2.1.2.2.1.1 4 | Counter32 |
| ifInUnknownP rotos | 1.3.6.1.2.1.2.2.1.1 5 | Counter32 |
| ifOutOctets | 1.3.6.1.2.1.2.2.1.1 6 | Counter32 |
| ifOutUcastPkt s | 1.3.6.1.2.1.2.2.1.1 7 | Counter32 |
| ifOutNUcastP kts | 1.3.6.1.2.1.2.2.1.1 8 | Counter32 |
| ifOutDiscards | 1.3.6.1.2.1.2.2.1.1 9 | Counter32 |
| ifOutErrors | 1.3.6.1.2.1.2.2.1.2 0 | Counter32 |
| ifOutQLen | 1.3.6.1.2.1.2.2.1.2 1 | Gauge32 |
| ifSpecific | 1.3.6.1.2.1.2.2.1.2 2 | OBJECT IDENTIFIER |

#### ifXTable Realtime Statistics

[MIB file]:

Please refer to rfc2863.mib. [Index description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| ifName | 1.3.6.1.2.1.31.1.1.1. 1 | DisplayString |

{ ifIndex }. Type 1 platform composite index [MIB variable description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifInMulticastPkts | 1.3.6.1.2.1.31.1.1.1. 2 | Counter32 |
| ifInBroadcastPkts | 1.3.6.1.2.1.31.1.1.1. 3 | Counter32 |
| ifOutMulticastPkts | 1.3.6.1.2.1.31.1.1.1. 4 | Counter32 |
| ifOutBroadcastPkts | 1.3.6.1.2.1.31.1.1.1. 5 | Counter32 |
| ifHCInOctets | 1.3.6.1.2.1.31.1.1.1. 6 | Counter64 |
| ifHCInUcastPkts | 1.3.6.1.2.1.31.1.1.1. 7 | Counter64 |
| ifHCInMulticastPkts | 1.3.6.1.2.1.31.1.1.1. 8 | Counter64 |
| ifHCInBroadcastPkt s | 1.3.6.1.2.1.31.1.1.1. 9 | Counter64 |
| ifHCOutOctets | 1.3.6.1.2.1.31.1.1.1. 10 | Counter64 |
| ifHCOutUcastPkts | 1.3.6.1.2.1.31.1.1.1. 11 | Counter64 |
| ifHCOutMulticastPkt s | 1.3.6.1.2.1.31.1.1.1. 12 | Counter64 |
| ifHCOutBroadcastP kts | 1.3.6.1.2.1.31.1.1.1. 13 | Counter64 |
| ifLinkUpDownTrapE nable | 1.3.6.1.2.1.31.1.1.1. 14 | INTEGER { enabled ( 1 ) , disabled ( 2 ) } |
| ifHighSpeed | 1.3.6.1.2.1.31.1.1.1. 15 | Gauge32 |
| ifPromiscuousMode | 1.3.6.1.2.1.31.1.1.1. 16 | TruthValue |
| ifConnectorPresent | 1.3.6.1.2.1.31.1.1.1. 17 | TruthValue |
| ifAlias | 1.3.6.1.2.1.31.1.1.1. 18 | DisplayString ( SIZE ( 0 .. 64 ) ) |
| ifCounterDiscontinui tyTime | 1.3.6.1.2.1.31.1.1.1. 19 | TimeStamp |

## OLT Ethernet Realtime Statistics

#### Realtime Statistics

[MIB file]:

Please refer to zxAnPonOltIfCurrPerfTable defined in ZTE-AN-PON-PERF-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOltIfRxOct | . |  |
| ets | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.1.2.1.1 |  |
| zxAnPonOltIfRxPkts | . |  |
|  | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.1.2.1.2 |  |
| zxAnPonOltIfRxOct | . |  |
| etRate | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.1.2.1.3 |  |
| zxAnPonOltIfRxPkt | . |  |
| Rate | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.1.2.1.4 |  |
| zxAnPonOltIfRxOct | . |  |
| etPeakRate | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.1.2.1.5 |  |
| zxAnPonOltIfRxPkt | . |  |
| PeakRate | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.1.2.1.6 |  |
| zxAnPonOltIfRxUca | . |  |
| stPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.1.2.1.7 |  |
| zxAnPonOltIfRxNUc | . |  |
| astPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.1.2.1.8 |  |
| zxAnPonOltIfRxMca | . |  |
| stPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.1.2.1.9 |  |
| zxAnPonOltIfRxBca | . |  |
| stPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.1.2.1.10 |  |
| zxAnPonOltIfRxDisc | . |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| ardPkts | 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.11 |  |
| zxAnPonOltIfRxErrP kts | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.12 |  |
| zxAnPonOltIfRxPdr | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.13 |  |
| zxAnPonOltIfRxPer | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.14 |  |
| zxAnPonOltIfRxCrc AlignErrors | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.15 |  |
| zxAnPonOltIfRxUnd ersizePkts | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.16 |  |
| zxAnPonOltIfRxOve rsizePkts | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.17 |  |
| zxAnPonOltIfRxPkts 64Octets | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.18 |  |
| zxAnPonOltIfRxPkts 65To127Octets | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.19 |  |
| zxAnPonOltIfRxPkts 128To255Octets | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.20 |  |
| zxAnPonOltIfRxPkts 256To511Octets | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.21 |  |
| zxAnPonOltIfRxPkts 512To1023Oct | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.22 |  |
| zxAnPonOltIfRxPkts 1024To1518Oct | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.23 |  |
| zxAnPonOltIfRxBw | . |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| Utilization | 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.24 |  |
| zxAnPonOltIfRxAvg BwUtilization | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.25 |  |
| zxAnPonOltIfRxFra gments | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.26 |  |
| zxAnPonOltIfRxJab bers | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.27 |  |
| zxAnPonOltIfTxOcte ts | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.44 |  |
| zxAnPonOltIfTxPkts | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.45 |  |
| zxAnPonOltIfTxOcte tRate | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.46 |  |
| zxAnPonOltIfTxPkt Rate | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.47 |  |
| zxAnPonOltIfTxOcte tPeakRate | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.48 |  |
| zxAnPonOltIfTxPktP eakRate | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.49 |  |
| zxAnPonOltIfTxUca stPkts | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.50 |  |
| zxAnPonOltIfTxNUc astPkts | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.51 |  |
| zxAnPonOltIfTxMca stPkts | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.52 |  |
| zxAnPonOltIfTxBca | . |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| stPkts | 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.53 |  |
| zxAnPonOltIfTxDisc ardPkts | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.54 |  |
| zxAnPonOltIfTxErrP kts | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.55 |  |
| zxAnPonOltIfTxPdr | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.56 |  |
| zxAnPonOltIfTxPer | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.57 |  |
| zxAnPonOltIfTxCrc AlignErrors | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.58 |  |
| zxAnPonOltIfTxUnd ersizePkts | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.59 |  |
| zxAnPonOltIfTxOve rsizePkts | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.60 |  |
| zxAnPonOltIfTxPkts 64Octets | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.61 |  |
| zxAnPonOltIfTxPkts 65To127Octets | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.62 |  |
| zxAnPonOltIfTxPkts 128To255Octets | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.63 |  |
| zxAnPonOltIfTxPkts 256To511Octets | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.64 |  |
| zxAnPonOltIfTxPkts 512To1023Oct | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.65 |  |
| zxAnPonOltIfTxPkts | . |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| 1024To1518Oct | 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.66 |  |
| zxAnPonOltIfTxBwU tilization | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.67 |  |
| zxAnPonOltIfTxAvg BwUtilization | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.68 |  |
| zxAnPonOltIfTxFrag ments | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.69 |  |
| zxAnPonOltIfTxJab bers | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.70 |  |
| zxAnPonOltIfCollisio ns | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.88 |  |
| zxAnPonOltIfDropE vents | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.89 |  |
| zxAnPonOltIfCurrPe rfCapability | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.99 |  |
| zxAnPonOltIfCurrPe rfReset | . 1.3.6.1.4.1.3902.1082.5 00.4.2.1.2.1.100 |  |

## ONU Ethernet Realtime Statistics

#### Realtime Performance

[MIB file]:

Please refer to zxAnPonOnuIfCurrPerfTable defined in ZTE-AN-PON-PERF-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfRxOc | . |  |
| tets | 1.3.6.1.4.1.3902.1082.5 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | 00.4.2.2.2.1.1 |  |
| zxAnPonOnuIfRxPk | . |  |
| ts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.2 |  |
| zxAnPonOnuIfRxOc | . |  |
| tetRate | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.3 |  |
| zxAnPonOnuIfRxPk | . |  |
| tRate | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.4 |  |
| zxAnPonOnuIfRxOc | . |  |
| tetPeakRate | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.5 |  |
| zxAnPonOnuIfRxPk | . |  |
| tPeakRate | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.6 |  |
| zxAnPonOnuIfRxUc | . |  |
| astPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.7 |  |
| zxAnPonOnuIfRxN | . |  |
| UcastPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.8 |  |
| zxAnPonOnuIfRxMc | . |  |
| astPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.9 |  |
| zxAnPonOnuIfRxBc | . |  |
| astPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.10 |  |
| zxAnPonOnuIfRxDi | . |  |
| scardPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.11 |  |
| zxAnPonOnuIfRxErr | . |  |
| Pkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.12 |  |
| zxAnPonOnuIfRxPd | . |  |
| r | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.13 |  |
| zxAnPonOnuIfRxPe | . |  |
| r | 1.3.6.1.4.1.3902.1082.5 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | 00.4.2.2.2.1.14 |  |
| zxAnPonOnuIfRxCr | . |  |
| cAlignErrors | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.15 |  |
| zxAnPonOnuIfRxUn | . |  |
| dersizePkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.16 |  |
| zxAnPonOnuIfRxOv | . |  |
| ersizePkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.17 |  |
| zxAnPonOnuIfRxPk | . |  |
| ts64Octets | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.18 |  |
| zxAnPonOnuIfRxPk | . |  |
| ts65To127Octets | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.19 |  |
| zxAnPonOnuIfRxPk | . |  |
| ts128To255Octets | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.20 |  |
| zxAnPonOnuIfRxPk | . |  |
| ts256To511Octets | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.21 |  |
| zxAnPonOnuIfRxPk | . |  |
| ts512To1023Oct | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.22 |  |
| zxAnPonOnuIfRxPk | . |  |
| ts1024To1518Oct | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.23 |  |
| zxAnPonOnuIfRxBw | . |  |
| Utilization | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.24 |  |
| zxAnPonOnuIfRxAv | . |  |
| gBwUtilization | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.25 |  |
| zxAnPonOnuIfRxFr | . |  |
| agments | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.26 |  |
| zxAnPonOnuIfRxJa | . |  |
| bbers | 1.3.6.1.4.1.3902.1082.5 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | 00.4.2.2.2.1.27 |  |
| zxAnPonOnuIfTxOc | . |  |
| tets | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.44 |  |
| zxAnPonOnuIfTxPkt | . |  |
| s | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.45 |  |
| zxAnPonOnuIfTxOc | . |  |
| tetRate | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.46 |  |
| zxAnPonOnuIfTxPkt | . |  |
| Rate | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.47 |  |
| zxAnPonOnuIfTxOc | . |  |
| tetPeakRate | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.48 |  |
| zxAnPonOnuIfTxPkt | . |  |
| PeakRate | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.49 |  |
| zxAnPonOnuIfTxUc | . |  |
| astPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.50 |  |
| zxAnPonOnuIfTxNU | . |  |
| castPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.51 |  |
| zxAnPonOnuIfTxMc | . |  |
| astPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.52 |  |
| zxAnPonOnuIfTxBc | . |  |
| astPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.53 |  |
| zxAnPonOnuIfTxDis | . |  |
| cardPkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.54 |  |
| zxAnPonOnuIfTxErr | . |  |
| Pkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.55 |  |
| zxAnPonOnuIfTxPd | . |  |
| r | 1.3.6.1.4.1.3902.1082.5 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | 00.4.2.2.2.1.56 |  |
| zxAnPonOnuIfTxPe | . |  |
| r | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.57 |  |
| zxAnPonOnuIfTxCr | . |  |
| cAlignErrors | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.58 |  |
| zxAnPonOnuIfTxUn | . |  |
| dersizePkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.59 |  |
| zxAnPonOnuIfTxOv | . |  |
| ersizePkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.60 |  |
| zxAnPonOnuIfTxPkt | . |  |
| s64Octets | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.61 |  |
| zxAnPonOnuIfTxPkt | . |  |
| s65To127Octets | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.62 |  |
| zxAnPonOnuIfTxPkt | . |  |
| s128To255Octets | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.63 |  |
| zxAnPonOnuIfTxPkt | . |  |
| s256To511Octets | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.64 |  |
| zxAnPonOnuIfTxPkt | . |  |
| s512To1023Oct | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.65 |  |
| zxAnPonOnuIfTxPkt | . |  |
| s1024To1518Oct | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.66 |  |
| zxAnPonOnuIfTxBw | . |  |
| Utilization | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.67 |  |
| zxAnPonOnuIfTxAv | . |  |
| gBwUtilization | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.68 |  |
| zxAnPonOnuIfTxFra | . |  |
| gments | 1.3.6.1.4.1.3902.1082.5 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | 00.4.2.2.2.1.69 |  |
| zxAnPonOnuIfTxJa | . |  |
| bbers | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.70 |  |
| zxAnPonOnuIfCollis | . |  |
| ions | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.88 |  |
| zxAnPonOnuIfDrop | . |  |
| Events | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.89 |  |
| zxAnPonOnuIfCurrP | . |  |
| erfCapability | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.99 |  |
| zxAnPonOnuIfCurrP | . |  |
| erfReset | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.4.2.2.2.1.100 |  |

## GPON OLT PON Layer Realtime Statistics

#### Realtime Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponUsCorrec | . |  |
| tNIdleGems | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.1 |  |
| zxAnGponUsCorrec | . |  |
| tIdleGems | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.2 |  |
| zxAnGponUsErrGe | . |  |
| ms | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.3 |  |
| zxAnGponUsGemP | . |  |
| ayloadBytes | 1.3.6.1.4.1.3902.1082.5 |  |

Please refer to zxAnGponSrvOltCurrPerfTable defined in ZTE-AN-GPON-SERVICE- MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | 00.10.2.2.2.2.1.4 |  |
| zxAnGponUsCorrec | . |  |
| tEtherFrm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.5 |  |
| zxAnGponUsErrEth | . |  |
| erFrm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.6 |  |
| zxAnGponUsPloam | . |  |
| Msgs | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.7 |  |
| zxAnGponUsErrPlo | . |  |
| amMsgs | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.8 |  |
| zxAnGponUsErrBits | . |  |
|  | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.9 |  |
| zxAnGponUsTotalBi | . |  |
| ts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.10 |  |
| zxAnGponUsCrcErr | . |  |
| Pkts | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.11 |  |
| zxAnGponDsEtherF | . |  |
| rm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.12 |  |
| zxAnGponDsPloam | . |  |
| Msgs | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.13 |  |
| zxAnGponOltReis | . |  |
|  | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.14 |  |
| zxAnGponCorrected | . |  |
| Bytes | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.15 |  |
| zxAnGponCorrected | . |  |
| Words | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.16 |  |
| zxAnGponUncorrect | . |  |
| edWords | 1.3.6.1.4.1.3902.1082.5 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | 00.10.2.2.2.2.1.17 |  |
| zxAnGponTotalRxW | . |  |
| ords | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.18 |  |
| zxAnGponUsBer | . |  |
|  | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.19 |  |
| zxAnGponOltPerfCa | . |  |
| pability | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.99 |  |
| zxAnGponOltCurrR | . |  |
| eset | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.2.2.2.2.1.100 |  |

## GPON ONU PON Layer Realtime Statistics

#### Realtime Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOnuUsCor | . |  |
| rectNIdleGems | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.1 |  |
| zxAnGponOnuUsGe | . |  |
| mPayloadBytes | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.2 |  |
| zxAnGponOnuUsCor | . |  |
| rectEtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.3 |  |
| zxAnGponOnuUsErr | . |  |
| EtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.4 |  |
| zxAnGponOnuUsPlo | . |  |
| amMsgs | 1.3.6.1.4.1.3902.1082.500 |  |

Please refer to zxAnGponSrvOnuCurrPerfTable defined in ZTE-AN-GPON-SERVICE- MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | .10.2.3.2.2.1.5 |  |
| zxAnGponOnuUsOm | . |  |
| ciMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.6 |  |
| zxAnGponOnuUsErr | . |  |
| Bits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.7 |  |
| zxAnGponOnuUsTot | . |  |
| alBits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.8 |  |
| zxAnGponOnuUsCrc | . |  |
| ErrPkts | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.9 |  |
| zxAnGponOnuUsLos | . |  |
| tBursts | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.10 |  |
| zxAnGponOnuDsCor | . |  |
| rectNIdleGems | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.11 |  |
| zxAnGponOnuDsPlo | . |  |
| amMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.12 |  |
| zxAnGponOnuDsOm | . |  |
| ciMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.13 |  |
| zxAnGponOnuReiis | . |  |
|  | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.14 |  |
| zxAnGponOnuCorre | . |  |
| ctedBytes | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.15 |  |
| zxAnGponOnuCorre | . |  |
| ctedWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.16 |  |
| zxAnGponOnuUncor | . |  |
| rectedWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.17 |  |
| zxAnGponOnuTotalR | . |  |
| xWords | 1.3.6.1.4.1.3902.1082.500 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | .10.2.3.2.2.1.18 |  |
| zxAnGponOnuUsBer | . |  |
|  | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.19 |  |
| zxAnGponOnuPerfC | . |  |
| apability | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.99 |  |
| zxAnGponOnuCurrR | . |  |
| eset | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.100 |  |

## Ethernet Interface Performance

[MIB file]:

ZTE-AN-INTERFACE-STATS-MIB.mib ZTE-AN-INTERFACE-ETH-PERF-MIB.mib

#### Realtime Performance

[Index description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| zxAnEthIfInOctets | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .1 | Counter64 |
| zxAnEthIfInPkts | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .2 | Counter64 |
| zxAnEthIfInUcastPkts | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .3 | Counter64 |
| zxAnEthIfInMcastPkts | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .4 | Counter64 |
| zxAnEthIfInBcastPkts | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 | Counter64 |

{ ifIndex }. Type 1 platform composite index. [MIB variable description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | .5 |  |
| zxAnEthIfInOversizedPkts | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .6 | Counter64 |
| zxAnEthIfInUndersizedPkts | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .7 | Counter64 |
| zxAnEthIfOutOctets | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .8 | Counter64 |
| zxAnEthIfOutPkts | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .9 | Counter64 |
| zxAnEthIfOutUcastPkts | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .10 | Counter64 |
| zxAnEthIfOutMcastPkts | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .11 | Counter64 |
| zxAnEthIfOutBcastPkts | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .12 | Counter64 |
| zxAnEthIfOutPausePkts | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .13 | Counter64 |
| zxAnEthIfOutDiscardPkts | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .14 | Counter64 |
| zxAnEthIfOutCollisions | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .15 | Counter64 |
| zxAnEthIfSingleCollisions | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .16 | Counter64 |
| zxAnEthIfMultipleCollisions | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .17 | Counter64 |
| zxAnEthIfFcsErrors | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .18 | Counter64 |
| zxAnEthIfAlignmentErrors | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .19 | Counter64 |
| zxAnEthIfReset | 1.3.6.1.4.1.3902.1082.30.31.2.2.1 .100 | INTEGER { perfReset ( 1 ) } |

#### 15-Minute Realtime Performance

[Index description]:

{ ifIndex }. Type 1 platform composite index. [MIB variable description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| zxAnEthIfC15MTimeElapsed | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.1 | Gauge32 ( 0 .. 899 ) |
| zxAnEthIfC15MInOctets | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.2 | Counter64 |
| zxAnEthIfC15MInPkts | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.3 | Counter64 |
| zxAnEthIfC15MInUcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.4 | Counter64 |
| zxAnEthIfC15MInMcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.5 | Counter64 |
| zxAnEthIfC15MInBcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.6 | Counter64 |
| zxAnEthIfC15MInOversizedPk ts | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.7 | Counter64 |
| zxAnEthIfC15MInUndersizedP kts | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.8 | Counter64 |
| zxAnEthIfC15MOutOctets | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.9 | Counter64 |
| zxAnEthIfC15MOutPkts | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.10 | Counter64 |
| zxAnEthIfC15MOutUcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.11 | Counter64 |
| zxAnEthIfC15MOutMcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.12 | Counter64 |
| zxAnEthIfC15MOutBcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.13 | Counter64 |
| zxAnEthIfC15MOutPausePkts | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.14 | Counter64 |
| zxAnEthIfC15MOutDiscardPkt s | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.15 | Counter64 |
| zxAnEthIfC15MOutCollisions | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.16 | Counter64 |
| zxAnEthIfC15MSingleCollision s | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.17 | Counter64 |
| zxAnEthIfC15MMultipleCollisio ns | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.18 | Counter64 |
| zxAnEthIfC15MFcsErrors | 1.3.6.1.4.1.3902.1082.30.32.2.2. | Counter64 |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | 1.19 |  |
| zxAnEthIfC15MAlignmentError s | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.20 | Counter64 |
| zxAnEthIfC15MReset | 1.3.6.1.4.1.3902.1082.30.32.2.2. 1.100 | INTEGER { perfReset ( 1 ) } |

#### 24-Hour Realtime Performance

[Index description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| zxAnEthIfC1DTimeElapsed | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.1 | Gauge32 ( 0 .. 86399 ) |
| zxAnEthIfC1DInOctets | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.2 | Counter64 |
| zxAnEthIfC1DInPkts | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.3 | Counter64 |
| zxAnEthIfC1DInUcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.4 | Counter64 |
| zxAnEthIfC1DInMcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.5 | Counter64 |
| zxAnEthIfC1DInBcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.6 | Counter64 |
| zxAnEthIfC1DInOversizedPkts | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.7 | Counter64 |
| zxAnEthIfC1DInUndersizedPkt s | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.8 | Counter64 |
| zxAnEthIfC1DOutOctets | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.9 | Counter64 |
| zxAnEthIfC1DOutPkts | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.10 | Counter64 |
| zxAnEthIfC1DOutUcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.11 | Counter64 |
| zxAnEthIfC1DOutMcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.3. | Counter64 |

{ ifIndex }. Type 1 platform composite index. [MIB variable description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | 1.12 |  |
| zxAnEthIfC1DOutBcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.13 | Counter64 |
| zxAnEthIfC1DOutPausePkts | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.14 | Counter64 |
| zxAnEthIfC1DOutDiscardPkts | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.15 | Counter64 |
| zxAnEthIfC1DOutCollisions | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.16 | Counter64 |
| zxAnEthIfC1DSingleCollisions | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.17 | Counter64 |
| zxAnEthIfC1DMultipleCollision s | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.18 | Counter64 |
| zxAnEthIfC1DFcsErrors | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.19 | Counter64 |
| zxAnEthIfC1DAlignmentErrors | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.20 | Counter64 |
| zxAnEthIfC1DReset | 1.3.6.1.4.1.3902.1082.30.32.2.3. 1.100 | INTEGER { perfReset ( 1 ) } |

#### 15-Minute History Performance

[Index description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| zxAnEthIfH15MIntervalNo | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.1 | Integer32 ( 1 .. 96 ) |
| zxAnEthIfH15MIntervalValidDa ta | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.2 | TruthValue |
| zxAnEthIfH15MDateTime | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.3 | DateAndTime |
| zxAnEthIfH15MInOctets | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.4 | Counter64 |
| zxAnEthIfH15MInPkts | 1.3.6.1.4.1.3902.1082.30.32.2.4. | Counter64 |

{ ifIndex }. Type 1 platform composite index. [MIB variable description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | 1.5 |  |
| zxAnEthIfH15MInUcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.6 | Counter64 |
| zxAnEthIfH15MInMcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.7 | Counter64 |
| zxAnEthIfH15MInBcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.8 | Counter64 |
| zxAnEthIfH15MInOversizedPk ts | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.9 | Counter64 |
| zxAnEthIfH15MInUndersizedP kts | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.10 | Counter64 |
| zxAnEthIfH15MOutOctets | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.11 | Counter64 |
| zxAnEthIfH15MOutPkts | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.12 | Counter64 |
| zxAnEthIfH15MOutUcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.13 | Counter64 |
| zxAnEthIfH15MOutMcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.14 | Counter64 |
| zxAnEthIfH15MOutBcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.15 | Counter64 |
| zxAnEthIfH15MOutPausePkts | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.16 | Counter64 |
| zxAnEthIfH15MOutDiscardPkt s | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.17 | Counter64 |
| zxAnEthIfH15MOutCollisions | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.18 | Counter64 |
| zxAnEthIfH15MSingleCollision s | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.19 | Counter64 |
| zxAnEthIfH15MMultipleCollisio ns | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.20 | Counter64 |
| zxAnEthIfH15MFcsErrors | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.21 | Counter64 |
| zxAnEthIfH15MAlignmentError s | 1.3.6.1.4.1.3902.1082.30.32.2.4. 1.22 | Counter64 |

#### 24-Hour History Performance

[Index description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| zxAnEthIfH1DIntervalNo | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.1 | Integer32 ( 1 .. 7 ) |
| zxAnEthIfH1DIntervalValidDat a | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.2 | TruthValue |
| zxAnEthIfH1DDateTime | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.3 | DateAndTime |
| zxAnEthIfH1DInOctets | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.4 | Counter64 |
| zxAnEthIfH1DInPkts | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.5 | Counter64 |
| zxAnEthIfH1DInUcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.6 | Counter64 |
| zxAnEthIfH1DInMcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.7 | Counter64 |
| zxAnEthIfH1DInBcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.8 | Counter64 |
| zxAnEthIfH1DInOversizedPkts | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.9 | Counter64 |
| zxAnEthIfH1DInUndersizedPkt s | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.10 | Counter64 |
| zxAnEthIfH1DOutOctets | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.11 | Counter64 |
| zxAnEthIfH1DOutPkts | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.12 | Counter64 |
| zxAnEthIfH1DOutUcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.13 | Counter64 |
| zxAnEthIfH1DOutMcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.14 | Counter64 |
| zxAnEthIfH1DOutBcastPkts | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.15 | Counter64 |

{ ifIndex }. Type 1 platform composite index. [MIB variable description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnEthIfH1DOutPausePkts | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.16 | Counter64 |
| zxAnEthIfH1DOutDiscardPkts | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.17 | Counter64 |
| zxAnEthIfH1DOutCollisions | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.18 | Counter64 |
| zxAnEthIfH1DSingleCollisions | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.19 | Counter64 |
| zxAnEthIfH1DMultipleCollision s | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.20 | Counter64 |
| zxAnEthIfH1DFcsErrors | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.21 | Counter64 |
| zxAnEthIfH1DAlignmentErrors | 1.3.6.1.4.1.3902.1082.30.32.2.5. 1.22 | Counter64 |

#### Performance Threshold Alarm Profile

[Index description]:

{ zxAnEthIfAlmProfileName, zxAnEthIfPerfVariable } [MIB variable description]:

zxAnEthIfAlmProfileEntry

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnEthIfAlmProfileName | 1.3.6.1.4.1.3902.1082.30.32.2.6. 1.1 | DisplayString ( SIZE ( 1 .. 32 ) ) |
| zxAnEthIfAlmProfileRowStatus | 1.3.6.1.4.1.3902.1082.30.32.2.8. 1.50 | RowStatus |

zxAnEthIfAlmProfileConfEntry

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnEthIfAlmProfileName | 1.3.6.1.4.1.3902.1082.30.32.2.6. 1.1 | DisplayString ( SIZE ( 1 .. 32 ) ) |
| zxAnEthIfPerfVariable | 1.3.6.1.4.1.3902.1082.30.32.2.6. 1.2 | OBJECT IDENTIFIER |
| zxAnEthIfRiseAlmThresh | 1.3.6.1.4.1.3902.1082.30.32.2.6. 1.3 | HCPerfCurrentCount |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnEthIfClrRiseAlmThre sh | 1.3.6.1.4.1.3902.1082.30.32.2.6. 1.4 | HCPerfCurrentCount |
| zxAnEthIfRiseWarnThresh | 1.3.6.1.4.1.3902.1082.30.32.2.6. 1.5 | HCPerfCurrentCount |
| zxAnEthIfClrRiseWarnThr esh | 1.3.6.1.4.1.3902.1082.30.32.2.6. 1.6 | HCPerfCurrentCount |
| zxAnEthIfFallWarnThresh | 1.3.6.1.4.1.3902.1082.30.32.2.6. 1.7 | HCPerfCurrentCount |
| zxAnEthIfClrFallWarnThre sh | 1.3.6.1.4.1.3902.1082.30.32.2.6. 1.8 | HCPerfCurrentCount |
| zxAnEthIfFallAlmThresh | 1.3.6.1.4.1.3902.1082.30.32.2.6. 1.9 | HCPerfCurrentCount |
| zxAnEthIfClrFallAlmThres h | 1.3.6.1.4.1.3902.1082.30.32.2.6. 1.10 | HCPerfCurrentCount |
| zxAnEthIfThreshUsed | 1.3.6.1.4.1.3902.1082.30.32.2.6. 1.11 | BITS { riseAlmThresh ( 0 ) , riseAlmClrThresh ( 1 ) , riseWarnThresh ( 2 ) , riseWarnClrThresh ( 3 ) , fallWarnThresh ( 4 ) , fallWarnClrThresh ( 5 ) , fallAlmThresh ( 6 ) , fallAlmClrThresh ( 7 ) } |
| zxAnEthIfAlmProfileConfR owStatus | 1.3.6.1.4.1.3902.1082.30.32.2.6. 1.12 | RowStatus |

#### Performance Alarm Profile Loading

[Index description]:

{ ifIndex }. Type 1 platform composite index. [MIB variable description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| zxAnEthIfAlmPrf | 1.3.6.1.4.1.3902.1082.30.32.2.7. 1.1 | DisplayString ( SIZE ( 1 .. 32 ) ) |
| zxAnEthIfAlmPrfApplyRowStat us | 1.3.6.1.4.1.3902.1082.30.32.2.7. 1.50 | RowStatus |

## VPORT Performance

#### Startup and Stop

[MIB file]:

| MIB Variable | OID | MIB Value | Description |
| --- | --- | --- | --- |
| zxAnGponVportCurr | . | RowStatus |  |
| RowStatus | 1.3.6.1.4.1.3902.1082. |  |  |
|  | 500.10.2.3.12.2.1.101 |  |  |

Please refer to zxAnGponSrvVportCurrPerfTable defined in ZTE-AN-GPON-SERVICE- MIB.mib.

#### Realtime Performance

[Index description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| zxAnSubIfIndex | 1.3.6.1.4.1.3902.1082.30.5.5.1.1.2 | Second-level index |
| zxAnBrgIfInOctets | 1.3.6.1.4.1.3902.1082.30.31.2.3.1.1 | Counter64 |
| zxAnBrgIfInUcastPkts | 1.3.6.1.4.1.3902.1082.30.31.2.3.1.2 | Counter64 |
| zxAnBrgIfInMcastPkts | 1.3.6.1.4.1.3902.1082.30.31.2.3.1.3 | Counter64 |
| zxAnBrgIfInBcastPkts | 1.3.6.1.4.1.3902.1082.30.31.2.3.1.4 | Counter64 |

{ ifIndex }. Type 4 or type 10 platform composite index. [MIB variable description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnBrgIfOutOctets | 1.3.6.1.4.1.3902.1082.30.31.2.3.1.5 | Counter64 |
| zxAnBrgIfOutUcastPkts | 1.3.6.1.4.1.3902.1082.30.31.2.3.1.6 | Counter64 |
| zxAnBrgIfOutMcastPkt s | 1.3.6.1.4.1.3902.1082.30.31.2.3.1.7 | Counter64 |
| zxAnBrgIfOutBcastPkts | 1.3.6.1.4.1.3902.1082.30.31.2.3.1.8 | Counter64 |
| zxAnBrgIfReset | 1.3.6.1.4.1.3902.1082.30.31.2.3.1.100 | INTEGER { perfReset ( 1 ) } |

[MIB description]:

Please refer to ZTE-AN-INTERFACE-STATS-MIB.mib.

zxAnBrgIfCurrStatsTable	OBJECT-MIB value

SYNTAX	SEQUENCE OF ZxAnBrgIfCurrStatsEntry MAX-ACCESS not-accessible

STATUS	current Description

"Current performance data table of bridge interfaces."

::= { zxAnIfPerfObjects 3 }

zxAnBrgIfCurrStatsEntry	OBJECT-MIB value SYNTAX	ZxAnBrgIfCurrStatsEntry

MAX-ACCESS not-accessible STATUS	current Description

"Current performance data entry of bridge interfaces."

INDEX  { ifIndex}

::= { zxAnBrgIfCurrStatsTable 1 }

ZxAnBrgIfCurrStatsEntry ::= SEQUENCE { zxAnBrgIfInOctets		Counter64, zxAnBrgIfInUcastPkts				 Counter64, zxAnBrgIfInMcastPkts					Counter64, zxAnBrgIfInBcastPkts				Counter64, zxAnBrgIfOutOctets			Counter64, zxAnBrgIfOutUcastPkts						 Counter64, zxAnBrgIfOutMcastPkts							Counter64, zxAnBrgIfOutBcastPkts						Counter64, zxAnBrgIfReset	INTEGER

}

#### 15-Minute Realtime Performance

[Index description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| zxAnSubIfIndex | 1.3.6.1.4.1.3902.1082.30.5.5.1.1.2 | 二级索引 |
| zxAnBrgIfC15MTimeElapse d | 1.3.6.1.4.1.3902.1082.30.33.2.2.1. 1 | Gauge32 ( 0 .. 899 ) |
| zxAnBrgIfC15MInOctets | 1.3.6.1.4.1.3902.1082.30.33.2.2.1. 2 | Counter64 |

{ ifIndex }. Type 4 or type 10 platform composite index. [MIB variable description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnBrgIfC15MInUcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.2.1. 3 | Counter64 |
| zxAnBrgIfC15MInMcastPkt s | 1.3.6.1.4.1.3902.1082.30.33.2.2.1. 4 | Counter64 |
| zxAnBrgIfC15MInBcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.2.1. 5 | Counter64 |
| zxAnBrgIfC15MOutOctets | 1.3.6.1.4.1.3902.1082.30.33.2.2.1. 6 | Counter64 |
| zxAnBrgIfC15MOutUcastPk ts | 1.3.6.1.4.1.3902.1082.30.33.2.2.1. 7 | Counter64 |
| zxAnBrgIfC15MOutMcastP kts | 1.3.6.1.4.1.3902.1082.30.33.2.2.1. 8 | Counter64 |
| zxAnBrgIfC15MOutBcastPk ts | 1.3.6.1.4.1.3902.1082.30.33.2.2.1. 9 | Counter64 |
| zxAnBrgIfC15MReset | 1.3.6.1.4.1.3902.1082.30.33.2.2.1. 100 | INTEGER { perfReset ( 1 ) } |

[MIB description]:

Refer to ZTE-AN-INTERFACE-BRG-PERF-MIB.mib.

zxAnBrgIfCurr15MinPerfTable	OBJECT-MIB value

SYNTAX	SEQUENCE OF ZxAnBrgIfCurr15MinPerfEntry MAX-ACCESS not-accessible

STATUS	current Description

"The 15 minutes interval performance data table of the bridge interfaces."

::= { zxAnBrgIfPerfObjects 2 }

zxAnBrgIfCurr15MinPerfEntry	OBJECT-Type

SYNTAX	ZxAnBrgIfCurr15MinPerfEntry MAX-ACCESS not-accessible

STATUS	current Description

"The 15 minutes interval performance data entry of the bridge interfaces."

INDEX  { ifIndex}

::= { zxAnBrgIfCurr15MinPerfTable 1 }

ZxAnBrgIfCurr15MinPerfEntry ::= SEQUENCE { zxAnBrgIfC15MTimeElapsed						 Gauge32, zxAnBrgIfC15MInOctets		Counter64, zxAnBrgIfC15MInUcastPkts				 Counter64, zxAnBrgIfC15MInMcastPkts					Counter64, zxAnBrgIfC15MInBcastPkts				Counter64, zxAnBrgIfC15MOutOctets			Counter64, zxAnBrgIfC15MOutUcastPkts						 Counter64, zxAnBrgIfC15MOutMcastPkts							Counter64, zxAnBrgIfC15MOutBcastPkts						Counter64, zxAnBrgIfC15MReset	INTEGER

}

#### 24-Hour Realtime Performance

[Index description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| zxAnSubIfIndex | 1.3.6.1.4.1.3902.1082.30.5.5.1.1.2 | Second-level index |
| zxAnBrgIfC1DTimeElapsed | 1.3.6.1.4.1.3902.1082.30.33.2.3.1. 1 | Gauge32 ( 0 .. 86399 ) |
| zxAnBrgIfC1DInOctets | 1.3.6.1.4.1.3902.1082.30.33.2.3.1. 2 | Counter64 |
| zxAnBrgIfC1DInUcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.3.1. 3 | Counter64 |
| zxAnBrgIfC1DInMcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.3.1. 4 | Counter64 |
| zxAnBrgIfC1DInBcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.3.1. 5 | Counter64 |
| zxAnBrgIfC1DOutOctets | 1.3.6.1.4.1.3902.1082.30.33.2.3.1. 6 | Counter64 |
| zxAnBrgIfC1DOutUcastPkt s | 1.3.6.1.4.1.3902.1082.30.33.2.3.1. 7 | Counter64 |
| zxAnBrgIfC1DOutMcastPkt s | 1.3.6.1.4.1.3902.1082.30.33.2.3.1. 8 | Counter64 |
| zxAnBrgIfC1DOutBcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.3.1. 9 | Counter64 |
| zxAnBrgIfC1DReset | 1.3.6.1.4.1.3902.1082.30.33.2.3.1. 100 | INTEGER { perfReset ( 1 ) } |

{ ifIndex }. Type 4 or type 10 platform composite index. [MIB variable description]:

MIB description：

Refer to ZTE-AN-INTERFACE-BRG-PERF-MIB.mib.

zxAnBrgIfCurr1DayPerfTable	OBJECT-MIB value

SYNTAX	SEQUENCE OF ZxAnBrgIfCurr1DayPerfEntry MAX-ACCESS not-accessible

STATUS	current

Description

"The 1 day interval performance data table of the bridge interfaces."

::= { zxAnBrgIfPerfObjects 3 }

zxAnBrgIfCurr1DayPerfEntry	OBJECT-Type SYNTAX	ZxAnBrgIfCurr1DayPerfEntry MAX-ACCESS not-accessible

STATUS	current Description

"The 1 day interval performance data entry of the bridge interfaces."

INDEX  { ifIndex}

::= { zxAnBrgIfCurr1DayPerfTable 1 }

ZxAnBrgIfCurr1DayPerfEntry ::= SEQUENCE { zxAnBrgIfC1DTimeElapsed					Gauge32, zxAnBrgIfC1DInOctets	Counter64, zxAnBrgIfC1DInUcastPkts			 Counter64, zxAnBrgIfC1DInMcastPkts				Counter64, zxAnBrgIfC1DInBcastPkts			Counter64, zxAnBrgIfC1DOutOctets		Counter64, zxAnBrgIfC1DOutUcastPkts					Counter64,

zxAnBrgIfC1DOutMcastPkts			Counter64, zxAnBrgIfC1DOutBcastPkts		Counter64, zxAnBrgIfC1DReset	INTEGER

}

#### 15-Minute History Performance

[Index description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| zxAnSubIfIndex | 1.3.6.1.4.1.3902.1082.30.5.5.1.1 .2 | Second-level index |
| zxAnBrgIfH15MIntervalNo | 1.3.6.1.4.1.3902.1082.30.33.2.4. 1.1 | Third-level index Integer32 ( 1 .. 96 ) |
| zxAnBrgIfH15MIntervalValidD ata | 1.3.6.1.4.1.3902.1082.30.33.2.4. 1.2 | TruthValue |
| zxAnBrgIfH15MDateTime | 1.3.6.1.4.1.3902.1082.30.33.2.4. 1.3 | DateAndTime |
| zxAnBrgIfH15MInOctets | 1.3.6.1.4.1.3902.1082.30.33.2.4. 1.4 | Counter64 |
| zxAnBrgIfH15MInUcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.4. 1.5 | Counter64 |
| zxAnBrgIfH15MInMcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.4. 1.6 | Counter64 |
| zxAnBrgIfH15MInBcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.4. 1.7 | Counter64 |
| zxAnBrgIfH15MOutOctets | 1.3.6.1.4.1.3902.1082.30.33.2.4. 1.8 | Counter64 |
| zxAnBrgIfH15MOutUcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.4. 1.9 | Counter64 |
| zxAnBrgIfH15MOutMcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.4. 1.10 | Counter64 |

{ ifIndex }. Type 4 or type 10 platform composite index. [MIB variable description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnBrgIfH15MOutBcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.4. 1.11 | Counter64 |

[MIB description]:

Refer to ZTE-AN-INTERFACE-BRG-PERF-MIB.mib.

zxAnBrgIfHis15MinPerfTable	OBJECT-MIB value

SYNTAX	SEQUENCE OF ZxAnBrgIfHis15MinPerfEntry MAX-ACCESS not-accessible

STATUS	current Description

"The 15 minutes interval history performance data table of the bridge interfaces."

::= { zxAnBrgIfPerfObjects 4 }

zxAnBrgIfHis15MinPerfEntry	OBJECT-MIB value SYNTAX	ZxAnBrgIfHis15MinPerfEntry

MAX-ACCESS not-accessible STATUS	current Description

"The 15 minutes interval performance data entry of the bridge interfaces."

INDEX { ifIndex,zxAnBrgIfH15MIntervalNo }

::= { zxAnBrgIfHis15MinPerfTable 1 }

ZxAnBrgIfHis15MinPerfEntry ::= SEQUENCE { zxAnBrgIfH15MIntervalNo		Integer32, zxAnBrgIfH15MIntervalValidData							TruthValue, zxAnBrgIfH15MDateTime			 DateAndTime, zxAnBrgIfH15MInOctets	Counter64, zxAnBrgIfH15MInUcastPkts				 Counter64, zxAnBrgIfH15MInMcastPkts					Counter64, zxAnBrgIfH15MInBcastPkts				Counter64, zxAnBrgIfH15MOutOctets			Counter64, zxAnBrgIfH15MOutUcastPkts						 Counter64, zxAnBrgIfH15MOutMcastPkts							 Counter64, zxAnBrgIfH15MOutBcastPkts						Counter64

}

#### 24-Hour History Performance

[Index description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| zxAnSubIfIndex | 1.3.6.1.4.1.3902.1082.30.5.5.1.1 .2 | Second-level index |
| zxAnBrgIfH1DIntervalNo | 1.3.6.1.4.1.3902.1082.30.33.2.5. 1.1 | Third-level index Integer32 ( 1 .. 7 ) |

{ ifIndex }. Type 4 or type 10 platform composite index. [MIB variable description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnBrgIfH1DIntervalValidDat a | 1.3.6.1.4.1.3902.1082.30.33.2.5. 1.2 | TruthValue |
| zxAnBrgIfH1DDateTime | 1.3.6.1.4.1.3902.1082.30.33.2.5. 1.3 | DateAndTime |
| zxAnBrgIfH1DInOctets | 1.3.6.1.4.1.3902.1082.30.33.2.5. 1.4 | Counter64 |
| zxAnBrgIfH1DInUcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.5. 1.5 | Counter64 |
| zxAnBrgIfH1DInMcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.5. 1.6 | Counter64 |
| zxAnBrgIfH1DInBcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.5. 1.7 | Counter64 |
| zxAnBrgIfH1DOutOctets | 1.3.6.1.4.1.3902.1082.30.33.2.5. 1.8 | Counter64 |
| zxAnBrgIfH1DOutUcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.5. 1.9 | Counter64 |
| zxAnBrgIfH1DOutMcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.5. 1.10 | Counter64 |
| zxAnBrgIfH1DOutBcastPkts | 1.3.6.1.4.1.3902.1082.30.33.2.5. 1.11 | Counter64 |

[MIB description]:

Refer to ZTE-AN-INTERFACE-BRG-PERF-MIB.mib.

zxAnBrgIfHis1DayPerfTable	OBJECT-MIB value

SYNTAX	SEQUENCE OF ZxAnBrgIfHis1DayPerfEntry MAX-ACCESS not-accessible

STATUS	current Description

"The 1 day interval history performance data table of the bridge interfaces."

::= { zxAnBrgIfPerfObjects 5 }

zxAnBrgIfHis1DayPerfEntry	OBJECT-MIB value SYNTAX	ZxAnBrgIfHis1DayPerfEntry

MAX-ACCESS not-accessible STATUS	current Description

"The 1 day interval performance data entry of the bridge interfaces."

INDEX { ifIndex,zxAnBrgIfH1DIntervalNo }

::= { zxAnBrgIfHis1DayPerfTable 1 }

ZxAnBrgIfHis1DayPerfEntry ::= SEQUENCE { zxAnBrgIfH1DIntervalNo		Integer32, zxAnBrgIfH1DIntervalValidData						 TruthValue, zxAnBrgIfH1DDateTime			 DateAndTime, zxAnBrgIfH1DInOctets	Counter64, zxAnBrgIfH1DInUcastPkts				 Counter64, zxAnBrgIfH1DInMcastPkts					Counter64, zxAnBrgIfH1DInBcastPkts				Counter64, zxAnBrgIfH1DOutOctets			Counter64, zxAnBrgIfH1DOutUcastPkts						Counter64, zxAnBrgIfH1DOutMcastPkts						 Counter64,

zxAnBrgIfH1DOutBcastPkts	Counter64

}

#### Performance Threshold Alarm Profile

[Index description]:

{ zxAnBrgIfAlmProfileName }, first-level index.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnBrgIfAlmProfileName | 1.3.6.1.4.1.3902.1082.30.33.2.6.1 | First-level index |
|  | .1 | DisplayString |
|  |  | ( SIZE ( 1 .. 32 ) ) |
| zxAnBrgIfPerfVariable | 1.3.6.1.4.1.3902.1082.30.33.2.6.1 | Second-level index |
|  | .2 | OBJECT |
|  |  | IDENTIFIER |
| zxAnBrgIfRiseAlmThresh | 1.3.6.1.4.1.3902.1082.30.33.2.6.1 | HCPerfCurrentCou |
|  | .3 | nt |
| zxAnBrgIfClrRiseAlmThre | 1.3.6.1.4.1.3902.1082.30.33.2.6.1 | HCPerfCurrentCou |
| sh | .4 | nt |
| zxAnBrgIfRiseWarnThres | 1.3.6.1.4.1.3902.1082.30.33.2.6.1 | HCPerfCurrentCou |
| h | .5 | nt |
| zxAnBrgIfClrRiseWarnThr | 1.3.6.1.4.1.3902.1082.30.33.2.6.1 | HCPerfCurrentCou |
| esh | .6 | nt |
| zxAnBrgIfFallWarnThresh | 1.3.6.1.4.1.3902.1082.30.33.2.6.1 | HCPerfCurrentCou |
|  | .7 | nt |
| zxAnBrgIfClrFallWarnThre | 1.3.6.1.4.1.3902.1082.30.33.2.6.1 | HCPerfCurrentCou |
| sh | .8 | nt |
| zxAnBrgIfFallAlmThresh | 1.3.6.1.4.1.3902.1082.30.33.2.6.1 | HCPerfCurrentCou |
|  | .9 | nt |
| zxAnBrgIfClrFallAlmThres | 1.3.6.1.4.1.3902.1082.30.33.2.6.1 | HCPerfCurrentCou |
| h | .10 | nt |
| zxAnBrgIfThreshUsed | 1.3.6.1.4.1.3902.1082.30.33.2.6.1 | BITS |
|  | .11 | { riseAlmThresh |
|  |  | ( 0 ) , |
|  |  | riseAlmClrThresh |

{ zxAnBrgIfPerfVariable }, second-level index. [MIB variable description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
|  |  | ( 1 ) , |
|  |  | riseWarnThresh |
|  |  | ( 2 ) , |
|  |  | riseWarnClrThresh |
|  |  | ( 3 ) , |
|  |  | fallWarnThresh ( 4 ) |
|  |  | , fallWarnClrThresh |
|  |  | ( 5 ) , fallAlmThresh |
|  |  | ( 6 ) , |
|  |  | fallAlmClrThresh ( 7 |
|  |  | ) } |
| zxAnBrgIfAlmProfileConfR | 1.3.6.1.4.1.3902.1082.30.33.2.6.1 | RowStatus |
| owStatus | .50 |  |

[MIB description]:

Refer to ZTE-AN-INTERFACE-BRG-PERF-MIB.mib.

zxAnBrgIfAlmProfileTable OBJECT-MIB value

SYNTAX	SEQUENCE OF ZxAnBrgIfAlmProfileEntry MAX-ACCESS not-accessible

STATUS	current Description

"The performance alarm threshold profile table of the bridge interfaces. It can be used to delete all of the performance alarm threshold profiles which has the same name configured in ‘zxAnBrgIfAlmProfileConfTable’."

::= { zxAnBrgIfPerfObjects 8 }

zxAnBrgIfAlmProfileEntry OBJECT-Type

SYNTAX	 ZxAnBrgIfAlmProfileEntry MAX-ACCESS not-accessible STATUS	current

Description

"The performance alarm threshold profile entry of the bridge interfaces."

INDEX  { zxAnBrgIfAlmProfileName}

::= { zxAnBrgIfAlmProfileTable 1 }

ZxAnBrgIfAlmProfileEntry ::= SEQUENCE { zxAnBrgIfAlmProfileRowStatus RowStatus

}

zxAnBrgIfAlmProfileConfTable OBJECT-MIB value

SYNTAX	SEQUENCE OF ZxAnBrgIfAlmProfileConfEntry MAX-ACCESS not-accessible

STATUS	current Description

"The performance alarm threshold profile table of the bridge interfaces."

::= { zxAnBrgIfPerfObjects 6 }

zxAnBrgIfAlmProfileConfEntry OBJECT-MIB value SYNTAX	ZxAnBrgIfAlmProfileConfEntry

MAX-ACCESS not-accessible STATUS	current Description

"The performance alarm threshold profile table of the bridge interfaces.These variables in the profile configuration is optional, if not configured, it means the correspongding alarm is not reported, but the alarm and clear alarm threshold configuration is needed to bind. "

INDEX { zxAnBrgIfAlmProfileName, zxAnBrgIfPerfVariable }

::= { zxAnBrgIfAlmProfileConfTable 1 }

ZxAnBrgIfAlmProfileConfEntry ::= SEQUENCE { zxAnBrgIfAlmProfileName				 DisplayString, zxAnBrgIfPerfVariable	OBJECT IDENTIFIER, zxAnBrgIfRiseAlmThresh				HCPerfCurrentCount, zxAnBrgIfClrRiseAlmThresh					 HCPerfCurrentCount, zxAnBrgIfRiseWarnThresh					HCPerfCurrentCount, zxAnBrgIfClrRiseWarnThresh				HCPerfCurrentCount, zxAnBrgIfFallWarnThresh				HCPerfCurrentCount, zxAnBrgIfClrFallWarnThresh					 HCPerfCurrentCount, zxAnBrgIfFallAlmThresh			HCPerfCurrentCount, zxAnBrgIfClrFallAlmThresh				 HCPerfCurrentCount, zxAnBrgIfThreshUsed		BITS,

zxAnBrgIfAlmProfileConfRowStatus RowStatus

}

#### Performance Alarm Profile Loading

[Index description]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| ifIndex | 1.3.6.1.2.1.2.2.1.1 | Type 1 platform composite index |
| zxAnSubIfIndex | 1.3.6.1.4.1.3902.1082.30.5.5.1.1 .2 | Second-level index |
| zxAnBrgIfAlmPrf | 1.3.6.1.4.1.3902.1082.30.33.2.7. 1.1 | DisplayString ( SIZE ( 1 .. 32 ) ) |
| zxAnBrgIfAlmPrfApplyRowStat us | 1.3.6.1.4.1.3902.1082.30.33.2.7. 1.50 | RowStatus |

{ ifIndex }. Type 4 or type 10 platform composite index. [MIB variable description]:

[MIB description]:

Refer to ZTE-AN-INTERFACE-BRG-PERF-MIB.mib.

zxAnBrgIfAlmProfileApplyTable OBJECT-MIB value

SYNTAX	SEQUENCE OF ZxAnBrgIfAlmProfileApplyEntry MAX-ACCESS not-accessible

STATUS	current Description

"The performance alarm threshold profile configuration table of the bridge interfaces."

::= { zxAnBrgIfPerfObjects 7 }

zxAnBrgIfAlmProfileApplyEntry OBJECT-MIB value SYNTAX	ZxAnBrgIfAlmProfileApplyEntry

MAX-ACCESS not-accessible STATUS	current Description

"The performance alarm threshold profile configuration entry of the bridge interfaces."

INDEX { ifIndex }

::= { zxAnBrgIfAlmProfileApplyTable 1 }

ZxAnBrgIfAlmProfileApplyEntry ::= SEQUENCE { zxAnBrgIfAlmPrf	DisplayString, zxAnBrgIfAlmPrfApplyRowStatus	RowStatus

}

## OLT Ethernet Performance

This function is enabled by default.

#### Realtime Performance

[MIB file]:

Please refer to zxAnPonOltIfCurrPerfTable defined in ZTE-AN-PON-PERF-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOltIfRxOctet | . |  |
| s | 1.3.6.1.4.1.3902.1082.500 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | .4.2.1.2.1.1 |  |
| zxAnPonOltIfRxPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.2 |  |
| zxAnPonOltIfRxOctet Rate | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.3 |  |
| zxAnPonOltIfRxPktR ate | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.4 |  |
| zxAnPonOltIfRxOctet PeakRate | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.5 |  |
| zxAnPonOltIfRxPktP eakRate | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.6 |  |
| zxAnPonOltIfRxUcas tPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.7 |  |
| zxAnPonOltIfRxNUc astPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.8 |  |
| zxAnPonOltIfRxMcas tPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.9 |  |
| zxAnPonOltIfRxBcas tPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.10 |  |
| zxAnPonOltIfRxDisc ardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.11 |  |
| zxAnPonOltIfRxErrP kts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.12 |  |
| zxAnPonOltIfRxPdr | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.13 |  |
| zxAnPonOltIfRxPer | . 1.3.6.1.4.1.3902.1082.500 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | .4.2.1.2.1.14 |  |
| zxAnPonOltIfRxCrcAl ignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.15 |  |
| zxAnPonOltIfRxUnde rsizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.16 |  |
| zxAnPonOltIfRxOver sizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.17 |  |
| zxAnPonOltIfRxPkts 64Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.18 |  |
| zxAnPonOltIfRxPkts 65To127Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.19 |  |
| zxAnPonOltIfRxPkts 128To255Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.20 |  |
| zxAnPonOltIfRxPkts 256To511Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.21 |  |
| zxAnPonOltIfRxPkts 512To1023Oct | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.22 |  |
| zxAnPonOltIfRxPkts 1024To1518Oct | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.23 |  |
| zxAnPonOltIfRxBwUt ilization | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.24 |  |
| zxAnPonOltIfRxAvgB wUtilization | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.25 |  |
| zxAnPonOltIfRxFrag ments | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.26 |  |
| zxAnPonOltIfRxJabb ers | . 1.3.6.1.4.1.3902.1082.500 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | .4.2.1.2.1.27 |  |
| zxAnPonOltIfTxOctet s | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.44 |  |
| zxAnPonOltIfTxPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.45 |  |
| zxAnPonOltIfTxOctet Rate | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.46 |  |
| zxAnPonOltIfTxPktR ate | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.47 |  |
| zxAnPonOltIfTxOctet PeakRate | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.48 |  |
| zxAnPonOltIfTxPktP eakRate | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.49 |  |
| zxAnPonOltIfTxUcast Pkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.50 |  |
| zxAnPonOltIfTxNUca stPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.51 |  |
| zxAnPonOltIfTxMcas tPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.52 |  |
| zxAnPonOltIfTxBcast Pkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.53 |  |
| zxAnPonOltIfTxDisca rdPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.54 |  |
| zxAnPonOltIfTxErrPk ts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.55 |  |
| zxAnPonOltIfTxPdr | . 1.3.6.1.4.1.3902.1082.500 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | .4.2.1.2.1.56 |  |
| zxAnPonOltIfTxPer | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.57 |  |
| zxAnPonOltIfTxCrcAl ignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.58 |  |
| zxAnPonOltIfTxUnde rsizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.59 |  |
| zxAnPonOltIfTxOver sizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.60 |  |
| zxAnPonOltIfTxPkts6 4Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.61 |  |
| zxAnPonOltIfTxPkts6 5To127Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.62 |  |
| zxAnPonOltIfTxPkts1 28To255Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.63 |  |
| zxAnPonOltIfTxPkts2 56To511Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.64 |  |
| zxAnPonOltIfTxPkts5 12To1023Oct | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.65 |  |
| zxAnPonOltIfTxPkts1 024To1518Oct | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.66 |  |
| zxAnPonOltIfTxBwUt ilization | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.67 |  |
| zxAnPonOltIfTxAvgB wUtilization | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.68 |  |
| zxAnPonOltIfTxFrag ments | . 1.3.6.1.4.1.3902.1082.500 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | .4.2.1.2.1.69 |  |
| zxAnPonOltIfTxJabb ers | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.70 |  |
| zxAnPonOltIfCollisio ns | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.88 |  |
| zxAnPonOltIfDropEv ents | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.89 |  |
| zxAnPonOltIfCurrPer fCapability | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.99 |  |
| zxAnPonOltIfCurrPer fReset | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.2.1.100 |  |

#### 15-Minute Realtime Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOltIfC15MTi meElapsed | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.1 |  |
| zxAnPonOltIfC15MR xOctets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.2 |  |
| zxAnPonOltIfC15MR xPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.3 |  |
| zxAnPonOltIfC15MR xUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.4 |  |
| zxAnPonOltIfC15MR xNUcastPkts | . 1.3.6.1.4.1.3902.1082.500 |  |

Please refer to zxAnPonOltIfCurr15MinPerfTable defined in ZTE-AN-PON-PERF- MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | .4.2.1.3.1.5 |  |
| zxAnPonOltIfC15MR xMcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.6 |  |
| zxAnPonOltIfC15MR xBcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.7 |  |
| zxAnPonOltIfC15MR xDiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.8 |  |
| zxAnPonOltIfC15MR xErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.9 |  |
| zxAnPonOltIfC15MR xCrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.10 |  |
| zxAnPonOltIfC15MR xUndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.11 |  |
| zxAnPonOltIfC15MR xOversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.12 |  |
| zxAnPonOltIfC15MR x64Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.13 |  |
| zxAnPonOltIfC15MR x65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.14 |  |
| zxAnPonOltIfC15MR x128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.15 |  |
| zxAnPonOltIfC15MR x256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.16 |  |
| zxAnPonOltIfC15MR x512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.17 |  |
| zxAnPonOltIfC15MR x1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | .4.2.1.3.1.18 |  |
| zxAnPonOltIfC15MR xFragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.19 |  |
| zxAnPonOltIfC15MR xJabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.20 |  |
| zxAnPonOltIfC15MT xOctets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.36 |  |
| zxAnPonOltIfC15MT xPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.37 |  |
| zxAnPonOltIfC15MT xUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.38 |  |
| zxAnPonOltIfC15MT xNUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.39 |  |
| zxAnPonOltIfC15MT xMcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.40 |  |
| zxAnPonOltIfC15MT xBcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.41 |  |
| zxAnPonOltIfC15MT xDiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.42 |  |
| zxAnPonOltIfC15MT xErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.43 |  |
| zxAnPonOltIfC15MT xCrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.44 |  |
| zxAnPonOltIfC15MT xUndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.45 |  |
| zxAnPonOltIfC15MT xOversizePkts | . 1.3.6.1.4.1.3902.1082.500 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
|  | .4.2.1.3.1.46 |  |
| zxAnPonOltIfC15MT xPkts64Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.47 |  |
| zxAnPonOltIfC15MT x65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.48 |  |
| zxAnPonOltIfC15MT x128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.49 |  |
| zxAnPonOltIfC15MT x256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.50 |  |
| zxAnPonOltIfC15MT x512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.51 |  |
| zxAnPonOltIfC15MT x1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.52 |  |
| zxAnPonOltIfC15MT xFragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.53 |  |
| zxAnPonOltIfC15MT xJabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.54 |  |
| zxAnPonOltIfC15MC ollisions | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.70 |  |
| zxAnPonOltIfC15MD ropEvents | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.71 |  |
| zxAnPonOltIfPerfCap ability | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.99 |  |
| zxAnPonOltIfC15MP erfReset | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.3.1.100 |  |

#### 24-Hour Realtime Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOltIfC1DTim eElapsed | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.1 |  |
| zxAnPonOltIfC1DRx Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.2 |  |
| zxAnPonOltIfC1DRx Pkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.3 |  |
| zxAnPonOltIfC1DRx UcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.4 |  |
| zxAnPonOltIfC1DRx NUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.5 |  |
| zxAnPonOltIfC1DRx McastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.6 |  |
| zxAnPonOltIfC1DRx BcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.7 |  |
| zxAnPonOltIfC1DRx DiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.8 |  |
| zxAnPonOltIfC1DRx ErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.9 |  |
| zxAnPonOltIfC1DRx CrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.10 |  |
| zxAnPonOltIfC1DRx UndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.11 |  |

Please refer to zxAnPonOltIfCurr1DayPerfTable defined in ZTE-AN-PON-PERF- MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOltIfC1DRx OversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.12 |  |
| zxAnPonOltIfC1DRx 64Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.13 |  |
| zxAnPonOltIfC1DRx 65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.14 |  |
| zxAnPonOltIfC1DRx 128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.15 |  |
| zxAnPonOltIfC1DRx 256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.16 |  |
| zxAnPonOltIfC1DRx 512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.17 |  |
| zxAnPonOltIfC1DRx 1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.18 |  |
| zxAnPonOltIfC1DRx Fragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.19 |  |
| zxAnPonOltIfC1DRx Jabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.20 |  |
| zxAnPonOltIfC1DTx Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.36 |  |
| zxAnPonOltIfC1DTx Pkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.37 |  |
| zxAnPonOltIfC1DTx UcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.38 |  |
| zxAnPonOltIfC1DTx NUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.39 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOltIfC1DTx McastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.40 |  |
| zxAnPonOltIfC1DTx BcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.41 |  |
| zxAnPonOltIfC1DTx DiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.42 |  |
| zxAnPonOltIfC1DTx ErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.43 |  |
| zxAnPonOltIfC1DTx CrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.44 |  |
| zxAnPonOltIfC1DTx UndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.45 |  |
| zxAnPonOltIfC1DTx OversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.46 |  |
| zxAnPonOltIfC1DTx6 4Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.47 |  |
| zxAnPonOltIfC1DTx6 5To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.48 |  |
| zxAnPonOltIfC1DTx1 28To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.49 |  |
| zxAnPonOltIfC1DTx2 56To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.50 |  |
| zxAnPonOltIfC1DTx5 12To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.51 |  |
| zxAnPonOltIfC1DTx1 024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.52 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOltIfC1DTx Fragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.53 |  |
| zxAnPonOltIfC1DTxJ abbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.54 |  |
| zxAnPonOltIfC1DCol lisions | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.70 |  |
| zxAnPonOltIfC1DDro pEvents | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.71 |  |
| zxAnPonOltIfC1DPer fReset | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.4.1.100 |  |

#### 15-Minute History Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOltIfH15MIn tervalNo | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.1 |  |
| zxAnPonOltIfH15MV alidData | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.2 |  |
| zxAnPonOltIfH15MD ateAndTime | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.3 |  |
| zxAnPonOltIfH15MR xOctets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.4 |  |
| zxAnPonOltIfH15MR xPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.5 |  |

Please refer to zxAnPonOltIfHis15MinPerfTable defined in ZTE-AN-PON-PERF- MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOltIfH15MR xUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.6 |  |
| zxAnPonOltIfH15MR xNUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.7 |  |
| zxAnPonOltIfH15MR xMcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.8 |  |
| zxAnPonOltIfH15MR xBcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.9 |  |
| zxAnPonOltIfH15MR xDiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.10 |  |
| zxAnPonOltIfH15MR xErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.11 |  |
| zxAnPonOltIfH15MR xCrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.12 |  |
| zxAnPonOltIfH15MR xUndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.13 |  |
| zxAnPonOltIfH15MR xOversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.14 |  |
| zxAnPonOltIfH15MR x64Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.15 |  |
| zxAnPonOltIfH15MR x65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.16 |  |
| zxAnPonOltIfH15MR x128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.17 |  |
| zxAnPonOltIfH15MR x256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.18 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOltIfH15MR x512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.19 |  |
| zxAnPonOltIfH15MR x1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.20 |  |
| zxAnPonOltIfH15MR xFragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.21 |  |
| zxAnPonOltIfH15MR xJabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.22 |  |
| zxAnPonOltIfH15MT xOctets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.36 |  |
| zxAnPonOltIfH15MT xPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.37 |  |
| zxAnPonOltIfH15MT xUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.38 |  |
| zxAnPonOltIfH15MT xNUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.39 |  |
| zxAnPonOltIfH15MT xMcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.40 |  |
| zxAnPonOltIfH15MT xBcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.41 |  |
| zxAnPonOltIfH15MT xDiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.42 |  |
| zxAnPonOltIfH15MT xErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.43 |  |
| zxAnPonOltIfH15MT xCrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.44 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOltIfH15MT xUndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.45 |  |
| zxAnPonOltIfH15MT xOversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.46 |  |
| zxAnPonOltIfH15MT xPkts64Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.47 |  |
| zxAnPonOltIfH15MT x65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.48 |  |
| zxAnPonOltIfH15MT x128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.49 |  |
| zxAnPonOltIfH15MT x256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.50 |  |
| zxAnPonOltIfH15MT x512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.51 |  |
| zxAnPonOltIfH15MT x1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.52 |  |
| zxAnPonOltIfH15MT xFragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.53 |  |
| zxAnPonOltIfH15MT xJabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.54 |  |
| zxAnPonOltIfH15MC ollisions | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.70 |  |
| zxAnPonOltIfH15MD ropEvents | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.5.1.71 |  |

#### 24-Hour History Performance

[MIB file]:

Please refer to zxAnPonOltIfHis1DayPerfTable defined in ZTE-AN-PON-PERF-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOltIfH1DInte rvalNo | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.1 |  |
| zxAnPonOltIfH1DVali dData | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.2 |  |
| zxAnPonOltIfH1DDat eAndTime | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.3 |  |
| zxAnPonOltIfH1DRx Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.4 |  |
| zxAnPonOltIfH1DRx Pkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.5 |  |
| zxAnPonOltIfH1DRx UcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.6 |  |
| zxAnPonOltIfH1DRx NUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.7 |  |
| zxAnPonOltIfH1DRx McastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.8 |  |
| zxAnPonOltIfH1DRx BcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.9 |  |
| zxAnPonOltIfH1DRx DiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.10 |  |
| zxAnPonOltIfH1DRx ErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.11 |  |
| zxAnPonOltIfH1DRx | . |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| CrcAlignErrors | 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.12 |  |
| zxAnPonOltIfH1DRx UndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.13 |  |
| zxAnPonOltIfH1DRx OversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.14 |  |
| zxAnPonOltIfH1DRx 64Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.15 |  |
| zxAnPonOltIfH1DRx 65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.16 |  |
| zxAnPonOltIfH1DRx 128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.17 |  |
| zxAnPonOltIfH1DRx 256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.18 |  |
| zxAnPonOltIfH1DRx 512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.19 |  |
| zxAnPonOltIfH1DRx 1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.20 |  |
| zxAnPonOltIfH1DRx Fragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.21 |  |
| zxAnPonOltIfH1DRx Jabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.22 |  |
| zxAnPonOltIfH1DTx Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.36 |  |
| zxAnPonOltIfH1DTx Pkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.37 |  |
| zxAnPonOltIfH1DTx | . |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| UcastPkts | 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.38 |  |
| zxAnPonOltIfH1DTx NUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.39 |  |
| zxAnPonOltIfH1DTx McastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.40 |  |
| zxAnPonOltIfH1DTx BcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.41 |  |
| zxAnPonOltIfH1DTx DiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.42 |  |
| zxAnPonOltIfH1DTx ErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.43 |  |
| zxAnPonOltIfH1DTx CrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.44 |  |
| zxAnPonOltIfH1DTx UndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.45 |  |
| zxAnPonOltIfH1DTx OversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.46 |  |
| zxAnPonOltIfH1DTx Pkts64Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.47 |  |
| zxAnPonOltIfH1DTx6 5To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.48 |  |
| zxAnPonOltIfH1DTx1 28To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.49 |  |
| zxAnPonOltIfH1DTx2 56To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.50 |  |
| zxAnPonOltIfH1DTx5 | . |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| 12To1023Octs | 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.51 |  |
| zxAnPonOltIfH1DTx1 024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.52 |  |
| zxAnPonOltIfH1DTx Fragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.53 |  |
| zxAnPonOltIfH1DTxJ abbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.54 |  |
| zxAnPonOltIfH1DCol lisions | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.70 |  |
| zxAnPonOltIfH1DDro pEvents | . 1.3.6.1.4.1.3902.1082.500 .4.2.1.6.1.71 |  |

## ONU Ethernet Performance

This function is enabled by default.

#### Realtime Performance

[MIB file]:

Please refer to zxAnPonOnuIfCurrPerfTable defined in ZTE-AN-PON-PERF-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfRxOct ets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.1 |  |
| zxAnPonOnuIfRxPkt s | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.2 |  |
| zxAnPonOnuIfRxOct etRate | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.3 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfRxPkt Rate | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.4 |  |
| zxAnPonOnuIfRxOct etPeakRate | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.5 |  |
| zxAnPonOnuIfRxPkt PeakRate | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.6 |  |
| zxAnPonOnuIfRxUca stPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.7 |  |
| zxAnPonOnuIfRxNU castPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.8 |  |
| zxAnPonOnuIfRxMc astPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.9 |  |
| zxAnPonOnuIfRxBca stPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.10 |  |
| zxAnPonOnuIfRxDis cardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.11 |  |
| zxAnPonOnuIfRxErr Pkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.12 |  |
| zxAnPonOnuIfRxPdr | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.13 |  |
| zxAnPonOnuIfRxPer | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.14 |  |
| zxAnPonOnuIfRxCrc AlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.15 |  |
| zxAnPonOnuIfRxUnd ersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.16 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfRxOve rsizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.17 |  |
| zxAnPonOnuIfRxPkt s64Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.18 |  |
| zxAnPonOnuIfRxPkt s65To127Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.19 |  |
| zxAnPonOnuIfRxPkt s128To255Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.20 |  |
| zxAnPonOnuIfRxPkt s256To511Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.21 |  |
| zxAnPonOnuIfRxPkt s512To1023Oct | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.22 |  |
| zxAnPonOnuIfRxPkt s1024To1518Oct | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.23 |  |
| zxAnPonOnuIfRxBw Utilization | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.24 |  |
| zxAnPonOnuIfRxAvg BwUtilization | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.25 |  |
| zxAnPonOnuIfRxFra gments | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.26 |  |
| zxAnPonOnuIfRxJab bers | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.27 |  |
| zxAnPonOnuIfTxOct ets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.44 |  |
| zxAnPonOnuIfTxPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.45 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfTxOct etRate | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.46 |  |
| zxAnPonOnuIfTxPkt Rate | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.47 |  |
| zxAnPonOnuIfTxOct etPeakRate | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.48 |  |
| zxAnPonOnuIfTxPkt PeakRate | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.49 |  |
| zxAnPonOnuIfTxUca stPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.50 |  |
| zxAnPonOnuIfTxNUc astPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.51 |  |
| zxAnPonOnuIfTxMca stPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.52 |  |
| zxAnPonOnuIfTxBca stPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.53 |  |
| zxAnPonOnuIfTxDisc ardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.54 |  |
| zxAnPonOnuIfTxErr Pkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.55 |  |
| zxAnPonOnuIfTxPdr | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.56 |  |
| zxAnPonOnuIfTxPer | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.57 |  |
| zxAnPonOnuIfTxCrc AlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.58 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfTxUnd ersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.59 |  |
| zxAnPonOnuIfTxOve rsizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.60 |  |
| zxAnPonOnuIfTxPkts 64Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.61 |  |
| zxAnPonOnuIfTxPkts 65To127Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.62 |  |
| zxAnPonOnuIfTxPkts 128To255Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.63 |  |
| zxAnPonOnuIfTxPkts 256To511Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.64 |  |
| zxAnPonOnuIfTxPkts 512To1023Oct | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.65 |  |
| zxAnPonOnuIfTxPkts 1024To1518Oct | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.66 |  |
| zxAnPonOnuIfTxBw Utilization | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.67 |  |
| zxAnPonOnuIfTxAvg BwUtilization | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.68 |  |
| zxAnPonOnuIfTxFra gments | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.69 |  |
| zxAnPonOnuIfTxJab bers | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.70 |  |
| zxAnPonOnuIfCollisi ons | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.88 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfDropE vents | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.89 |  |
| zxAnPonOnuIfCurrP erfCapability | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.99 |  |
| zxAnPonOnuIfCurrP erfReset | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.2.1.100 |  |

#### 15-Minute Realtime Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfC15M TimeElapsed | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.1 |  |
| zxAnPonOnuIfC15M RxOctets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.2 |  |
| zxAnPonOnuIfC15M RxPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.3 |  |
| zxAnPonOnuIfC15M RxUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.4 |  |
| zxAnPonOnuIfC15M RxNUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.5 |  |
| zxAnPonOnuIfC15M RxMcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.6 |  |
| zxAnPonOnuIfC15M RxBcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.7 |  |

Please refer to zxAnPonOnuIfCurr15MinPerfTable defined in ZTE-AN-PON-PERF- MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfC15M RxDiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.8 |  |
| zxAnPonOnuIfC15M RxErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.9 |  |
| zxAnPonOnuIfC15M RxCrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.10 |  |
| zxAnPonOnuIfC15M RxUndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.11 |  |
| zxAnPonOnuIfC15M RxOversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.12 |  |
| zxAnPonOnuIfC15M Rx64Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.13 |  |
| zxAnPonOnuIfC15M Rx65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.14 |  |
| zxAnPonOnuIfC15M Rx128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.15 |  |
| zxAnPonOnuIfC15M Rx256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.16 |  |
| zxAnPonOnuIfC15M Rx512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.17 |  |
| zxAnPonOnuIfC15M Rx1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.18 |  |
| zxAnPonOnuIfC15M RxFragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.19 |  |
| zxAnPonOnuIfC15M RxJabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.20 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfC15M TxOctets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.36 |  |
| zxAnPonOnuIfC15M TxPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.37 |  |
| zxAnPonOnuIfC15M TxUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.38 |  |
| zxAnPonOnuIfC15M TxNUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.39 |  |
| zxAnPonOnuIfC15M TxMcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.40 |  |
| zxAnPonOnuIfC15M TxBcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.41 |  |
| zxAnPonOnuIfC15M TxDiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.42 |  |
| zxAnPonOnuIfC15M TxErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.43 |  |
| zxAnPonOnuIfC15M TxCrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.44 |  |
| zxAnPonOnuIfC15M TxUndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.45 |  |
| zxAnPonOnuIfC15M TxOversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.46 |  |
| zxAnPonOnuIfC15M TxPkts64Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.47 |  |
| zxAnPonOnuIfC15M Tx65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.48 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfC15M Tx128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.49 |  |
| zxAnPonOnuIfC15M Tx256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.50 |  |
| zxAnPonOnuIfC15M Tx512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.51 |  |
| zxAnPonOnuIfC15M Tx1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.52 |  |
| zxAnPonOnuIfC15M TxFragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.53 |  |
| zxAnPonOnuIfC15M TxJabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.54 |  |
| zxAnPonOnuIfC15M Collisions | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.70 |  |
| zxAnPonOnuIfC15M DropEvents | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.71 |  |
| zxAnPonOnuIfPerfC apability | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.99 |  |
| zxAnPonOnuIfC15M PerfReset | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.3.1.100 |  |

#### 24-Hour Realtime Performance

[MIB file]:

Please refer to zxAnPonOnuIfCurr1DayPerfTable defined in ZTE-AN-PON-PERF- MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfC1DTi meElapsed | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.1 |  |
| zxAnPonOnuIfC1DR xOctets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.2 |  |
| zxAnPonOnuIfC1DR xPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.3 |  |
| zxAnPonOnuIfC1DR xUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.4 |  |
| zxAnPonOnuIfC1DR xNUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.5 |  |
| zxAnPonOnuIfC1DR xMcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.6 |  |
| zxAnPonOnuIfC1DR xBcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.7 |  |
| zxAnPonOnuIfC1DR xDiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.8 |  |
| zxAnPonOnuIfC1DR xErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.9 |  |
| zxAnPonOnuIfC1DR xCrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.10 |  |
| zxAnPonOnuIfC1DR xUndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.11 |  |
| zxAnPonOnuIfC1DR xOversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.12 |  |
| zxAnPonOnuIfC1DR x64Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.13 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfC1DR x65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.14 |  |
| zxAnPonOnuIfC1DR x128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.15 |  |
| zxAnPonOnuIfC1DR x256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.16 |  |
| zxAnPonOnuIfC1DR x512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.17 |  |
| zxAnPonOnuIfC1DR x1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.18 |  |
| zxAnPonOnuIfC1DR xFragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.19 |  |
| zxAnPonOnuIfC1DR xJabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.20 |  |
| zxAnPonOnuIfC1DT xOctets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.36 |  |
| zxAnPonOnuIfC1DT xPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.37 |  |
| zxAnPonOnuIfC1DT xUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.38 |  |
| zxAnPonOnuIfC1DT xNUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.39 |  |
| zxAnPonOnuIfC1DT xMcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.40 |  |
| zxAnPonOnuIfC1DT xBcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.41 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfC1DT xDiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.42 |  |
| zxAnPonOnuIfC1DT xErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.43 |  |
| zxAnPonOnuIfC1DT xCrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.44 |  |
| zxAnPonOnuIfC1DT xUndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.45 |  |
| zxAnPonOnuIfC1DT xOversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.46 |  |
| zxAnPonOnuIfC1DT x64Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.47 |  |
| zxAnPonOnuIfC1DT x65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.48 |  |
| zxAnPonOnuIfC1DT x128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.49 |  |
| zxAnPonOnuIfC1DT x256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.50 |  |
| zxAnPonOnuIfC1DT x512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.51 |  |
| zxAnPonOnuIfC1DT x1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.52 |  |
| zxAnPonOnuIfC1DT xFragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.53 |  |
| zxAnPonOnuIfC1DT xJabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.54 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfC1DC ollisions | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.70 |  |
| zxAnPonOnuIfC1DDr opEvents | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.71 |  |
| zxAnPonOnuIfC1DP erfReset | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.4.1.100 |  |

#### 15-Mintue History Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfH15MI ntervalNo | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.1 |  |
| zxAnPonOnuIfH15M ValidData | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.2 |  |
| zxAnPonOnuIfH15M DateAndTime | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.3 |  |
| zxAnPonOnuIfH15M RxOctets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.4 |  |
| zxAnPonOnuIfH15M RxPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.5 |  |
| zxAnPonOnuIfH15M RxUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.6 |  |
| zxAnPonOnuIfH15M RxNUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.7 |  |

Please refer to zxAnPonOnuIfHis15MinPerfTable defined in ZTE-AN-PON-PERF- MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfH15M RxMcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.8 |  |
| zxAnPonOnuIfH15M RxBcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.9 |  |
| zxAnPonOnuIfH15M RxDiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.10 |  |
| zxAnPonOnuIfH15M RxErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.11 |  |
| zxAnPonOnuIfH15M RxCrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.12 |  |
| zxAnPonOnuIfH15M RxUndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.13 |  |
| zxAnPonOnuIfH15M RxOversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.14 |  |
| zxAnPonOnuIfH15M Rx64Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.15 |  |
| zxAnPonOnuIfH15M Rx65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.16 |  |
| zxAnPonOnuIfH15M Rx128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.17 |  |
| zxAnPonOnuIfH15M Rx256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.18 |  |
| zxAnPonOnuIfH15M Rx512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.19 |  |
| zxAnPonOnuIfH15M Rx1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.20 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfH15M RxFragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.21 |  |
| zxAnPonOnuIfH15M RxJabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.22 |  |
| zxAnPonOnuIfH15M TxOctets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.36 |  |
| zxAnPonOnuIfH15M TxPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.37 |  |
| zxAnPonOnuIfH15M TxUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.38 |  |
| zxAnPonOnuIfH15M TxNUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.39 |  |
| zxAnPonOnuIfH15M TxMcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.40 |  |
| zxAnPonOnuIfH15M TxBcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.41 |  |
| zxAnPonOnuIfH15M TxDiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.42 |  |
| zxAnPonOnuIfH15M TxErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.43 |  |
| zxAnPonOnuIfH15M TxCrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.44 |  |
| zxAnPonOnuIfH15M TxUndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.45 |  |
| zxAnPonOnuIfH15M TxOversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.46 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfH15M TxPkts64Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.47 |  |
| zxAnPonOnuIfH15M Tx65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.48 |  |
| zxAnPonOnuIfH15M Tx128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.49 |  |
| zxAnPonOnuIfH15M Tx256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.50 |  |
| zxAnPonOnuIfH15M Tx512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.51 |  |
| zxAnPonOnuIfH15M Tx1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.52 |  |
| zxAnPonOnuIfH15M TxFragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.53 |  |
| zxAnPonOnuIfH15M TxJabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.54 |  |
| zxAnPonOnuIfH15M Collisions | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.70 |  |
| zxAnPonOnuIfH15M DropEvents | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.5.1.71 |  |

#### 24-Hour History Performance

[MIB file]:

Please refer to zxAnPonOnuIfHis1DayPerfTable defined in ZTE-AN-PON-PERF- MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfH1DIn tervalNo | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.1 |  |
| zxAnPonOnuIfH1DV alidData | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.2 |  |
| zxAnPonOnuIfH1DD ateAndTime | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.3 |  |
| zxAnPonOnuIfH1DR xOctets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.4 |  |
| zxAnPonOnuIfH1DR xPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.5 |  |
| zxAnPonOnuIfH1DR xUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.6 |  |
| zxAnPonOnuIfH1DR xNUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.7 |  |
| zxAnPonOnuIfH1DR xMcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.8 |  |
| zxAnPonOnuIfH1DR xBcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.9 |  |
| zxAnPonOnuIfH1DR xDiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.10 |  |
| zxAnPonOnuIfH1DR xErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.11 |  |
| zxAnPonOnuIfH1DR xCrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.12 |  |
| zxAnPonOnuIfH1DR xUndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.13 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfH1DR xOversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.14 |  |
| zxAnPonOnuIfH1DR x64Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.15 |  |
| zxAnPonOnuIfH1DR x65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.16 |  |
| zxAnPonOnuIfH1DR x128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.17 |  |
| zxAnPonOnuIfH1DR x256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.18 |  |
| zxAnPonOnuIfH1DR x512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.19 |  |
| zxAnPonOnuIfH1DR x1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.20 |  |
| zxAnPonOnuIfH1DR xFragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.21 |  |
| zxAnPonOnuIfH1DR xJabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.22 |  |
| zxAnPonOnuIfH1DT xOctets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.36 |  |
| zxAnPonOnuIfH1DT xPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.37 |  |
| zxAnPonOnuIfH1DT xUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.38 |  |
| zxAnPonOnuIfH1DT xNUcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.39 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfH1DT xMcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.40 |  |
| zxAnPonOnuIfH1DT xBcastPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.41 |  |
| zxAnPonOnuIfH1DT xDiscardPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.42 |  |
| zxAnPonOnuIfH1DT xErrPkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.43 |  |
| zxAnPonOnuIfH1DT xCrcAlignErrors | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.44 |  |
| zxAnPonOnuIfH1DT xUndersizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.45 |  |
| zxAnPonOnuIfH1DT xOversizePkts | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.46 |  |
| zxAnPonOnuIfH1DT xPkts64Octets | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.47 |  |
| zxAnPonOnuIfH1DT x65To127Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.48 |  |
| zxAnPonOnuIfH1DT x128To255Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.49 |  |
| zxAnPonOnuIfH1DT x256To511Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.50 |  |
| zxAnPonOnuIfH1DT x512To1023Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.51 |  |
| zxAnPonOnuIfH1DT x1024To1518Octs | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.52 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonOnuIfH1DT xFragments | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.53 |  |
| zxAnPonOnuIfH1DT xJabbers | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.54 |  |
| zxAnPonOnuIfH1DC ollisions | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.70 |  |
| zxAnPonOnuIfH1DDr opEvents | . 1.3.6.1.4.1.3902.1082.500 .4.2.2.6.1.71 |  |

## GPON OLT PON Layer Performance

#### Realtime Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponUsCorrect NIdleGems | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.1 |  |
| zxAnGponUsCorrectI dleGems | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.2 |  |
| zxAnGponUsErrGem s | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.3 |  |
| zxAnGponUsGemPa yloadBytes | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.4 |  |
| zxAnGponUsCorrect EtherFrm | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.5 |  |

Please refer to zxAnGponSrvOltCurrPerfTable defined in ZTE-AN-GPON-SERVICE- MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponUsErrEthe rFrm | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.6 |  |
| zxAnGponUsPloam Msgs | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.7 |  |
| zxAnGponUsErrPloa mMsgs | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.8 |  |
| zxAnGponUsErrBits | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.9 |  |
| zxAnGponUsTotalBit s | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.10 |  |
| zxAnGponUsCrcErrP kts | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.11 |  |
| zxAnGponDsEtherFr m | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.12 |  |
| zxAnGponDsPloam Msgs | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.13 |  |
| zxAnGponOltReis | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.14 |  |
| zxAnGponCorrected Bytes | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.15 |  |
| zxAnGponCorrected Words | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.16 |  |
| zxAnGponUncorrect edWords | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.17 |  |
| zxAnGponTotalRxWo rds | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.18 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponUsBer | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.19 |  |
| zxAnGponOltPerfCa pability | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.99 |  |
| zxAnGponOltCurrRe set | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.2.1.100 |  |

#### 15-Minute Realtime Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOltC15MTi meElapsed | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.1 |  |
| zxAnGponC15MUsC orrectNIdleGems | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.2 |  |
| zxAnGponC15MUsC orrectIdleGems | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.3 |  |
| zxAnGponC15MUsE rrGemFrm | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.4 |  |
| zxAnGponC15MUsG emPayloadBytes | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.5 |  |
| zxAnGponC15MUsC orrectEtherFrm | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.6 |  |
| zxAnGponC15MUsE rrEtherFrm | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.7 |  |
| zxAnGponC15MUsPl | . |  |

Please refer to zxAnGponSrvOltCurr15MinPerfTable defined in ZTE-AN-GPON- SERVICE-MIB.mib.

| oamMsgs | 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.8 |  |
| --- | --- | --- |
| zxAnGponC15MUsE rrPloamMsgs | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.9 |  |
| zxAnGponC15MUsE rrBits | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.10 |  |
| zxAnGponC15MUsT otalBits | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.11 |  |
| zxAnGponC15MUsC rcErrPkts | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.12 |  |
| zxAnGponC15MDsEt herFrm | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.13 |  |
| zxAnGponC15MDsPl oamMsgs | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.14 |  |
| zxAnGponC15MReis | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.15 |  |
| zxAnGponC15MCorr ectedBytes | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.16 |  |
| zxAnGponC15MCorr ectedWords | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.17 |  |
| zxAnGponC15MUnc orrectedWords | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.18 |  |
| zxAnGponC15MTotal RxWords | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.19 |  |
| zxAnGponOltC15MR eset | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.4.1.100 |  |

#### 24-Hour Realtime Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOltC1DTi meElapsed | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.1 |  |
| zxAnGponC1DUsCor rectNIdleGems | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.2 |  |
| zxAnGponC1DUsCor rectIdleGems | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.3 |  |
| zxAnGponC1DUsErr GemFrm | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.4 |  |
| zxAnGponC1DUsGe mPayloadBytes | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.5 |  |
| zxAnGponC1DUsCor rectEtherFrm | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.6 |  |
| zxAnGponC1DUsErr EtherFrm | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.7 |  |
| zxAnGponC1DUsPlo amMsgs | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.8 |  |
| zxAnGponC1DUsErr PloamMsgs | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.9 |  |
| zxAnGponC1DUsErr Bits | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.10 |  |
| zxAnGponC1DUsTot alBits | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.11 |  |

Please refer to zxAnGponSrvOltCurr1DayPerfTable defined in ZTE-AN-GPON- SERVICE-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponC1DUsCrc ErrPkts | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.12 |  |
| zxAnGponC1DDsEth erFrm | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.13 |  |
| zxAnGponC1DDsPlo amMsgs | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.14 |  |
| zxAnGponC1DReis | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.15 |  |
| zxAnGponC1DCorre ctedBytes | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.16 |  |
| zxAnGponC1DCorre ctedWords | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.17 |  |
| zxAnGponC1DUncor rectedWords | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.18 |  |
| zxAnGponC1DTotalR xWords | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.19 |  |
| zxAnGponOltC1DRe set | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.5.1.100 |  |

#### 15-Minute History Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOltH15MIn | . |  |
| tervalNo | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.1 |  |

Please refer to zxAnGponSrvOltHis15MinPerfTable defined in ZTE-AN-GPON- SERVICE-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOltH15MV | . |  |
| alidData | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.2 |  |
| zxAnGponOltH15MD | . |  |
| ateAndTime | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.3 |  |
| zxAnGponH15MUsC | . |  |
| orrectNIdleGems | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.4 |  |
| zxAnGponH15MUsC | . |  |
| orrectIdleGems | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.5 |  |
| zxAnGponH15MUsE | . |  |
| rrGemFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.6 |  |
| zxAnGponH15MUsG | . |  |
| emPayloadBytes | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.7 |  |
| zxAnGponH15MUsC | . |  |
| orrectEtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.8 |  |
| zxAnGponH15MUsE | . |  |
| rrEtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.9 |  |
| zxAnGponH15MUsPl | . |  |
| oamMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.10 |  |
| zxAnGponH15MUsE | . |  |
| rrPloamMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.11 |  |
| zxAnGponH15MUsE | . |  |
| rrBits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.12 |  |
| zxAnGponH15MUsT | . |  |
| otalBits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.13 |  |
| zxAnGponH15MUsC | . |  |
| rcErrPkts | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.14 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponH15MDsEt | . |  |
| herFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.15 |  |
| zxAnGponH15MDsPl | . |  |
| oamMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.16 |  |
| zxAnGponH15MReis | . |  |
|  | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.17 |  |
| zxAnGponH15MCorr | . |  |
| ectedBytes | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.18 |  |
| zxAnGponH15MCorr | . |  |
| ectedWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.19 |  |
| zxAnGponH15MUnc | . |  |
| orrectedWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.20 |  |
| zxAnGponH15MTotal | . |  |
| RxWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.21 |  |

#### 24-Hour History Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOltH1DInt | . |  |
| ervalNo | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.1 |  |
| zxAnGponOltH1DVali | . |  |
| dData | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.2 |  |
| zxAnGponOltH1DDat | . |  |
| eAndTime | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.3 |  |

Please refer to zxAnGponSrvOltHis1DayPerfTable defined in ZTE-AN-GPON- SERVICE-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponH1DUsCor | . |  |
| rectNIdleGems | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.4 |  |
| zxAnGponH1DUsCor | . |  |
| rectIdleGems | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.5 |  |
| zxAnGponH1DUsErr | . |  |
| GemFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.6 |  |
| zxAnGponH1DUsGe | . |  |
| mPayloadBytes | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.7 |  |
| zxAnGponH1DUsCor | . |  |
| rectEtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.8 |  |
| zxAnGponH1DUsErr | . |  |
| EtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.9 |  |
| zxAnGponH1DUsPlo | . |  |
| amMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.10 |  |
| zxAnGponH1DUsErr | . |  |
| PloamMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.11 |  |
| zxAnGponH1DUsErr | . |  |
| Bits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.12 |  |
| zxAnGponH1DUsTot | . |  |
| alBits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.13 |  |
| zxAnGponH1DUsCrc | . |  |
| ErrPkts | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.14 |  |
| zxAnGponH1DDsEth | . |  |
| erFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.15 |  |
| zxAnGponH1DDsPlo | . |  |
| amMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.16 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponH1DReis | . |  |
|  | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.17 |  |
| zxAnGponH1DCorre | . |  |
| ctedBytes | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.18 |  |
| zxAnGponH1DCorre | . |  |
| ctedWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.19 |  |
| zxAnGponH1DUncor | . |  |
| rectedWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.20 |  |
| zxAnGponH1DTotalR | . |  |
| xWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.7.1.21 |  |

## GPON ONU PON Layer Performance

#### Realtime Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOnuUsCor | . |  |
| rectNIdleGems | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.1 |  |
| zxAnGponOnuUsGe | . |  |
| mPayloadBytes | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.2 |  |
| zxAnGponOnuUsCor | . |  |
| rectEtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.3 |  |
| zxAnGponOnuUsErr | . |  |
| EtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.4 |  |

Please refer to zxAnGponSrvOnuCurrPerfTable defined in ZTE-AN-GPON-SERVICE- MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOnuUsPlo | . |  |
| amMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.5 |  |
| zxAnGponOnuUsOm | . |  |
| ciMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.6 |  |
| zxAnGponOnuUsErr | . |  |
| Bits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.7 |  |
| zxAnGponOnuUsTot | . |  |
| alBits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.8 |  |
| zxAnGponOnuUsCrc | . |  |
| ErrPkts | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.9 |  |
| zxAnGponOnuUsLos | . |  |
| tBursts | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.10 |  |
| zxAnGponOnuDsCor | . |  |
| rectNIdleGems | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.11 |  |
| zxAnGponOnuDsPlo | . |  |
| amMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.12 |  |
| zxAnGponOnuDsOm | . |  |
| ciMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.13 |  |
| zxAnGponOnuReiis | . |  |
|  | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.14 |  |
| zxAnGponOnuCorre | . |  |
| ctedBytes | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.15 |  |
| zxAnGponOnuCorre | . |  |
| ctedWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.16 |  |
| zxAnGponOnuUncor | . |  |
| rectedWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.17 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOnuTotalR | . |  |
| xWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.18 |  |
| zxAnGponOnuUsBer | . |  |
|  | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.19 |  |
| zxAnGponOnuPerfC | . |  |
| apability | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.99 |  |
| zxAnGponOnuCurrR | . |  |
| eset | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.2.1.100 |  |

#### 15-Minute Realtime Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOnuC15M | . |  |
| TimeElapsed | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.4.1.1 |  |
| zxAnGponOnuC15M | . |  |
| UsCorrectNIdleGem | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.2 |  |
| zxAnGponOnuC15M | . |  |
| UsGemPayloadBytes | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.3 |  |
| zxAnGponOnuC15M | . |  |
| UsCorrectEtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.4 |  |
| zxAnGponOnuC15M | . |  |
| UsErrEtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.5 |  |
| zxAnGponOnuC15M | . |  |
| UsPloamMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.6 |  |

Please refer to zxAnGponSrvOnuCurr15MinPerfTable defined in ZTE-AN-GPON- SERVICE-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOnuC15M | . |  |
| UsOmciMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.7 |  |
| zxAnGponOnuC15M | . |  |
| UsErrBits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.8 |  |
| zxAnGponOnuC15M | . |  |
| UsTotalBits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.9 |  |
| zxAnGponOnuC15M | . |  |
| UsCrcErrPkts | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.10 |  |
| zxAnGponOnuC15M | . |  |
| UsLostBursts | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.11 |  |
| zxAnGponOnuC15M | . |  |
| DsCorrectNIdleGem | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.12 |  |
| zxAnGponOnuC15M | . |  |
| DsPloamMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.13 |  |
| zxAnGponOnuC15M | . |  |
| DsOmciMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.14 |  |
| zxAnGponOnuC15M | . |  |
| Reiis | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.15 |  |
| zxAnGponOnuC15M | . |  |
| CorrectedBytes | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.16 |  |
| zxAnGponOnuC15M | . |  |
| CorrectedWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.17 |  |
| zxAnGponOnuC15M | . |  |
| UncorrectedWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.18 |  |
| zxAnGponOnuC15M | . |  |
| TotalRxWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.19 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOnuC15M | . |  |
| Reset | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.4.1.100 |  |

#### 24-Hour Realtime Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOnuC1DTi | . |  |
| meElapsed | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.5.1.1 |  |
| zxAnGponOnuC1DU | . |  |
| sCorrectNIdleGems | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.2 |  |
| zxAnGponOnuC1DU | . |  |
| sGemPayloadBytes | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.3 |  |
| zxAnGponOnuC1DU | . |  |
| sCorrectEtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.4 |  |
| zxAnGponOnuC1DU | . |  |
| sErrEtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.5 |  |
| zxAnGponOnuC1DU | . |  |
| sPloamMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.6 |  |
| zxAnGponOnuC1DU | . |  |
| sOmciMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.7 |  |
| zxAnGponOnuC1DU | . |  |
| sErrBits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.8 |  |
| zxAnGponOnuC1DU | . |  |
| sTotalBits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.9 |  |

Please refer to zxAnGponSrvOnuCurr1DayPerfTable defined in ZTE-AN-GPON- SERVICE-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOnuC1DU | . |  |
| sCrcErrPkts | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.10 |  |
| zxAnGponOnuC1DU | . |  |
| sLostBursts | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.11 |  |
| zxAnGponOnuC1DD | . |  |
| sCorrectNIdleGems | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.12 |  |
| zxAnGponOnuC1DD | . |  |
| sPloamMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.13 |  |
| zxAnGponOnuC1DD | . |  |
| sOmciMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.14 |  |
| zxAnGponOnuC1DR | . |  |
| eiis | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.15 |  |
| zxAnGponOnuC1DC | . |  |
| orrectedBytes | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.16 |  |
| zxAnGponOnuC1DC | . |  |
| orrectedWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.17 |  |
| zxAnGponOnuC1DU | . |  |
| ncorrectedWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.18 |  |
| zxAnGponOnuC1DT | . |  |
| otalRxWords | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.19 |  |
| zxAnGponOnuC1DR | . |  |
| eset | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.5.1.100 |  |

#### 15-Minute History Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOnuH15MI | . |  |
| ntervalNo | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.6.1.1 |  |
| zxAnGponOnuH15M | . |  |
| ValidData | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.2 |  |
| zxAnGponOnuH15M | . |  |
| DateAndTime | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.3 |  |
| zxAnGponOnuH15M | . |  |
| UsCorrectNIdleGem | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.4 |  |
| zxAnGponOnuH15M | . |  |
| UsGemPayloadBytes | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.5 |  |
| zxAnGponOnuH15M | . |  |
| UsCorrectEtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.6 |  |
| zxAnGponOnuH15M | . |  |
| UsErrEtherFrm | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.7 |  |
| zxAnGponOnuH15M | . |  |
| UsPloamMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.8 |  |
| zxAnGponOnuH15M | . |  |
| UsOmciMsgs | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.9 |  |
| zxAnGponOnuH15M | . |  |
| UsErrBits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.10 |  |
| zxAnGponOnuH15M | . |  |
| UsTotalBits | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.11 |  |
| zxAnGponOnuH15M | . |  |
| UsCrcErrPkts | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.2.2.6.1.12 |  |
| zxAnGponOnuH15M | . |  |

Please refer to zxAnGponSrvOnuHis15MinPerfTable defined in ZTE-AN-GPON- SERVICE-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| UsLostBursts | 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.6.1.13 |  |
| zxAnGponOnuH15M DsCorrectNIdleGem | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.6.1.14 |  |
| zxAnGponOnuH15M DsPloamMsgs | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.6.1.15 |  |
| zxAnGponOnuH15M DsOmciMsgs | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.6.1.16 |  |
| zxAnGponOnuH15M Reiis | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.6.1.17 |  |
| zxAnGponOnuH15M CorrectedBytes | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.6.1.18 |  |
| zxAnGponOnuH15M CorrectedWords | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.6.1.19 |  |
| zxAnGponOnuH15M UncorrectedWords | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.6.1.20 |  |
| zxAnGponOnuH15M TotalRxWords | . 1.3.6.1.4.1.3902.1082.500 .10.2.3.2.6.1.21 |  |

#### 24-Hour History Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponOnuH1DIn | . |  |
| tervalNo | 1.3.6.1.4.1.3902.1082.500 |  |
|  | .10.2.3.2.7.1.1 |  |
| zxAnGponOnuH1DV | . |  |

Please refer to zxAnGponSrvOnuHis1DayPerfTable defined in ZTE-AN-GPON- SERVICE-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| alidData | 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.2 |  |
| zxAnGponOnuH1DD ateAndTime | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.3 |  |
| zxAnGponOnuH1DU sCorrectNIdleGems | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.4 |  |
| zxAnGponOnuH1DU sGemPayloadBytes | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.5 |  |
| zxAnGponOnuH1DU sCorrectEtherFrm | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.6 |  |
| zxAnGponOnuH1DU sErrEtherFrm | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.7 |  |
| zxAnGponOnuH1DU sPloamMsgs | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.8 |  |
| zxAnGponOnuH1DU sOmciMsgs | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.9 |  |
| zxAnGponOnuH1DU sErrBits | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.10 |  |
| zxAnGponOnuH1DU sTotalBits | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.11 |  |
| zxAnGponOnuH1DU sCrcErrPkts | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.12 |  |
| zxAnGponOnuH1DU sLostBursts | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.13 |  |
| zxAnGponOnuH1DD sCorrectNIdleGems | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.14 |  |
| zxAnGponOnuH1DD | . |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| sPloamMsgs | 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.15 |  |
| zxAnGponOnuH1DD sOmciMsgs | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.16 |  |
| zxAnGponOnuH1DR eiis | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.17 |  |
| zxAnGponOnuH1DC orrectedBytes | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.18 |  |
| zxAnGponOnuH1DC orrectedWords | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.19 |  |
| zxAnGponOnuH1DU ncorrectedWords | . 1.3.6.1.4.1.3902.1082.500 .10.2.2.2.7.1.20 |  |
| zxAnGponOnuH1DT otalRxWords | . 1.3.6.1.4.1.3902.1082.500 .10.2.3.2.7.1.21 |  |

## GPON GEMPORT Realtime Statistics

#### Realtime Performance

[MIB file]:

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponGemPortR xOctets | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.1 |  |
| zxAnGponGemPortR xUcastPkts | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.2 |  |
| zxAnGponGemPortR | . |  |

Please refer to zxAnGponSrvGemPortCurrPerfTable defined in ZTE-AN-GPON- SERVICE-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| xBcastPkts | 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.3 |  |
| zxAnGponGemPortR xMcastPkts | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.4 |  |
| zxAnGponGemPortR xDiscardPkts | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.5 |  |
| zxAnGponGemPortT xOctets | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.6 |  |
| zxAnGponGemPortT xUcastPkts | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.7 |  |
| zxAnGponGemPortT xBcastPkts | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.8 |  |
| zxAnGponGemPortT xMcastPkts | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.9 |  |
| zxAnGponGemPortT xDiscardPkts | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.10 |  |
| zxAnGponGemPortR xOctetRate | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.11 |  |
| zxAnGponGemPortR xOctetPeakRate | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.12 |  |
| zxAnGponGemPortT xOctetRate | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.13 |  |
| zxAnGponGemPortT xOctetPeakRate | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.14 |  |
| zxAnGponGemPortP erfCapability | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.99 |  |
| zxAnGponGemPortC | . |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| urrReset | 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.100 |  |
| zxAnGponGemPortC urrRowStatus | . 1.3.6.1.4.1.3902.1082.50 0.10.2.3.11.2.1.101 |  |

# Alarms and Notifications

## System Control Alarm and Notification

[MIB file]:

ZTE-AN-CHASSIS-MIB.mib ZTE-AN-ENVMON-MIB.mib

ZTE-AN-DATA-BACKUP-MIB.mib ZTE-AN-REMOTE-UNIT-SW-MIB.mib

#### System Cold Start Notification

| TripOID | .1.3.6.1.6.3.1.1.5.1 | Notification |
| --- | --- | --- |
| Variable | None |  |

#### Active and Standby Switchover Notification

| TripOID | . 1.3.6.1.4.1.3902.1082.10.1.3.1.2 | Notification |
| --- | --- | --- |
| Variable | None | Card operating status |

#### Active and Standby Data Synchronization Failure Notification

| TripOID |  |  | Notification |
| --- | --- | --- | --- |
| Variable | . | zxAnCardOperStatus | Card |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1.5 |  | operating |
|  |  |  | status |

#### Card Abnormal Operating Alarm

##### Alarm

| TripOI D | . 1.3.6.1.4.1.3902.1082.10.1.3.2.1 | Major Alarm |
| --- | --- | --- |

|  | .1.3.6.1.2.1.2.2.1.1 | ifIndex | ifIndex |  |
| --- | --- | --- | --- | --- |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1. | zxAnCardConfMainType | Configuratio |  |
|  | 2 |  | n card type |  |
|  | . | zxAnCardActualMainTyp | Actual | card |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1. | e | type |  |
|  | 3 |  |  |  |
| Variabl e | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 4 | zxAnCardActualType | Card name |  |
|  | . | zxAnCardAdminStatus | Card |  |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1. |  | managemen |  |
|  | 6 |  | t status |  |
|  | . | zxAnCardOperStatus | Card |  |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1. |  | operating |  |
|  | 5 |  | status |  |

Card operating status (. 1.3.6.1.4.1.3902.1082.10.1.2.4.1.5) value: 1 inService

2 notInService 3.hwOnline

- hwOffline

- configuring

- configFailed

- MIB value Mismatch 8 deactived

- faulty

- invalid

- noPower

##### Alarm Recovery

| TripOI | . 1.3.6.1.4.1.3902.1082.10.1.3.2.2 | Alarm |
| --- | --- | --- |

| D |  |  | recovery |  |
| --- | --- | --- | --- | --- |
|  | .1.3.6.1.2.1.2.2.1.1 | ifIndex | ifIndex |  |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1. | zxAnCardConfMainType | Configuratio |  |
|  | 2 |  | n card type |  |
|  | . | zxAnCardActualMainTyp | Actual | card |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1. | e | type |  |
|  | 3 |  |  |  |
| Variabl e | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1. 4 | zxAnCardActualType | Card name |  |
|  | . | zxAnCardAdminStatus | Card |  |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1. |  | managemen |  |
|  | 6 |  | t status |  |
|  | . | zxAnCardOperStatus | Card |  |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1. |  | operating |  |
|  | 5 |  | status |  |

#### Memory Overload Alarm

##### Alarm

| TripOI D | . 1.3.6.1.4.1.3902.1082.10.1.3.2.22 |  | Minor alarm |
| --- | --- | --- | --- |
|  | . | zxAnCardMemUsage | Memory |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1 |  | usage |
| Variabl e | .11 |  | percentag e |
|  | . | zxAnCardMemUsageThresh | Memory |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1 | old | usage |
|  | .12 |  | threshold |

##### Alarm Recovery

| TripOI D | . 1.3.6.1.4.1.3902.1082.10.1.3.2.23 |  | Alarm recovery |
| --- | --- | --- | --- |
| Variabl | . | zxAnCardMemUsage | Memory |
| e | 1.3.6.1.4.1.3902.1082.10.1.2.4.1 |  | usage |
|  | .11 |  | percentag |
|  |  |  | e |
|  | . | zxAnCardMemUsageThresh | Memory |

|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1 | old | usage |
| --- | --- | --- | --- |
|  | .12 |  | threshold |

#### CPU Overload Alarm

##### Alarm

| TripOI D | . 1.3.6.1.4.1.3902.1082.10.1.3.2.20 |  | Minor alarm |
| --- | --- | --- | --- |
|  | . | zxAnCardCpuLoad | Current |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1.9 |  | CPU |
|  |  |  | utilizatio |
| Variabl e |  |  | n rate |
|  | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1.1 | zxAnCardCpuLoadThreshol d | CPU utilizatio |
|  | 0 |  | n rate |
|  |  |  | threshol |
|  |  |  | d |

##### Alarm Recovery

| TripOI D | . 1.3.6.1.4.1.3902.1082.10.1.3.2.21 |  | Alarm recovery |
| --- | --- | --- | --- |
|  | . | zxAnCardCpuLoad | Current |
|  | 1.3.6.1.4.1.3902.1082.10.1.2.4.1.9 |  | CPU |
|  |  |  | utilizatio |
| Variabl e |  |  | n rate |
|  | . 1.3.6.1.4.1.3902.1082.10.1.2.4.1.1 | zxAnCardCpuLoadThreshol d | CPU utilizatio |
|  | 0 |  | n rate |
|  |  |  | threshol |
|  |  |  | d |

#### Card Version Update Failure Alarm

##### Alarm

| TripOID | .1.3.6.1.4.1.3902.1015.2.2.2.10 |  | Common alarm |
| --- | --- | --- | --- |
| Variable | .1.3.6.1.4.1.3902.1015.2.1.2.5.1.2 | zxAnVerUpdateStatus | Version |

|  |  |  | update status |
| --- | --- | --- | --- |
|  | .1.3.6.1.4.1.3902.1015.2.1.2.5.1.3 | zxAnVerUpdateReason | Version update reason |

##### Alarm Recovery

| TripOID | .1.3.6.1.4.1.3902.1015.2.2.2.11 |  | Alarm recovery |
| --- | --- | --- | --- |
| Variable | .1.3.6.1.4.1.3902.1015.2.1.2.5.1.2 | zxAnVerUpdateStatus | Version update status |
|  | .1.3.6.1.4.1.3902.1015.2.1.2.5.1.3 | zxAnVerUpdateReason | Version update reason |

#### Card Auto Update Alarm

| TripOI D | .1.3.6.1.4.1.3902.1082.20.30.3.1 |  | Common alarm |
| --- | --- | --- | --- |
| Variabl | . | zxAnSwAutoUpdateFini | Auto update |
| e | 1.3.6.1.4.1.3902.1082.20.30.3.1 | shed | finished |
|  | .1 |  |  |
|  | . | zxAnSwAutoUpdateSw | Number of |
|  | 1.3.6.1.4.1.3902.1082.20.30.3.1 | Differ | auto updated |
|  | .2 |  | files |
|  | . | zxAnSwAutoUpdateSw | Check failsure |
|  | 1.3.6.1.4.1.3902.1082.20.30.3.1 | ChkFailed |  |
|  | .2 |  |  |

#### Card Version Download Alarm

| TripOI D | .1.3.6.1.4.1.3902.1082.20.50.3.1 |  | Common alarm |
| --- | --- | --- | --- |
| Variabl | . | zxAnFileFtpDownloadFinish | Downloa |
| e | 1.3.6.1.4.1.3902.1082.20.50.3.1. | ed | d |
|  | 1 |  | finished |
|  | . | zxAnFileFtpUploadFinished | Downloa |
|  | 1.3.6.1.4.1.3902.1082.20.50.3.1. |  | d |
|  | 1 |  | finished |

#### Card Backup Alarm

| TripOID | .1.3.6.1.4.1.3902.1082.20.45.3 |  | Common alarm |
| --- | --- | --- | --- |
| Variable | .1.3.6.1.4.1.3902.1082.20.45.3.1 | zxAnDataBackupFinished | Backup finished |

#### High Ambient Temperature Alarm

##### Alarm

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.1.1 |  | High ambient temperatu re alarm |
| --- | --- | --- | --- |
|  | . | zxAnEnvTemp | Operating |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1. |  | status |
| Variabl | 5.1.3 |  |  |
| e | . | zxAnEnvTempHighAlmThre |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1. | shold |  |
|  | 5.1.4 |  |  |

##### Alarm Recovery

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.1.2 |  | High ambient temperatu re alarm recovery |
| --- | --- | --- | --- |
|  | . | zxAnEnvTemp | Operating |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1. |  | status |
| Variabl | 5.1.3 |  |  |
| e | . | zxAnEnvTempHighAlmThre |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1. | shold |  |
|  | 5.1.4 |  |  |

#### Sensor Abnormal Alarm

##### Alarm

| TripOID | .1.3.6.1.4.1.3902.1082.10.10.3.1.5 | Common |
| --- | --- | --- |

|  |  |  | alarm |
| --- | --- | --- | --- |
| Variable | None |  | Actual alarm |

#### Ambient Temperature Danger Alarm

##### Alarm

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.1.6 |  | Commo n alarm |
| --- | --- | --- | --- |
|  | . | zxAnEnvTemp | Operati |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1. |  | ng |
| Variabl | 5.1.3 |  | status |
| e | . | zxAnEnvTempCriticalAlmThre |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1. | shold |  |
|  | 5.1.5 |  |  |

##### Alarm Recovery

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.1.7 |  | Alarm recover y |
| --- | --- | --- | --- |
|  | . | zxAnEnvTemp | Operati |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1. |  | ng |
| Variabl | 5.1.3 |  | status |
| e | . | zxAnEnvTempCriticalAlmThre |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1. | shold |  |
|  | 5.1.5 |  |  |

#### Low Ambient Temperature Alarm

##### Alarm

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.1.8 |  | Commo n alarm |
| --- | --- | --- | --- |
| Variabl | . | zxAnEnvTemp | Operatin |
| e | 1.3.6.1.4.1.3902.1082.10.10.2.1.5 |  | g status |
|  | .1.3 |  |  |
|  | . | zxAnEnvTempLowAlmThres |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1.5 | hold |  |

|  | .1.6 |  |  |
| --- | --- | --- | --- |

##### Alarm Recovery

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.1.9 |  | Alarm recovery |
| --- | --- | --- | --- |
|  | . | zxAnEnvTemp | Operatin |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1.5 |  | g status |
| Variabl | .1.3 |  |  |
| e | . | zxAnEnvTempLowAlmThres |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1.5 | hold |  |
|  | .1.6 |  |  |

#### Card Offline Alarm

##### Alarm

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.1.30 |  | Commo n alarm |
| --- | --- | --- | --- |
|  | . | zxAnEnvCardShutdownRea | Operatin |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1.1. | son | g status |
|  | 1 |  |  |
|  | . | zxAnCardTemp |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1.6. |  |  |
| Variabl | 1.2 |  |  |
| e | . | zxAnEnvTemp |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1.5. |  |  |
|  | 1.3 |  |  |
|  | . | zxAnEnvPowerMode |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.3.1. |  |  |
|  | 5 |  |  |

##### Alarm Recovery

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.1.31 |  | Alarm recovery |
| --- | --- | --- | --- |
| Variabl e | . 1.3.6.1.4.1.3902.1082.10.10.2.1.1. 1 | zxAnEnvCardShutdownRea son | Operatin g status |
|  | . | zxAnCardTemp |  |

|  | 1.3.6.1.4.1.3902.1082.10.10.2.1.6. |  |  |
| --- | --- | --- | --- |
|  | 1.2 |  |  |
|  | . | zxAnEnvTemp |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.1.5. |  |  |
|  | 1.3 |  |  |
|  | . | zxAnEnvPowerMode |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.3.1. |  |  |
|  | 5 |  |  |

#### Environmental Monitoring Interface Down Alarm

##### Alarm

| TripOID | .1.3.6.1.4.1.3902.1082.10.10.3.2.1 |  | Common alarm |
| --- | --- | --- | --- |
| Variable | None |  | Operating status |

##### Alarm Recovery

##### Environmental Monitoring Interface Down Alarm Recovery

| TripOID | .1.3.6.1.4.1.3902.1082.10.10.3.2.2 |  | Alarm recovery |
| --- | --- | --- | --- |
| Variable | None |  |  |

#### Power Abnormal Alarm

##### Alarm

| TripOID | .1.3.6.1.4.1.3902.1082.10.10.3.3.1 |  | Common alarm |
| --- | --- | --- | --- |
| Variable | None |  | Operating status |

##### Alarm Recovery

| TripOID | .1.3.6.1.4.1.3902.1082.10.10.3.3.2 | Alarm recovery |
| --- | --- | --- |

| Variable | None |  |  |
| --- | --- | --- | --- |

#### Power Supply Overvoltage Alarm

##### Alarm

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.3.3 |  | Commo n alarm |
| --- | --- | --- | --- |
|  | . | zxAnPowerSupplyInVoltage | Operati |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.3.1 |  | ng |
| Variabl | 1.1.2 |  | status |
| e | . | zxAnPowerInVoltageUpperT |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.3.1 | hresh |  |
|  | 1.1.4 |  |  |

##### Alarm Recovery

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.3.4 |  | Alarm recover y |
| --- | --- | --- | --- |
|  | . | zxAnPowerSupplyInVoltage | Operati |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.3.1 |  | ng |
| Variabl | 1.1.2 |  | status |
| e | . | zxAnPowerInVoltageUpperT |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.3.1 | hresh |  |
|  | 1.1.4 |  |  |

#### Power Supply Undervoltage Alarm

##### Alarm

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.3.5 |  | Commo n alarm |
| --- | --- | --- | --- |
|  | . | zxAnPowerSupplyInVoltage | Operati |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.3.1 |  | ng |
| Variabl | 1.1.2 |  | status |
| e | . | zxAnPowerInVoltageLowerT |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.3.1 | hresh |  |
|  | 1.1.5 |  |  |

##### Alarm Recovery

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.3.6 |  | Alarm recover y |
| --- | --- | --- | --- |
|  | . | zxAnPowerSupplyInVoltage | Operati |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.3.1 |  | ng |
| Variabl | 1.1.2 |  | status |
| e | . | zxAnPowerInVoltageLowerT |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.3.1 | hresh |  |
|  | 1.1.5 |  |  |

#### Power Down Alarm

##### Alarm

| TripOID | .1.3.6.1.4.1.3902.1082.10.10.3.3.7 |  | Common alarm |
| --- | --- | --- | --- |
| Variable | None |  | Operating status |

##### Alarm Recovery

| TripOID | .1.3.6.1.4.1.3902.1082.10.10.3.3.8 |  | Alarm recovery |
| --- | --- | --- | --- |
| Variable | None |  |  |

#### Power Supply Emergency Alarm

##### Alarm

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.3.9 |  | Commo n alarm |
| --- | --- | --- | --- |
|  | . | zxAnEnvEmergencyPowerSave | Operati |
|  | 1.3.6.1.4.1.3902.1082.10.10.2. | Enable | ng |
| Variabl | 3.1.1 |  | status |
| e | . | zxAnEnvPowerMode |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2. |  |  |
|  | 3.1.5 |  |  |

##### Alarm Recovery

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.3.10 |  | Alarm recover y |
| --- | --- | --- | --- |
|  | . | zxAnEnvEmergencyPowerSave | Operati |
|  | 1.3.6.1.4.1.3902.1082.10.10.2. | Enable | ng |
| Variabl | 3.1.1 |  | status |
| e | . | zxAnEnvPowerMode |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2. |  |  |
|  | 3.1.5 |  |  |

#### Fan Offline Alarm

##### Alarm

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.4.1 |  | Common alarm |
| --- | --- | --- | --- |
| Variabl e | . 1.3.6.1.4.1.3902.1082.10.10.2.4.11.1. 6 | zxAnEnvFanOnlineStatu s | Operatin g status |

##### Alarm Recovery

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.4.2 |  | Alarm recovery |
| --- | --- | --- | --- |
| Variabl e | . 1.3.6.1.4.1.3902.1082.10.10.2.4.11.1. 6 | zxAnEnvFanOnlineStatu s | Operatin g status |

#### Fan Card Offline Alarm

##### Alarm

| TripOID | .1.3.6.1.4.1.3902.1082.10.10.3.4.3 |  | Common alarm |
| --- | --- | --- | --- |
| Variable | None |  | Operating status |

##### Alarm Recovery

| TripOID | .1.3.6.1.4.1.3902.1082.10.10.3.4.4 |  | Alarm recovery |
| --- | --- | --- | --- |
| Variable | None |  |  |

#### Fan Abnormal Alarm

##### Alarm

| TripOID | .1.3.6.1.4.1.3902.1082.10.10.3.4.5 |  | Common alarm |
| --- | --- | --- | --- |
| Variabl e | . 1.3.6.1.4.1.3902.1082.10.10.2.4.11.1. 5 | zxAnEnvFanOperStatu s | Operatin g status |

##### Alarm Recovery

| TripOID | .1.3.6.1.4.1.3902.1082.10.10.3.4.6 |  | Alarm recover y |
| --- | --- | --- | --- |
| Variabl | . | zxAnEnvFanOperStatu |  |
| e | 1.3.6.1.4.1.3902.1082.10.10.2.4.11.1.5 | s |  |

#### Environmetal Monitoring Device Abnormal Alarm

##### Alarm

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.5.1 |  | Commo n alarm |
| --- | --- | --- | --- |
|  | . | zxAnEnvDevMonSwitchCurrSt | Operati |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.5. | atus | ng |
| Variabl | 3.1.5 |  | status |
| e | . | zxAnEnvDevMonSwitchTrapE |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.5. | nable |  |
|  | 3.1.3 |  |  |

##### Alarm Recovery

| TripOI D | .1.3.6.1.4.1.3902.1082.10.10.3.5.2 | Alarm recover |
| --- | --- | --- |

|  |  |  | y |
| --- | --- | --- | --- |
|  | . | zxAnEnvDevMonSwitchCurrSt | Operati |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.5. | atus | ng |
| Variabl | 3.1.5 |  | status |
| e | . | zxAnEnvDevMonSwitchTrapE |  |
|  | 1.3.6.1.4.1.3902.1082.10.10.2.5. | nable |  |
|  | 3.1.3 |  |  |

#### Version Update Alarm

##### Alarm

| TripOID | .1.3.6.1.4.1.3902.1082.20.40.3 |  |  |
| --- | --- | --- | --- |
| Variable | .1.3.6.1.4.1.3902.1082.20.40.3.1 | zxAnRuSwUpdated |  |

## Common Alarm and Notification

#### Uplink Interface Linkdown Alarm

[MIB file]:

IF-MIB.mib

##### Alarm

| TripOID | .1.3.6.1.4.1.3902.1082.30.20.3.2 |  | Severe alarm |
| --- | --- | --- | --- |
| Variable | .1.3.6.1.2.1.2.2.1.7 | ifAdminStatus | Management status |
|  | .1.3.6.1.2.1.2.2.1.8 | ifOperStatus | Operating status |
|  | .1.3.6.1.2.1.2.2.1.3 | ifMIB value | Interface type |

##### Alarm Recovery

| TripOID | .1.3.6.1.4.1.3902.1082.30.20.3.2 |  | Alarm Recovery |
| --- | --- | --- | --- |
| Variable | .1.3.6.1.2.1.2.2.1.7 | ifAdminStatus | Management status |
|  | .1.3.6.1.2.1.2.2.1.8 | ifOperStatus | Operating status |
|  | .1.3.6.1.2.1.2.2.1.3 | ifMIB value | Interface type |

#### DoS Attack Alarm

[MIB file]:

ZTE-AN-SECURITY-CPUDEFEND-MIB.mib

##### Alarm

| TripOI D | .1.3.6.1.4.1.3902.1082.70.25.3.1.1 | General Alarm |
| --- | --- | --- |

|  | .1.3.6.1.2.1.2.2.1.1 | ifIndex | IfIndex |
| --- | --- | --- | --- |
|  | . | zxAnSubIfIndex | zxAnSubIfInd |
|  | 1.3.6.1.4.1.3902.1082.30.5.5.1. |  | ex |
|  | 1.2 |  |  |
| Variabl | . | zxAnSecDosAttackSrc | Source MAC |
| e | 1.3.6.1.4.1.3902.1082.70.25.2.1 | Mac | address |
|  | .2.1.1 |  |  |
|  | . | zxAnSecDosAttackVid | Port VLAN |
|  | 1.3.6.1.4.1.3902.1082.70.25.2.1 |  |  |
|  | .2.1.2 |  |  |

##### Alarm Recovery

| TripOI D | .1.3.6.1.4.1.3902.1082.70.25.3.1.2 |  | Alarm Recovery |
| --- | --- | --- | --- |
|  | .1.3.6.1.2.1.2.2.1.1 | ifIndex | IfIndex |
|  | . | zxAnSubIfIndex | zxAnSubIfInd |
|  | 1.3.6.1.4.1.3902.1082.30.5.5.1. |  | ex |
|  | 1.2 |  |  |
| Variabl | . | zxAnSecDosAttackSrc | Source MAC |
| e | 1.3.6.1.4.1.3902.1082.70.25.2.1 | Mac | address |
|  | .2.1.1 |  |  |
|  | . | zxAnSecDosAttackVid | Port VLAN |
|  | 1.3.6.1.4.1.3902.1082.70.25.2.1 |  |  |
|  | .2.1.2 |  |  |

#### User Interface LinkUp Notification

| TripOID | .1.3.6.1.4.1.3902.1082.30.20.3.1 |  | Notification |
| --- | --- | --- | --- |
| Variable | .1.3.6.1.2.1.2.2.1.7 | ifAdminStatus | Management status |
|  | .1.3.6.1.2.1.2.2.1.8 | ifOperStatus | Operating status |
|  | .1.3.6.1.2.1.2.2.1.3 | ifType | Interface type |

[MIB file]: IF-MIB.mib

#### User Interface LinkDown Notification

[MIB file]:

IF-MIB.mib

| TripOID | .1.3.6.1.4.1.3902.1082.30.20.3.2 |  | Notification |
| --- | --- | --- | --- |
| Variable | .1.3.6.1.2.1.2.2.1.7 | Management status | Management status |
|  | .1.3.6.1.2.1.2.2.1.8 | Operating status | Operating status |
|  | .1.3.6.1.2.1.2.2.1.3 | Interface type | Interface type |

#### Serial Interface or Management Interface Login Notification

[MIB file]:

ZTE-AN-CLI-MIB.mib

| TripOI D | .1.3.6.1.4.1.3902.1082.20.10.3.1 |  | Notification |
| --- | --- | --- | --- |
|  | . | zxAnCliCrftTerminalLastLog | Login type |
|  | 1.3.6.1.4.1.3902.1082.2 | inMIB value |  |
|  | 0.10.2.1.3 |  |  |
| Variabl e | . 1.3.6.1.4.1.3902.1082.2 0.10.2.3.3.1.3 | zxAnCliActiveUserName | Login username |
|  | . | zxAnCliActiveUserLocation | Login location |
|  | 1.3.6.1.4.1.3902.1082.2 |  |  |
|  | 0.10.2.3.3.1.8 |  |  |

#### Serial Interface or Management Interface Logout Notification

[MIB file]:

ZTE-AN-CLI-MIB.mib

| TripOI D | .1.3.6.1.4.1.3902.1082.20.10.3.2 |  | Notification |
| --- | --- | --- | --- |
| Variabl e | . 1.3.6.1.4.1.3902.1082.20 .10.2.1.3 | Login type | Logout type |

|  | . 1.3.6.1.4.1.3902.1082.20 .10.2.3.3.1.3 | Login username | Logout username |
| --- | --- | --- | --- |
|  | . | Login location | Logout |
|  | 1.3.6.1.4.1.3902.1082.20 |  | location |
|  | .10.2.3.3.1.8 |  |  |

## PON Alarm and Notification

[MIB file]:

Please refer to the definitions in ZTE-AN-PON-BASE-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonRxPowerHi | . |  |
| ghAlm | 1.3.6.1.4.1.3902.1082.5 | "Alarm for received power." |
|  | 00.1.3.1.1 |  |
| zxAnPonRxPowerHi ghClr | . 1.3.6.1.4.1.3902.1082.5 00.1.3.1.2 | "Restore of alarm for received power." |
| zxAnPonRxPowerL | . |  |
| owAlm | 1.3.6.1.4.1.3902.1082.5 | "Alarm for received power." |
|  | 00.1.3.1.3 |  |
| zxAnPonRxPowerL owClr | . 1.3.6.1.4.1.3902.1082.5 00.1.3.1.4 | "Restore of alarm for received power." |

#### PON Public Alarm and Notification

[MIB file]:

Please refer to the definitions in ZTE-AN-PON-BASE-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnPonRxPowerHi | . |  |
| ghAlm | 1.3.6.1.4.1.3902.1082.5 | "Alarm for received power." |
|  | 00.1.3.1.1 |  |
| zxAnPonRxPowerHi ghClr | . 1.3.6.1.4.1.3902.1082.5 00.1.3.1.2 | "Restore of alarm for received power." |
| zxAnPonRxPowerL | . |  |
| owAlm | 1.3.6.1.4.1.3902.1082.5 | "Alarm for received power." |
|  | 00.1.3.1.3 |  |
| zxAnPonRxPowerL owClr | . 1.3.6.1.4.1.3902.1082.5 00.1.3.1.4 | "Restore of alarm for received power." |

#### GPON Alarm and Notification

[MIB file]:

Please refer to the definitions in ZTE-AN-GPON-SERVICE-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponSrvOltLos | . |  |
| iAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.1 |  |
| zxAnGponSrvOltLos | . |  |
| iClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.2 |  |
| zxAnGponSrvOltLofi | . |  |
| Alm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.3 |  |
| zxAnGponSrvOltLofi | . |  |
| Clr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.4 |  |
| zxAnGponSrvOltDo | . |  |
| wiAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.5 |  |
| zxAnGponSrvOltDo | . |  |
| wiClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.6 |  |
| zxAnGponSrvOltSfi | . |  |
| Alm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.7 |  |
| zxAnGponSrvOltSfi | . |  |
| Clr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.8 |  |
| zxAnGponSrvOltSdi | . |  |
| Alm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.9 |  |
| zxAnGponSrvOltSdi | . |  |
| Clr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.10 |  |
| zxAnGponSrvOltLcd | . |  |
| giAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.13 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponSrvOltLcd | . |  |
| giClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.14 |  |
| zxAnGponSrvOltRdi | . |  |
| iAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.15 |  |
| zxAnGponSrvOltRdi | . |  |
| iClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.16 |  |
| zxAnGponSrvOltSuf | . |  |
| iAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.17 |  |
| zxAnGponSrvOltSuf | . |  |
| iClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.18 |  |
| zxAnGponSrvOltDfi | . |  |
| Alm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.19 |  |
| zxAnGponSrvOltDfi | . |  |
| Clr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.20 |  |
| zxAnGponSrvOltLo | . |  |
| aiAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.21 |  |
| zxAnGponSrvOltLo | . |  |
| aiClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.22 |  |
| zxAnGponSrvOltDgi | . |  |
| Alm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.23 |  |
| zxAnGponSrvOltDgi | . |  |
| Clr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.24 |  |
| zxAnGponSrvOltLo | . |  |
| amiAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.25 |  |
| zxAnGponSrvOltLo | . |  |
| amiClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.26 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponSrvOltMe | . |  |
| miAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.27 |  |
| zxAnGponSrvOltMe | . |  |
| miClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.28 |  |
| zxAnGponSrvOltMis | . |  |
| iAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.29 |  |
| zxAnGponSrvOltMis | . |  |
| iClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.30 |  |
| zxAnGponSrvOltPe | . |  |
| eiAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.31 |  |
| zxAnGponSrvOltPe | . |  |
| eiClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.32 |  |
| zxAnGponSrvOltLos | . |  |
| Alm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.61 |  |
| zxAnGponSrvOltLos | . |  |
| Clr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.62 |  |
| zxAnGponSrvOltLof | . |  |
| Alm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.63 |  |
| zxAnGponSrvOltLof | . |  |
| Clr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.64 |  |
| zxAnGponSrvOnuR | . |  |
| egister | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.65 |  |
| zxAnGponSrvOnuU | . |  |
| nregister | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.66 |  |
| zxAnGponSrvOnuA | . |  |
| uthPass | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.67 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponSrvOnuA | . |  |
| uthFailed | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.68 |  |
| zxAnGponSrvLinkO | . |  |
| nuDyingGaspAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.69 |  |
| zxAnGponSrvLinkO | . |  |
| nuDyingGaspClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.70 |  |
| zxAnGponSrvOltFa | . |  |
| kedSN | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.73 |  |
| zxAnGponSrvOnuC | . |  |
| onstantOpticalAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.74 |  |
| zxAnGponSrvOnuC | . |  |
| onstantOpticalClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.75 |  |
| zxAnGponSrvOnuR | . |  |
| xFiberFlexAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.76 |  |
| zxAnGponSrvOnuR | . |  |
| xFiberFlexClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.77 |  |
| zxAnGponSrvOnuU | . |  |
| nauthOnuOnline | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.80 |  |
| zxAnGponSrvOltTfA | . |  |
| lm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.81 |  |
| zxAnGponSrvOltTfC | . |  |
| lr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.82 |  |
| zxAnGponSrvOltFa | . |  |
| kedPwd | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.83 |  |
| zxAnGponSrvOnuA | . |  |
| utoAuthSuccess | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.84 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponSrvOltLo | . |  |
| ei | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.10.3.1.85 |  |

#### GPON ONU Remote Alarm and Notification

[MIB file]:

Please refer to the definitions in ZTE-AN-GPON-REMOTE-ONU-MIB.mib.

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponRmEthUn | . |  |
| iLanLosAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.1 |  |
| zxAnGponRmEthUn | . |  |
| iLanLosClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.2 |  |
| zxAnGponRmEthUn | . |  |
| iLoopIndicatAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.3 |  |
| zxAnGponRmEthUn | . |  |
| iLoopIndicatClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.4 |  |
| zxAnGponRmOnuE | . |  |
| quipAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.73 |  |
| zxAnGponRmOnuE | . |  |
| quipClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.74 |  |
| zxAnGponRmOnuP | . |  |
| owerAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.75 |  |
| zxAnGponRmOnuP | . |  |
| owerClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.76 |  |
| zxAnGponRmOnuB | . |  |
| atteryMissingAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.77 |  |
| zxAnGponRmOnuB | . |  |
| atteryMissingClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.78 |  |
| zxAnGponRmOnuB | . |  |
| atteryFailAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.79 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponRmOnuB | . |  |
| atteryFailClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.80 |  |
| zxAnGponRmOnuB | . |  |
| atteryLowAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.81 |  |
| zxAnGponRmOnuB | . |  |
| atteryLowClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.82 |  |
| zxAnGponRmOnuP | . |  |
| hyIntrusionAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.83 |  |
| zxAnGponRmOnuP | . |  |
| hyIntrusionClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.84 |  |
| zxAnGponRmOnuS | . |  |
| elfTestFailAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.85 |  |
| zxAnGponRmOnuS | . |  |
| elfTestFailClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.86 |  |
| zxAnGponRmOnuD | . |  |
| yingGaspAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.87 |  |
| zxAnGponRmOnuD | . |  |
| yingGaspClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.88 |  |
| zxAnGponRmOnuT | . |  |
| empYellowAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.89 |  |
| zxAnGponRmOnuT | . |  |
| empYellowClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.90 |  |
| zxAnGponRmOnuT | . |  |
| empRedAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.91 |  |
| zxAnGponRmOnuT | . |  |
| empRedClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.92 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponRmOnuV | . |  |
| oltageYellowAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.93 |  |
| zxAnGponRmOnuV | . |  |
| oltageYellowClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.94 |  |
| zxAnGponRmOnuV | . |  |
| oltageRedAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.95 |  |
| zxAnGponRmOnuV | . |  |
| oltageRedClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.96 |  |
| zxAnGponRmAniLo | . |  |
| wRxPowerAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.123 |  |
| zxAnGponRmAniLo | . |  |
| wRxPowerClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.124 |  |
| zxAnGponRmAniHi | . |  |
| ghRxPowerAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.125 |  |
| zxAnGponRmAniHi | . |  |
| ghRxPowerClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.126 |  |
| zxAnGponRmAniSf | . |  |
| Alm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.127 |  |
| zxAnGponRmAniSf | . |  |
| Clr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.128 |  |
| zxAnGponRmAniSd | . |  |
| Alm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.129 |  |
| zxAnGponRmAniSd | . |  |
| Clr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.130 |  |
| zxAnGponRmAniLo | . |  |
| wTransPowerAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.131 |  |

| MIB Variable | OID | Description |
| --- | --- | --- |
| zxAnGponRmAniLo | . |  |
| wTransPowerClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.132 |  |
| zxAnGponRmAniHi | . |  |
| ghTransPowerAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.133 |  |
| zxAnGponRmAniHi | . |  |
| ghTransPowerClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.134 |  |
| zxAnGponRmAniLa | . |  |
| serBiasCurrentAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.135 |  |
| zxAnGponRmAniLa | . |  |
| serBiasCurrentClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.136 |  |
| zxAnGponRmOnuHi | . |  |
| ghCpuUsageAlm | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.169 |  |
| zxAnGponRmOnuHi | . |  |
| ghCpuUsageClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.170 |  |
| zxAnGponRmOnuHi | . |  |
| ghMemoryUsageAl | 1.3.6.1.4.1.3902.1082.5 |  |
| m | 00.20.3.1.171 |  |
| zxAnGponRmOnuHi | . |  |
| ghMemoryUsageClr | 1.3.6.1.4.1.3902.1082.5 |  |
|  | 00.20.3.1.172 |  |

# 10